| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0260 |
| --- | --- |

| Title: | Cross-check of JVET-S0234: Mismatch between text specification and reference software on chroma residual scaling when ACT is enabled |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Zhao | Email: | jie.zhao@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This is cross-check report for JVET-S0234: “Mismatch between text specification and reference software on chroma residual scaling when ACT is enabled”.

Software is examined and confirmed that it matches with what’s proposed in JVET-S0234. Partial results obtained so far also match with those provided in JVET-S0234.

.

# Introduction

JVET-S0234 identifies a mismatch between VVC draft 9 and test model software VTM-9.0 on chroma residual scaling when the ACT is applied. In VVC draft 9, when the ACT is applied to one CU, it residual samples are firstly converted from YCgCo space to RGB space before the chroma residual scaling is applied. However, in VTM-9.0, the chroma residual samples are directly scaled in the YCgCo space which are then converted back to the RGB space. JVET-S0234 provided software to fix this mismatch.

Software is downloaded from JEVT-S0234. It was examined, and confirmed that when ACT is enabled, chroma residual scaling is performed after the residual being converted back to RGB domain as proposed in JVET-S0234.

The spec text change in 8.7.5.3 regarding the following condition tu_cb_coded_flag, tu_cr_coded_flag, tuCbfChroma  is also reflected in software. When ACT is enabled, Chroma residual scaling does not depend on the tu_cb_coded_flag and tu_cr_coded_flag. Otherwise, it does.

# Results

The results are evaluated on VTM9.0 [2]. Reference is VTM9 following non-4:2:0 common test conditions [3] and tested on RGB sequences.

## RGB Screen Content

## In VTM9, for RGB screen content test sequences, 13 out of the 14 sequences, LMCS has already been disabled by encoder decision. So change in JVET-S0234 only affected one sequence ArenaOfValor. These results match with the data sheet provided in JVET-S0234.

Table 2.1 Performance of JVET-S0234 for RGB screen content

|  |  | AI |  |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FlyingGraphics | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Desktop | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Console | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| ChineseEditing | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Web_Browsing | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Map | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Programming | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SlideShow | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| ArenaOfValor | -0.9% | 0.3% | -0.5% | -0.5% | -0.2% | -0.8% |  |  |  |
| Robot | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| GlassHalf | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Basketball_Screen | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| MissionControlClip2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| MissionControlClip3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

## RGB Natural Content

Table 2.2 shows the result of JEVT-S0234 on RGB natural sequences. These results match with the data sheet provided in JVET-S0234.

Table 2.2 Performance of S0234 for RGB natural content

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.89% | 0.01% | -0.90% | 101% | 100% |
| Kimono | -0.25% | 0.00% | -0.62% | 101% | 102% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 100% | 102% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 103% | 104% |
| VenueVu | 0.00% | 0.00% | 0.00% | 100% | 100% |
| DucksAndLegs | -0.48% | 0.13% | -0.68% | 100% | 104% |
| OldTownCross | -1.45% | 0.22% | -0.40% | 102% | 100% |
| ParkScene | -0.20% | 0.04% | -0.14% | 101% | 100% |
| Overall | -0.41% | 0.05% | -0.34% | 100% | 102% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -2.08% | -0.23% | -1.86% | 101% | 100% |
| Kimono | -0.83% | -0.19% | -0.87% | 101% | 104% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 100% | 105% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 102% | 104% |
| VenueVu | 0.00% | 0.00% | 0.00% | 101% | 103% |
| DucksAndLegs | -1.35% | -0.12% | -0.77% | 99% | 101% |
| OldTownCross | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| ParkScene | 0.01% | 0.06% | -0.06% | 100% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| Kimono | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| EBULupoCandlelight | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 102% | 101% |
| VenueVu | 0.00% | 0.00% | 0.00% | 101% | 98% |
| DucksAndLegs | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| OldTownCross | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| ParkScene | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 9)”, JVET-R2001, April 2020, Teleconference

- VTM-9.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-9.0.

- Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-R2013, April 2020, Teleconference