| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0261-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-S0232-v1 Removal of redundant clipping operations on inter prediction samples in forward luma mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jie Chen | Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of the first version of JVET-S0232 which proposes to remove clipping operation on inter prediction samples in forward luma mapping as proposed in JVET-S0232-v1[1]. It is reported that the coding results are identical with those reported by the proponent.

- Introduction

When LMCS is applied, for inter and CIIP modes, forward luma mapping is applied to convert the inter prediction samples of luma component before they are added with the corresponding residual samples. In VTM-9.0, a clipping operation is applied to the mapped inter predictors before added with corresponding residues. It is asserted in the JVET-S0232-v1 that the above clipping operations are redundant. Thus, the JVET-S0232-v1 proposes to remove the redundant clipping operations. (And later in the second version of JVET-S0232, it is reported that there could be overflow in 16-bit and the clipping operation are not redundant.)

This contribution crosschecks the removal of the mentioned clipping operations proposed in JVET-S0232-v1. Simulation results prove the redundancy of the removed clipping operations in internal 10-bit coding by providing bit-exact BD-rate performance in CTC.

Test Result

The test results are shown in Table 1 to Table 3. The running time is not reliable due to the cluster issue.

Table 1. Simulation results of JVET-S0232-v1 in AI

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | 0.00% | 0.00% | 0.00% | 105% | 106% | 0.00% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 103% | 101% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 101% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 104% | 106% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 109% | 109% | 0.00% |

Table 2. Simulation results of JVET-S0232-v1 in RA

|  |  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 97% | 0.00% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 98% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 99% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 98% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% |

Table 3. Simulation results of JVET-S0232-v1 in LDB

|  |  |  | Low delay B Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |  |
|  | Y | U | V | EncT | DecT | YUV |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 95% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 98% | 96% | 0.00% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! |
| Class D | 0.00% | 0.00% | 0.00% | 96% | 95% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 91% | 87% | 0.00% |

References

[1] X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Removal of redundant clipping operations on inter prediction samples in forward luma mapping, JVET-S0232-v1, Teleconference, June, 2020.