| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0267-v7 |
| --- | --- |

| Title: | JVET AHG on extending VVC with learning based coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shan Liu, Xiang Li, Wei Wang (Tencent) Elena Alshina (Huawei) Kei Kawamura, Kyohei Unno, Yoshitaka Kidani (KDDI) Ping Wu (ZTE) Andrew Segall (Sharp) Mathias Wien (RWTH Aachen) Jonathan Pfaff, Heiko Schwarz, Benjamin Bross (Fraunhofer HHI) Xianglin Wang, Xiaoyu Xiu, Yi-Wen Chen (Kwai) Zhenzhong Chen (Wuhan University) Jill Boyce (Intel) Yu-Wen Huang (MediaTek) Feng Wu, Dong Liu (USTC) M. Karczewicz, J. Chen (Qualcomm) Dan Grois (Comcast Cable) Fabrice Le Léannec, Franck Galpin, Edouard Francois (InterDigital) | Tel: Email: | {shanl, xlxiangli, rickweiwang} @tencent.com elena.alshina@huawei.com ki-kawamura@kddi.com pwuzte@gmail.com asegall@sharplabs.com wien@lfb.rwth-aachen.de firstname.lastname@hhi.fraunhofer.de {xianglinwang, xiaoyuxiu, yiwenchen }@kwai.com zzchen@whu.edu.cn jill.boyce@intel.com yuwen.huang@mediatek.com {fengwu, dongeliu}@ustc.edu.cn {martak, cjianle}@qti.qualcomm.com grois@ieee.org Edouard.Francois@InterDigital.com |
| Source: | Tencent, Huawei, KDDI, ZTE, Sharp, RWTH Aachen, Fraunhofer HHI, Kwai, Wuhan University, Intel, MediaTek, USTC, Qualcomm, Comcast Cable, InterDigital |  |  |

_____________________________

# Abstract

With VVC v1 being completed, it is proposed to further explore compression technologies beyond VVC v1 in JVET. Neural Network (NN) based coding tool is reported as an interesting topic to explore. In fact, Neural Network (NN) based coding tools were studied in JVET AHG9 during VVC v1 development. NN based coding tools in a broad range of technology options were proposed, and coding performance improvement was demonstrated during the two-year study. This study was paused in 2019 to prioritize the completion of VVC v1. For now, it is proposed to re-establish this AHG in order to build on its previous work in JVET, with the goal of developing a potential VVC extension supporting learning based video coding tools.

# Overview

With VVC v1 being completed, it is desirable to further explore compression technologies and features beyond VVC v1. NN based video coding tools have demonstrated encouraging coding performance gains in recent years. Among the total 46 responses to the joint Call for Proposals (CfP) on video compression with capability beyond HEVC, six included learning-based tools with another four NN based coding tool proposals. These responses and proposals reported compression benefits against or on top of conventional methods [1]. A JVET ad-hoc group (AHG9) was set up for investigating the compression performance and complexity of neural network-based coding tools in early 2018 [2], with initial mandates as shown below.

- Investigate the benefit of deep learning technology in video compression.

- Investigate the complexity impact of using deep learning in video compression.

- Investigate deep learning based coding tools such as CNN loop filter.

- Investigate the relationship between CNN filter and ALF, and other loop filters.

- Investigate the performance of CNN filter used as an in-loop filter or a post-processing filter.

- Investigate the impact of QP on CNN filter.

AHGs with substantially the same charter were set up for many meeting cycles. At least 40 technical contributions were received by the AHG, and two Core Experiments were established to conduct more thorough investigation, simulation, tool comparison and evaluation. Two output documents (JVET-L1006 and JVET-M1006) were issued and used to provide thorough evaluation results as well as an evaluation matrix and methodology. Additionally, a dedicated branch for software sharing, cross check and reference was established. It is asserted that this should be considered a good “dry run” for organized testing of this type of video coding tools for VVC.

The series of AHGs on NN tools for VVC ended after the P meeting, as it was necessary to prioritize work on other aspects of the VVC design in order to finalize the VVC v1 standard on time. With the VVC v1 standard now completes, we propose that the AHG be re-established at this meeting to build on its previous work with the goal of developing a potential VVC extension.

# Proposal

It is proposed to create an AHG at this meeting targeting work in JVET towards a VVC extension with learning based coding tools. The proposed mandates are:

- Develop learning based coding tools for an extension of VVC in JVET.

- Investigate the complexity impact of learning based coding tools and develop complexity measurement.

- Investigate the benefit of universal versus selectable networks, both in terms of compression benefit and complexity.

- Investigate SEI messages for NN based post processing.

- Investigate the impact of training materials on the performance of neural network coding tools.

- Investigate the impact of the training process on performance and complexity.

- Identify video data sets available publicly or for standardization purposes and try to build common training video data set.

# Summary of previous work

## Suggestions on complexity analysis and reporting template

The complexity of learning based coding tools is an important issue to be seriously considered for a VVC extension. Previously, AHG9 reported complexity analysis of NN based tools and produced evaluation measurement and reporting template for this purpose. Please refer document JVET-M1006 [3] for detailed report.

## Suggestions on core experiments

More than 100 experts participated the AHG activities and proposed 40 contributions through the study. Detailed contribution list may be found in appendix. Core Experiments were conducted to compare the coding performance and complexity among several NN based filtering tools, using the same software base, test conditions, performance and complexity evaluation matrix. Experts from more than 25 institutes participated the Core Experiments.

Core Experiments and the summary of CE results are provided in [4][5]. We can see that NN based filters bring over 4.6% gain (AI, Y) over VTM-4.0 and up to nearly 4% gain (AI, Y) over VTM-5.0. Higher coding gains were observed for chroma components in general.

It is suggested to keep such CE practices including crosscheck activities as stableness of learning based coding tools and duplicable results are important when targeting a VVC extension.

It is also suggested to study appropriate quality metric to evaluate the coding performance of learning based coding tools.

## Suggestions on software Repository and CTC

Based on AHG discussion and for better comparison and alignment with related CE work, it is suggested to follow VVC v1 CTC JVET-N1010 to evaluate the coding performance of NN based tools. For optional tests, short tests such as one intra period may be used.

To ease study and crosscheck, it is suggested to follow earlier practice of CE on NN based tools to commit code to VTM git repository https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM under AHG or CE branch. As an example, following figure shows the branches of previous CE10 on NN based coding tools.

# References

- “JVET AHG report 9: Neural Networks in Video Coding (AHG9)”, S. Liu, L. Wang, P. Wu, H. Yang, ISO/IEC JTC1/SC29/WG11 JVET-J0009.

- “Deep Learning-Based Technology in Responses to the Joint Call for Proposals on Video Compression with Capability beyond HEVC”, D. Liu, Z. Chen, S. Liu, F. Wu, IEEE Transactions on Circuits and Systems for Video Technology, 2019.

- “Methodology and reporting template for neural network coding tool testing”, Y. Li, S. Liu, K. Kawamura, ISO/IEC JTC1/SC29/WG11 JVET-M1006.

- “CE13: Summary Report on Neural Network based Filter for Video Coding”, Y. Li, S. Liu, K. Kawamura, ISO/IEC JTC1/SC29/WG11 JVET-N0033.

- “CE10: Summary Report on Neural Network based Filter for Video Coding”, Y. Li, S. Liu, K. Kawamura, ISO/IEC JTC1/SC29/WG11 JVET-O0030.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.

Huawei does not have any current or pending patent rights relating to the technology described in this contribution.

KDDI does not have any current or pending patent rights relating to the technology described in this contribution.

ZTE does not have any current or pending patent rights relating to the technology described in this contribution.

Sharp Labs of America does not have any current or pending patent rights relating to the technology described in this contribution.

RWTH Aachen does not have any current or pending patent rights relating to the technology described in this contribution.

Fraunhofer HHI does not have any current or pending patent rights relating to the technology described in this contribution.

Kwai does not have any current or pending patent rights relating to the technology described in this contribution.

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.

Intel does not have any current or pending patent rights relating to the technology described in this contribution.

MediaTek does not have any current or pending patent rights relating to the technology described in this contribution.

USTC does not have any current or pending patent rights relating to the technology described in this contribution.

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.

Comcast Cable does not have any current or pending patent rights relating to the technology described in this contribution.

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.

# Appendix

- JVET-I0022, “Convolutional Neural Network Filter (CNNF) for intra frame”, L. Zhou, X. Song, J. Yao, L. Wang, F. Chen (Hikvision), Gwangju, KR, 20–26 Jan. 2018

- JVET-J0034, “AHG9: CNN-based driving of block partitioning for intra slices encoding”, F. Galpin, F. Racapé, P. Bordes, F. Le Léannec, E. François (Technicolor), San Diego, US, 10–20 Apr. 2018

- JVET-J0043, “AHG9: Convolutional Neural Network Filter for inter frame”, J. Yao, X. Song, S. Fang, L. Wang (Hikvision), San Diego, US, 10–20 Apr. 2018

- JVET-J0076, “AHG9: Results of CNN filter in JVET-I0022 as in-loop filter and post-processing filter”, L. Zhao, X. Li, S. Liu (Tencent), San Diego, US, 10–20 Apr. 2018

- JVET-J0037, “Intra Prediction Modes based on Neural Networks”, J. Pfaff, P Helle, D. Maniry, S. Kaltenstadler, B. Stallenberger, P. Merkle, M. Siekmann, H. Schwarz, D. Marpe, T. Wiegand (HHI), San Diego, US, 10–20 Apr. 2018

- JVET-J0014, “Description of SDR, HDR and 360° video coding technology proposal by Fraunhofer HHI”, M. Albrecht, C. Bartnik, S. Bosse, J. Brandenburg, B. Bross, J. Erfurt, V. George, P. Haase, P. Helle, C. Helmrich, A. Henkel, T. Hinz, S. de Luxan Hernandez, S. Kaltenstadler, P. Keydel, H. Kirchhoffer, C. Lehmann, W.-Q. Lim, J. Ma, D. Maniry, D. Marpe, P. Merkle, T. Nguyen, J. Pfaff, J. Rasch, R. Rischke, C. Rudat, M. Schaefer, T. Schierl, H. Schwarz, M. Siekmann, R. Skupin, B. Stallenberger, J. Stegemann, K. Suehring, G. Tech, G. Venugopal, S. Walter, A. Wieckowski, T. Wiegand, M. Winken (Fraunhofer HHI), San Diego, US, 10–20 Apr. 2018

- JVET-J0018, “Description of SDR video coding technology proposal by MediaTek”, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei (MediaTek), San Diego, US, 10–20 Apr. 2018

- JVET-J0022, “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version”, P. Bordes, Y. Chen, C. Chevance, E. François, F. Galpin, M. Kerdranvat, F. Hiron, P. de Lagrange, F. Le Léannec, K. Naser, T. Poirier, F. Racapé, G. Rath, A. Robert, F. Urban, T. Viellard (Technicolor), Y. Chen, W.-J. Chien, H.-C. Chuang, M. Coban, J. Dong, H. E. Egilmez, N. Hu, M. Karczewicz, A. Ramasubramonian, D. Rusanovskyy, A. Said, V. Seregin, G. Van Der Auwera, K. Zhang, L. Zhang (Qualcomm), San Diego, US, 10–20 Apr. 2018

- JVET-J0031, “Description of SDR video coding technology proposal by University of Bristol”, D. Bull, F. Zhang, M. Afonso (Univ. of Bristol), San Diego, US, 10–20 Apr. 2018

- JVET-J0032, “Description of SDR video coding technology proposal by University of Science and Technology of China, Peking University, Harbin Institute of Technology, and Wuhan University (IEEE 1857.10 Study Group)”, F. Wu, D. Liu, J. Xu, B. Li, H. Li, Z. Chen, L. Li, F. Chen, Y. Dai, L. Guo, Y. Li, Y. Li, J. Lin, C. Ma, N. Yan (USTC), W. Gao, S. Ma, R. Xiong, Y. Xu, J. Li (Peking Univ.), X. Fan, N. Zhang, Y. Wang, T. Zhang, M. Gao (Harbin Inst. Tech.), Z. Chen, Y. Zhou, X. Pan, Y. Li, F. Liu, Y. Wang (Wuhan Univ.), San Diego, US, 10–20 Apr. 2018

- JVET-K0158, “AHG9: Separable Convolutional Neural Network Filter with Squeeze-and-Excitation block”, T. Hashimoto, E. Sasaki, T. Ikai (Sharp), Ljubljana, SI, 10–18 July 2018

- JVET-K0222, “AHG9: Convolution neural network loop filter”, Y.-L. Hsiao, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), Ljubljana, SI, 10–18 July 2018

- JVET-K0391, “AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao (Tencent), Ljubljana, SI, 10–18 July 2018

- JVET-K0266, “CE3: Non-linear weighted intra prediction (Test 6.1.1)”, P. Merkle, J. Pfaff, P. Helle, R. Rischke, M. Schäfer, B. Stallenberger, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI), Ljubljana, SI, 10–18 July 2018

- JVET-L0242, “AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao (Tencent), Macao, CN, 03–12 Oct. 2018

- JVET-L0383, “AHG9: Convolution Neural Network Filter”, K. Kawamura, Y. Kidani, S. Naito (KDDI), Macao, CN, 03–12 Oct. 2018

- JVET-M0159, “AHG9: Convolutional neural network loop filter”, Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0215, “AHG9-related: CNN-based lambda-domain rate control for intra frames”, Y. Li, D. Liu, Z. Chen (USTC), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0351, “AHG9: Convolutional Neural Network Filter (CNNF) for Intra Frame”, C. Lin, J. Yao, L. Wang (Hikvision), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0508, “AHG9: Test Results of Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0510, “AHG9: CNN-based in-loop filter proposed by USTC”, Y. Dai, D. Liu, Y. Li, F. Wu (USTC), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0566, “Adaptive convolutional neural network loop filter”, H. Yin, R. Yang, X. Fang, S. Ma, Y. Yu (Intel), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0691, “AHG9: Complexity analysis about neural network video coding tools”, Y. Li, Z. Chen (WHU), S. Liu (Tencent), Marrakech, MA, 9–18 Jan. 2019

- JVET-N0110, “CE13-1.1: Convolutional neural network loop filter”, Y.-L. Hsiao, O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, Geneva CH, 19–27 March 2019

- JVET-N0169, “CE13-2.1: Convolutional Neural Network Filter (CNNF) for Intra Frame”, J. Yao, L.Wang, Geneva CH, 19–27 March 2019

- JVET-N0254, “CE13: Dense Residual Convolutional Neural Network based In-Loop Filter (Test 2.2 and 2.3)”, Y. Wang, Z. Chen, Y. Li, L. Zhao, S. Liu, X. Li, Geneva CH, 19–27 March 2019

- JVET-N0480, “CE13-1.2: Adaptive convolutional neural network loop filter”, H. Yin, R. Yang, X. Fang, S. Ma, Geneva, CH, 19–27 March 2019

- JVET-N0513, “CE13: Experimental results of CNN-based In-Loop Filter (USTC)”, Y. Dai, D. Liu, N. Yan, F. Wu, Geneva, CH, 19–27 March 2019

- JVET-N0133, “CE13-related: In-loop filter with only CNN-based filter”, S. Wan, M.-Z.Wang, H. Gong, C.-Y. Zou, Y.-Z. Ma, J.-Y. Huo, Y.-F. Yu, Y. Liu, Geneva CH, 19–27 March 2019

- JVET-N0065, “AHG17 & AHG9: Comments on carriage of coding tool parameters in Adaptation Parameter Set”, M. Li, P. Wu, Geneva CH, 19–27 March 2019

- JVET-O0056, “CE10-1.2: Convolutional neural network loop filter”, Y.-L. Hsiao, O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), Gothenburg, SE, 3–12 July 2019

- JVET-O0063, “CE10-1.7: Adaptive convolutional neural network loop filter”, H. Yin, R. Yang, X. Fang, S. Ma (Intel), Gothenburg, SE, 3–12 July 2019

- JVET-O0079, “CE10: Integrated in-loop filter based on CNN (Tests 2.1, 2.2 and 2.3) ”, S. Wan, M.-Z.Wang, H. Gong, C.-Y. Zou (NPU), Y.-Z. Ma, J.-Y. Huo (Xidian Univ.), Y.-F. Yu, Y. Liu (OPPO), Gothenburg, SE, 3–12 July 2019

- JVET-O0101, “CE10: Dense Residual Convolutional Neural Network based In-Loop Filter (Tests 2.5 and 2.7) ”, Y. Wang, T. Ouyang, C. Zou, Y. Li, Z. Chen(Wuhan Univ.), L. Zhao, S. Liu, X. Li(Tencent), Gothenburg, SE, 3–12 July 2019

- JVET-O0131, “CE10-1.10/CE10-1.11: Evaluation results of CNN-based filtering with on-line learning model”, Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI), Gothenburg, SE, 3–12 July 2019

- JVET-O0132, “CE10-2.10/CE10-2.11: Evaluation results of CNN-based filtering with off-line learning model”, Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI), Gothenburg, SE, 3–12 July 2019

- JVET-O0347, “CE10:Neural Network based Filter for Video Coding”, H. Zhao, Y. Dai, D. Liu, N. Yan, H. Li (USTC), X. Chen, H. Yang (Huawei), Gothenburg, SE, 3–12 July 2019

- JVET-O0157, “Non-CE10: A CNN based in-loop filter for intra frame”, L. Xu, J. Zhu, K. Kazui (Fujitsu), Gothenburg, SE, 3–12 July 2019

- JVET-O0348, “CE10-Related: Classifier Based Adaptive Convolutional Neural Network Loop Filter”, H. Yin, R. Yang, X. Fang, S. Ma (Intel), Gothenburg, SE, 3–12 July 2019

- JVET-P0489, “AHG9: Multiple Convolution Neural Networks For Sequence-Independent Processing”, H.Yin, R.Yang, X.Fang, Z.Gao, R.Yang, Geneva CH, 1–11 October 2019