| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0067 |
| --- | --- |

| Title: | Bit Stream Feature Analyzer (BSFA) for Coding Tool Statistics based on VTM-10.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matthias Kränzler Christian Herglotz André Kaup | Tel: Email: | matthias.kraenzler@fau.de christian.herglotz@fau.de andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander University Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This contribution presents an analyzer add-on for the VTM-10.0 decoder. For any VVC-conform bit stream, the analyzer counts the occurrence of all major coding tools and coding modes used for improving the compression efficiency. Examples are tools like intra prediction, transformations, residual coefficients, and in-loop filtering operations on slice level, block level, or pel level. The authors report that the tool is lightweight in terms of complexity and that the results of the analyzer can be used for further interpretation and assessment of coding tools. Furthermore, results can be used for novel encoding techniques targeting energy efficient coding or mode decisions based on machine learning.

# Introduction

In the JVET meeting in Brussels, an analyzer was proposed that is capable of counting the frequency of occurrence of a large variety of different coding modes and coding tools. For this meeting, the analyzer was updated to be conform to the recently standardized VVC specifications. Furthermore, the software is extended to analyze the latest coding tools such as GEO.

The proposed tool is a valuable add-on to existing tools such as tracing [1] and DECODER_DEBUG_STATISTICS, where the former is much more complex and returns highly detailed information on the explicit block structure and the use of coding modes. The latter tool returns information on the usage of coded bits, which is important in the analysis of the CABAC engine. In contrast, the proposed bit stream feature analyzer (BSFA) provides the following information at a significantly smaller complexity than the already available approaches:

- Frequency of occurrence of coding tools like inter / intra prediction blocks, transformations, refinement, coefficient coding, and in-loop filtering.

- Evaluation on block-level.

- Overall sequence-wise statistics.

Furthermore, it is claimed that the following benefits can be drawn:

- Information on the use of newly introduced coding modes.

- Investigation of the influence of external parameters (e.g., QP, encoder configuration, tools on/off).

- In-depth analysis [8] of coding mode statistics.

- Training data for encoder optimizations [9].

- Exploitation for energy modeling [2,6].

- Exploitation for decoding-energy optimal encoding [3].

- Exploitation for no-reference quality evaluation on mode 3 of Rec. ITU-T P.1204.3 [7]

We propose to include the analyzer software into the VTM-reference software.

# Bit Stream Analysis in Decoding

To implement the analyzer, the decoder of VTM-10.0 was extended with additional functionality. The main extension is a structure called featureCounter, which includes integer variables that count the occurrence of features. During decoding, after parsing and interpretation of the bit stream information, the variables are incremented when a certain feature occurs. After finishing the overall decoding process, the counted numbers are written to an ASCII text file, which can be read using software like Matlab or Python.

Version VTM-10.0 of the analyzer provides information for all features listed in Table 1. The features comprise parsing, prediction, transformation, block size, and loop filter information. For further clarification, we distinguish features by a reference unit. Namely, features are counted on slice level (slice level features), CTB level (CTB features), block level (block features), on pel level (pel features), or on boundary level (boundary features). Block level features are counted depending on the block dimensions (e.g., CU dimensions) as explained in Section 2.1.

In the current implementation, feature counting is enabled by setting a command line flag

--BSFA="targetTextFile.m",

which causes the decoder to write an ASCII-text file in the source folder of the bit stream, which is directly Matlab-readable.

## Block-Wise Feature Counting

Most of the features are counted block-wise, which reflects the multi-type tree coding structure implemented in VTM. Allowed block dimensions in counting include a single pel (size 1x1) up to CTU size of 128x128 pels. The proposed analyzer considers all possible square and rectangular block sizes by using a matrix structure, where each entry corresponds to a single block size. The matrix has dimensions 8x8, which corresponds to block dimensions in powers of two. The resulting matrix in terms of block dimensions is defined as

,

where the first dimension indicates the width and the second dimension the height of the corresponding block.

## Currently Implemented Features

The implemented features are categorized according to the main components of a hybrid decoder, which are the categories GENERAL, PREDICTION, TRANSFORM, PARSING, and IN-LOOP. All features, depending on their category, are listed in Table 1.

Table 1 Features implemented in the VTM analyzer software. Newly implemented features are indicated by bold letters.

| Category | Index | Feature name | Level |
| --- | --- | --- | --- |
| GENERAL | 1 | ISlice | Slice |
|  | 2 | PSlice | Slice |
|  | 3 | BSlice | Slice |
|  | 4 | QP | Slice |
| PREDICTION | 5 | intraBlock | Block |
|  | 6 | intraPlanarLuma | Block |
|  | 7 | intraDCLuma | Block |
|  | 8 | intraHVDLuma | Block |
|  | 9 | intraAngLuma | Block |
|  | 10 | intraPlanarChroma | Block |
|  | 11 | intraDCChroma | Block |
|  | 12 | intraHVDChroma | Block |
|  | 13 | intraAngChroma | Block |
|  | 14 | intraCrossComp | Block |
|  | 15 | intraMIP | Block |
|  | 16 | intraPDPC | Block |
|  | 17 | intraIBC | Block |
|  | 18 | intraSubPartitionsHor | Block |
|  | 19 | intraSubPartitionsVer | Block |
|  | 20 | interBlocks | Block |
|  | 21 | interInter | Block |
|  | 22 | interMerge | Block |
|  | 23 | interSkip | Block |
|  | 24 | interAffine | Block |
|  | 25 | interAffineInter | Block |
|  | 26 | interAffineMerge | Block |
|  | 27 | interAffineSkip | Block |
|  | 28 | interSubblockMerge | Block |
|  | 29 | interTriangleSplit | Block |
|  | 30 | uniPredPel | Pel |
|  | 31 | biPredPel | Pel |
|  | 32 | fracPelHor | Pel |
|  | 33 | fracPelVer | Pel |
|  | 34 | fracPelBoth | Pel |
|  | 35 | copyPel | Pel |
|  | 37 | affinefracPelHor | Pel |
|  | 38 | affinefracPelVer | Pel |
|  | 39 | affinefracPelBoth | Pel |
|  | 40 | affinecopyPel | Pel |
|  | 41 | dmvr | Block |
|  | 42 | Bdof | Block |
|  | 43 | Geo | Block |
| TRANSFORM | 44 | Transform | Block |
|  | 45 | TransformDCT2 Hor | Block |
|  | 46 | TransformDCT8 Hor | Block |
|  | 47 | TransformDST7 Hor | Block |
|  | 48 | TransformDCT2 Ver | Block |
|  | 49 | TransformDCT8 Ver | Block |
|  | 50 | TransformDST7 Ver | Block |
|  | 51 | TransformSkip | Block |
|  | 52 | TransformLFNST | Block |
| PARSING | 53 | nCoeff | Pel |
|  | 54 | coeffG1 | Pel |
|  | 55 | valueOfCoeff | Log-domain |
| IN-LOOP | 56 | Bs0 | Boundary |
|  | 57 | Bs1 | Boundary |
|  | 58 | Bs2 | Boundary |
|  | 59 | SAOLumaEO | CTB |
|  | 60 | SAOLumaBO | CTB |
|  | 61 | SAOChromaEO | CTB |
|  | 62 | SAOChromaBO | CTB |
|  | 63 | ALFLuma | CTB |
|  | 64 | ALFChroma | CTB |

Most of the features are described in detail in [4] and [5]. New features (indicated in bold) count the number of blocks using decoder side motion vector derivation (DMVR), bi-direction optical flow (BDOF), geometric partitioning (GEO), affine predicted pixels.

It shall be noted that the implemented list exhaustively counts all new incorporated prediction modes. However, it is possible that further modes or other side information might be of interest, such that more counters could be implemented in the future.

## Extension with New Features

To include new features for counting, the following steps must be undertaken:

- Add a new feature to the structure featureStruct in TypeDef.h.

- Set the initial value to zero.

- Increment counter after parsing (mostly in the function decompressCTU() in DecCu.cpp).

- Make a new output fprintf() in the if clause if (usePrunedFeatureSet == 0) in DecApp.cpp.

Afterwards, the newly introduced feature is printed to the output file and can be further used for analysis.

# Applications

The proposed feature analyzer can be used for several applications. One example is the analysis of the occurrence of block sizes as shown in [5]. Furthermore, results can be used for energy modeling [6] to perform research targeting energy efficient decoding solutions [3]. Finally, an extensive analysis of coded bit streams might help in finding fast and efficient encoder implementations [9].

# Concluding Remarks

This contribution introduced an analyzer add-on for the VTM-10.0 decoder software. It enables to obtain detailed statistics on the use of compression tools such as prediction modes, transformation, coefficient coding, CU block sizes, and in-loop filtering. For further assessment and in-depths investigations of coding tools, it is proposed to include the analyzer into the VTM-reference software.

The source code is freely available for download and further use under the BSD License at https://gitlab.lms.tf.fau.de/LMS/vtm-analyzer .

# Patent rights declaration(s)

The Friedrich-Alexander University Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.

# References

[1] J. Sauer, J. Schneider, M. Bläser, M. Wien, “Reference software extension for coding block statistics”, JVET-K0149, July 2018.

[2] C. Herglotz, D. Springer, M. Reichenbach, B. Stabernack, and A. Kaup, „Modeling the energy consumption of the HEVC decoding process”, IEEE Transactions on Circuits and Systems for Video Technology, 28:217–229, Jan 2018.

[3] C. Herglotz, A. Heindel, and A. Kaup. Decoding energy distortion optimization for video coding. IEEE Transactions on Circuits and Systems for Video Technology, 29:171-182, Jan. 2019.

[4] C. Herglotz, M. Kränzler, A. Kaup, “Bit Stream Analyzer for Coding Tool Statistics”, JVET-P0085, October 2019.

[5] C. Herglotz, M. Kränzler, A. Kaup, “Bit Stream Feature Analyzer (BSFA) for Coding Tool Statistics Based on VTM-7.0”, JVET-Q0049, January 2020.

[6] M. Kränzler, C. Herglotz, A. Kaup, “Decoding Energy Modeling for Versatile Video Coding”, accepted for IEEE International Conference on Image Processing (ICIP), October 2020.

[7] R. Rao, S. Göring, P- List, W. Robitza, B. Feiten, U. Wüstenhagen, A. Raake, "Bitstream-Based Model Standard for 4K/UHD: ITU-T P.1204.3 — Model Details, Evaluation, Analysis and Open Source Implementation," Proc. Twelfth International Conference on Quality of Multimedia Experience (QoMEX), Athlone, Ireland, 2020, pp. 1-6.

[8] O. Bougacha, H. Kibeya, N. Belhadj, M. A. Ben Ayed and N. Masmoudi, "Statistical analysis of intra prediction in HEVC video encoder," Proc. International Image Processing, Applications and Systems (IPAS), Hammamet, 2016, pp. 1-5.

[9] G. Chi, X. Jin and Q. Dai, "A quad-tree and statistics based fast CU depth decision algorithm for 3D-HEVC," Proc. IEEE International Conference on Multimedia and Expo Workshops (ICMEW), Chengdu, 2014, pp. 1-5.