| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting: by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0070-v1 |
| --- | --- |

| Title: | AHG9/AHG8: Scalability dimension SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Li Zhang1, Kai Zhang1, Zhipin Deng2 (Bytedance) 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a new SEI message, named scalability dimension SEI message, for signalling of scalability dimension information for a VVC bitstream, including 1) whether the bitstream is a multiview bitstream, and if yes, the view ID of each layer, and 2) whether one or more layers in the bitstream represent auxiliary information such as alpha, depth, etc., and if yes, which layers represent what.

# Introduction

VVC supports multi-layer scalabilities. However, given a VVC multi-layer bitstream, it is unknown whether the bitstream of an OLS is a multiview bitstream or just a bitstream consisting of multiple layers with SNR and/or spatial scalabilities. Furthermore, given a VVC multi-layer bitstream, it is unknown whether there is one or more layers representing auxiliary information such as alpha, depth, etc., and if yes, which layers represent what.

To address these issues, an SEI message, named scalability dimension SEI message, is proposed.

It is understood that for fully support of multiview and other applications, camera parameters and other multiview information like those signalled in the multiview acquisition information SEI message and other SEI messages specified in HEVC subclause G.14 would also be needed. Therefore, the proposal here is intended to be a starting point.

# Proposal

The syntax and semantics of the proposed scalability dimension SEI message are as follows:

## Scalability dimension SEI message syntax

| scalability_dimension( payloadSize ) { | Descriptor |
| --- | --- |
| sd_max_layers_minus1 | u(6) |
| sd_multiview_info_flag | u(1) |
| sd_auxilary_info_flag | u(1) |
| if( sd_multiview_info_flag ¦ ¦ sd_auxilary_info_flag ) { |  |
| if( sd_multiview_info_flag ) |  |
| sd_view_id_len | u(4) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| if( sd_multiview_info_flag ) |  |
| sd_view_id_val[ i ] | u(v) |
| if( sd_auxilary_info_flag ) |  |
| sd_aux_id[ i ] | u(8) |
| } |  |
| } |  |
| } |  |

## Scalability dimension SEI message semantics

The scalability dimension SEI message provides the scalability dimension information for each layer in bitstreamInScope (defined below), such as 1) when bitstreamInScope may be a multiview bitstream, the view ID of each layer; and 2) when there may be auxilary information (such as deppth or alpha) carried by one or more layers in bitstreamInScope, the auxilary ID of each layer.

The bitstreamInScope is the sequence of AUs that consists, in decoding order, of the AU containing the current scalability dimension SEI message, followed by zero or more AUs, including all subsequent AUs up to but not including any subsequent AU that contains a scalability dimension SEI message.

sd_max_layers_minus1 plus 1 indicates the maximum number of layers in bitstreamInScope.

sd_multiview_info_flag equal to 1 indicates that bitstreamInScope may be a multiview bitstream and the sd_view_id_val[ ] syntax elements are present in the scalability dimension SEI message. sd_multiview_flag equal to 0 indicates that bitstreamInScope is not a multiview bitstream and the sd_view_id_val[ ] syntax elements are not present in the scalability dimension SEI message.

sd_auxilary_info_flag equal to 1 indicates that there may be auxilary information carried by one or more layers in bitstreamInScope and the sd_aux_id[ ] syntax elements are present in the scalability dimension SEI message. sd_auxilary_info_flag equal to 0 indicates that there is no auxilary information carried by one or more layers in bitstreamInScope and the sd_aux_id[ ] syntax elements are not present in the scalability dimension SEI message.

sd_view_id_len specifies the length, in bits, of the sd_view_id_val[ i ] syntax element.

sd_view_id_val[ i ] specifies the view ID of the i-th layer in bitstreamInScope. The length of the sd_view_id_val[ i ] syntax element is sd_view_id_len bits. When not present, the value of sd_view_id_val[ i ] is inferred to be equal to 0.

sd_aux_id[ i ] equal to 0 indicates that the i-th layer in bitstreamInScope does not contain auxilary pictures. sd_aux_id[ i ] greater than 0 indicates the type of auxiliary pictures in the i-th layer in bitstreamInScope as specified in Table 1.

Table 1 – Mapping of sd_aux_id[ i ] to the type of auxiliary pictures

| sd_aux_id[ i ] | Name | Type of auxiliary pictures |
| --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3..127 |  | Reserved |
| 128..159 |  | Unspecified |
| 160..255 |  | Reserved |

NOTE 1 – The interpretation of auxiliary pictures associated with sd_aux_id in the range of 128 to 159, inclusive, is specified through means other than the sd_aux_id value.

sd_aux_id[ i ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, for bitstreams conforming to this version of this Specification. Although the value of sd_aux_id[ i ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, in this version of this Specification, decoders shall allow values of sd_aux_id[ i ] in the range of 0 to 255, inclusive.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).