| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting: by teleconference, 7 – 16 October 2020 | Document: JVET-T0084 |
| --- | --- |

| Title: | AHG12: Transform coefficients range extension for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Takeshi Chujoh Eiichi Sasaki Tomohiro Ikai | Tel: Email: | +81-50-5213-6338 zhou.tianyang@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a format range extension design for high bit-depth coding. In the VVC versin1, the transform coefficient range is always equal to 15 regardless of bit-depth. It is asserted that this design is not sufficient in high bit-depth coding.

This proposal proposes to extend BitDepth + 5 for high bit-depth coding to argue this value would be sufficient and it is consistent in 10 bit-depth case. It is reported that the experimental result show bd-rate gain is 1.03 %, 1.05 %, 1.02 % in AI-12, LDB-12, RA-12 (qp =-13, -8, -3, 2, 7, 12) and 14.83 %, 9.72 %, 8.82 % in AI-16, LDB-16, RA-16 (qp=-33, -28, -23, 18, 13, 8) respectively.

It is also reported that this high precision design shows comparable or better coding performance in 12, 14 and 16 bit case compared to BitDepth + 6 design.

Note: The current VTM software provides an extension for transform coefficient range when JVET_R0351_HIGH_BIT_DEPTH_ENABLED macro is enabled in build and the extended precision flag is equal to 1 where BitDepth + 6 is used for transform coefficient range instead of 16.

Introduction

In the current VVC software, when JVET_R0351_HIGH_BIT_DEPTH_ENABLED is enabled the extended precision flag was specified in SPS. The decoder processes scaling (IQ), and IDCT to derive residual. In this process, the intermediate value range for transform is restricted to the so-called max transform coefficients range (maxLog2TransformRange) and shifting operations are carried out to adjust the value range. Specifically >> bdShift (bdShift1) in scaling, >> 7 in vertical IDCT and 20 – BitDepth (bdShift2) in horizontal IDCT.

Depending on extended_precision_processing_flag, maxLog2TransformRange, bdShift1 and bdShift2 are derived as follows.

If extended_precision_processing_flag is equal to 0 (a.k.a VVC version1).

- maxLog2TransformRange = 15

- bdShift1 = BitDepth + rectNonTsFlag +	( ( Log2 ( nTbW ) + Log2 ( nTbH ) ) / 2 ) − 5 + sh_dep_quant_used_flag

- bdShift2 = Max (20 – BitDepth, 0)

Otherwise (extended_precision_processing_flag is equal to 1)

- maxLog2TransformRange = extended_precision_processing_flag ? Max( 15, BitDepthY + 6 ) : 15

- bdShift1 = BitDepthY + Log2( nTbS ) + 10 − log2TransformRange

- bdShift2 = Max( 20 − BitDepth, extended_precision_processing_flag ? 11 : 0 )

There are two problems.

For extended_precision_processing_flag is equal to 0, the transform coefficient range (constant 15) is not sufficient in high bit-depth range

For extended_precision_processing_flag is equal to 1, the appropriate transform range value has not been studied in VVC design.

This contribution proposes to use BitDepth + 5 and bdShift to 10 in the range of 10 to 16 bit case.

Compared to the HEVC RExt design where the range is BitDepth + 6 and bdShift is equal to 11, it is asserted that 1 bit less range would be beneficial to reduce the multiplier burdens when we enable this feature in 12, 14 bit profile as well. The range and bdShift value in 10 bit case are not changed according to the flag.

For simplicity, a part of bdShift1 (here in bdShift1 offset) is shown in Table 1.

Table 1. Summary of bdShift values in the range of 8 to 16 in BitDepth

| Where | BitDepth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |
| VVC |  |  |  |  |  |  |  |  |  |  |
| maxLog2TrDynamicRange | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| bdShift1 base | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | bitDepth -5 |
| bdShift2 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 20-bitdepth |
| HEVC RExt |  |  |  |  |  |  |  |  |  |  |
| maxLog2TrDynamicRange | 15 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | Max(bitDepth + 6, 15) |
| bdShift1 base | 3 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| bdShift2 | 12 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | Max(20-bitDepth, 11) |
| Proposal |  |  |  |  |  |  |  |  |  |  |
| maxLog2TrDynamicRange | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | bitDepth + 5 |
| bdShift1 base | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| bdShift2 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |

Experiment

Test sequences and test condition

The test sequences and condition follows AHG12 suggested CTC[3]. The test software used is the latest VTM 10.0 with

#define JVET_R0351_HIGH_BIT_DEPTH_ENABLED                 1

Tests results

Anchor: VTM-10.0 with RExt__HIGH_BIT_DEPTH_SUPPORT = 1, ExtendedPrecision = 0

Test1 (Proposal): VTM-10.0 with RExt__HIGH_BIT_DEPTH_ENABLED = 1, ExtendedPrecision = 1 and maxLog2TrDynamicRange = BitDepth + 5

Note: Table 1 results following AHG12[3] CTC format sheet.

Table 2. Results of proposal (BitDetph + 5)

|  |  |  | AI-12 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -0.74% | -1.06% | -1.06% | 109% | 91% |
| PQ444 | -1.27% | -1.77% | -1.90% | 102% | 102% |
| PQ422 | -1.07% | -1.27% | -1.36% | 102% | 102% |
| Overall | -1.03% | -1.37% | -1.44% | 104% | 98% |
|  |  |  | LDB-12 |  |  |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -0.78% | -0.72% | -0.73% | 114% | 94% |
| PQ444 | -1.28% | -2.10% | -2.12% | 101% | 106% |
| PQ422 | -1.09% | -1.70% | -1.61% | 101% | 106% |
| Overall | -1.05% | -1.50% | -1.49% | 82% | 81% |
|  |  |  | RA-12 |  |  |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -0.70% | -0.66% | -0.66% | 109% | 95% |
| PQ444 | -1.31% | -1.93% | -2.01% | 101% | 95% |
| PQ422 | -1.04% | -1.54% | -1.51% | 101% | 95% |
| Overall | -1.02% | -1.37% | -1.39% | 105% | 95% |

|  |  |  | AI-16 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -14.83% | -16.53% | -14.66% |  |  |
| Overall | -14.83% | -16.53% | -14.66% |  |  |
|  |  |  | LDB-16 |  |  |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -9.72% | -9.33% | -9.35% |  |  |
| Overall | -9.72% | -9.33% | -9.35% |  |  |
|  |  |  | RA-16 |  |  |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -8.82% | -8.77% | -8.81% |  |  |
| Overall | -8.82% | -8.77% | -8.81% |  |  |

Additional results (Extended range vs 12 to 16bit-depth)

We conducted additional test with different extended range from BitDepth + 4 to BitDepth +6 and different bit-depth ranging from 12bit to 16bit. The following shows a summary, it is asserted the proposal have best balance of coding performance.

Table 3 Summary resutls

|  | AI-12 | AI-14 | AI-16 |
| --- | --- | --- | --- |
| BitDepth + 4 | 0.10% / 0.13% | 0.04% / 0.03% | -0.33% / -0.04% |
| BitDepth + 5 (Proposal) | 0.11% / 0.10% | 0.00% / 0.01% | -0.26% / -0.04% |
| BitDepth + 6 (Anchor) | 0% | 0% | 0% |
| BitDepth + 7 | -0.03% / -0.03% | 0.07% / 0.00% | 0.83% / 0.14% |

SVT / PQ422

Best case are high lighted in cyan.

The table 2 shows the coding efficiency of setting different maxLog2TrDynamicRange value.

Table 4 Results on AI-12, AI-14, AI-16

|  |  | AI-12 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-10.0 ExtendedPrecision = 1 |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Test 1 BitDepth + 5 | SVT | 0.11% | 0.14% | 0.14% | 104% | 102% |
|  | PQ422 | 0.10% | 0.11% | 0.14% | 101% | 101% |
| Test 2 BitDepth + 4 | SVT | 0.10% | 0.09% | 0.09% | 101% | 101% |
|  | PQ422 | 0.13% | 0.12% | 0.10% | 99% | 101% |
| Test 3 BitDepth + 7 | SVT | -0.03% | -0.04% | -0.04% | 99% | 100% |
|  | PQ422 | -0.03% | -0.03% | -0.03% | 103% | 100% |
|  |  | AI-14 |  |  |  |  |
|  |  | Over VTM-10.0 ExtendedPrecision = 1 |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Test 1 BitDepth + 5 | SVT | 0.00% | 0.00% | 0.00% | 103% | 100% |
|  | PQ422 | 0.01% | 0.02% | 0.01% | 100% | 100% |
| Test 2 BitDepth + 4 | SVT | 0.04% | 0.04% | 0.04% | 101% | 100% |
|  | PQ422 | 0.03% | 0.05% | 0.05% | 98% | 100% |
| Test 3 BitDepth + 7 | SVT | 0.07% | 0.05% | 0.05% | 100% | 100% |
|  | PQ422 | 0.00% | 0.01% | 0.00% | 100% | 100% |
|  |  | AI-16 |  |  |  |  |
|  |  | Over VTM-10.0 ExtendedPrecision = 1 |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Test 1 BitDepth + 5 | SVT | -0.26% | -0.21% | -0.21% | 100% | 99% |
|  | PQ422 | -0.04% | -0.04% | -0.03% | 97% | 100% |
| Test 2 BitDepth + 4 | SVT | -0.33% | -0.26% | -0.26% | 102% | 99% |
|  | PQ422 | -0.04% | -0.04% | -0.04% | 98% | 99% |
| Test 3 BitDepth + 7 | SVT | 0.83% | 0.81% | 0.81% | 101% | 101% |
|  | PQ422 | 0.14% | 0.15% | 0.15% | 96% | 100% |

References

[1]	A. Browne, S. Keating and K. Sharman, “High bit depth coding”, Joint Video Experts Team Document, JVET-R0351, teleconference, April 2020.

[2]	B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 10)”, Joint Video Experts Team Document, JVET-S2001, teleconference, September 2020.

[3]	T. Ikai, A. Browne, X. Xiu, “AHG12: Preliminary common test conditions for high bitdepth video coding”, JVET-T0047, teleconference, October 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).