| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0130-v3 |
| --- | --- |

| Title: | BoG Report: Neural Network Technology |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall | Email: | asegall@sharplabs.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This is a DRAFT report of activities from the BoG on Neural Network Technology.  The BoG held the following meetings during the 20th JVET meeting:

- October 13 – 20:20 – 21:20

- October 14 – 21:40 – 23:20

- October 15 – 21:45 – 24:05

Recommendations from the BoG are summarized as:

Test Conditions

Recommendation: Reduce Class A1/A2 to 3s for LDB/LDP.

Recommendation: Use the sequences from the BVI-DVC data and JVET FTP data set that are available for use by JVET for the training set.  Do not include any sequences that are included in the test set.

Recommendation: Create a list of exact sequences to be included in the training set and provide in the CTC.  This should include the location of the sequence and md5sum.

Recommendation: Output document with common test conditions for neural network-based video coding technology based on BoG output.

EE Experiment

Recommendation: Create EE on neural network-based video coding technology chaired by E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. Ma and H.Wang.

Recommendation: Include loop filter proposals JVET-T0057, JVET-T0069, JVET-T0079, JVET-T088, JVET-T0094 and JVET-T0128 in the CE.  Additionally, include JVET-T0058 and JVET-T0125.

Recommendation: Review EE description.

AhG Mandates

Recommendation (Revisit): Create mandate for AhG to create results with temporal filtering enabled.

Recommendation: Add mandate to AhG to study and identify sequences from the UGC data set for future inclusion of the test set.

# Mandates

The BoG was established with the following mandates:

- Develop a common test procedure for both end-to-end and hybrid neural network methods.  The common test procedure should include a unified CTC, reporting template and training procedure

- Define an exploration experiment on NN loop filtering, with the main purposing of serving as a test of the common test procedure.

# Common Test Procedures

The following questions should be considered in developing the common test procedures:

- How to harmonize the existing common test conditions developed in JVET-T0041 and DNNVC (JVET-T0129)

- How to measure complexity and what measurements should be mandatory in the common test procedure?

- What limitations should be applied in training?

  - Should we define the training set?  And, if so, what should it be?

The group agreed that these were the main questions to be answered.

Common Test Conditions

The following table was discussed within the BoG with the goal of (i) capturing a comparison between the test conditions developed in JVET-T0041 and JVET-T0129 and (ii) defining the test conditions to be used as a way forward.

|  | JVET-T0041 | JVET-T0129 | Way Forward |
| --- | --- | --- | --- |
| Quantization Parameter |  |  |  |
| Fixed QP | 22, 27, 32, 37, 42 | 27, 32, 37, 42 | 22, 27, 32, 37, 42 |
| Bit-rate Target | - | ±10% of Anchor | ±10% of 27, 32, 37, 42 |
| Objective Metrics | PSNR, MS-SSIM | PSNR, MS-SSIM | PSNR, MS-SSIM |
| Sequences |  |  |  |
| Classes (AI) | A1, A2, B, C, E, D, F | A1, A2, B, C, E, D, F, H2(O) | A1, A2, B, C, E, D, F, H2(O) |
| Classes (RA) | A1, A2, B, C, E(O), D, F | A1, A2, B, C, D, F, H2(O) | A1, A2, B, C, D, F, H2(O) |
| Classes (LDB) | A1(O), A2(O), B, C, E, D, F | B, C, E, D, F | A1(O), A2(O), B, C, E, D, F |
| Classes (LDP) | A1(O), A2(O), B, C, E, D, F | B, C, E, D, F | A1(O), A2(O), B(O), C(O), E(O), D(O), F(O) |
| Anchor | VTM 10.0 or newer | VTM10.0 or newer | VTM10.0 or newer |

Table A

One participant commented that proponents should be encouraged to report on as many optional classes as possible.

One participant suggested to reduce the length of Class A1/A2 sequences if they are included in the LDP/LDB tests.  Multiple participants commented that this would be a good way forward.

Recommendation: Reduce Class A1/A2 to 3s for LDB/LDP.

One participant suggested to crop the Class A1/A2 sequences if they are included in LDP/LDB.

Open participant commented that including Class A1/A2 could be useful for evaluating NN technologies, as these technologies do not always perform well on high resolution sequences since they are often trained with low resolution patches.  Another participant commented that the encoding time for an end-to-end NN solution may be much faster than the VTM.

One participant commented that enabling the temporal filter in the anchor could be useful for the common test conditions.

Recommendation (Revisit): Create mandate for AhG to create results with temporal filtering enabled.

During the second BoG meeting, the following discussion occurred:

One participant commented that the temporal filtering should be aligned with any decision in JVET.

One participant commented that LDP is optional in the JVET SDR CTC.  A second participant supported this approach.

The group agreed to use and reflect the status of the test sequences captured in the “Way Forward” column in the draft common test conditions.

Complexity Metrics

The following table was discussed within the BoG with the goal of (i) capturing a comparison between the complexity metrics and information requested in JVET-T0041 and JVET-T0129 and (ii) defining the complexity metrics to be used as a way forward.  Note that M corresponds to mandatory in the table below and O corresponds to optional.

|  | JVET-T0041 | JVET-T0129 | Way Forward |
| --- | --- | --- | --- |
| Platform |  |  |  |
| GPU Type (including Memory Size) | M | M | M |
| Framework | M | M | M |
| Number of GPUs per task |  |  | M |
| Sum of Peak TeraFLOP |  |  | O |
| Inference and Training |  |  | M |
| Training |  |  |  |
| Epoch | M | O | M |
| Batch size | M | O | M |
| Number Iterations |  | O | O |
| Training Time | M |  | M |
| Patch size | O |  | O |
| Learning rate | O | O | O |
| Optimizer | O | O | O |
| Loss function | O | O | O |
| Preprocessing (Y/N) | O |  | O |
| Training sets | M |  | M |
| Training configuration per R-D point. This could include which QP points where used for training each R-D point. | M |  | M |
| Learning Curve (Training and Validation) |  |  | M |
| Inference |  |  |  |
| Architecture / Network Summary / Graphical |  | O | M |
| Total Conv. Layers | O |  |  |
| Total FC Layers | O |  |  |
| Framework | O |  | N/A |
| Parameters – Number (Per Model and/or Total) | O |  | M |
| Parameter – Precision | O | O | M |
| Parameter – Memory (Per Model and/or Total) | O |  | M (Auto) |
| Total Memory | O |  |  |
| Total MACs (or per pixel) | O |  | M |
| Any changes to network configuration or weights required to generate rate points |  | O |  |
| Peak Memory Usage |  | M |  |
| Patch and Batch size |  |  |  |
| Run Time: Proponent may choose to report one of the below: - CPU run-time - GPU run-time - CPU+GPU run-time Participants should provide a description of what run-time is reported and the environment used for computing the run-time, including the patch size, batch size and number of GPUs |  |  | M |

One participant commented that the memory size should account for the input size.  Another participant suggested that this could be accomplished by normalizing for a given input resolution.  (For example, a 4K input size.)

One participant commented that the MAC should also account for input size.

The group agreed to use and reflect the status of the metrics captured in the “Way Forward” column in the draft common test conditions.

Recommendation: Output document with common test conditions for neural network-based video coding technology based on BoG output.

Training Procedure

One participant asked if we should mandate the split of the training set into training and validation sets, for example a 80-20 split.

One participant commented that it could be beneficial to allow using a subset of the training set.  Another participant expressed support for this. Allowing the use of a subset of the training setwas generally agreed in the BoG.

One participant commented that it would be desirable to make sure there was no overlap between the training, validation and test sets.  Ensuring that there was no overlap between the training, validation and test sets was generally agreed in the BoG.

One participant asked if it would be allowed to use training data outside of the training set defined in the CTC.  One participant commented that it would be desirable if any outside dataset could be contributed to the JVET activity.

One participant commented that using the outside training data would make it difficult to understand if the benefit of the proposal came from its design or the training data used.

Another participant commented that reporting performance using an training data set outside the CTC and, additionally, the CTC data set could provide better understanding of the proposal.

It was commented that it should be required to report the performance of the proposal using the CTC data set.  If a participant chooses to report results using a non-CTC data set as supplemental information, this additional information would always be appreciated.   The comment expressed in the previous two sentences was generally agreed in the BoG.

It was commented that participants should provide a description on how the training, validation and/or test sets were constructed.  Requesting this description was generally agreed in the BoG.

One participant asked how we would verify that the limitations were followed.  Multiple participants commented that there must be some element of trust between participants.

Another participant commented that it could be beneficial to have an unknown test set in the future to verify performance, and that this procedure could also encourage participants to follow the limitations.

The group then discussed the selection of content to be used for the training and validation set.

The following table was included in JVET-T0041 and suggested by multiple participants to be useful for the discussion.

### Video sequences with reported license terms

| No. | Data base | Web | Availability Check | Priority |
| --- | --- | --- | --- | --- |
| A.1 General content datasets |  |  |  |  |
| 2 | UGC | https://media.withyoutube.com/ | CC BY 3.0 | 2 |
| 3 | SJTU | http://medialab.sjtu.edu.cn/web4k/index.html | Only Campfire Party content available? | 3 |
| 5 | CDVL | https://www.cdvl.org/ | Standards committees can use CDVL R&D content within subjective tests to validate objective video quality models (e.g., ATIS, VQEG, ITU). | 3 |
| 8 | JVET FTP | ftp.ient.rwth-aachen.de ftp.tnt.uni-hannover.de/testsequences/ | copyright information comes with content | 1 |
| 10 | Sports-1M | http://cs.stanford.edu/people/karpathy/deepvideo/ | CC BY 3.0 | 2 |
| 11 | Youtube-8M | https://research.google.com/youtube8m/download.html | CC BY 4.0 | 2 |
| 15 | BVI-DVC | https://vilab.blogs.bristol.ac.uk/?p=2375 | A Training Database for Deep Video Compression 104 video sequences sourced by U. of Bristol. | 1 |
| 16 | Xiph.org | https://media.xiph.org/video/derf/ | copyright information comes with content | 1 |
| A.2 Datasets for specific tasks |  |  |  |  |
| 1 | HMDB51 | http://serre-lab.clps.brown.edu/resource/hmdb-a-large-human-motion-database/ | CC BY 4.0 | 2 |
| 6 | AVA | https://research.google.com/ava/ | CC BY 4.0 | 2 |

Priority: 1 – with explicit claim that VCEG/MPEG/JCTVC/JVET can use for standardization purpose; 2 –CC BY 3.0 or 4.0 license; 3 – additional registration or clarification from content owner is needed.

It was reported by one participant that U. of Bristol has expressed a willingness to provide a sub-set of the BVI-DVC data set for standardization.

One participant suggested that including the DIV2K data set could be valuable.

One participant suggested using the BVI-DVC data set as a starting point.  However, it was encouraged to study and identify additional data sets for use.

One participant commented that the CTC should include a list of the exact sequences to be used.

One participant commented that using the JVET FTP data set would also be useful to include in the training set.

One participant commented that using the UGC data set would be useful to include in the training set.

One participant commented that the combination of the BVI-DVC data and JVET FTP data set should result in roughly 200 sequences.

Recommendation: Use the sequences from the BVI-DVC data and JVET FTP data set that are available for use by JVET for the training set.  Do not include any sequences that are included in the test set.

Recommendation: Create a list of exact sequences to be included in the training set and provided in the CTC.  This should include the location of the sequence and md5sum.

Recommendation: Add mandate to AhG to study and identify sequences from the UGC data set for future inclusion in the test set.

# Exploration Experiment

It is noted that the main purpose of the exploration experiment is to test the common test procedure.

It was proposed to define the exploration experiment to focus on loop filter technologies.  The contributions include JVET-T0057, JVET-T0069, JVET-T0079, JVET-T0088, JVET-T0094, and JVET-T0128.

One proponent suggested including JVET-T0125 in the EE.  Multiple participants commented that this is a technology that has not been proposed to JVET previously and that it would be valuable and interesting to understand its performance more.

One proponent suggested including JVET-T0058 in the EE.  One participant voiced support for including the technology in the EE.

One participant commented that it isn’t necessary for all technology proposed at the meeting to be in the EE to be further explored and that any information about these proposals would be highly encouraged at the next meeting as a non-EE or AHG related proposal.

The group then discussed the following questions and came to the following agreements within the BoG:

Q1: What should be the test in the EE?

A1: Test technology using the NN-CTC

Q2: How much deviation is allowed between the technology proposed as the current meeting and what is tested in the EE?

A2: Participants should define their test within 2 weeks of the meeting, and the tests should be similar to the input contribution.  Additionally, participants are encouraged to consider the following when defining their test:

  - Changes in parameter precision, mis-match between training and test QPs, number of layers, and/or position of filter (including as a post-filter).

  - Exercising aspects that are different from other technology in the EE.

Q3: Will participants share software as part of the EE process?

A3: Yes, we will plan to share software and pre-trained model within JVET

Q4: What cross-check procedure will be used in the EE?

A4: Cross check inference only

The BoG then identified potential coordinates for the EE and assigned the action item to create a draft of the EE using the agreements captured above.

EE Coordinators: E. Alshina, S. Liu, R.-L. Liao, Y Li, H.Wang, W. Chen, and Z.Ma

Recommendation: Create EE on neural network-based video coding technology chaired by E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. Ma and H. Wang.

Recommendation: Include loop filter proposals JVET-T0057, JVET-T0069, JVET-T0079, JVET-T088, JVET-T0094 and JVET-T0128 in the CE.  Additionally, include JVET-T0058 and JVET-T0125.

Recommendation: Review EE description.