| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0131 |
| --- | --- |

| Title: | Results for GOP-based temporal filtering for SDR and HDR on top of GOP 32 configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Jack Enhorn, Jacob Ström, Lukasz Litwic, Per Wennersten, Christopher Hollman, Martin Pettersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution provides results for GOP-based temporal filtering on top of the GOP 32 configuration for random access in SDR and HDR. It also provides an abbreviated description on how the filter works by filtering every 8th or 4th frame, and that the filter is turned off for low or negative QPs. The contribution also discusses questions raised at the meeting regarding the filter behavior for parallel operation, HDR sequences, scene cuts, fades and chroma formats. The proponents state that they did not find any problematic behavior of the filter in these cases.

The BD-rate impact is as follows:

SDR RA: -4.1%, -7.2%, -7.0% (Y, U, V)

HDR RA: -9.4%, -7.3, -1.5%, -1.7% (deltaE, psnrL, wpsnrY, psnrY)

To be updated to reflect HDR performance when HLG part is ready.

# Introduction

At the October JVET meeting, it was discussed whether the GOP-based temporal filter should be included in the common test conditions. During that discussion there were some questions about the filter, and this document is an attempt to answer these questions, as well as provide information on the performance of the filter for GOP 32 configuration.

The GOP-based temporal filter was first proposed for HM in JCTVC-AI0023, and later for VTM in JVET-O0549 and included into VTM-7.0 and HM-16.21. It was discovered by software coordinators that the performance in VTM-10.0 had dropped since VTM-8.0. This was due to an integration bug in another tool.  The bug was located and a ticket on VTM was filed (#1400) and later a fix was submitted in a merge request https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/merge_requests/1902.

Since the term “Motion compensated temporal filtering” (MCTF) is typically used for a broad range of filters in the industry, we use the name “GOP-based temporal filtering” or GBTF in this document.

In JVET-T0063 it was proposed to use GOP hierarchy of 32 pictures for CTC for SDR and HDR. In this document we provide performance for GBTF on top of that. Since GBTF did not work properly in VTM-10.0 due to the bug mentioned above, the comparison is made with the bugfixed version of VTM-10.0.

# Short description of temporal filtering

The temporal filter process includes a hierarchical motion estimation scheme to produce motion compensated reference pictures. Two reference pictures that precede the current picture in output order and additionally, for random access, two pictures that follow the current picture in output order are motion compensated relative to the current picture. A bilateral filter is then applied to calculate the resulting filtered samples. The weights in the bilateral filter depend on the frame positions in the GOP structure, the temporal sample difference and on the QP value. The filtered pictures are used for encoding and the original unfiltered pictures are used for BD-rate calculations.

GBTF is applied to every 8th picture for random access coding and every 4th picture for low delay coding according to the current GBTF-parameters in the VTM configuration files. However, since GOP-8 has been adopted for low delay coding, a better setting may be to filter every 8th frame there as well. Therefore, we have provided results for both types of filterings for LDB below. No future frames are used for temporal filtering in low delay coding. The filter is not applied to all intra coding.

The overall filter strength, s0, is set according to the equation below for RA:

if (POC % 16 == 0)

s0 = 1.5

else if (POC % 8 == 0)

s0 = 0.95

For the LD case, s0 = 0.40

Both HM and VTM also supports non-4:2:0 content. When the filter was integrated into HM, the software was adapted to work with non-YUV-4:2:0 content and with higher bit depths, as described in JCTVC-AK0003. It was this version that was later ported to VTM, so higher bit depths and non-4:2:0 content is expected to work also for VTM.

GBTF filters both luma and chroma, but the filter strengths are different for luma and chroma.

## Behavior at low QPs

The filter turns itself off when the input QP<17. This also means that it is turned off when the input QP is negative. The only information that the filter needs is the input QP and the GOP structure. This means that the filter can run completely off-line (disconnected from the rest of the encoder) if necessary.

# Regarding sequential and parallel mode of operation

When parallel simulations are performed, there are typically two approaches that are used. The first approach is to have the complete source sequence available and then have VTM to encode segments of the full length in parallel. Finally, the parallelly encoded bitstreams are merged and decoded for PSNR computation. To confirm that temporal filtering can be achieved with the same results for both sequential and parallel processing using this first approach, MD5’s for bitstreams with sequential coding for class D were compared with the corresponding merged parallelly encoded bitstreams. The comparison shows a match.

The second approach is to instead split the source sequence into segments before coding. After coding and decoding each segment, the .yuv segments are merged. In some environments, this second approach may be preferable. For that to work properly the temporal filter requires slightly overlapping segments, two frames before the first frame to be encoded and two frames after the last frame. This can be obtained with the current implementation of GBTF in VTM by providing overlapping segments to the encoder and instruct it to start encoding at the third frame and end encoding two frames before the end. The proponents do not use this second approach for parallel encoding, so this has not been tested, but is expected to work.

# Results

The objective performance (BDR) for the proposal is shown below in comparison to VTM-10.0 [3] using SDR CTC [4] and HDR CTC [5] with the modification of setting the intra period to be a multiple of 32 and using a random access hierarchy of a GOP of 32. The objective results show a nice gain in coding efficiency. Encoding and decoding time is measured on simulations run on a cluster with machines of similar capability but is not reliable.

## SDR Random access

|  | Over VTM-10.0 with gop 32 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2,71% | -7,23% | -6,10% | 91% | 96% |
| Class A2 | -5,20% | -10,16% | -9,90% | 93% | 103% |
| Class B | -6,32% | -8,29% | -8,46% | 93% | 92% |
| Class C | -1,64% | -3,80% | -3,54% | 99% | 107% |
| Class E |  |  |  |  |  |
| Overall | -4,13% | -7,25% | -6,96% | 94% | 99% |
| Class D | 0,72% | -3,45% | -3,35% | 99% | 104% |
| Class F | -0,15% | -0,76% | -0,93% | 103% | 107% |

## HDR Random access

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 with gop 32 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -9,39% | -7,27% | -1,52% | -0,74% | -0,07% | -1,71% | -1,04% | -0,29% | 90% | 92% |
| Class H2 |  |  |  |  |  | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | -9,39% | -7,27% | -1,52% | -0,74% | -0,07% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Additional results for SDR low-delay B

|  | Over VTM-10.0, filter every 4th frame |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,78% | -1,48% | -2,18% | 100% | 112% |
| Class C | 0,47% | 0,20% | -0,10% | 94% | 100% |
| Class E | -1,19% | -1,35% | -1,64% | 100% | 110% |
| Overall | -0,46% | -0,89% | -1,35% | 98% | 107% |
| Class D | 1,16% | 1,16% | -0,18% | 103% | 105% |
| Class F | 0,45% | 0,30% | 0,71% | 103% | 114% |

|  | Over VTM-10.0, filter every 8th frame |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1,11% | -1,32% | -2,05% | 101% | 110% |
| Class C | 0,44% | 0,29% | 0,16% | 100% | 104% |
| Class E | -1,09% | -1,14% | -1,78% | 100% | 109% |
| Overall | -0,59% | -0,74% | -1,25% | 100% | 108% |
| Class D | 0,97% | 1,16% | 0,08% | 101% | 105% |
| Class F | 0,30% | -0,24% | 0,45% | 103% | 113% |

## Viewing of particular encoded sequences

At the October JVET meeting, there were some questions about how the temporal filtering may be affected by chroma leakage that is sometimes visible in HDR sequences. In particular, it was raised that the Market3_1920x1080p_50_10b_pq_709_ct2020_420_rev1 sequence should be scrutinized, especially in the awning and price tag areas. We had a look at this sequence and could not find anything out of the ordinary.

Another concern that was raised at the meeting was whether there could be artifacts at scene cuts. Already at the meeting it was noted that since the bilateral filter reduces the contribution from sufficiently different intensities to zero, this is unlikely to be a big problem. We had a look at the sequence MeridianHDR2_3840x2160p_60_P3PQ_YCbCr420_10b which contains a scene cut and could not find any issues in the surrounding images.

A similar concern is potential artifacts during fades. Here we had a look at SlideShow_1280x720_20 , which contains some kind of fades. Nothing out of the ordinary was found when looking at this sequence.

It is of course possible that the filter may have problems with other sequences containing scene cuts, fades, etc even though it did not run into problems with these particular ones.

# References

[1] P. Wennersten, R. Sjöberg, J. Enhorn, “AHG10: Encoder-only GOP-based temporal filter,” Joint Video Experts Team, doc. JVET-O0549, version 2, Gothenburg, July 2019.

[2] K. Andersson, J. Enhorn, R. Sjöberg, J. Ström, L. Litwic, P. Wennersten, “Suggestion to upgrading random access configuration in CTC with GOP 32 configuration”, JVET-T0063, Teleconference, October 2020.

[3] VTM-10.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-10.0

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, JVET-N1010, Geneva, Switzerland, March 2019.

[5] A. Segall, E. François, W. Husak, S. Iwamura and D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-P2011, Geneva, Switzerland, October 2019.

[6] K. Sühring, B. Li, K. Sharman, V. Seregin, G. Tech, A. Tourapis, “JCT-VC AHG report: Software development and software technical evaluation (AHG3)”, JCTVC-AK0003, Geneva, Switzerland, October 2019.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).