| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0022-r2 |
| --- | --- |

| Title: | CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomonori Hashimoto Hong-Jheng Jhu Dmytro Rusanovskyy | Email: | adrian.browne@sony.com tomonori.hashimoto@sharp.co.jp jhuhong-jheng@kwai.com dmytror@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report for the Core Experiment on entropy coding for high bit depth and high bit rate coding. A total of 14 tests have been completed within the CE between the JVET T and U meetings to study and evaluate technologies related to Rice parameter derivation. Both regular residual coding (RRC) (5 tests) and transform skip residual coding (TSRC) (3 tests) have been tested. In addition 6 tests have been completed in a second stage on combinations of the previous tests.

In this report, coding performance and complexity are reported and analyzed. In particular, test results against VTM-11.0rc1 anchors are provided to show the coding efficiency and complexity trade-off of each tool. Where appropriate results are also provided for lossless conditions. Crosschecking results for the performed tests are submitted as separate documents and their summaries are integrated in this contribution.

# Test specification

## CE-1 (RRC)

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| CE-1.1 | Dmytro Rusanovskyy (Qualcomm) | JVET-U0064 | On the Rice parameter derivation for high bit-depth coding | Adrian Browne (Sony) |
| CE-1.2 | Dmytro Rusanovskyy (Qualcomm) | JVET-U0064 | On the Rice parameter derivation for high bit-depth coding | Adrian Browne (Sony) |
| CE-1.3 | Tomonori Hashimoto (Sharp) | JVET-U0050 | Rice parameter derivation by using a formula for RRC | Hong-Jheng Jhu (Kwai) |
| CE-1.4 | Adrian Browne (Sony) | JVET-U0057 | Modification of Rice parameter selection for high bit depths using the value of previously coded coefficients within the slice. | Dmytro Rusanovskyy (Qualcomm) |
| CE-1.5 | Adrian Browne (Sony) | JVET-U0057 | Simplification of CE-1.4 | Dmytro Rusanovskyy (Qualcomm) |

## CE-2 (TSRC)

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| CE-2.1 | Hong-Jheng Jhu (Kwai) | JVET-U0075 | Slice based Rice parameter selection for transform skip residual coding | Tomonori Hashimoto (Sharp) |
| CE-2.2 | Adrian Browne (Sony) | JVET-U0057 | Rice parameter selection using the value of previously coded coefficients both locally (within the TB) and globally (within the slice). | Hong-Jheng Jhu (Kwai) |
| CE-2.3 | Adrian Browne (Sony) | JVET-U0057 | Rice parameter selection using the value of previously coded coefficients within the slice. Simplification of CE-2.2 | Hong-Jheng Jhu (Kwai) |

## CE-3 (Combinations)

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| CE-3.1 | Tomonori Hashimoto (Sharp) | JVET-U0050 | Combination of CE-1.3 and CE-2.1 | Kenji Kondo (Sony) |
| CE-3.2 | Adrian Browne (Sony) | JVET-U0058 | Combination of CE-1.5 and CE-2.1 | Tomonori Hashimoto (Sharp) |
| CE-3.3 | Adrian Browne (Sony) | JVET-U0059 | Combination of CE-1.5 and CE-2.3 | Dmytro Rusanovskyy (Qualcomm) |
| CE-3.4 | Dmytro Rusanovskyy (Qualcomm) | JVET-U0065 | Combination of CE-1.12 and CE-2.1 | Adrian Browne (Sony) |
| CE-3.5 | Dmytro Rusanovskyy (Qualcomm) | JVET-U0066 | Combination of CE-1.2 and CE-1.4/1.5 | Hong-Jheng Jhu (Kwai) |
| CE-3.6 | Dmytro Rusanovskyy (Qualcomm) | JVET-U0066 | Combination of CE-1.3 and CE-3.5 | Hong-Jheng Jhu (Kwai) |

# Test descriptions

## CE-1.1: Method of JVET-T0105 without locally adaptivity

In this test, method proposed in JVET-T0105 with globally derived adjusment for rice parameter derivation is being evaluated. To extend the range of the Rice parameters, it is proposed to scale values locSumAbs with a right shift, if locSumAbs exceed certain threshold, such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with shift value if scaling process was taken a place for a given locSumAbs value.

Adjustment parameters are determined from signalled syntax elemments, without local adaptation within a block (no dependency on locSumAbs). Two subtests were planned:

- Test CE-1.1.a: SPS-level control information is used to derive adjustment parameters.

- Test CE-1.1.b: Syntax elements controlling adjustment parameters are signalled at the sub-SPS levels, e.g. at slice level.

During CE development, CE-1.1.a test was omitted. Only sub-test with slice level signaling of the derivation parameter was conducted. Detailed description of this method and specification text are provided in JVET-U0064.

## CE-1.2: Method of JVET-T0105 with local adaptivity

In this test, method proposed in JVET-T0105 with locally adaptive adjusment for Rice parameter derivation is being evaluated. Similarly to the Test CE-1.1, to extend the range of the Rice parameters, it is proposed to scale values locSumAbs with a shift, if locSumAbs exceed certain threshold, such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with shift value if scaling process was taken a place for a given locSumAbs value. A shift parameter for locSumAbs scaling and cRiceParam increment is derived at the encoder side and signalled at the slice header. Motivated by the history usage of CE-1.4 and CE-1.5, additional sub-test was conducted (CE-1.2.b) targeting improving accuracy of the Rice derivation for coeffitients with no accract template information available. Detailed description of this method and specification text are provided in JVET-U0064.

## CE-1.3: JVET-U0050

The proposal of JVET-U0050 introduces a modification to the VVC Rice parameters derivation method for regular residual coding (RRC). It is proposed to use a formula instead of conventional look-up table. Specifically, the rice parameter value is predicted based on an adjusted value of the locSumAbs using linear prediction with log2 operation. The linear prediction calicurate floorLog2(a * locSumAbs + b) + c. The linear prediction parameter (a, b and c) depend on syntax (i.e. abs_reminder or dec_abs_level). An enablied flag for the proposed method is signalled in SPS. This flag doesn’t affect the process if bitdepth is equal or less than 10.

## CE-1.4: RRC technique in JVET-U0057

The RRC component of the method previously proposed in JVET-T0072. For this experiment the TSRC component of the modification described in JVET-T0072 is disabled and standard VVC TSRC is used.

The implementation has been improved from that in JVET-T0072 by computing the parameters for updating counters and deriving Rice parameters at the point the counters are updated. In addition, the counters and associated parameters are stored within the structure used for CABAC contexts (as proposed but not implemented in JVET-T0072). Finally a fix has been implemented to automatically adjust the Rice parameter selected based on whether a parity bit has been coded.

## CE-1.5: RRC simplification in JVET-U0057

The RRC component of the simplification previously proposed in JVET-T0072. For this experiment the TSRC component of the modification described in JVET-T0072 is disabled and standard VVC TSRC is used.

This experiment simplifies CE-1.4 as the parameters for updating counters and deriving Rice parameters are computed at the end of each transform block. In addition, the counters are only updated when the coefficient being coded is on the leading diagonal of the transform block.

## CE-2.1: TSRC technique in JVET-U0075

Method proposed in JVET-U0075, it is proposed to explicitly signal the Rice parameter for each slice to indicate the Rice parameter for the binary codewords of abs_remainder. In the proposed method, one control flag is signaled in sequence parameter set to indicate the signaling of Rice parameter for the transform skip blocks is enabled or disabled. When the control flag is signaled as enabled, one syntax element is further signaled for each transform skip slice to indicate the Rice parameter of that slice. When the control flag is signaled as disabled (e.g. set equal to “0”), no further syntax element is signaled at lower level to indicate the Rice parameter for the transform skip slice and a default Rice parameter (e.g. 1) is used for all the transform skip slice. Detailed description of this method and specification text are provided in JVET-U0075.

## CE-2.2: TSRC technique in JVET-U0057

The TSRC component of the method previously proposed in JVET-T0072. For this experiment the RRC component of the modification described in JVET-T0072 is disabled and standard VVC RRC is used.

The implementation has been improved from that in JVET-T0072 by computing the parameters for updating counters and deriving Rice parameters at the point the counters are updated. In addition, the counters and associated parameters are stored within the structure used for CABAC contexts (as proposed but not implemented in JVET-T0072). Finally a fix has been implemented to automatically adjust the Rice parameter selected based on whether a parity bit has been coded.

## CE-2.3: TSRC simplification in JVET-U0057

The TSRC component of the simplification previously proposed in JVET-T0072. In addition, this experiment replaces the usage of locSumAbs and g_auiGoRiceParsCoeff with an offset derived from the value of the selected counter. For this experiment the RRC component of the modification described in JVET-T0072 is disabled and standard VVC RRC is used.

This experiment simplifies CE-1.4 as the parameters for updating counters and deriving Rice parameters are computed at the end of each transform block. In addition, the counters are only updated when the coefficient being coded is on the leading diagonal of the transform block.

## CE-3.1: Combination of CE-1.3 and CE-2.1

This experiment combines the RRC coding modifications from CE-1.3 with the TSRC coding modifications in CE-2.1 and is described in JVET-U0050.

## CE-3.2: Combination of CE-1.5 and CE-2.1

This experiment combines the RRC coding modifications from CE-1.5 with the TSRC coding modifications in CE-2.1 and is described in JVET-U0058.

## CE-3.3: Combination of CE-1.5 and CE-2.3

This experiment combines the RRC coding modifications from CE-1.5 with the TSRC coding modifications in CE-2.3 and is described in JVET-U0059.

## CE-3.4: Combination of CE-1.2 and CE-2.1

This experiment will blend the RRC coding modifications from CE-1.2.a with the TSRC coding modifications in CE-2.1. Detailed description of this method and specification text are provided in JVET-U0065.

## CE-3.5: Combination of CE-1.2 and CE-1.4/1.5

This experiment will investigate integration of the classification ellements of CE-1.4/1.5 with rice parameter inheritance mechanism and the Rice parameter derivation of the CE-1.2.b. Detailed description of this method and specification text are provided in JVET-U0066.

## CE-3.6: Combination of CE-1.3 and CE-3.5

This experiment will investigate integration of Rice parameter derivation method of CE1.3 with with Rice parameter inheritance mechanism of the CE1.2.b modified with classification ellements of CE-1.4/1.5. Detailed description of this method and specification text are provided in JVET-U0066.

# Simulation results

Tables 1 -3 show simulation results conducted with CTC for HBD/HBR lossy coding(JVET-T2018)

Table 1. Simulation results for AI configuration

| AI | HDR PQ |  |  |  |  | HDR HLG |  |  |  |  | SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Enc | Dec | Y | U | V | Enc | Dec | G | B | R | Enc | Dec |
| CE-1.1 | -0.60% | -0.67% | -0.67% | 82% | 90% | -0.22% | -0.24% | -0.24% | 83% | 87% | -13.86% | -13.23% | -13.23% | 90% | 83% |
| CE-1.2a | -0.76% | -0.79% | -0.79% | 92% | 83% | -0.31% | -0.31% | -0.32% | 90% | 82% | -14.54% | -13.82% | -13.82% | 97% | 91% |
| CE-1.2b | -0.76% | -0.79% | -0.79% | 92% | 83% | -0.31% | -0.31% | -0.32% | 90% | 82% | -14.79% | -13.97% | -13.98% | 98% | 91% |
| CE-1.3 | -0.92% | -0.91% | -0.90% | 100% | 96% | -0.50% | -0.41% | -0.41% | 102% | 99% | -14.33% | -13.76% | -13.78% | 102% | 87% |
| CE-1.4 | -0.94% | -0.89% | -0.91% | 104% | 100% | -0.60% | -0.45% | -0.45% | 103% | 101% | -16.00% | -14.70% | -14.71% | 107% | 106% |
| CE-1.5 | -0.92% | -0.85% | -0.85% | 102% | 100% | -0.63% | -0.47% | -0.46% | 102% | 102% | -15.96% | -14.57% | -14.60% | 107% | 110% |
| CE-2.1 | -0.12% | -0.09% | -0.08% | 99% | 97% | -0.10% | -0.05% | -0.05% | 100% | 100% | -11.84% | -11.48% | -11.43% | 101% | 96% |
| CE-2.2 | -0.18% | -0.20% | -0.19% | 102% | 102% | -0.07% | -0.05% | -0.05% | 101% | 101% | -11.99% | -11.58% | -11.53% | 106% | 104% |
| CE-2.3 | -0.15% | -0.14% | -0.13% | 101% | 100% | -0.17% | -0.12% | -0.12% | 100% | 100% | -8.74% | -8.30% | -8.26% | 103% | 99% |
| CE-3.1 | -0.92% | -0.87% | -0.86% | 108% | 103% | -0.54% | -0.43% | -0.43% | 104% | 100% | -14.62% | -13.95% | -13.94% | 106% | 90% |
| CE-3.2 | -0.92% | -0.81% | -0.81% | 103% | 100% | -0.63% | -0.47% | -0.46% | 102% | 101% | -15.54% | -14.39% | -14.37% | 105% | 106% |
| CE-3.3 | -0.92% | -0.84% | -0.84% | 102% | 100% | -0.67% | -0.51% | -0.51% | 101% | 102% | -15.96% | -14.60% | -14.61% | 107% | 110% |
| CE-3.4 | -0.75% | -0.74% | -0.75% | 88% | 99% | -0.36% | -0.33% | -0.33% | 85% | 96% | -14.66% | -13.95% | -13.93% | 95% | 89% |
| CE-3.5 | -0.95% | -0.87% | -0.88% | 105% | 104% | -0.46% | -0.32% | -0.32% | 104% | 102% | -15.88% | -14.77% | -14.77% | 98% | 97% |
| CE-3.5.b | -0.99% | -0.90% | -0.92% | 104% | 107% | -0.55% | -0.41% | -0.41% | 105% | 105% | -15.86% | -14.74% | -14.74% | 113% | 102% |
| CE-3.6 | -1.05% | -0.92% | -0.91% | 103% | 101% | -0.56% | -0.37% | -0.37% | 103% | 101% | -15.76% | -14.79% | -14.80% | 115% | 96% |

Table 2. Simulation results for LDB configuration

| LDB | HDR PQ |  |  |  |  | HDR HLG |  |  |  |  | SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Enc | Dec | Y | U | V | Enc | Dec | G | B | R | Enc | Dec |
| CE-1.1 | -0.05% | -0.06% | -0.07% | 99% | 95% | -0.03% | -0.04% | -0.04% | 94% | 101% | -8.43% | -7.79% | -7.92% | 101% | 94% |
| CE-1.2a | -0.03% | -0.04% | -0.04% | 89% | 89% | 0.03% | 0.02% | 0.03% | 99% | 99% | -8.85% | -8.37% | -8.42% | 103% | 100% |
| CE-1.2b | -0.03% | -0.04% | -0.04% | 89% | 89% | 0.03% | 0.02% | 0.03% | 99% | 99% | -8.94% | -8.57% | -8.58% | 114% | 106% |
| CE-1.3 | 0.07% | 0.08% | 0.09% | 89% | 86% | -0.03% | 0.01% | 0.01% | 101% | 99% | -8.86% | -8.15% | -8.31% | 104% | 94% |
| CE-1.4 | -0.09% | -0.10% | -0.13% | 103% | 101% | -0.08% | -0.08% | -0.07% | 103% | 102% | -9.38% | -8.96% | -8.97% | 107% | 109% |
| CE-1.5 | -0.09% | -0.10% | -0.12% | 101% | 100% | -0.09% | -0.07% | -0.07% | 102% | 101% | -9.40% | -8.95% | -8.96% | 104% | 108% |
| CE-2.1 | 0.01% | -0.01% | 0.01% | 98% | 100% | 0.00% | -0.01% | -0.01% | 99% | 100% | -7.19% | -6.91% | -6.89% | 99% | 100% |
| CE-2.2 | -0.02% | -0.03% | -0.06% | 102% | 101% | 0.00% | -0.01% | -0.01% | 102% | 100% | -7.10% | -6.79% | -6.78% | 101% | 104% |
| CE-2.3 | -0.01% | -0.01% | -0.01% | 100% | 100% | 0.00% | -0.01% | -0.01% | 101% | 100% | -4.65% | -4.37% | -4.37% | 100% | 99% |
| CE-3.1 | 0.06% | 0.08% | 0.10% | 105% | 104% | -0.03% | 0.01% | 0.01% | 104% | 102% | -8.99% | -8.60% | -8.62% | 109% | 97% |
| CE-3.2 | -0.07% | -0.09% | -0.12% | 101% | 104% | -0.09% | -0.07% | -0.07% | 102% | 101% | -9.43% | -8.96% | -8.96% | 103% | 107% |
| CE-3.3 | -0.10% | -0.10% | -0.11% | 101% | 101% | -0.09% | -0.07% | -0.07% | 102% | 101% | -9.43% | -8.94% | -8.95% | 104% | 109% |
| CE-3.4 | -0.03% | -0.02% | -0.03% | 88% | 89% | 0.03% | 0.03% | 0.03% | 86% | 93% | -8.97% | -8.59% | -8.60% | 105% | 96% |
| CE-3.5 | -0.03% | -0.05% | -0.06% | 101% | 100% | 0.05% | 0.06% | 0.06% | 103% | 105% | -9.53% | -9.14% | -9.15% | 102% | 107% |
| CE3.5.b | -0.08% | -0.10% | -0.11% | 101% | 102% | -0.02% | 0.00% | 0.00% | 107% | 106% | -9.52% | -9.13% | -9.14% | 121% | 115% |
| CE-3.6 | 0.10% | 0.12% | 0.12% | 99% | 97% | 0.01% | 0.05% | 0.05% | 108% | 110% | -9.46% | -9.10% | -9.12% | 127% | 115% |

Table 3. Simulation results for RA configuration

| RA | HDR PQ |  |  |  |  | HDR HLG |  |  |  |  | SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Enc | Dec | Y | U | V | Enc | Dec | G | B | R | Enc | Dec |
| CE-1.1 | -0.12% | -0.14% | -0.16% | 89% | 95% |  |  |  |  |  | -7.90% | -7.98% | -8.00% | 91% | 87% |
| CE-1.2a | -0.13% | -0.14% | -0.14% | 77% | 78% | 0.00% | 0.00% | 0.00% | 96% | 103% | -8.41% | -8.49% | -8.47% | 85% | 85% |
| CE-1.2b | -0.13% | -0.14% | -0.14% | 77% | 78% | 0.00% | 0.00% | 0.00% | 96% | 103% | -8.56% | -8.62% | -8.60% | 91% | 90% |
| CE-1.3 | -0.05% | -0.01% | 0.01% | 100% | 102% | -0.04% | -0.01% | 0.00% | 109% | 108% | -8.30% | -8.33% | -8.37% | 98% | 88% |
| CE-1.4 | -0.20% | -0.19% | -0.21% | 103% | 102% | -0.09% | -0.08% | -0.07% | 103% | 101% | -9.00% | -8.99% | -8.97% | 107% | 108% |
| CE-1.5 | -0.19% | -0.17% | -0.20% | 102% | 101% | -0.10% | -0.07% | -0.07% | 101% | 101% | -9.06% | -9.01% | -8.98% | 103% | 107% |
| CE-2.1 | 0.00% | -0.01% | 0.00% | 99% | 100% | 0.00% | 0.00% | 0.00% | 100% | 101% | -6.75% | -7.04% | -6.99% | 100% | 100% |
| CE-2.2 | -0.05% | -0.04% | -0.05% | 102% | 101% | 0.00% | 0.00% | 0.00% | 101% | 101% | -6.67% | -6.95% | -6.90% | 101% | 103% |
| CE-2.3 | -0.02% | -0.02% | -0.02% | 101% | 100% | 0.00% | -0.01% | 0.00% | 100% | 100% | -4.10% | -4.37% | -4.34% | 100% | 99% |
| CE-3.1 | -0.03% | 0.00% | 0.01% | 105% | 125% | -0.04% | -0.01% | 0.00% | 100% | 89% | -8.57% | -8.57% | -8.56% | 106% | 94% |
| CE-3.2 | -0.18% | -0.17% | -0.19% | 102% | 104% | -0.10% | -0.07% | -0.07% | 102% | 100% | -9.04% | -9.00% | -8.96% | 104% | 106% |
| CE-3.3 | -0.19% | -0.18% | -0.19% | 102% | 101% | -0.09% | -0.07% | -0.07% | 102% | 101% | -9.09% | -9.00% | -8.97% | 104% | 107% |
| CE-3.4 | -0.14% | -0.12% | -0.14% | 86% | 93% | 0.00% | 0.00% | 0.01% | 105% | 109% | -8.56% | -8.62% | -8.59% | 88% | 88% |
| CE-3.5 | -0.17% | -0.16% | -0.16% | 104% | 103% | 0.02% | 0.03% | 0.03% | 110% | 113% | -9.15% | -9.18% | -9.15% | 95% | 98% |
| CE3.5.b | -0.20% | -0.18% | -0.19% | 105% | 105% |  |  |  |  |  |  |  |  |  |  |
| CE-3.6 | -0.01% | 0.03% | 0.04% | 107% | 107% |  |  |  |  |  | -9.05% | -9.08% | -9.06% | 110% | 97% |

Table 4. Simulation results for SVT RGB and AI configuration

| AI |  |  | SVT 16 RGB |  |  |  |  |  |  |  |  |  |  | SVT 12 RGB |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G | B | R | Enc | Dec | G | B | R | Enc | Dec |
| CE-1.1 |  |  | -25.10% |  |  | -23.98% |  | -23.99% |  | 98% |  | 76% |  | -2.62% |  |  | -2.48% |  | -2.48% |  | 83% |  | 89% |  |  |
| CE-1.2a |  |  | -25.96% |  |  | -24.73% |  | -24.73% |  | 95% |  | 78% |  | -3.11% |  |  | -2.91% |  | -2.91% |  | 99% |  | 103% |  |  |
| CE-1.2b |  |  | -26.47% |  |  | -25.04% |  | -25.05% |  | 97% |  | 80% |  | -3.11% |  |  | -2.91% |  | -2.91% |  | 99% |  | 103% |  |  |
| CE-1.3 |  |  | -25.45% |  |  | -24.54% |  | -24.57% |  | 103% |  | 76% |  | -3.21% |  |  | -2.99% |  | -2.99% |  | 102% |  | 98% |  |  |
| CE-1.4 |  |  | -27.78% |  |  | -25.68% |  | -25.68% |  | 107% |  | 105% |  | -4.22% |  |  | -3.72% |  | -3.73% |  | 107% |  | 108% |  |  |
| CE-1.5 |  |  | -27.67% |  |  | -25.41% |  | -25.41% |  | 109% |  | 111% |  | -4.26% |  |  | -3.74% |  | -3.75% |  | 105% |  | 108% |  |  |
| CE-3.5 |  |  | -27.55% |  |  | -25.76% |  | -25.76% |  | 102% |  | 94% |  | -4.20% |  |  | -3.78% |  | -3.78% |  | 94% |  | 101% |  |  |
| CE-3.5.b |  |  | -27.54% |  |  | -25.71% |  | -25.71% |  | 119% |  | 99% |  | -4.18% |  |  | -3.77% |  | -3.78% |  | 106% |  | 105% |  |  |
| CE-3.6 |  |  | -27.36% |  |  | -25.84% |  | -25.85% |  | 122% |  | 89% |  | -4.16% |  |  | -3.75% |  | -3.75% |  | 108% |  | 102% |  |  |
| CE-2.1 | -22.24% | -21.35% | -21.30% | 101% | 92% | -1.44% | -1.61% | -1.56% | 100% | 100% |
| CE-2.2 | -22.50% | -21.54% | -21.50% | 108% | 103% | -1.48% | -1.61% | -1.56% | 103% | 104% |
| CE-2.3 |  |  | -16.24% |  |  | -15.24% |  | -15.20% |  | 105% |  | 97% |  | -1.24% |  |  | -1.37% |  | -1.32% |  | 101% |  | 101% |  |  |
| CE-3.1 | -25.98% | -24.77% | -24.77% | 107% | 79% | -3.27% | -3.13% | -3.11% | 106% | 101% |
| CE-3.2 | -26.86% | -24.99% | -24.97% | 106% | 104% | -4.22% | -3.79% | -3.77% | 104% | 108% |
| CE-3.3 | -27.67% | -25.42% | -25.45% | 109% | 112% | -4.25% | -3.77% | -3.76% | 105% | 109% |
| CE-3.4 | -26.17% | -24.87% | -24.87% | 106% | 83% | -3.15% | -3.03% | -3.00% | 85% | 94% |

Table 5. Simulation results for SVT RGB and LDB configuration

| LDB |  | SVT 16 RGB |  |  |  |  |  |  |  |  |  |  |  |  | SVT 12 RGB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | G |  | B |  | R |  | Enc |  |  | Dec |  |  |  | G | B |  |  | R |  |  | Enc |  |  | Dec |  |  |  |
| CE-1.1 |  | -16.56% |  | -15.20% |  |  | -15.47% |  |  | 100% |  |  | 90% |  | -0.29% |  |  | -0.38% |  |  | -0.38% |  |  | 103% |  |  | 98% |  |
| CE-1.2a |  | -17.19% |  | -16.18% |  |  | -16.28% |  |  | 106% |  |  | 94% |  | -0.51% |  |  | -0.56% |  |  | -0.56% |  |  | 100% |  |  | 106% |  |
| CE-1.2b |  | -17.37% |  | -16.57% |  |  | -16.60% |  |  | 129% |  |  | 107% |  | -0.51% |  |  | -0.56% |  |  | -0.56% |  |  | 100% |  |  | 106% |  |
| CE-1.3 |  | -17.23% |  | -15.77% |  |  | -16.08% |  |  | 104% |  |  | 85% |  | -0.49% |  |  | -0.54% |  |  | -0.54% |  |  | 103% |  |  | 103% |  |
| CE-1.4 |  | -17.78% |  | -16.96% |  |  | -16.98% |  |  | 111% |  |  | 100% |  | -0.98% |  |  | -0.96% |  |  | -0.96% |  |  | 104% |  |  | 108% |  |
| CE-1.5 |  | -17.75% |  | -16.87% |  |  | -16.89% |  |  | 105% |  |  | 109% |  | -1.05% |  |  | -1.02% |  |  | -1.02% |  |  | 102% |  |  | 108% |  |
| CE-3.5 |  | -18.05% |  | -17.29% |  |  | -17.30% |  |  | 105% |  |  | 108% |  | -1.01% |  |  | -1.00% |  |  | -1.01% |  |  | 99% |  |  | 106% |  |
| CE-3.5.b |  | -18.02% |  | -17.23% |  |  | -17.25% |  |  | 126% |  |  | 115% |  | -1.02% |  |  | -1.02% |  |  | -1.03% |  |  | 116% |  |  | 115% |  |
| CE-3.6 |  | -18.15% |  | -17.43% |  |  | -17.46% |  |  | 134% |  |  | 112% |  | -0.77% |  |  | -0.77% |  |  | -0.78% |  |  | 119% |  |  | 117% |  |
| CE-2.1 |  | -13.99% |  | -13.39% |  |  |  | -13.39% |  |  | 100% |  | 99% |  |  | -0.38% |  |  | -0.42% |  |  | -0.39% |  |  | 99% |  | 100% |  |
| CE-2.2 |  | -13.82% |  | -13.22% |  |  | -13.22% |  |  | 100% |  |  | 107% |  | -0.37% |  |  | -0.36% |  |  | -0.34% |  |  | 101% |  |  | 101% |  |
| CE-2.3 |  | -9.01% |  | -8.49% |  |  | -8.51% |  |  | 98% |  |  | 98% |  | -0.28% |  |  | -0.24% |  |  | -0.23% |  |  | 101% |  |  | 101% |  |
| CE-3.1 |  | -17.42% |  | -16.60% |  |  | -16.63% |  |  | 112% |  |  | 91% |  | -0.57% |  |  | -0.61% |  |  | -0.61% |  |  | 106% |  |  | 102% |  |
| CE-3.2 |  | -17.76% |  | -16.88% |  |  | -16.89% |  |  | 106% |  |  | 108% |  | -1.09% |  |  | -1.04% |  |  | -1.03% |  |  | 101% |  |  | 107% |  |
| CE-3.3 |  | -17.76% |  | -16.88% |  |  | -16.90% |  |  | 106% |  |  | 110% |  | -1.10% |  |  | -1.00% |  |  | -1.01% |  |  | 103% |  |  | 108% |  |
| CE-3.4 |  | -17.31% |  | -16.50% |  |  | -16.52% |  |  | 112% |  |  | 97% |  | -0.63% |  |  | -0.68% |  |  | -0.67% |  |  | 97% |  |  | 96% |  |

Table 6. Simulation results for SVT RGB and RA configuration

| RA | SVT 16 RGB |  |  |  |  |  |  |  | SVT 12 RGB |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G | B | R |  | Enc |  | Dec |  | G | B |  | R |  | Enc |  | Dec |  |
| CE-1.1 | -15.36% | -15.50% |  | -15.54% |  | 90% |  | 83% | -0.44% |  | -0.45% |  | -0.45% |  | 92% |  | 91% |
| CE-1.2a | -16.12% | -16.31% |  | -16.27% |  | 90% |  | 83% | -0.69% |  | -0.67% |  | -0.67% |  | 80% |  | 86% |
| CE-1.2b | -16.43% | -16.58% |  | -16.54% |  | 101% |  | 94% | -0.69% |  | -0.67% |  | -0.67% |  | 80% |  | 86% |
| CE-1.3 | -15.98% | -16.10% |  | -16.18% |  | 92% |  | 82% | -0.61% |  | -0.56% |  | -0.57% |  | 105% |  | 95% |
| CE-1.4 | -16.86% | -16.97% |  | -16.92% |  | 110% |  | 108% | -1.15% |  | -1.01% |  | -1.02% |  | 105% |  | 107% |
| CE-1.5 | -16.86% | -16.97% |  | -16.92% |  | 110% |  | 108% | -1.15% |  | -1.01% |  | -1.02% |  | 105% |  | 107% |
| CE-3.5 | -17.13% | -17.31% |  | -17.25% |  | 100% |  | 98% | -1.16% |  | -1.04% |  | -1.05% |  | 90% |  | 97% |
| CE-3.5.b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE-3.6 | -17.21% | -17.39% |  | -17.35% |  | 115% |  | 94% | -0.90% |  | -0.76% |  | -0.77% |  | 104% |  | 100% |
| CE-2.1 | -13.07% | -13.65% |  | -13.57% |  | 101% |  | 98% | -0.44% |  | -0.43% |  | -0.40% |  | 98% |  | 101% |
| CE-2.2 | -12.90% | -13.51% |  | -13.44% |  | 101% |  | 105% | -0.45% |  | -0.39% |  | -0.37% |  | 101% |  | 101% |
| CE-2.3 | -7.90% | -8.51% |  | -8.46% |  | 99% |  | 97% | -0.30% |  | -0.23% |  | -0.23% |  | 101% |  | 100% |
| CE-3.1 | -16.44% | -16.52% |  | -16.50% |  | 106% |  | 86% | -0.70% |  | -0.63% |  | -0.62% |  | 105% |  | 101% |
| CE-3.2 | -16.83% | -16.91% |  | -16.85% |  | 106% |  | 105% | -1.26% |  | -1.08% |  | -1.08% |  | 101% |  | 106% |
| CE-3.3 | -16.91% | -16.95% |  | -16.90% |  | 106% |  | 109% | -1.28% |  | -1.04% |  | -1.05% |  | 102% |  | 106% |
| CE-3.4 | -16.34% | -16.51% |  | -16.46% |  | 90% |  | 84% | -0.78% |  | -0.73% |  | -0.72% |  | 87% |  | 92% |

# CE-3.5 Additional Results (CE-3.5.b)

Input contribution JVET-U0070 provided additional simulation results for CE-3.5 test. The proposal describes a content adaptive encoder algorithm and simplified high level syntax for design in CE-3.5. Software for the test is available in CE-3.5 GIT repository. Reported simulation results were produced by following CE tests description CTC. Cross-check for this test is available in JVET-U0113.

Tables 7 -9 show simulation results conducted with CTC for HBD/HBR lossy coding (JVET-T2018).

Table 7. Simulation results for AI configuration.

| AI | HDR PQ |  |  |  |  | HDR HLG |  |  |  |  | SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Enc | Dec | Y | U | V | Enc | Dec | R | G | B | Enc | Dec |
| CE-3.5.b | -0.99% | -0.90% | -0.92% | 104% | 107% | -0.55% | -0.41% | -0.41% | 105% | 105% | -15.86% | -14.74% | -14.74% | 113% | 102% |

Table 8. Simulation results for LDB configuration.

| LDB | HDR PQ |  |  |  |  | HDR HLG |  |  |  |  | SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Enc | Dec | Y | U | V | Enc | Dec | R | G | B | Enc | Dec |
| CE-3.5.b | -0.08% | -0.10% | -0.11% | 101% | 102% | -0.02% | 0.00% | 0.00% | 107% | 106% | -9.52% | -9.13% | -9.14% | 121% | 115% |

Table 9. Simulation results for RA configuration.

| RA | HDR PQ |  |  |  |  | HDR HLG |  |  |  |  | SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Enc | Dec | Y | U | V | Enc | Dec | R | G | B | Enc | Dec |
| CE-3.5.b | -0.20% | -0.18% | -0.19% | 105% | 105% |  |  |  |  |  |  |  |  |  |  |

# Additional results

In addition to the high bit depth CTC, supplementary tests have also been conducted for experiments targeting TSRC using the TGM RGB 4:4:4 sequences within the non-420 CTC (JVET-Q2013). These tests have been conducted using the non-420 test conditions but with RExt__HIGH_BIT_DEPTH_SUPPORT = 1 and InternalBitDepth = 12 at QPs -8, -3, 2, 7, 12.

For comparison, CE-1.4, which is the RRC method with the best coding efficiency in CE-1, is also tested using the same test conditions and compared with TSRC methods, CE-2.1, CE-2.2 and CE-2.3, and combined methods, CE-3.1, CE-3.2, CE-3.3 and CE-3.4.

Table 10. Simulation results for TGM tess sequences

| TGM 1080p | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | EncT | DecT |
| CE-1.4 | -0.48% | -0.51% | -0.48% | 104% | 101% |
| CE-2.1 | -4.89% | -4.67% | -4.72% | 100% | 99% |
| CE-2.2 | -5.52% | -5.28% | -5.32% | 102% | 100% |
| CE-2.3 | -5.02% | -4.78% | -4.84% | 101% | 99% |
| CE-3.1 | -4.71% | -4.51% | -4.54% | 100% | 97% |
| CE-3.2 | -5.05% | -4.83% | -4.87% | 103% | 99% |
| CE-3.3 | -5.15% | -4.93% | -4.97% | 103% | 100% |
| CE-3.4 | -5.07% | -4.86% | -4.90% | 102% | 99% |

# Crosschecker report

Table 11. Summary of cross-check reports

| Test | Crosschecker | Crosscheck report |
| --- | --- | --- |
| CE-1.1 | Sony | results: Cross-check results match proponent’s with neglible discrepancies at QP=-13 and below code verification: Consistent with CE description. draft text verification: further comments: Encoder / decoder run-times did not match |
| CE-1.2 | Sony | results: Three small discrepancies in SVT LDB (2 in subtest (a), 1 in subtest (b)) code verification: Consistent with CE description. draft text verification: Subtest (a) draft text provided – matches source code further comments: Encoder / decoder run-times did not match |
| CE-1.3 | Kwai | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description. draft text verification: Same as the software further comments: Encoder / decoder run-times did not match for PQ in LDB |
| CE-1.4 | Qualcomm | results: Simulation results for CTC and completed lossless (Intra and LDB) tests match to results reported by the proponent’s in term of coding performance. code verification: Consistent with CE description. draft text verification: further comments: |
| CE-1.5 | Qualcomm | results: Simulation results for CTC and completed lossless (Intra and LDB) tests match to results reported by the proponent’s in term of coding performance. code verification: Consistent with CE description. draft text verification: further comments: Reporting CTC excel provided by the proponent featured a typo for PQ classes (PSNR is reported instead of wPSNR). Summary results provided in Tables 1-3 of this document (CE report) are correct. |
| CE-2.1 | Sharp | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description. draft text verification: same as the software further comments: |
| CE-2.2 | Kwai | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description. draft text verification: Same as the software further comments: |
| CE-2.3 | Kwai | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description. draft text verification: Same as the software further comments: |
| CE-3.1 | Sony | results: Cross-check results completely match proponent’s code verification: Consistent with CE description. draft text verification: further comments: |
| CE-3.2 | Sharp | results: Partially cross-check results match proponent’s code verification: Consistent with CE description. draft text verification: further comments: |
| CE-3.3 | Qualcomm | results: Simulation results for completed QP points of CTC tests match to results reported by the proponent’s in term of coding performance. code verification: Consistent with CE description. draft text verification: further comments: |
| CE-3.4 | Sony | results: Cross-check results completely match proponent’s code verification: Consistent with CE description. draft text verification: further comments: Encoder / decoder run-times did not match |
| CE-3.5 | Kwai | results: Simulation results for completed QP points of CTC tests match to results reported by the proponent’s in term of coding performance. code verification: Consistent with CE description. draft text verification: further comments: |
| CE-3.6 | Kwai | results: Simulation results for completed QP points of CTC tests match to results reported by the proponent’s in term of coding performance. code verification: Consistent with CE description. draft text verification: further comments: Encoder / decoder run-times did not match for SVT RGB |

# Complexity analysis and memory requirements

## CE-1

Table 12.1. Summary of complexity estimates per test.

| Test | Complexity | Memory requirement |
| --- | --- | --- |
| CE-1.1 | 2 conditions, 1 shifts, 1 adds per Rice call Number of operations per 4x4 TU , condition/shift/adds/MSB find: 32/16/16, 64 in total. | 4 storages, 32 bits in ROM/RAM. |
| CE-1.2.a | 1 condition, 2 shifts, 1 add, 1 MSB find. Number of operations per 4x4 TU , condition/shift/adds/MSB find: 16/32/16/16, 80 in total. | No memory increase. |
| CE-1.2.b | 10 conditions, 31 shifts, 12 adds, 16 MSB finds, 69 in total. | 12 bits of storage for history counters. |
| CE-1.3 | 1 condition, 1 mult (1shift+1add), 2 adds, 1 Most Significant Bit (MSB) find per Rice call Number of operations per 4x4 TU , condition/shift/adds/MSB find: 16/16/48/16, 96 in total. | No memory increase. |
| CE-1.4 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (per coeff): 3 conditionals, 2 adds, 1 fixed shift For selecting a parameter set (per coeff): 2 conditional, 1 add For updating associated parameters (per coeff): 6 adds, 3 fixed shifts, 2 variable shifts Total per 4x4 TU: 352 80 conditionals, 48 variable shifts, 64 fixed shifts, 160 adds | Eight stores each comprising 1*10 bit, 2 * 24 bit, 2*6 bit unsigned values |
| CE-1.5 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (4 per block): 3 conditionals, 2 adds, 1 fixed shift For selecting a parameter set (per coeff): 2 conditionals, 1 add For updating associated parameters (3 per block): 6 adds, 3 fixed shifts, 2 variable shifts Total per 4x4 TU: 133 44 conditionals, 22 variable shifts,9 fixed shifts, 58 adds | Eight stores, each comprising a single 10 bit unsigned value. |

## CE-2

Table 12.2. Summary of complexity estimates per test.

| Test | Complexity | Memory requirement |
| --- | --- | --- |
| CE-2.1 | 1 condition and 1 add per slice. | No memory increase. |
| CE-2.2 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (per coeff): 3 conditionals, 3 adds, 1 fixed shift For updating associated parameters (per coeff): 6 adds, 3 fixed shifts, 2 variable shifts Total per 4x4 TU: 320 48 conditionals, 48 variable shifts, 64 fixed shifts, 160 adds | Two stores each comprising 1*10 bit, 2 * 24 bit, 2*6 bit unsigned values |
| CE-2.3 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (4 per block): 3 conditionals, 3 adds, 1 fixed shift For updating associated parameters (1 per block): 6 adds, 3 fixed shifts, 2 variable shifts Total per 4x4 TU: 71 12 conditionals, 18 variable shifts, 7 fixed shifts, 34 adds | Two stores, each comprising a single 10 bit unsigned value. |

## CE-3

Table 12.3. Summary of complexity estimates per test.

| Test | Complexity | Memory requirement |
| --- | --- | --- |
| CE-3.1 | 1 condition, 1 mult (1shift+1add), 2 adds, 1 Most Significant Bit (MSB) find per Rice call. Number of operations per 4x4 TU , condition/shift/adds/MSB find: 16/16/48/16, 96 in total. | No memory increase. |
| CE-3.2 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (4 per block): 3 conditionals, 2 adds, 1 fixed shift For selecting a parameter set (per coeff): 2 conditionals, 1 add For updating associated parameters (3 per block): 6 adds, 3 fixed shifts, 2 variable shifts Total per 4x4 TU: 133 44 conditionals, 22 variable shifts,9 fixed shifts, 58 adds | Eight stores each comprising 1*10 bit, 2 * 24 bit, 2*6 bit unsigned values |
| CE-3.3 | For coding a value (per coeff): 1 variable shift, 1 add For updating a counter (4 per block): 3 conditionals, 2 adds, 1 fixed shift For selecting a parameter set (per coeff): 2 conditionals, 1 add For updating associated parameters (3 per block): 6 adds, 3 fixed shifts, 2 variable shifts Total per 4x4 TU: 133 44 conditionals, 22 variable shifts,9 fixed shifts, 58 adds | Eight stores, each comprising a single 10 bit unsigned value. |
| CE-3.4 | 1 condition, 2 shift, 1 add, 1 MSB find. 16/32/16/16, total 80. | No memory increase. |
| CE-3.5 | Number of operations per 4x4 TU , condition/shift/adds/MSB find: CE-1.2.a: 16/32/16/16 History update: 1/1/2/1 History usage: 0/1/19/0 Total number: 17/34/37/17Total 105 per 4x4 TU. | 12 bits of storage for history counters. |
| CE-3.6 | Number of operations per 4x4 TU , condition/shift/adds/MSB find: CE-1.3: 16/16/48/16 History update: 1/1/2/1 History usage: 0/1/19/0 Total number: 17/18/71/17 Total 121 per 4x4 TU. | 12 bits of storage for history counters. |

# Visualization of gain vs complexity for CE tests

To visualize identified gain vs. complexity ratios, total number of operations per TU size of 4x4 estimated in Section 7, have been agreated in Table along with coding results (average over 3 color components), presented in section 3.

To visualize the results, CE tests are color marked. Example of such table for AI case, and Figures for AI/LDB and RA coding cases are shown below.

Table 13. Summary of coding gain and number of operations per 4x4 TU per test, AI case.

| AI | HDR PQ |  |  |  | HDR HLG |  |  |  | SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Average | Y | U | V | Average | R | G | B | Average | num Ops |
| CE-1.1 | -0.60% | -0.67% | -0.67% | -0.65% | -0.22% | -0.24% | -0.24% | -0.23% | -13.86% | -13.23% | -13.23% | -13.44% | 64 |
| CE-1.2a | -0.76% | -0.79% | -0.79% | -0.78% | -0.31% | -0.31% | -0.32% | -0.31% | -14.54% | -13.82% | -13.82% | -14.06% | 80 |
| CE-1.2b | -0.76% | -0.79% | -0.79% | -0.78% | -0.31% | -0.31% | -0.32% | -0.31% | -14.79% | -13.97% | -13.98% | -14.25% | 69 |
| CE-1.3 | -0.92% | -0.91% | -0.90% | -0.91% | -0.50% | -0.41% | -0.41% | -0.44% | -14.33% | -13.76% | -13.78% | -13.96% | 96 |
| CE-1.4 | -0.94% | -0.89% | -0.91% | -0.91% | -0.60% | -0.45% | -0.45% | -0.50% | -16.00% | -14.70% | -14.71% | -15.14% | 352 |
| CE-1.5 | -0.92% | -0.85% | -0.85% | -0.87% | -0.63% | -0.47% | -0.46% | -0.52% | -15.96% | -14.57% | -14.60% | -15.04% | 133 |
| CE-2.1 | -0.12% | -0.09% | -0.08% | -0.10% | -0.10% | -0.05% | -0.05% | -0.07% | -11.84% | -11.48% | -11.43% | -11.58% | 0 |
| CE-2.2 | -0.18% | -0.20% | -0.19% | -0.19% | -0.07% | -0.05% | -0.05% | -0.06% | -11.99% | -11.58% | -11.53% | -11.70% | 320 |
| CE-2.3 | -0.15% | -0.14% | -0.13% | -0.14% | -0.17% | -0.12% | -0.12% | -0.14% | -8.74% | -8.30% | -8.26% | -8.43% | 71 |
| CE-3.1 | -0.92% | -0.87% | -0.86% | -0.88% | -0.54% | -0.43% | -0.43% | -0.47% | -14.62% | -13.95% | -13.94% | -14.17% | 96 |
| CE-3.2 | -0.92% | -0.81% | -0.81% | -0.85% | -0.63% | -0.47% | -0.46% | -0.52% | -15.54% | -14.39% | -14.37% | -14.77% | 133 |
| CE-3.3 | -0.92% | -0.84% | -0.84% | -0.87% | -0.67% | -0.51% | -0.51% | -0.56% | -15.96% | -14.60% | -14.61% | -15.06% | 133 |
| CE-3.4 | -0.75% | -0.74% | -0.75% | -0.75% | -0.36% | -0.33% | -0.33% | -0.34% | -14.66% | -13.95% | -13.93% | -14.18% | 80 |
| CE-3.5 | -0.95% | -0.87% | -0.88% | -0.90% | -0.46% | -0.32% | -0.32% | -0.37% | -15.88% | -14.77% | -14.77% | -15.14% | 105 |
| CE-3.5.b | -0.99% | -0.90% | -0.92% | -0.94% | -0.55% | -0.41% | -0.41% | -0.46% | -15.86% | -14.74% | -14.74% | -15.11% | 105 |
| CE-3.6 | -1.05% | -0.92% | -0.91% | -0.96% | -0.56% | -0.37% | -0.37% | -0.43% | -15.76% | -14.79% | -14.80% | -15.12% | 121 |

## AI case:

Note: horizontal axis is clipped at 150 ops.

Note: horizontal axis is clipped at 150 ops.

Note: horizontal axis is clipped at 150 ops.

## LDB case

## RA case

# Input contributions for CE

## CE contributions

- JVET-U0050, “CE-1.3 and CE-3.1: Rice parameter derivation for high bit-depth coding”, T. Hashimoto, T. Ikai (Sharp)

- JVET-U0057, “CE-1.4, CE-1.5, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths”, A. Browne, S. Keating, K. Sharman (Sony)

- JVET-U0058, “CE-3.2: Combination of CE-1.5 and CE-2.1”, A. Browne, S. Keating, K. Sharman (Sony)

- JVET-U0059, “CE-3.3: Combination of CE-1.5 and CE-2.3”, A. Browne, S. Keating, K. Sharman (Sony)

- JVET-U0064, “CE-1.1 and CE-1.2: On the Rice parameter derivation for high bit-depth coding”, D.Rusanovskyy, L. P. Van, M. Karczewicz (Qualcomm)

- JVET-U0065, “CE-3.4: Combination of CE-1.2 and CE-2.1”, D.Rusanovskyy, L. P. Van, M. Karczewicz (Qualcomm)

- JVET-U0066, “CE-3.5 and CE-3.6: Combination of CE-1.2, CE-1.4/1.5 and CE-1.3”, D.Rusanovskyy, L. P. Van, M. Karczewicz (Qualcomm)

- JVET-U0075, “CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang (Kwai Inc.)

- JVET-T2018, “Common test conditions for high bit depth and high bit rate video coding”, A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu.

- JVET-U0070, “CE Related: On signalling and encoder optimization for Rice parameter derivation”, D. Rusanovskyy, M. Coban, M. Karczewicz (Qualcomm).