| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0043 |
| --- | --- |

| Title: | Game video sequences proposal for the SDR HD low delay VVC verification test |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fabrice Le Léannec Gaëlle Martin-Cocher 975 Avenue des Champs Blancs, 35510 Cesson-Sévigné, France | Tel: Email: | fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

# Introduction

This contribution proposes three game video sequences for the SDR HD low delay VVC verification test.

These sequences are provided by the DERF TWITCH, and are part of the sequences considered in the 3GPP SA4 group for 5G video codecs evaluation [2].

In the following, the sequences are introduced, together with their characteristics and copyright information. Then the VTM-10.0 performances relative to HM-16.22 are provided.

# Proposed sequences

## Minecraft

The Minecraft test sequence is an extract from a famous sandbox video game developed by Mojang Studios in which players explore a blocky, procedurally-generated 3D world with infinite terrain, and may discover and extract raw materials, craft tools and items, and build structures or earthworks. Figure 1 shows a screenshot of the Minecraft test sequence provided by DERF-TWITCH.

Figure 1: Screenshot of Minecraft test sequence

## CS:GO

The CS:GO test sequence from Twitch is an extract from a multiplayer first-person shooter video game developed by Valve and Hidden Path Entertainment where two opposing teams, known as the Terrorists and the Counter-Terrorists, compete in game modes to complete objectives, such as securing a location to plant or defuse a bomb and rescuing or guarding hostages. Figure 2 shows two screenshots of the CS:GO test sequence provided by DERF-TWITCH.

Figure 2: Screenshots of CS:GO test sequence

## StarCraft

The StarCraft test sequence from Twitch is an extract from a famous Real Time Strategy game developed and published by Blizzard Entertainment where players enter a tournament of 5 games and chose to play one of three alien categories with distinct characteristic and abilities. Figure 3 shows a screenshot of the StarCraft test sequence provided by DERF-TWITCH.

Figure 3: Screenshot of StarCraft test sequence

# Properties, repository and copyright

Properties

The Minecraft, CS:GO and StarCraft test sequences provided by DERF-TWITCH have the following properties:

- Resolution: 1920 x1080

- Scan: Progressive

- Frame rate: 60 fps

- Bit depth: 8-bit

- Length: 3600 frames (60s)

- Color format: 4:2:0

- Colour Components: Y’CbCr

- Colour space: ITU-R BT.709

Repository

The proposed sequence can be downloaded in y4m format at following URL: https://media.xiph.org/video/derf/twitch/y4m/

and can be converted into YUV format with ffmpeg by using the following exemplary command line:

ffmpeg -i input.y4m output.yuv

Copyright information

The copyright under which the video sequences from DERF-TWITCH is made available on that link:

https://media.xiph.org/video/derf/twitch/copyright.txt

which states that “These video game recordings are placed in the public domain. The recordings may contain copyrighted work which can only be used in compliance with fair use.”

# Rate distortion performances

The following Table 1 provides the comparative BD-PSNR performances of the VTM-10.0 and HM-16.22 in low delay B configuration. For each sequence, the first 600 frames are encoded with QP 22, 27, 32, and 37.

Table 1: VTM-10 rate Distortion performances compared to HM-16.22, on the three proposed sequences.

# References

[1] M. Wien and V. Baroncini, “AHG4: Agenda and report of the AHG meeting on the SDR HD Verification Tests on 2020-09-03,” JVET-T0045, Sep. 2020.

[2] 3GPP TR 26.955 V0.4.0 (2020-11), 3rd Generation Partnership Project; Technical Specification Group TSG SA; 5G Video Codec Characteristics (Release 17), Annex C https://www.3gpp.org/ftp//Specs/archive/26_series/26.955/26955-040.zip

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.