| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0045-v1 |
| --- | --- |

| Title: | AHG9: Picture output suppression SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian Jacob Ström | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes a new picture output suppression SEI message to be included in the VSEI specification and to be supported by VVC version 2.

It is claimed that the proposed SEI message is useful for providing spatial and SNR scalability support for VVC decoders that support Main 10 (4:2:0 or 4:4:4) but do not support any of the Multilayer Main 10 (4:2:0 or 4:4:4) profiles. Spatial and SNR scalability in this proposal are supported by treating temporal sublayers as scalability layers and using the picture output suppression SEI message to indicate which temporal sublayers, or which specific pictures, to not output.

The proposed picture output suppression SEI message syntax looks as follows:

| picture_output_suppression( payloadSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < MaxNumTemporalSublayers; i++ ) |  |
| pos_temporal_sublayer_no_output_flag[ i ] | u(1) |
| pos_persistence_flag | u(1) |
| } |  |

pos_temporal_sublayer_no_output_flag[ i ] equal to 1 specifies that pictures in the current layer with TemporalId equal to i are not output. pos_temporal_sublayer_no_output_flag[ i ] equal to 0 specifies that pictures in the current layer with TemporalId equal to i may or may not be output.

The persistence of the SEI message is controlled by the pos_persistence_flag syntax element. When pos_persistence_flag is equal to 0 the SEI message is applied to the current decoded picture only. When pos_persistence_flag is equal to 1 the SEI message applies to the current decoded picture and subsequent pictures in the CVLS until a new picture output suppression SEI message is received. This is in line with many other SEI messages.

It is acknowledged that for some typical use cases, the frame rate of the proposed solution is effectively doubled. It is claimed however, that in these use cases the required level does not need to be higher compared to using the Multilayer Main 10 profile.

# Introduction

VVC version 1 defines the Main 10 (4:2:0 and 4:4:4) and the Multilayer Main 10 (4:2:0 and 4:4:4) video profiles. While the Multilayer Main 10 profiles support the full set of multilayer features in VVC including scalability layers, the Main 10 profiles only support having a single layer per bitstream.

A new feature in VVC, compared to previous standards, is the reference picture resampling (RPR) which allows the size of the reference pictures and the output pictures to vary for pictures in the same layer. RPR is supported by all VVC video profiles.

The main idea in this contribution is that for some typical use cases, scalability support can be provided using RPR in combination with information sent in an SEI message, without the decoder having to support any of the Multilayer Main 10 profiles. We believe that bringing scalability support to a wider range of decoders is advantageous, even if it is of a limited kind.

# Proposal

We propose to include an output suppression SEI message for the VSEI specification and to support it in VVC version 2.

By using the proposed output suppression SEI message it is possible to support spatial and SNR scalability use cases with the VVC Main 10 video profiles (4:2:0 or 4:4:4) that do not support any of the Multilayer Main 10 profiles. For spatial scalability, this is done by using the reference picture resampling (RPR) feature of VVC in combination with treating temporal sublayers as layers. Basically, pictures that would be in different scalability layers are instead put in different temporal sublayers. All pictures of one temporal sublayer have the same spatial output resolution and inter prediction across temporal sublayers is done using RPR. For SNR scalability, using RPR would not be necessary. SNR scalability is supported by using different fidelity in different temporal sublayers.

The proposed solution can support up to 7 spatial/SNR scalability layers for VVC. The solution would also support a combination of temporal sublayers and spatial/SNR scalability layers, such as 2 spatial/SNR layers with 3 temporal sublayers each. Independent layers can also be supported.

A simple example of the solution is shown below where two temporal sublayers are used to represent two layers for spatial scalability, a base layer T0 with TemporalId = 0 and an enhancement layer T1 with TemporalId = 1. The pictures in T1 are encoded with a higher resolution, e.g. 4K, than the resolution of the pictures in T0, e.g. 1080p, and the pictures in T1 use RPR for referencing pictures in T0. To extract and output the pictures of the base layer T0, the bitstream is pruned by removing all pictures in the temporal sublayer T1. Thus, extraction of the base layer T0 can be done already in VVC Main 10 without any SEI messages.

Figure 1: The base layer T0 is extracted and output by pruning the bitstream and removing the enhancement layer T1 before decoding.

To only output the pictures of the higher resolution enhancement layer T1, the picture output suppression SEI message is sent with an indication to not output the pictures of temporal sublayer T0. If the decoder would not get the indication to suppress the output for T0 it would output all pictures in temporal sublayers T0 and T1. Thus, a decoder not capable of understanding the SEI message would output every second picture in full resolution, and every second picture in a lower resolution. This would most often not be a desired output, and this limits the applicability of this SEI message to cases where it is known that the decoder can process the SEI message. This is similar in spirit to other types of SEI messages, such as the SEI message for cube maps, which is only used in cases when it is known that the decoder can process the SEI message.

Figure 2: The picture output suppression SEI message is used to indicate that only enhancement layer T1 is to be output after decoding.

## 2.1 Proposed syntax and semantics for VVC

Proposed syntax and semantics with added text in yellow on top of VVC (JVET-T2001-v2) are shown below.

## D.2	General SEI payload

## D.2.1	General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| … |  |
| else if( payloadType = = 204 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sample_aspect_ratio_info( payloadSize ) |  |
| else if( payloadType = = 205 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| picture_output_suppression( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| … |  |
| if( more_data_in_payload( ) ) { |  |
| … |  |
| } |  |
| } |  |

…

### D.9.6	Use of the frame-field information SEI message

For purposes of interpretation of the frame-field information SEI message, the variable FixedPicRateWithinCvsFlag is set equal to fixed_pic_rate_within_cvs_flag[ Htid ].

…

### D.9.7	Use of the picture output suppression SEI message

For purposes of interpretation of the picture output suppression SEI message, the variable MaxNumTemporalSublayers is set equal to 7.

## Proposed syntax and semantics for VSEI

Proposed syntax and semantics on top of VSEI (JVET-S2007-v7) are shown below:

8.18	Picture output suppression SEI message

8.18.1	Picture output suppression SEI message syntax

| picture_output_suppression( payloadSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < MaxNumTemporalSublayers; i++ ) |  |
| pos_temporal_sublayer_no_output_flag[ i ] | u(1) |
| pos_persistence_flag | u(1) |
| } |  |

8.18.2	Picture output suppression SEI message semantics

The picture output suppression SEI message may be used for identifying pictures that are recommended to not output or be displayed.

NOTE 1 – For example, an encoder may use the picture output suppression SEI message to hint to a decoder that a single picture or a set of pictures may be discarded after output. Discarding pictures is optional and does not affect the decoding process specified in this Specification.

pos_temporal_sublayer_no_output_flag[ i ] equal to 1 specifies that pictures in the current layer with TemporalId equal to i are not output. pos_temporal_sublayer_no_output_flag[ i ] equal to 0 specifies that pictures in the current layer with TemporalId equal to i may or may not be output. When not present, temporal_sublayer_no_output_flag[ i ] is inferred to be equal to 0.

pos_persistence_flag specifies the persistence of the picture output suppression SEI message for the current layer.

pos_persistence_flag equal to 0 specifies that the picture output suppression SEI message applies to the current decoded picture only.

pos_persistence_flag equal to 1 specifies that the picture output suppression SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a picture output suppression SEI message is output that follows the current picture in output order.

NOTE 2 – A previous persistant picture output suppression SEI message can be cancelled by a new SEI message where all pos_temporal_sublayer_no_output_flag[ i ] syntax elements are set equal to 0.

# Analysis of required level

In this section the required level is studied for the Multilayer Main 10 profile and the proposed SEI message solution. Table 1 shows the maximum picture rates, max luma picture size in samples and max luma sample rate at the Main tier for levels 4.1 to level 6.2 for some example picture sizes.

Table 1	Maximum picture rates (pictures per second) at the Main tier, level 4.1 to level 6.2 for some example picture sizes when MinCbSizeY is equal to 64. The values are taken from Tables 138 and 139 in VVC.

| Level: |  |  |  | 4.1 | 5.0 | 5.1 | 5.2 | 6.0 | 6.1 | 6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max luma picture size (samples): |  |  |  | 2 228 224 | 8 912 896 | 8 912 896 | 8 912 896 | 35 651 584 | 35 651 584 | 35 651 584 |
| Max luma sample rate (samples/sec) |  |  |  | 133 693 440 | 267 386 880 | 534 773 760 | 1 069 547 520 | 1 069 547 520 | 2 139 095 040 | 4 278 190 080 |
| Format nickname | Luma width | Luma height | Luma picture size |  |  |  |  |  |  |  |
| … | … | … | … | … | … | … | … | … | … | … |
| 1080 HD | 1 920 | 1 080 | 2 088 960 | 64.0 | 128.0 | 256.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 2K×1K | 2 048 | 1 024 | 2 097 152 | 63.7 | 127.5 | 255.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 2K×1080 | 2 048 | 1 080 | 2 228 224 | 60.0 | 120.0 | 240.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 4XGA | 2 048 | 1 536 | 3 145 728 | - | 85.0 | 170.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 16VGA | 2 560 | 1 920 | 4 915 200 | - | 54.4 | 108.8 | 217.6 | 217.6 | 300.0 | 300.0 |
| 3616×1536 (2.35:1) | 3 616 | 1 536 | 5 603 328 | - | 47.7 | 95.4 | 190.8 | 190.8 | 300.0 | 300.0 |
| 3672×1536 (2.39:1) | 3 680 | 1 536 | 5 701 632 | - | 46.8 | 93.7 | 187.5 | 187.5 | 300.0 | 300.0 |
| 3840×2160 (4*HD) | 3 840 | 2 160 | 8 355 840 | - | 32.0 | 64.0 | 128.0 | 128.0 | 256.0 | 300.0 |
| 4K×2K | 4 096 | 2 048 | 8 388 608 | - | 31.8 | 63.7 | 127.5 | 127.5 | 255.0 | 300.0 |
| 4096×2160 | 4 096 | 2 160 | 8 912 896 | - | 30.0 | 60.0 | 120.0 | 120.0 | 240.0 | 300.0 |
| 4096×2304 (16:9) | 4 096 | 2 304 | 9 437 184 | - | - | - | - | 113.3 | 226.6 | 300.0 |
| 4096×3072 | 4 096 | 3 072 | 12 582 912 | - | - | - | - | 85.0 | 170.0 | 300.0 |
| 7680×4320 | 7 680 | 4 320 | 33 423 360 | - | - | - | - | 32.0 | 64.0 | 128.0 |
| 8192×4096 | 8 192 | 4 096 | 33 554 432 | - | - | - | - | 31.8 | 63.7 | 127.5 |
| 8192×4320 | 8 192 | 4 320 | 35 651 584 |  | - | - | - | 30.0 | 60.0 | 120.0 |

Table 2 and Table 3 show the minimum required levels needed for the Multilayer Main 10 profile and the proposed SEI message solution for the Main 10 profile, for two typical spatial scalability use cases. In Table 2 the comparison is for spatial scalability with a 1080p60 base layer and 4K enhancement layer and in Table 3 the comparison is for spatial scalability with a 4K base layer and 8K enhancement layer, all at 60 fps output framerate.

Note that for the proposed SEI message solution the level can be set per temporal sublayer using the PTL syntax element sublayer_level_idc[i] where i is the TemporalId of the sublayer.

From the tables it can be seen that despite the actual frame rate is doubled for the proposed SEI message solution, the required levels are the same as for the Multilayer Main 10 profile.

Table 2	Comparison of required level for spatial scalability with 1080p60 base layer and 4K enhancement layer for the multilayer main 10 profile and the proposed SEI message solution for Main 10 profile.

|  | Multilayer Main 10 profile |  |  |  | Proposed SEI message solution for Main 10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma picture size | Actual frame rate | luma sample rate (samples/sec) | Required Level | Luma picture size | Actual frame rate | luma sample rate (samples/sec) | Required level |
| Base layer (1920x1080@60) | 2,088,960 | 60 | 125,337,600 | 4.1 | 2,088,960 | 60 | 125,337,600 | 4.1 |
| Enhancement layer (4096x2160@60) | 8,912,896 | 60 | 534,773,760 | - | 8,912,896 | 60 | 534,773,760 | - |
| Base + Enh. Layers | 8,912,896* | 60 | 660,111,360 | 5.2 | 8,912,896 | 120 | 660,111,360 | 5.2 |

Table 3	Comparison of required level for spatial scalability with 4K base layer and 8K enhancement layer for the multilayer main 10 profile and the proposed SEI message solution for Main 10 profile.

|  | Multilayer Main 10 profile |  |  |  | Proposed SEI message solution for Main 10 profile |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma picture size | Actual frame rate | luma sample rate (samples/sec) | Required Level | Luma picture size | Actual frame rate | luma sample rate (samples/sec) | Required level |
| Base layer (4096x2160@60) | 8,912,896 | 60 | 534,773,760 | 5.1 | 8,912,896 | 60 | 534,773,760 | 5.1 |
| Enhancement layer (8192x4096@60) | 33,554,432 | 60 | 2,013,265,920 | - | 33,554,432 | 60 | 2,013,265,920 | - |
| Base+Enh. Layers | 33,554,432* | 60 | 2,548,039,680 | 6.2 | 33,554,432 | 120 | 2,548,039,680 | 6.2 |

* The maximum luma picture size of the two layers have been used here. Unclear whether the luma picture size should instead be summed up for the layers when the Multilayer Main 10 profile is used.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).