| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0048 |
| --- | --- |

| Title: | Evaluation of Template Matching Prediction for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Leannec Tangi Poirier Franck Galpin | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This document studies the coding efficiency of the classical coding tool of template matching. It is reported that this tool provides AI coding gain of 0.2% to 0.5%  and up to 16% for TGM class.

# Introduction

Template matching prediction (TMP) is a special intra prediction mode that copies the best prediction block from the reconstructed part of the current frame, whose L-shaped templated matches the current template. This is illustrated in Figure 1. For a predefined search range, the encoder searches for the most similar template to the current template in the reconstructed part of the current frame, and uses the corresponding block as a prediction block. The encoder then signals the usage of this mode, and the inverse operation is made at the decoder side.

Figure 1 template matching prediction

# Adapting TMP to VVC

For signaling the TMP usage, a CU-level flag “tmp_enabled_flag” is used. The modification to the CU syntax table is as follows:

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if (sps_TMP_enabled_flag && cbWidth <= MaxTMPSize && cbHeight <= MaxTMPSize) |  |
| intra_TMP_flag | ue(v) |
| else{ |  |
| if( sps_mip_enabled_flag ) |  |
| intra_mip_flag | ae(v) |
| if( intra_mip_flag ) { |  |
| intra_mip_transposed_flag[ x0 ][ y0 ] | ae(v) |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx | ae(v) |
| if( sps_isp_enabled_flag && intra_luma_ref_idx = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) && !cu_act_enabled_flag ) |  |
| intra_subpartitions_mode_flag | ae(v) |
| if( intra_subpartitions_mode_flag = = 1 ) |  |
| intra_subpartitions_split_flag | ae(v) |
| if( intra_luma_ref_idx = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| … |  |

That is, if TMP is used, then the rest of intra modes are inferred to be unused.

Further, the context-based entropy coding of the TMP flag is the same as for MIP:

| … | 0 | na | na | na | na | Na |
| --- | --- | --- | --- | --- | --- | --- |
| intra_mip_flag | ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) > 1 ) ? 3 : ( 0,1,2 (subclause 9.3.4.2.2) ) | na | na | na | na | Na |
| tmp_enabled_flag | ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) > 1 ) ? 3 : ( 0,1,2 (subclause 9.3.4.2.2) ) | na | na | na | na | na |

The search range is hard coded and multiple values are tested to study the gain-complexity trade-off.

# Experimental Results

TMP is implemented on top of VTM-8.0 and tested under the common test conditions JVET-T2010. The additional TGM class in tested. The values of tested search range are: 64, 128 and 256. The experimental results are shown in the tables below:

Table 1: Search range equal to 64

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.05% | -0.01% | 151% | 114% |
| Class A2 | -0.33% | -0.34% | -0.34% | 145% | 104% |
| Class B | -0.28% | -0.23% | -0.29% | 156% | 112% |
| Class C | -0.03% | -0.16% | -0.10% | 137% | 125% |
| Class E | -0.41% | -0.43% | -0.54% | 146% | 130% |
| Overall | -0.21% | -0.22% | -0.25% | 147% | 117% |
| Class D | 0.07% | 0.02% | -0.04% | 129% | 116% |
| Class F | -1.07% | -1.00% | -0.99% | 121% | 146% |
| Class TGM | -3.43% | -3.22% | -3.21% | 119% | 163% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.07% | -0.02% | 106% | 92% |
| Class A2 | -0.11% | -0.09% | 0.04% | 104% | 101% |
| Class B | -0.08% | 0.01% | -0.17% | 107% | 100% |
| Class C | 0.01% | 0.08% | 0.00% | 105% | 85% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.02% | -0.05% | 106% | 94% |
| Class D | 0.12% | -0.04% | -0.02% | 102% | 85% |
| Class F | -0.80% | -0.95% | -0.76% | 104% | 94% |
| Class TGM | -1.63% | -1.60% | -1.60% | 104% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.15% | 0.03% | 108% | 97% |
| Class C | -0.04% | -0.13% | 0.19% | 106% | 91% |
| Class E | -0.13% | -0.32% | 0.66% | 105% | 92% |
| Overall | -0.05% | -0.06% | 0.24% | 106% | 94% |
| Class D | 0.06% | -0.47% | 0.00% | 107% | 101% |
| Class F | -0.43% | -0.07% | -0.90% | 106% | 92% |
| Class TGM | -1.01% | -1.19% | -1.27% | 103% | 93% |

Table 2 search range equal to 128

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.03% | -0.02% | 314% | 150% |
| Class A2 | -0.68% | -0.95% | -0.90% | 308% | 181% |
| Class B | -0.53% | -0.56% | -0.64% | 336% | 211% |
| Class C | -0.12% | -0.18% | -0.06% | 259% | 193% |
| Class E | -0.66% | -0.75% | -0.88% | 305% | 223% |
| Overall | -0.41% | -0.47% | -0.49% | 304% | 192% |
| Class D | -0.01% | -0.05% | -0.08% | 203% | 162% |
| Class F | -2.70% | -2.57% | -2.53% | 202% | 415% |
| Class TGM | -8.55% | -8.13% | -8.16% | 202% | 721% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.10% | 132% | 105% |
| Class A2 | -0.29% | -0.28% | 0.06% | 128% | 105% |
| Class B | -0.17% | -0.09% | -0.14% | 138% | 115% |
| Class C | 0.00% | 0.02% | -0.07% | 136% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.08% | -0.07% | 134% | 107% |
| Class D | 0.14% | 0.22% | -0.03% | 123% | 97% |
| Class F | -2.07% | -2.08% | -2.10% | 137% | 136% |
| Class TGM | -4.37% | -4.29% | -4.27% | 135% | 174% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 0.15% | 0.04% | 134% | 101% |
| Class C | -0.06% | -0.26% | 0.25% | 131% | 99% |
| Class E | -0.12% | 0.02% | -0.59% | 119% | 88% |
| Overall | -0.08% | -0.02% | -0.04% | 129% | 97% |
| Class D | 0.06% | -0.11% | 0.54% | 121% | 101% |
| Class F | -1.07% | -0.73% | -1.29% | 128% | 107% |
| Class TGM | -2.83% | -3.10% | -3.24% | 124% | 143% |

Table 3: search range equal to 256

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.03% | -0.09% | 869% | 270% |
| Class A2 | -0.76% | -1.07% | -1.06% | 847% | 353% |
| Class B | -0.64% | -0.72% | -0.80% | 933% | 472% |
| Class C | -0.14% | -0.24% | -0.14% | 596% | 382% |
| Class E | -0.77% | -0.72% | -0.85% | 820% | 513% |
| Overall | -0.48% | -0.56% | -0.59% | 804% | 397% |
| Class D | -0.01% | -0.10% | -0.09% | 334% | 242% |
| Class F | -5.70% | -5.64% | -5.74% | 433% | 1317% |
| Class TGM | -13.55% | -13.04% | -13.06% | 491% | 3124% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | -0.05% | 224% | 128% |
| Class A2 | -0.32% | -0.22% | -0.13% | 209% | 123% |
| Class B | -0.22% | -0.18% | -0.16% | 245% | 144% |
| Class C | 0.01% | 0.10% | -0.10% | 227% | 118% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.07% | -0.12% | 228% | 129% |
| Class D | 0.07% | 0.37% | 0.09% | 159% | 111% |
| Class F | -3.78% | -3.99% | -3.90% | 247% | 265% |
| Class TGM | -7.15% | -7.01% | -6.97% | 238% | 518% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.00% | -0.02% | 224% | 109% |
| Class C | -0.09% | 0.00% | 0.07% | 210% | 112% |
| Class E | -0.05% | 0.29% | -0.53% | 169% | 93% |
| Overall | -0.05% | 0.07% | -0.12% | 204% | 105% |
| Class D | -0.02% | 0.06% | 0.68% | 150% | 107% |
| Class F | -1.64% | -1.74% | -2.59% | 189% | 157% |
| Class TGM | -4.69% | -5.15% | -5.15% | 188% | 356% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).