| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0050 |
| --- | --- |

| Title: | CE-1.3 and CE-3.1: Rice parameter derivation for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-50-5213-6337 tomonori.hashimoto@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution reports a result of CE-1.3: Rice parameter derivation for high bit-depth coding, which has been proposed in JVET-T0085 to use a formula instead of conventional look-up table in the derivation of rice parameter for RRC, is evaluated. It is reported the gain compared to the HBD anchor is

- 0.92 %, 0.50 % and 14.33 % in bdrateY in PQ, HLG and SVT for AI

- -0.07 %, 0.03 % and 8.86 % in bdrateY in PQ, HLG and SVT for LDB

- 0.03 %, 0.04 % and 8.30 % in bdrateY in PQ, HLG and SVT for RA

This contribution also reports a result of CE-3.1: Combination of CE-1.3 and CE-2.1. CE-1.3 changes the rice parameters derivation method only for RRC and can be straight forwardly combined with TSRC only method of CE-2. It is reported the gain compared to the HBD anchor is

- 0.92 %, 0.54 % and 14.62 % in bdrateY in PQ, HLG and SVT for AI

- -0.06 %, 0.03 % and 8.99 % in bdrateY in PQ, HLG and SVT for LDB

- 0.03 %, 0.04 % and 8.57 % in bdrateY in PQ, HLG and SVT for RA

It is proposed to adopt this method into the next VVC draft / VTM for the version 2.

Introduction

The proposed method is a modified rice parameters derivation method for regular residual coding (RRC) in JVET-T0085 [2]. It is proposed to use a formula instead of conventional look-up table. Specifically, the rice parameter value is predicted based on an adjusted value of the locSumAbs using linear prediction with log2 operation.

It is noted the current VVC derives the rice parameter for RRC with look-up table referring to the neighboring transform coefficients within a template.

Figure 1. Template area referred by the current coefficient

Proposed Methods

The proposed method derives cRiceParam by a simple linear formula in the case an enabling flag is equal to 1 and bitDepth is larger than 10.

cRiceParam = Max(0, floorLog2(a * locSumAbs0 + b) – 5)

Where the variable a and b are derived depending on syntax. i.e. abs_remainder (baseLevel is 4) or dec_abs_level (baseLevel is 0) as following:

- Depending on baseLevel, the variables a and b are derived as follows:

  - If baseLevel is not equal to 0, a is set equal to 4 and b is set equal to 0.

  - Otherwise (baseLevel is equal to 0), a is set equal to 9 and b is set equal to 20.

locSumAbs0 is the sum of absolute coefficient values of neighboring blocks (before subtraction considering baseLevel) as below.

locSumAbs = 0
if( xC < ( 1  <<  log2TbWidth ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC + 1 ][ yC ]
	if( xC < ( 1  <<  log2TbWidth ) − 2 )
		locSumAbs  +=  AbsLevel[ xC + 2 ][ yC ]
	if( yC < ( 1  <<  log2TbHeight ) − 1 )
		locSumAbs  +=  AbsLevel[ xC + 1 ][ yC + 1 ]	(1517)
}
if( yC < ( 1  <<  log2TbHeight ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC ][ yC + 1 ]
	if( yC < ( 1  <<  log2TbHeight ) − 2 )
		locSumAbs  +=  AbsLevel[ xC ][ yC + 2 ]
} 
if ( riceAdaptationFlag )
	locSumAbs0 = locSumAbs // use the value before subtraction
locSumAbs = Clip3( 0, 31, locSumAbs - baseLevel * 5 )

Enabled flag

An enabled flag is signalled in SPS extension. This flag doesn’t affect the process if bitdepth is equal or less than 10.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rice_adaptation_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

rice_adaptation_enabled_flag equal to 1 specifies that the Rice parameter for the binarization of abs_remaining[ ] and dec_abs_level may be derived by a formula. rice_adaptation_enabled_flag equal to 0 specifies that the Rice parameter for the binarization of abs_remaining[ ] and dec_abs_level is derived by a look-up table. When not present, the value of rice_adaptation_enabled_flag is inferred to be equal to 0.

The variable riceAdaptationFlag is derived as follows:

- If rice_adaptation_enabled_flag is equal to 1 and BitDepth is greater than 10, riceAdaptationFlag is set equal to 1.

- Otherwise (rice_adaptation_enabled_flag is equal to 0 or BitDepth is less than or equal to 10), riceAdaptationFlag is set equal to 0.

Experimental Results

The simulations were conducted according to JVET-T2022 [3]. Following tables show the results.

HBD CTC

HDR PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.87% | -0.84% | -0.82% | -0.87% | -0.84% | -0.81% | 104% | 99% |
| PQ422 | -0.96% | -0.97% | -0.98% | -0.97% | -0.97% | -0.98% | 103% | 99% |
| Overall | -0.92% | -0.90% | -0.90% | -0.92% | -0.91% | -0.90% | 103% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.07% | 0.06% | 0.04% | 0.06% | 0.06% | 0.04% | 102% | 101% |
| PQ422 | 0.09% | 0.10% | 0.12% | 0.08% | 0.10% | 0.13% | 102% | 103% |
| Overall | 0.08% | 0.08% | 0.08% | 0.07% | 0.08% | 0.09% | 102% | 102% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.05% | -0.04% | -0.04% | -0.05% | -0.04% | -0.03% | 102% | 103% |
| PQ422 | 0.00% | 0.03% | 0.04% | -0.01% | 0.02% | 0.05% | 102% | 105% |
| Overall | -0.02% | -0.01% | 0.00% | -0.03% | -0.01% | 0.01% | 102% | 104% |
| Overall PQ | -0.29% | -0.28% | -0.27% | -0.29% | -0.28% | -0.27% | 102% | 102% |

HDR HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.49% | -0.40% | -0.41% | 104% | 101% |
| HLG422 | -0.50% | -0.42% | -0.42% | 104% | 101% |
| Overall | -0.50% | -0.41% | -0.41% | 104% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.01% | 0.02% | 102% | 100% |
| HLG422 | -0.05% | 0.00% | 0.01% | 102% | 101% |
| Overall | -0.03% | 0.01% | 0.01% | 102% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.03% | 0.01% | 0.01% | 106% | 98% |
| HLG422 | -0.06% | -0.02% | -0.02% | 107% | 100% |
| Overall | -0.04% | -0.01% | 0.00% | 107% | 99% |
| Overall HLG | -0.19% | -0.14% | -0.13% | 104% | 100% |

SVT RGB

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -25.45% | -24.54% | -24.57% | 106% | 77% |
| SVT12 | -3.21% | -2.99% | -2.99% | 105% | 99% |
| Overall | -14.33% | -13.76% | -13.78% | 105% | 88% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -17.23% | -15.77% | -16.08% | 106% | 87% |
| SVT12 | -0.49% | -0.54% | -0.54% | 105% | 103% |
| Overall | -8.86% | -8.15% | -8.31% | 106% | 95% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -15.98% | -16.10% | -16.18% | 106% | 88% |
| SVT12 | -0.61% | -0.56% | -0.57% | 105% | 101% |
| Overall | -8.30% | -8.33% | -8.37% | 105% | 95% |
| Overall RGB | -10.50% | -10.08% | -10.16% | 105% | 93% |

Lossless

| PQ | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-1.3 |  |
| PQ444 | 2.4 | 2.5 | -3.95% |
| PQ422 | 2.2 | 2.3 | -5.00% |
| Overall | 2.3 | 2.4 | -4.47% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 87% |  |  |
| HLG | All Intra |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-1.3 |  |
| HLG444 | 1.9 | 2.0 | -3.38% |
| HLG422 | 1.8 | 1.8 | -4.28% |
| Overall | 1.8 | 1.9 | -3.83% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 90% |  |  |
| SVT | All Intra |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-1.3 |  |
| SVT16 | 0.7 | 1.2 | -41.51% |
| SVT12 | 1.0 | 1.3 | -22.46% |
| Overall | 0.8 | 1.2 | -31.98% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 72% |  |  |

Combination test (CE-3.1)

CE-1.3 changes the rice parameters derivation method only for RRC and can be combined with one for TSRC in CE-2. In this contribution, the results for CE-3.1, the combination of CE-1.3 and CE-2.1, are also reported.

HBD CTC

HDR PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.87% | -0.82% | -0.80% | -0.88% | -0.83% | -0.80% | 105% | 100% |
| PQ422 | -0.95% | -0.90% | -0.91% | -0.96% | -0.92% | -0.91% | 111% | 107% |
| Overall | -0.91% | -0.86% | -0.85% | -0.92% | -0.87% | -0.86% | 108% | 103% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.06% | 0.05% | 0.03% | 0.05% | 0.05% | 0.04% | 105% | 103% |
| PQ422 | 0.08% | 0.11% | 0.14% | 0.07% | 0.11% | 0.15% | 105% | 104% |
| Overall | 0.07% | 0.08% | 0.09% | 0.06% | 0.08% | 0.10% | 105% | 104% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.04% | -0.03% | -0.03% | -0.05% | -0.04% | -0.03% | 105% | 125% |
| PQ422 | -0.01% | 0.04% | 0.05% | -0.01% | 0.04% | 0.05% | 105% | 126% |
| Overall | -0.03% | 0.00% | 0.01% | -0.03% | 0.00% | 0.01% | 105% | 125% |
| Overall PQ | -0.29% | -0.26% | -0.25% | -0.30% | -0.26% | -0.25% | 106% | 111% |

HDR HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.53% | -0.42% | -0.41% | 105% | 99% |
| HLG422 | -0.55% | -0.45% | -0.45% | 104% | 101% |
| Overall | -0.54% | -0.43% | -0.43% | 104% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.02% | 0.01% | 105% | 103% |
| HLG422 | -0.05% | 0.00% | 0.01% | 103% | 100% |
| Overall | -0.03% | 0.01% | 0.01% | 104% | 102% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.02% | 0.00% | 0.01% | 100% | 87% |
| HLG422 | -0.06% | -0.02% | -0.01% | 100% | 91% |
| Overall | -0.04% | -0.01% | 0.00% | 100% | 89% |
| Overall HLG | -0.20% | -0.14% | -0.14% | 103% | 97% |

SVT RGB

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -25.98% | -24.77% | -24.77% | 107% | 79% |
| SVT12 | -3.27% | -3.13% | -3.11% | 106% | 101% |
| Overall | -14.62% | -13.95% | -13.94% | 106% | 90% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -17.42% | -16.60% | -16.63% | 112% | 91% |
| SVT12 | -0.57% | -0.61% | -0.61% | 106% | 102% |
| Overall | -8.99% | -8.60% | -8.62% | 109% | 97% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -16.44% | -16.52% | -16.50% | 106% | 86% |
| SVT12 | -0.70% | -0.63% | -0.62% | 105% | 101% |
| Overall | -8.57% | -8.57% | -8.56% | 106% | 94% |
| Overall RGB | -10.73% | -10.38% | -10.37% | 107% | 93% |

Conclusion

This contribution reports a result of CE-1.3: Rice parameter derivation for high bit-depth coding, and CE-3.1, which is proposed to use a formula instead of conventional look-up table for the derivation of rice parameter for RRC. It is proposed to adopt this method into the next VVC draft / VTM for Version 2.

References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10,” JVET-T2001, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 20th Meeting, by teleconference, 7 – 16 Oct. 2020.

[2] T. Hashimoto, T. Ikai, “AHG12: Rice parameter derivation for high bit-depth coding”, JVET-T0085

[3] A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, “CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-T2022

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).