      - Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

      - Sequence parameter set RBSP semantics

extended_precision_processing_flag equal to 1 specifies that an extended dynamic range may be used for transform coefficients and transform processing. extended_precision_processing_flag equal to 0 specifies that the extended dynamic range is not used. When not present, the value of extended_precision_processing_flag is inferred to be equal to 0.

The variable ExtendedPrecisionFlag is derived as follows:

–	If extended_precision_processing_flag is equal to 1 and BitDepth is greater than 10, ExtendedPrecisionFlag is set equal to 1.

–	Otherwise (extended_precision_processing_flag is equal to 0 or BitDepth is less than or equal to 10), ExtendedPrecisionFlag is set equal to 0.

      - 7.4.12.11 Residual coding semantics

The array AbsLevel[ xC ][ yC ] represents an array of absolute values of transform coefficient levels for the current transform block and the array AbsLevelPass1[ xC ][ yC ] represents an array of partially reconstructed absolute values of transform coefficient levels for the current transform block. The array indices xC and yC specify the transform coefficient location ( xC, yC ) within the current transform block. When the value of AbsLevel[ xC ][ yC ] is not specified in clause 7.3.11.11, it is inferred to be equal to 0. When the value of AbsLevelPass1[ xC ][ yC ] is not specified in clause 7.3.11.11, it is inferred to be equal to 0.

The variables CoeffMin and CoeffMax specifying the minimum and maximum transform coefficient values are derived as follows:

CoeffMin = −( 1  << (ExtendedPrecisionFlag ? Min(20, BitDepth + 6) : 15) )		(189)

CoeffMax = ( 1  << (ExtendedPrecisionFlag ? Min(20, BitDepth + 6) : 15) ) − 1		(190)

  - 8.7	Scaling, transformation and array construction process

    - 8.7.3	Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable predMode specifying the prediction mode of the coding unit,

- a variable cIdx specifying the colour component of the current block.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP and the variable QpActOffset are derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y		(1123)

QpActOffset = cu_act_enabled_flag[ xTbY ][ yTbY ] ? −5 : 0	(1124)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

qP = Qp′CbCr		(1125)

QpActOffset = cu_act_enabled_flag[ xTbY ][ yTbY ] ? 1 : 0	(1126)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(1127)

QpActOffset = cu_act_enabled_flag[ xTbY ][ yTbY ] ? 1 : 0	(1128)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr		(1129)

QpActOffset = cu_act_enabled_flag[ xTbY ][ yTbY ] ? 3 : 0	(1130)

The variable Log2TransformRange is derived as follows

Log2TransformRange = ExtendedPrecisionFlag ? Min(20, BitDepth + 6) : 15

The quantization parameter qP is modified and the variables rectNonTsFlag and bdShift are derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

qP = Clip3( 0, 63 + QpBdOffset, qP + QpActOffset )	(1131)

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 ) ? 1 : 0	(1132)

bdShift = BitDepth + rectNonTsFlag +		(1133)
		( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) + 10 − Log2TransformRange  + sh_dep_quant_used_flag

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

qP = Clip3( QpPrimeTsMin, 63 + QpBdOffset, qP + QpActOffset )	(1134)

rectNonTsFlag = 0		(1135)

bdShift = 10			(1136)

### Transformation process for scaled transform coefficients

#### 8.7.4.1	General

#### ….

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- When nTbH is greater than 1, each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.4 for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list d[ x ][ y ] with y = 0..nonZeroH − 1 and the transform type variable trType set equal to trTypeVer as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- When nTbH and nTbW are both greater than 1, the intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(1163)

- When nTbH is equal to 1, g[ x ][ 0 ] is set equal to d[ x ][ 0 ] for x = 0..nTbW − 1.

- When nTbW is greater than 1, each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.4 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1 and the transform type variable trType set equal to trTypeHor as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

- When nTbW is equal to 1, r[ 0 ][ y ] is set equal to e[ 0 ][ y ] for y = 0..nTbH − 1.

- The nTbW x nTbH array res[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1 is derived as follows:

bdShift = ( nTbH > 1  &&  nTbW > 1 ) ? 5 + Log2TransformRange – BitDepth : 6 + Log2TransformRange – BitDepth	(1164)

res[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(1165)

#### 9.3.3.11	Binarization process for abs_remainder[ ]

Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the binarization of the syntax element.

The variables lastAbsRemainder and lastRiceParam are derived as follows:

- If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0.

- Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this clause.

The rice parameter cRiceParam is derived as follows:

- If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 and sh_ts_residual_coding_disabled_flag is equal to 0, the Rice parameter cRiceParam is set equal to 1.

- Otherwise, the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[ ] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

The variable cMax is derived from cRiceParam as:

cMax = 6  <<  cRiceParam		(1526)

The binarization of the syntax element abs_remainder[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of abs_remainder[ n ], prefixVal, is derived as follows:

prefixVal = Min( cMax, abs_remainder[ n ] )		(1527)

- The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.3.3.3 for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 6 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of abs_remainder[ n ], suffixVal, is derived as follows:

suffixVal = abs_remainder[ n ] − cMax		(1528)

- The suffix bin string is specified by invoking the limited k-th order EGk binarization process as specified in clause 9.3.3.6 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1, variable maxPreExtLen set equal to 26 − Log2TransformRange and variable truncSuffixLen set equal to Log2TransformRange as input.

#### 9.3.3.12	Binarization process for dec_abs_level[ ]

Input to this process is a request for a binarization of the syntax element dec_abs_level[ n ], the colour component cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the binarization of the syntax element.

The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for dec_abs_level[ ] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 0, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

The variable cMax is derived from cRiceParam as:

cMax = 6  <<  cRiceParam		(1529)

The binarization of dec_abs_level[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of dec_abs_level[ n ], prefixVal, is derived as follows:

prefixVal = Min( cMax, dec_abs_level[ n ] )		(1530)

- The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.3.3.3 for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 6 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of dec_abs_level[ n ], suffixVal, is derived as follows:

suffixVal = dec_abs_level[ n ] − cMax		(1531)

- The suffix bin string is specified by invoking the limited k-th order EGk binarization process as specified in clause 9.3.3.6 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1, maxPreExtLen set equal to 26 − Log2TransformRange and variable truncSuffixLen set equal to Log2TransformRange as input.