| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0052_v2 |
| --- | --- |

| Title: | AHG8: Transform coefficients range extension for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Takeshi Chujoh Eiichi Sasaki Tomohiro Ikai | Tel: Email: | +81-50-5213-6338 zhou.tianyang@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes an extended coefficient precision for high bit-depth coding. Compared to the VVC version1, where the transform coefficient range is always equal to 15 regardless of bit-depth, this proposal uses Min(20, BitDepth + 6) for the coefficient range. An enabled flag is signaled in SPS extension but the method is only enabled when the flag is equal to 1 and the bit-depth is greater than 10 to avoid 10 bit case changes.

The experimental results are reported (anchor: extended precision = 0, test: extended precision = 1):

Method 1 (sps_extention_flag, Min(BitDepth + 6, 20) ):

- 0.64 %, 1.43 % and 5.89 % in bdrateY in PQ, HLG and SVT in AI

- 0.58 %, 1.71 % and 5.22 % in bdrateY in PQ, HLG and SVT in LDB

- 0.54 %, xxx % and 4.62 % in bdrateY in PQ, HLG and SVT in RA

Method 2 (no flag, Max(Min(BitDepth + 5, 20),15) ):

- 0.61 %, 1.33 % and 5.86 % in bdrateY in PQ, HLG and SVT in AI

- 0.54 %, 1.49 % and 5.17 % in bdrateY in PQ, HLG and SVT in LDB

- 0.56 %, 1.40 % and 4.58 % in bdrateY in PQ, HLG and SVT in RA

v2 added no ExtendedPrecisionFlag solution (Method2) where extended range is always used when the bit-depth is greater than 10. There is no change compared to VVC version 1 since the extended range is disabled when bit-depth is equal to or less than 10 bit. It is asserted that no flag solution could significantly reduce verification cost.

Introduction

In the current VVC design, the extended precision (extended coefficient range) has not been specified.

However, the VTM software supports the extended precision with an enabling macro named JVET_R0351_HIGH_BIT_DEPTH_ENABLED.

In VTM and HEVC RExt, when extended precision is enabled,

- Scaling shift value (bdShift) is reduced while transform shift value is increased depending on bitdepth

- Coefficient range (i.e. clipping value) is increased depending on bitdepth

In addition to the above, the following changes are applied.

- residual coding in EG(k) is changed to extend the suffix length to enable the enlarged coefficient signaling. Specifically, truncSuffixLen is set equal to Log2TransformRange

- To keep the maximum length of Truncated Rice (TR) binarization and EG(k) binarization being equal to 32, the maxPreExtLen in EG(k) is also set equal to 26 - Log2TransformRange

Proposal

Method 1:

The proposed method 1 derives ExtendedPrecisionFlag as follow to prevent 10 bit case changes.

ExtendedPrecisionFlag = extended_precision_processing_flag && bitDepth > 10.

It is proposed to derive Log2TransformRange, bdShift1 (bdShift in 8.7.3	Scaling process for transform coefficients), bdShift2 (bdShift in 8.7.4	Transformation process for scaled transform coefficients), truncSuffixLen and maxPreExtLen as follows.

If ExtendedPrecisionFlag is equal to 0.

- Log2TransformRange = 15

- bdShift1 = BitDepth + rectNonTsFlag +	( ( Log2 ( nTbW ) + Log2 ( nTbH ) ) / 2 ) − 5 + sh_dep_quant_used_flag

- bdShift2 = ( nTbH > 1  &&  nTbW > 1 ) ? ( 20 − BitDepth ) : ( 21 − BitDepth )

- truncSuffixLen = 15

- maxPreExtLen = 11

Otherwise (ExtendedPrecisionFlag is equal to 1)

- Log2TransformRange = ExtendedPrecisionFlag ? Max( 15, Min(20, BitDepthY + 6) ) : 15

- bdShift1 = BitDepthY + rectNonTsFlag +	( ( Log2 ( nTbW ) + Log2 ( nTbH ) ) / 2 ) + 10 − log2TransformRange+ sh_dep_quant_used_flag

- bdShift2 = ( nTbH > 1  &&  nTbW > 1 ) ? 5 + Log2TransformRange – BitDepth : 6 + Log2TransformRange – BitDepth

- truncSuffixLen = Log2TransformRange

- maxPreExtLen = 32 – 6 – truncSuffixLen = 26 – Log2TransformRange

Method 2:

The proposed method 2 derives without ExtendedPrecisionFlag, and there is no change compared to VVC version 1 when bit-depth is equal to or less than 10 bit.

- Log2TransformRange = Max( 15, Min(20, BitDepthY + 5 ) )

- bdShift1 = BitDepthY + rectNonTsFlag +	( ( Log2 ( nTbW ) + Log2 ( nTbH ) ) / 2 ) + 10 – log2TransformRange+ sh_dep_quant_used_flag

- bdShift2 = ( nTbH > 1  &&  nTbW > 1 ) ? 5 + Log2TransformRange – BitDepth : 6 + Log2TransformRange – BitDepth

- truncSuffixLen = Log2TransformRange

- maxPreExtLen = 32 – 6 – truncSuffixLen = 26 – Log2TransformRange

Discussions

Best precision in the experiment

At the last meeting, JVET-S0228 shows that experimental results of increased precision case (bitDepth + 4, 5, 6, and 7) in 12bit, 14bit, and 16 bit case. And it shows the increased precision doesn’t necessarily improve the coding efficiency. Table1 shows the result of increasing the coefficient case in VTM 11.0.

Table 1 Summary results of extended precision in 12, 14 and 16 bit-depth case

|  | AI-12bit | AI-14bit | AI-16bit |
| --- | --- | --- | --- |
| bitDepth+4 | 0.15% / 0.47% / 0.19% | 0.01% / 0.03% / 0.04% | ………/ / -6.82% |
| bitDepth+5 | 0.03% / 0.11% / 0.06% | 0.00% / 0.01% / 0.00% | ………/ / -4.87% |
| bitDepth+6 (Anchor) | 0% | 0% | 0% |
| bitDepth+7 | -0.01% / -0.02% -0.02% | 0.01% / 0.00% /0.06% | / / 9.20% |

PQ444, 422 / HLG444, 422 / SVT

Best case are high lighted in cyan.

As above, the increased precision degrade coding efficiency in 16 bit case. In our investigation, one reason of the degradation in increased precision’s case is that increasing Log2TransformRange  results in the reduction of maxPreExtLen due to the 32 bit length constraint in TR + EG(k) binarization.

When maxPreExtLen becomes smaller, and the large coefficient residuals cannot be represented efficiently.

Relationship between precision and maxPreExtLen

As mentioned in the introduction, To keep the maximum length of Truncated Rice (TR) binarization and EG(k) binarization being equal to 32, the maxPreExtLen is set equal to 26 - Log2TransformRange. Thus maxPreExtLen becomes smaller as Log2TransformRange increases as sown Table 2.

- maxPreExtLen = 32 – 6 – truncSuffixLen = 26 – Log2TransformRange

Table 2. Example of TR and EG(k) binarization length in the 16 bit-depth case.

|  | TR | EG(k) |  | Sum |
| --- | --- | --- | --- | --- |
| Log2TransformRange | length of prefixVal | maxPreExtLen | truncSuffixLen (=Log2TransformRange) |  |
| 15 | 6 | 11 | 15 | 32 |
| BitDepth+3 | 6 | 7 | 19 | 32 |
| BitDepth+4 | 6 | 6 | 20 | 32 |
| BitDepth+5 | 6 | 5 | 21 | 32 |
| BitDepth+6 | 6 | 4 | 22 | 32 |
| BitDepth+7 | 6 | 3 | 23 | 32 |

To alleviate this problem, this contribution proposes to clip the coefficient range up to 20 to keep maxPreExtLen equal to 6.

|  | TR | EG(k) |  | Sum |
| --- | --- | --- | --- | --- |
| Log2TransformRange | length of prefixVal | maxPreExtLen | truncSuffixLen |  |
| 20 = Min(BitDepth+6, 20) | 6 | 6 | 20 | 32 |

It is asserted that setting a clipping of Log2TransformRange to 20 has evidence that gives the best coding performance in 16 bit case as well as in 12 and 14 bit case.

Experiment

The test sequences and condition follows JVET-T2018 CTC[3] but ExtendedPrecision is equal to 0 for the anchor in all bit-depth case.

Software: VTM11.0 with CE-1.3 in JVET-U0050

Anchor: ExtendedPrecision = 0, Log2TransformRange = 15

Test:   ExtendedPrecision = 1, Log2TransformRange = Min (20, BitDepth + 6)

Table 3. Results of proposal method 1.

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.68% | -1.18% | -1.19% | 104% | 96% |
| PQ422 | -0.60% | -0.79% | -0.79% | 102% | 96% |
| Overall | -0.64% | -0.98% | -0.99% | 103% | 96% |
| HLG444 | -1.74% | -3.03% | -3.31% | 106% | 92% |
| HLG422 | -1.12% | -1.94% | -2.06% | 103% | 90% |
| Overall | -1.43% | -2.48% | -2.69% | 104% | 91% |
| SVT16 | -11.23% | -9.76% | -9.96% | 97% | 81% |
| SVT12 | -0.55% | -1.16% | -1.17% | 100% | 93% |
| Overall | -5.89% | -5.46% | -5.56% | 98% | 87% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.58% | -0.70% | -0.77% | 104% | 125% |
| PQ422 | -0.51% | -0.51% | -0.56% | 105% | 126% |
| Overall | -0.54% | -0.61% | -0.66% | 105% | 125% |
| HLG444 | -2.16% | -2.86% | -3.00% | 98% | 99% |
| HLG422 |  |  |  |  |  |
| Overall |  |  |  |  |  |
| SVT16 | -8.50% | -8.76% | -8.79% | 102% | 83% |
| SVT12 | -0.74% | -0.78% | -0.79% | 101% | 93% |
| Overall | -4.62% | -4.77% | -4.79% | 102% | 88% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.61% | -0.71% | -0.76% | 90% | 82% |
| PQ422 | -0.55% | -0.48% | -0.50% | 98% | 94% |
| Overall | -0.58% | -0.60% | -0.63% | 94% | 88% |
| HLG444 | -2.30% | -2.92% | -3.07% | 99% | 99% |
| HLG422 | -1.11% | -1.79% | -1.84% | 98% | 93% |
| Overall | -1.71% | -2.36% | -2.46% | 98% | 96% |
| SVT16 | -9.55% | -8.85% | -8.87% | 103% | 82% |
| SVT12 | -0.89% | -0.81% | -0.82% | 100% | 100% |
| Overall | -5.22% | -4.83% | -4.84% | 102% | 91% |

Table 4. Results of proposal method 2.

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 |  | -0.65% |  | -1.12% |  | -1.13% |  | 121% |  | 116% |  |  |
| PQ422 |  | -0.57% |  | -0.74% |  | -0.75% |  | 119% |  | 114% |  |  |
| Overall |  | -0.61% |  | -0.93% |  | -0.94% |  | 120% |  | 115% |  |  |
| HLG444 |  | -1.62% |  | -2.73% |  | -2.99% |  | 124% |  | 110% |  |
| HLG422 |  | -1.03% |  | -1.76% |  | -1.87% |  | 120% |  | 105% |  |  |
| Overall |  | -1.33% |  | -2.25% |  | -2.43% |  | 122% |  | 108% |  |  |
| SVT16 |  | -11.23% |  | -9.76% |  | -9.96% |  | 97% |  | 81% |  |  |
| SVT12 |  | -0.49% |  | -1.02% |  | -1.02% |  | 118% |  | 111% |  |  |
| Overall |  | -5.86% |  | -5.39% |  | -5.49% |  | 107% |  | 96% |  |  |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 |  | -0.56% |  | -0.68% |  | -0.73% |  | 104% |  | 127% |  |  |
| PQ422 |  | -0.49% |  | -0.48% |  | -0.51% |  | 105% |  | 134% |  |  |
| Overall |  | -0.53% |  | -0.58% |  | -0.62% |  | 105% |  | 131% |  |  |
| HLG444 |  | -1.91% |  | -2.62% |  | -2.75% |  | 98% |  | 101% |  |
| HLG422 |  | -0.84% |  | -1.57% |  | -1.64% |  | 95% |  | 95% |  |  |
| Overall |  | -1.37% |  | -2.09% |  | -2.19% |  | 97% |  | 98% |  |  |
| SVT16 |  | -8.50% |  | -8.76% |  | -8.79% |  | 102% |  | 83% |  |
| SVT12 |  | -0.65% |  | -0.71% |  | -0.71% |  | 101% |  | 99% |  |  |
| Overall |  | -4.58% |  | -4.74% |  | -4.75% |  | 102% |  | 91% |  |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 |  | -0.57% |  | -0.68% |  | -0.72% |  | 91% |  | 83% |  |  |
| PQ422 |  | -0.53% |  | -0.46% |  | -0.47% |  | 102% |  | 98% |  |  |
| Overall |  | -0.55% |  | -0.57% |  | -0.59% |  | 96% |  | 90% |  |  |
| HLG444 |  | -2.02% |  | -2.65% |  | -2.79% |  | 102% |  | 99% |  |
| HLG422 |  | -0.96% |  | -1.65% |  | -1.66% |  | 102% |  | 95% |  |
| Overall |  | -1.49% |  | -2.15% |  | -2.23% |  | 102% |  | 97% |  |  |
| SVT16 |  | -9.55% |  | -8.85% |  | -8.87% |  | 103% |  | 82% |  |
| SVT12 |  | -0.78% |  | -0.73% |  | -0.73% |  | 106% |  | 102% |  |
| Overall |  | -5.17% |  | -4.79% |  | -4.80% |  | 105% |  | 92% |  |

Combination test with CE Rice parameter proposal (extended precision with CE-1.3)

Software: VTM11.0 with CE-1.3 in JVET-U0050 (GolombRiceParameterAdaptation=1) [5].

Anchor: ExtendedPrecision = 0 in 12 bit, 1 in 16bit, Log2TransformRange = 15

Test:   ExtendedPrecision = 1,                Log2TransformRange = Min (20, BitDepth + 6).

The table 4 shows the coding efficiency of proposal method over CE-1.3 results.

Table 5. Results of proposal method 1.

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1.3 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.70% | -1.19% | -1.21% | 102% | 96% |
| PQ422 | -0.62% | -0.82% | -0.84% | 100% | 96% |
| Overall | -0.66% | -1.00% | -1.03% | 101% | 96% |
| HLG444 | -1.73% | -2.98% | -3.26% | 99% | 87% |
| HLG422 | -1.09% | -1.91% | -2.03% | 98% | 87% |
| Overall | -1.41% | -2.44% | -2.64% | 99% | 87% |
| SVT16 | -1.22% | -0.62% | -0.66% | 117% | 101% |
| SVT12 | -0.76% | -1.26% | -1.27% | 95% | 86% |
| Overall | -0.99% | -0.94% | -0.96% | 106% | 94% |
|  |  |  | RA |  |  |
|  |  |  | Over CE-1.3 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.57% | -0.69% | -0.77% | 99% | 118% |
| PQ422 | -0.51% | -0.50% | -0.57% | 99% | 116% |
| Overall | -0.54% | -0.60% | -0.67% | 99% | 117% |
| HLG444 | -2.09% | -2.82% | -2.97% | 86% | 83% |
| HLG422 | -0.87% | -1.65% | -1.73% | 85% | 84% |
| Overall | -1.48% | -2.24% | -2.35% | 85% | 83% |
| SVT16 | -0.27% | -0.59% | -0.53% | 113% | 101% |
| SVT12 | -0.63% | -0.66% | -0.66% | 98% | 93% |
| Overall | -0.45% | -0.62% | -0.59% | 106% | 97% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE-1.3 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.60% | -0.70% | -0.73% | 100% | 97% |
| PQ422 | -0.53% | -0.44% | -0.50% | 100% | 96% |
| Overall | -0.57% | -0.57% | -0.61% | 100% | 97% |
| HLG444 | -2.24% | -2.88% | -3.02% | 93% | 93% |
| HLG422 | -1.04% | -1.73% | -1.80% | 93% | 89% |
| Overall | -1.64% | -2.31% | -2.41% | 93% | 91% |
| SVT16 | -0.36% | -0.28% | -0.24% | 116% | 106% |
| SVT12 | -0.76% | -0.66% | -0.66% | 99% | 92% |
| Overall | -0.56% | -0.47% | -0.45% | 107% | 99% |

Conclusion

It is proposed to adopt Min( bitDepth + 6, 20) transform coefficients precision in the case extended precision flag is equal to 1 and bitDepth is larger than 10 into the next VVC specification and VTM.

It is also proposed the second solution where no flag is used and Max(Min( bitDepth + 5, 20), 15) is set for transform coefficients precision.

References

[1]	A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, Joint Video Experts Team Document, JVET-T2018, teleconference, October 2020.

[2] B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10,” JVET-T2001, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 20th Meeting, by teleconference, 7 – 16 Oct. 2020.

[3]	T. Zhou, T. Chujoh, E. Sasaki, T. Ikai “AHG12: Transform coefficients range extension for high bit-depth coding”, JVET-T0084, teleconference, October 2020.

[4]	A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, “CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-T2022

[5]	T. Hashimoto, T. Ikai, “CE-1.3 and CE-3.1: Rice parameter derivation for high bit-depth coding”, JVET-U0050

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).