| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0055 |
| --- | --- |

| Title: | AHG11: Multi-density network for in-loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhao Wang, Ru-Ling Liao, Changyue Ma, Yan Ye (Alibaba Group). | Email: | baixiu.wz@alibaba-inc.com |
| Source: | Alibaba |  |  |

_____________________________

- Abstract

This contribution is an improved version of the contribution JVET-U0054 where a neural network based in-loop filter (NNLF) is proposed. This contribution proposes a multi-density network to be used in NNLF and the other information is similar to JVET-U0054. The experimental results show that our method can achieve 5.60%, 14.92% and 15.24% coding gain on VTM-10.0 for Y, U and V components under RA configuration.

Introduction

In JVET-U0054 [1], a neural network based in-loop filter (NNLF) scheme was proposed. In this contribution, a multi-density network (MDN) to be used in NNLF is proposed to further improve the coding performance. The NNLF is located between Deblocking and SAO, and CTU-level signalling is conducted.

Network architecture

For the reconstructed frame before NNLF, it will first be converted into YUV444 by up-sampling the chroma components if the original color planes don’t own the same size. Neighboring samples are copied directly to simplify the up-sampling process. During inference, only the top-left sample of each 2x2 chroma block is saved.

The overall architecture of the proposed multi-density network is illustrated in Figure 1, which consists of three parts: pre-processing, density learning and post-processing. The core design lies in the multi-density block, which consists of a basic branch and a density branch. The basic branch contains some Resblocks to process the feature map in full resolution. The density branch is added to learn the density correlations among feature samples and generate a weight map. In density branch, a down-sampling layer is firstly applied to obtain the half-resolution representations. One Resblock follows and then an up-sampling layer is concatenated to generate weight map in the same size as the feature map in the basic branch. Finally, the weight map of the density branch and the feature map of the basic branch are fused by element-wise product.

The architecture of down-sampling procedure, up-sampling procedure and Resblock are shown in Figure 2. In the down-sampling procedure, a 3x3 convolution with stride of 2 is used to realize half down-sampling process. After ReLU activation, additional 1x1 convolutional layer and activation function is further applied to enhance the non-linearity learning ability. With respect to the up-sampling procedure, the pixel-shuffle is adopted to expand the spatial resolution and then conduct the ReLU activation. The 1x1 convolution is also added in the up-sampling layer; then, the sigmoid function is adopted in the activation step to generate the density representations between the value of 0 and 1. The structure of Resblock in our implementation is shown in Figure 2-(c), which contains two convolutional layers and the by-pass identity mapping.

Figure 1. Illustration of the proposed multi-density network.

Figure 2. Down-sampling (a), up-sampling (b) and Resblock (c) in multi-density network.

Training

The training conditions specified in JVET-T2006 [4] are closely followed. Random 200 sequences in BVI-DVC are selected and encoded by VTM10.0 with RA configuration. Each selected sequence is encoded 5 times with 4 input QPs (22, 27, 32, 37, 42). The reconstructed images are then converted into YUV444 format and split into four sub-sets according to the frame-level QP {(min-34, 35-40, 41-46, 47-max)}. For each sub-set, a model is trained and the detailed information are summarized in below Table 1.

Table 1 Training information

| Training information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | 4x Telsa-V100-32G |
|  | CPU Type | Intel(R) Xeon(R) Platinum 8163 CPU @ 2.50GHz |
|  | Framework: | Pytorch 1.2.0 |
|  | Epoch: | 2000 |
|  | Batch size: | 32 |
|  | Training time | 70 hours |
|  | Total Parameter Number | 3.44M/model |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Training set | BVI-DVC |
| Optional | Patch size | 128x128x3 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Loss function: | L2 |

Inference

Detailed inference information is summarized in below Table 2.

Table 2 Inference information

| Inference information |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Intel(R) Xeon(R) Platinum 8163 CPU @ 2.50GHz |
|  | Framework: | PyTorch 1.2.0 |
|  | Number of GPUs per Task | 1 |
|  | Total Parameter Number | 3.44M/model |
|  | Parameter Precision (Bits) | 32 (F) |
| Optional | Total Conv. Layers | 66+32 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | frame |

Simulation results

The common test conditions defined in JVET-T2006 are closely followed. The performance on VTM-10.0 under RA configurations are shown in Table 3. We also apply the proposed method into the HHi VVenC-0.1.0.0 codec [5] and the experimental results under RA configuration are shown in Table 4.

Table 3 Test 1: experimental results on VTM-10.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.60% | -9.07% | -9.67% | 99% | 1750% |
| Class A2 | -5.14% | -15.55% | -10.66% | 97% | 1696% |
| Class B | -5.47% | -13.39% | -15.04% | 96% | 1938% |
| Class C | -4.83% | -17.44% | -17.17% | 91% | 2071% |
| Class D | -6.85% | -18.22% | -21.18% | 103% | 2500% |
| Overall | -5.60% | -14.92% | -15.24% | 97% | 1998% |

Table 4 Test 2: experimental results on VVenC-0.1.0.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VVenC-0.1.0.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.84% | -9.10% | -9.70% | 171% |  |
| Class A2 | -5.15% | -13.85% | -9.43% | 160% |  |
| Class B | -5.43% | -12.50% | -14.22% | 160% |  |
| Class C | -4.82% | -17.26% | -17.24% | 141% |  |
| Class D | -6.63% | -19.20% | -20.81% | 217% |  |
| Overall | -5.57% | -14.59% | -14.77% | 174% |  |

Reference

- Z. Wang, R.-L. Liao, C.Y. Ma, Y. Ye (Alibaba), “EE: Neural network based in-loop filtering”, JVET-U0054, 21th Meeting: by teleconference, Feb, 2021.

- E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. Ma and H. Wang, “Description of Exploration Experiments on NN-based video coding”, JVET-T2023, 20th Meeting: by teleconference, 7 – 16 October 2020.

- He K, Zhang X, Ren S, et al. Deep residual learning for image recognition[C]. Proceedings of the IEEE conference on computer vision and pattern recognition. 2016: 770-778.

- S. Liu, A. Segall, E. Alshina, R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-T2006, 20th Meeting: by teleconference, 7 – 16 October 2020.

- https://github.com/fraunhoferhhi/vvenc

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).