| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0062 |
| --- | --- |

| Title: | CE related: On Rice Parameter Derivation with Content Adaptation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Kyohei Unno 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 80 5066 9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes a rice parameter selection method of regular residual coding for operation range extensions of VVC. Since a distribution of residual coefficient values becomes wider for high bit depths and high through-put, the current rice parameter range makes a performance degradation. Such the distribution depends on not an internal bit depth but the content itself. To address this issue, this contribution decomposes the locSumAbs value into shift parameter and index parameter of the pre-defined look-up table.

Problem Statement

In the current VVC, a rice parameter for regular residual coding is derived using pre-defined look-up table. First, locSumAbs value is calculated as the sum of absolute of five available neighbouring coefficients. Then, locSumAbs value is updated as follows:

locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 )

where baseLevel is 4 or 0 based on the remainder coding or bypass coding, respectively.

The rice parameter is derived using this locSumAbs with the LUT. In the current design, the rice parameter range is constrained from 0 to 3.

For the operation range extensions, both the coefficients and locSumAbs value may significantly increase. In this case, the maximum rice parameter is insufficient, which make the performance degradation.

Proposed method

Proposal approach

This contribution proposes a rice parameter derivation method for the over 10 bit coding. It means that the proposed method does not affect the coding performance in VVC version 1 condition.

In order to re-use the existing pre-defined look-up table, locSumAbs value is decomposed into shift value and index value. After that, the rice parameter is looked up by the index and modified by shift value.

Proposed text change

The green high lighted parts are added to the latest VVC version 1 specification.

      - 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

The variables shiftRice is set equal to 0.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process:

locSumAbs = 0
if( xC < ( 1  <<  log2TbWidth ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC + 1 ][ yC ]
	if( xC < ( 1  <<  log2TbWidth ) − 2 )
		locSumAbs  +=  AbsLevel[ xC + 2 ][ yC ]
	if( yC < ( 1  <<  log2TbHeight ) − 1 )
		locSumAbs  +=  AbsLevel[ xC + 1 ][ yC + 1 ]	(1518)
}
if( yC < ( 1  <<  log2TbHeight ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC ][ yC + 1 ]
	if( yC < ( 1  <<  log2TbHeight ) − 2 )
		locSumAbs  +=  AbsLevel[ xC ][ yC + 2 ]
}
shiftRice = FloorLog2( locSumAbs  >>  3 ) )
localSumAbs = localSumAbs  >>  ShiftRice

Variable localSumAbs is updated as follows:

locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 )

Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. The variable cRiceParam is then refined as:

cRiceParam = cRiceParam + shiftRice

When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows:

ZeroPos[ n ] = ( QState < 2 ? 1 : 2 )  <<  cRiceParam	(1519)

Table 1 – Specification of cRiceParam based on locSumAbs

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

Experimental Results

The proposed method is implemented on the VTM-11.0. Simulation conditions follows JVET-T2018. Experimental results are summarized as in the following tables.

Table 2 Summary results for the HDR PQ conditions

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.68% | -0.73% | -0.72% | -0.64% | -0.68% | -0.67% | 103% | 100% |
| PQ422 | -0.78% | -0.85% | -0.86% | -0.73% | -0.78% | -0.78% | 102% | 99% |
| Overall | -0.73% | -0.79% | -0.79% | -0.68% | -0.73% | -0.72% | 102% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.05% | -0.06% | -0.08% | -0.04% | -0.05% | -0.06% | 102% | 101% |
| PQ422 |  |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.06% | -0.08% | -0.04% | -0.05% | -0.06% | 102% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.14% | -0.15% | -0.14% | -0.13% | -0.14% | -0.13% | 102% | 101% |
| PQ422 |  |  |  |  |  |  |  |  |
| Overall | -0.14% | -0.15% | -0.14% | -0.13% | -0.14% | -0.13% | 102% | 101% |
| Overall PQ | -0.52% | -0.56% | -0.57% | -0.48% | -0.52% | -0.51% | 102% | 100% |

Table 3 Partial results for the HDR HLG conditions. LDB and RA results are under simulation.

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.32% | -0.33% | -0.33% | 103% | 100% |
| HLG422 | -0.35% | -0.35% | -0.36% | 103% | 100% |
| Overall | -0.33% | -0.34% | -0.35% | 103% | 100% |

Table 4 Summary results for the SVT conditions.

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -25.44% | -24.53% | -24.56% | 98% | 81% |
| SVT12 | -3.01% | -2.82% | -2.82% | 100% | 99% |
| Overall | -14.23% | -13.68% | -13.69% | 99% | 90% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -17.21% | -15.74% | -16.05% | 101% | 90% |
| SVT12 | -0.52% | -0.59% | -0.59% | 102% | 103% |
| Overall | -8.87% | -8.16% | -8.32% | 102% | 96% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 |  |  |  |  |  |
| SVT12 | -0.66% | -0.65% | -0.65% | 103% | 102% |
| Overall | -0.66% | -0.65% | -0.65% | 103% | 102% |
| Overall RGB | -11.19% | -10.67% | -10.72% | 100% | 93% |

Conclusion

In this contribution, we proposed the rice parameter derivation method for high bit-depth and high through-put coding. It is recommended that the proposed method is adopted to the next version of VVC specification.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

References

- A. Browne, T. Ikai, D. Rusanovskyy, and X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-T2018, teleconference, 7–16 October 2020.

- A. Browne, T. Hashimoto, H.-J. Jhu, and D. Rusanovskyy, “CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-T2022, teleconference, 7–16 October 2020.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).