| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0065 |
| --- | --- |

| Title: | CE-3.4: Combination of CE-1.2 and CE-2.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Luong Pham Van Muhammed Coban Marta Karczewicz | Email: | dmytror@qti.qualcomm.com lphamvan@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution describes a CE-3.4 test, which combines modification to the Rice parameters selection method for regular residual coding (RRC) being tested in CE-1.2 and transform skip residual coding (TSRC) method tested in CE-2.1. Simulation results, and assessments of complexity and memory requirement change are provided as per CE requirement in JVET-T2022.

# Test description

This test combines the RRC coding modifications from CE-1.2.a described in JVET-U0064 with the TSRC coding modifications tested in CE-2.1 and described in JVET-U0075.

The specification text in part of RRC coding is accordingly modified as follows:

7.3.7	Slice header syntax

| … |  |
| --- | --- |
| if( sps_dep_quant_enabled_flag ) |  |
| sh_dep_quant_used_flag | u(1) |
| if( sps_high_bit_depth ) |  |
| sh_scale_rice | ue(v) |
| if( sps_sign_data_hiding_enabled_flag && !sh_dep_quant_used_flag ) |  |
| sh_sign_data_hiding_used_flag | u(1) |

sh_scale_rice specifies the scale value utilized for derivation of the Rice parameter, the value of sh_scale_rice shall be in the range of 1 to 2, inclusive. When sh_scale_rice is not present, it is inferred to be equal to 0.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) } if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } Variable shift is derived as following: scalledCoef = (localSumAbs * sh_scale_rice) >> 3 shift = (scallPreCoef == 0) ? 0 : Floor( Log2( scalledCoef) ) locSumAbs = locSumAbs >> shift Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. cRiceParam is then is refined as: cRiceParam = cRiceParam + shift

Worst case complexity analysis:

It is assessed that worst case complexity increase is accounted for 1 condition, 2 shift operations, 1 add and 1 floorLog2() (equivalent of MSB find operation) for each Golomb-Rice coded transform coefficient. For worst case scenario (4x4 TU with each coefficient codded with Rice), complexity can be assessed in 16/32/16/16 for condition/shift/adds/MSB find respectively.

Memory increase analysis:

It is estimated that method requires no ROM increase.

# CE-3.4.a Simulation results for HBD CTC

Simulation results have been conducted according to JVET-T2018. Presented run time estimates may not be accurate and can be updated.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.72% | -0.70% | -0.70% | -0.72% | -0.71% | -0.71% | 88% | 101% |
| PQ422 | -0.78% | -0.77% | -0.79% | -0.78% | -0.79% | -0.80% | 89% | 96% |
| Overall | -0.75% | -0.74% | -0.75% | -0.75% | -0.75% | -0.75% | 88% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | -0.01% | -0.03% | -0.03% | -0.01% | -0.03% | 87% | 87% |
| PQ422 | -0.03% | -0.02% | -0.02% | -0.04% | -0.02% | -0.02% | 89% | 91% |
| Overall | -0.03% | -0.02% | -0.03% | -0.03% | -0.02% | -0.02% | 88% | 89% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.13% | -0.13% | -0.13% | -0.13% | -0.12% | -0.13% | 82% | 89% |
| PQ422 | -0.14% | -0.12% | -0.14% | -0.15% | -0.13% | -0.13% | 89% | 97% |
| Overall | -0.14% | -0.12% | -0.14% | -0.14% | -0.12% | -0.13% | 86% | 93% |
| Overall PQ | -0.41% | -0.40% | -0.37% | -0.42% | -0.41% | -0.42% | 88% | 95% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.34% | -0.30% | -0.30% | 85% | 97% |
| HLG422 | -0.38% | -0.36% | -0.36% | 86% | 95% |
| Overall | -0.36% | -0.33% | -0.33% | 85% | 96% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.04% | 0.05% | 0.04% | 88% | 95% |
| HLG422 | 0.01% | 0.00% | 0.02% | 85% | 91% |
| Overall | 0.03% | 0.03% | 0.03% | 86% | 93% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.01% | 0.02% | 105% | 112% |
| HLG422 | -0.01% | -0.01% | 0.00% | 106% | 105% |
| Overall | 0.00% | 0.00% | 0.01% | 105% | 109% |
| Overall HLG | -0.17% | -0.16% | -0.16% | 90% | 98% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -26.17% | -24.87% | -24.87% | 106% | 83% |
| SVT12 | -3.15% | -3.03% | -3.00% | 85% | 94% |
| Overall | -14.66% | -13.95% | -13.93% | 95% | 89% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -17.31% | -16.50% | -16.52% | 112% | 97% |
| SVT12 | -0.63% | -0.68% | -0.67% | 97% | 96% |
| Overall | -8.97% | -8.59% | -8.60% | 105% | 96% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -16.34% | -16.51% | -16.46% | 90% | 84% |
| SVT12 | -0.78% | -0.73% | -0.72% | 87% | 92% |
| Overall | -8.56% | -8.62% | -8.59% | 88% | 88% |
| Overall RGB | -12.04% | -11.57% | -11.56% | 95% | 90% |

# CE-3.4 Simulation results for HBD CTC lossless

Simulation results have been conducted according to JVET-T2022. Presented run time estimates may not be accurate and can be updated.

| PQ | All Intra |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | Ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.4 |  | VTM11.0rc1 | CE-3.4 |  |
| PQ444 | 2.4 | 2.5 | -3.34% | 3.0 | 3.0 | -0.82% |
| PQ422 | 2.2 | 2.3 | -4.41% | 2.8 | 2.8 | -1.29% |
| Overall | 2.3 | 2.4 | -3.88% | 2.9 | 2.9 | -1.06% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| HLG | All Intra |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.4 |  | VTM11.0rc1 | CE-3.4 |  |
| HLG444 | 1.9 | 2.0 | -2.90% | 2.1 | 2.1 | -0.92% |
| HLG422 | 1.8 | 1.8 | -2.52% | 2.0 | 2.0 | -0.68% |
| Overall | 1.8 | 1.9 | -2.71% | 2.0 | 2.1 | -0.80% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| SVT | All Intra |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.4 |  | VTM11.0rc1 | CE-3.4 |  |
| SVT16 | 0.7 | 1.2 | -41.15% | 0.7 | 1.2 | -43.56% |
| SVT12 | 1.0 | 1.3 | -21.92% | 1.0 | 1.3 | -21.25% |
| Overall | 0.8 | 1.2 | -31.54% | 0.8 | 1.2 | -32.41% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

# References

[1]	Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 10)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- S2001, 19th Meeting: Teleconference, 22 June – 01 July 2020.

[2] JVET-T0105, AHG12: On the Rice parameter derivation for high bit-depth coding, L. Pham Van, D. Rusanovskyy, M. Karczewicz (Qualcomm).

[3]	JVET-T2018, "Common test conditions for high bit depth and high bit rate video coding", A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu.

[4]	JVET-T2022, "CE on Entropy Coding for High Bit Depth and High Bit Rate Coding", A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy.

[5]	JVET-U0022, "CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding", A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).