| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0069-v1 |
| --- | --- |

| Title: | AHG8: CABAC-bypass alignment for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Jie Chen Yan Ye Ru-Ling Liao 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1-408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

HEVC range extension supports CABAC bypass alignment flag to improve the throughput of the CABAC engine in high bit-rate applications. This contribution proposes to add the same CABAC-bypass alignment concept to VVC, where the value of the ivlCurrRange of the CABAC engine is set to 256 right before starting of the bypass coding of a coefficient group. Two alignment options are proposed. In the first option, only the bypass alignment is applied only to coefficient coding within a coefficient group (CG), without affecting the coding of sb_coded_flag of a transform block (TB). In the second option, in addition to CABAC bypass alignment, it is also proposed to switch to bypass coding of sb_coded_flag after the limit of context coded bins has been reached for the current TB. It is asserted that with the second option, alignment is needed only once after the limit of the context coded bins has been reached for the TB. In the proposed methods, the bypass coded bins can be decoded just by looking at the most-significant-but-one bits of ivlOffset which ultimately allows decoding of multiple bypass coded bins in parallel. In order to allow the flexibility of enable/disable the proposed tool, both sequence and picture level flags are also proposed.

Following results are reported

As compared to VTM-11.0:

- Option1:

  - SVT sequences: x.xx%(G), x.xx% (B), x.xx% (R)

  - HLG sequences: x.xx%(Y), x.xx% (U), x.xx% (V)

  - PQ sequences: 1.21%(Y), 1.51% (U), 1.48% (V)

- Option2:

  - SVT sequences: x.xx%(G), x.xx% (B), x.xx% (R)

  - HLG sequences: x.xx%(Y), x.xx% (U), x.xx% (V)

  - PQ sequences: 1.21%(Y), 1.50% (U), 1.48% (V)

As compared to CE-1.2.a:

- Option1:

  - SVT sequences: 0.58%(G), 0.60% (B), 0.60% (R)

  - HLG sequences: x.xx%(Y), x.xx% (U), x.xx% (V)

  - PQ sequences: 1.21% (Y), 1.52% (U), 1.48% (V)

- Option2:

  - SVT sequences: 0.58%(G), 0.60% (B), 0.60% (R)

  - HLG sequences: x.xx%(Y), x.xx% (U), x.xx% (V)

  - PQ sequences: 1.21% (Y), 1.52% (U), 1.48% (V)

# Problem Statement

The CABAC process in VVC is a sequential process where the evaluation of each iteration depends on the outcome of the previous iteration. At the higher bit depths and higher bit rates operating ranges, the serial nature of CABAC decoding process may impact the throughput of the codec. Since there is an upper-limit of the maximum number of context coded bins, at high bit-rate the bypass coding bins are the dominant portion in the bitstream, compared to the context coded bins. Table 1 shows the per pixel distribution of the number of context coded bins of 16 bit, RGB, SVT sequences at QP = -33. (VTM-10.0 was used to generate the data). As shown in Table 1, significant number of bins in the bitstream are bypass coded.

Table 1: Number of context and bypass coded bins of SVT16 sequences ( QP = -33)

|  | Number of context coded bins/pixel | Number of by-pass coded bins/pixel |
| --- | --- | --- |
| All Intra | 5.218 | 34.394 |
| Low delay | 5.375 | 29.985 |
| Randomaccess | 5.150 | 36.845 |

The decoding process of a bypass coded bin is not so straight forward. In VVC, the status of the arithmetic decoding engine is represented by the variables ivlCurrRange and ivlOffset. In order to decode a bypass coded bin the parameter ivlCurrRange is updated first ( as shown in Figure 1) . Then, the updated ivlCurrRange is compared with the ivlOffset. If the ivlOffset is greater than or equal to ivlCurrRange, the bin value is equal to 1 and ivlOffset is updated; otherwise, the bin value is set to 0. This serial decoding of bypass coding may impact the throughput of the codec especially at high bit-rate.

Figure 1: Decoding process of a bypass coded bin in VVC

# Proposed methods

In order to improve the throughput of the CABAC engine, HEVC range extension supports aligning the CABAC process prior to coding of bypass data, which allows easy, simultaneous decoding of multiple bypass bins [1]. In this contribution, it is proposed to add the same CABAC bypass alignment concept as that in HEVC range extension into VVC.  In the proposed method, using the same process as in HEVC range extension, the value of the ivlCurrRange of CABAC engine is set to 256 right before starting of the bypass coding of a coefficient group. After aligning the ivlCurrRange to 256, the decoding process of a bypass coded bin can be simplified (as shown in Figure 2).

Figure 2: Decoding of a bypass coded bin while ivlCurrRange == 256. The figure is extracted from [1]

As shown in Figure 2, the values of the bypass coded bins are directly visible from ivlOffset. In the proposed method, the bypass decoding process can be implemented using a shift register and multiple bypass coded bins can be decoded in parallel, as follows:

- ivlOffset ← (ivlOffset << n) | read_bits(n)

- binVals ← n most-significant-but-one bits of ivlOffset

- remove the n interpreted bits from ivlOffset

Based on the CABAC bypass alignment process in HEVC range extension shown above, this contribution adapts the process to VVC in the following manners:

- The alignment process is applied to both regular residual coding and transform skip residual coding;

- Two alignment options are proposed, one modifying the process for only coefficients within a CG, and the other also including switching sb_coded_flag to bypass coding when certain conditions are met;

- Different high-level syntax designs are proposed to allow more flexible switching of the alignment process

## Option 1: CABAC bypass alignment

In option 1, the proposed CABAC bypass alignment is applied only to coefficients within a CG, and executed before starting of the bypass coding pass. The coding method of sb_coded_flag is exactly same as VVC. In the following pseudo code, the variable bypass_alignment_enable_flag is set to 1 to enable the proposed method. If the value of bypass_alignment_enable_flag is equal to 0, the proposed method is disabled and the behavior is same as current VVC. The detail of the  high level signaling flags are described in the section 2.3.  The variables remBinsPass1 and RemCcbs (same variables are used in the residual coding syntax tables of VVC specification JVET-T2001) represent the remaining number of context coded bins available in the TB in regular and transform-skip residual coding, respectively. The red italic portions are the additional process as compared to the current VVC.

In case of regular residual coding:

- remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - context_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (remBinsPass1  >=  4 )

      - Pass 1:

        - sig_coeff_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 0 ] ; remBinsPass1--;

        - par_level_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 1 ] ; remBinsPass1--;

    - if (bypass_alignment_enable_flag)

      - CABAC bypass alignment (i.e.set ivlCurrRange to 256)

    - Pass 2: abs_remainder, dec_abs_level

    - Pass 3: coeff_sign_flag

In case of transform-skip residual coding:

- RemCcbs = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - context_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (RemCcbs >= 4)

      - Pass 1:

        - sig_coeff_flag; RemCcbs--;

        - coeff_sign_flag; RemCcbs--;

        - abs_level_gtx_flag[ ][ 0 ] ; RemCcbs--;

        - par_level_flag; RemCcbs--;

    - if (RemCcbs >= 4)

      - Pass 2:

        - abs_level_gtx_flag [ ][ 1 ]; RemCcbs--;

        - abs_level_gtx_flag [ ][ 2 ] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 3] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 4 ] ; RemCcbs--;

    - if (bypass_alignment_enable_flag)

      - CABAC-bypass alignment (i.e.set ivlCurrRange to 256)

    - pass 3: abs_remainder, coeff_sign_flag

Figure 3: Proposed option 1

Figure 3 shows the bit-stream format of option 1. The red arrows show the proposed alignment points within a transform block. The alignment is performed at each coefficient group before starting the decoding/encoding of the bypass coded coefficients. In this option, as depicted in Figure 3, it may be necessary to perform the alignment multiple times even after the maximum limit of the context coded bins has been reached for the current TB. This is because the sb_coded_flag is always context coded, so even after the limit of maximum number of context coded bins is reached, the switching between context coded bins and bypass coded bins means that bypass alignment could occur multiple times.

## Option 2: CABAC bypass alignment + context/bypass coding of sb_coded_flag

One drawback of option 1 is that the CABAC bypass alignment is required even after reaching to the maximum limit of the context coded bins. In order to guarantee that there is no context coded bins after reaching to the maximum limits, a method of coding of sb_coded_flag was proposed in JVET-P0370 [4]. In JVET-P0370, the sb_coded_flag (of both RRC and TSRC) are bypass coded after reaching to the number of context coded bins to the maximum limit.   In order to reduce the number of bypass alignments, in option 2, the proposed alignment method (described in option 1) is combined with sb_coded_flag coding method proposed in JVET-P0370 when CABAC bypass alignment is enabled.

Following is the pseudo-code of option 2. The variable bypass_alignment_enable_flag is set to 1 to enable the proposed method. If the value of bypass_alignment_enable_flag is equal to 0, the proposed method is disabled and the behavior is same as current VVC. The detail of the signaling of high level control flags of the proposed method is described in section 2.3. The variables remBinsPass1 and RemCcbs (same variables are used in the residual coding syntax tables of VVC specification JVET-T2001) represent the remaining number of context coded bins available in the TB in regular and transform-skip residual coding, respectively. The red italic portions are the additional process as compared to the current VVC.

In case of regular residual coding:

- remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - if (remBinsPass1  >=  4 || bypass_alignment_enable_flag == 0)

    - CABACBypasIntervalAlign = 0

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (remBinsPass1  >=  4)

      - Pass 1:

        - sig_coeff_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 0 ] ; remBinsPass1--;

        - par_level_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 1 ] ; remBinsPass1--;

    - If (CABACBypasIntervalAlign == 0 && bypass_alignment_enable_flag)

      - CABAC bypass alignment (i.e.set ivlCurrRange to 256)

      - CABACBypasIntervalAlign = 1

    - Pass 2: abs_remainder, dec_abs_level

    - Pass 3: coeff_sign_flag

In case of transform-skip residual coding:

- RemCcbs = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - if (RemCcbs >= 4 || bypass_alignment_enable_flag == 0)

    - CABACBypasIntervalAlign = 0

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (RemCcbs >= 4)

      - Pass 1:

        - sig_coeff_flag; RemCcbs--;

        - coeff_sign_flag; RemCcbs--;

        - abs_level_gtx_flag[ ][ 0 ] ; RemCcbs--;

        - par_level_flag; RemCcbs--;

    - if (RemCcbs >= 4)

      - Pass 2:

        - abs_level_gtx_flag [ ][ 1 ]; RemCcbs--;

        - abs_level_gtx_flag [ ][ 2 ] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 3] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 4 ] ; RemCcbs--;

    - If (CABACBypasIntervalAlign == 0 && bypass_alignment_enable_flag)

      - CABAC-bypass alignment (i.e.set ivlCurrRange to 256)

      - CABACBypasIntervalAlign = 1

    - Pass 3: abs_remainder, coeff_sign_flag

Figure 4 shows an example of the bit-stream of option 2. It shows that after the number of context coded bins has reach the limit for the current TB, only bypass coding bins are processed and the bypass alignment is necessary at most once. As shown in Figure 4, the proposed method reduces the number of alignments as compared to Figure 3.

Figure 4: Proposed option 2

## Proposed high level syntax changes

The proposed CABAC bypass alignment (i.e. set the value of ivlCurrRange to 256) has benefits of simplifying high-throughput design of the entropy decoder, however, it may not be suitable for all application scenarios. In order to maintain backward compatibility with version 1, it is proposed to signal ON/OFF flag to enable/disable the proposed alignment method. Following signaling methods are proposed.

### SPS level signaling

The SPS flags are signaled to enable/disable the proposed alignment in the sequence level. This is the same as HEVC range extension. At first, sps_range_extension_flag is added to support the VVC range extension. If sps_range_extension_flag  is equal to 1, sps_cabac_bypass_alignment_enabled_flag  is signaled to control the proposed alignment process. If the sps_cabac_bypass_alignment_enabled_flag is equal to 1, the proposed alignment method may be enabled for that sequence. The sps_extension_7bits are reserved for other future extensions. Following are the semantic of the proposed flags (in the following semantic italic part are valid only for option 2):

sps_range_extension_flag equal to 1 specifies that the sps_range_extension( ) syntax structure is present in the SPS RBSP syntax structure. sps_range_extension_flag equal to 0 specifies that this syntax structure is not present. When not present, the value of sps_range_extension_flag is inferred to be equal to 0.

sps_cabac_bypass_alignment_enabled_flag equal to 1 specifies that the value of ivlCurrRange may be aligned prior to bypass decoding of the syntax elements sb_coded_flag[][],  abs_remainder[ ], dec_abs_level[ n ], and coeff_sign_flag[]. sps_cabac_bypass_alignment_enabled_flag equal to 0 specifies that the value of ivlCurrRange is not aligned prior to bypass decoding. When not present, the value of sps_cabac_bypass_alignment_enabled_flag is inferred to be equal to 0.

sps_extension_7bits equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. When present, sps_extension_7bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Values of sps_extension_7bits not equal to 0 are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of sps_extension_7bits to be not equal to 0 and shall ignore all sps_extension_data_flag syntax elements in an SPS NAL unit. When not present, the value of sps_extension_7bits is inferred to be equal to 0.

Table 2: SPS syntax of the proposed method

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| sps_extension_flag | u(1) |
| if (sps_extension_flag) |  |
| { |  |
| sps_range_extension_flag | u(1) |
| sps_extension_7bits | u(7) |
| } |  |
| if (sps_range_extension_flag) |  |
| sps_range_extension( ) |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 3: SPS range extension syntax of the proposed method

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| sps_cabac_bypass_alignment_enabled_flag | u(1) |
| } |  |

### Picture level signaling

The proposed bypass alignment method is primarily suitable for very high bit rate coding. However, even for very high bit rate coding, the bit-rate may still vary significantly picture to picture. At same quantization parameter, the coded bit-rate depends on the complexity/characteristics of the picture. The bit-rate of a picture may also depend on the GOP structure. Generally, pictures at higher temporal layers are coded as lower bitrate than pictures at lower temporal layer. Table 4 shows the variation of the coded bitrate with different temporal id of SVT16 sequences. VTM-11.0 with QP=-33 is used to generate the data. As shown in Table 4, there is a significant difference in bitrate between temporal id 0 and 5. For OldTownCross sequence, frames with temporal layer 0 are coded as 7.08 Gbps, whereas, the frames with temporal layer 5 are coded as 4.91 Gbps.

Table 4: Variation of bit-rate with different temporal id

| SVT16 QP = -33 | Coded bitrate in Gbps in different temporal id |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| OldTownCross | 7. 08 | 6.51 | 6.52 | 5.67 | 5.25 | 4.91 |
| CrowdRun | 6.73 | 6.14 | 6.13 | 5.31 | 4.79 | 4.43 |

Based on the above rational, in addition to SPS level control, additional picture level flag is also proposed to enable/disable of the proposed alignment process in the picture level.

The semantic of the proposed picture level flag is defined as follows (in the following semantic italic part are valid only for option 2):

ph_cabac_bypass_alignment_flag equal to 1 specifies that the value of ivlCurrRange may be aligned prior to bypass decoding of the syntax elements sb_coded_flag[][],  abs_remainder[ ], dec_abs_level[ n ], and coeff_sign_flag[ ] for the slices of the current picture. ph_cabac_bypass_alignment_flag equal to 0 specifies that the value of ivlCurrRange is not aligned prior to bypass decoding of the slices of the current picture. When not present, the value of ph_cabac_bypass_alignment_flag is inferred to be equal to 0.

As shown in Table 5, the proposed picture level flag is conditioned on the SPS flag. The ph_cabac_bypass_alignment_flag is signaled if SPS flag  sps_cabac_bypass_alignment_enabled_flag is equal to 1.

Table 5: Picture level signaling of the proposed method based on the value of SPS flag

| picture_header_structure( ) { |  |
| --- | --- |
| if( pps_dbf_info_in_ph_flag ) { |  |
| ph_deblocking_params_present_flag | u(1) |
| if( ph_deblocking_params_present_flag ) { |  |
| if( !pps_deblocking_filter_disabled_flag ) |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_luma_beta_offset_div2 | se(v) |
| ph_luma_tc_offset_div2 | se(v) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if(sps_cabac_bypass_alignment_enabled_flag) |  |
| ph_cabac_bypass_alignment_flag | u(1) |
| if( pps_picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| } |  |

# Simulation results

The proposed methods are implemented on top of VTM-11.0. Regarding high level syntax part, only SPS flag is implemented. The performance of the proposed methods are evaluated based on the common test conditions suggested in [2]. Table 6, Table 7 and Table 8 show the results of option 1 of SVT, HLG, and PQ sequences, respectively as compared to VTM-11.0. The results of option 2 are tabulated in Table 9, Table 10, and Table 11.

Table 6: Results of option 1 on top of VTM-11.0 ( SVT sequences)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.24% | 0.29% | 0.29% | 100% | 103% |
| SVT12 | 0.48% | 0.54% | 0.54% | 100% | 101% |
| Overall | 0.36% | 0.41% | 0.41% | 100% | 102% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.42% | 0.42% | 0.43% | 101% | 103% |
| SVT12 | 0.71% | 0.73% | 0.73% | 100% | 102% |
| Overall | 0.57% | 0.58% | 0.58% | 101% | 102% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 |  |  |  |  |  |
| SVT12 |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Overall RGB |  |  |  |  |  |

Table 7: Results of option 1 on top of VTM-11.0 ( HLG sequences)

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.98% | 1.04% | 1.04% | 100% | 99% |
| HLG422 | 0.94% | 0.99% | 0.99% | 100% | 99% |
| Overall | 0.96% | 1.02% | 1.01% | 100% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 |  |  |  |  |  |
| HLG422 |  |  |  |  |  |
| Overall |  |  |  |  |  |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 |  |  |  |  |  |
| HLG422 |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Overall HLG |  |  |  |  |  |

Table 8: Results of option 1 on top of VTM-11.0 ( PQ sequences)

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.08% | 1.15% | 1.20% | 1.07% | 1.13% | 1.19% | 102% | 98% |
| PQ422 | 0.98% | 1.05% | 1.05% | 0.97% | 1.03% | 1.03% | 101% | 99% |
| Overall | 1.03% | 1.10% | 1.12% | 1.02% | 1.08% | 1.11% | 101% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.40% | 1.92% | 1.74% | 1.37% | 1.85% | 1.73% | 100% | 99% |
| PQ422 | 1.32% | 1.69% | 1.69% | 1.29% | 1.65% | 1.68% | 100% | 99% |
| Overall | 1.36% | 1.80% | 1.72% | 1.33% | 1.75% | 1.70% | 100% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.34% | 1.85% | 1.71% | 1.30% | 1.79% | 1.69% | 101% | 99% |
| PQ422 | 1.32% | 1.64% | 1.58% | 1.28% | 1.60% | 1.58% | 100% | 99% |
| Overall | 1.33% | 1.74% | 1.65% | 1.29% | 1.69% | 1.64% | 100% | 99% |
| Overall PQ | 1.24% | 1.55% | 1.50% | 1.21% | 1.51% | 1.48% | 101% | 99% |

Table 9: Results of option 2 on top of VTM-11.0 ( SVT sequences)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.24% | 0.29% | 0.29% | 100% | 103% |
| SVT12 | 0.48% | 0.54% | 0.54% | 100% | 101% |
| Overall | 0.36% | 0.41% | 0.41% | 100% | 102% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.42% | 0.42% | 0.43% | 101% | 102% |
| SVT12 | 0.71% | 0.73% | 0.73% | 100% | 101% |
| Overall | 0.57% | 0.58% | 0.58% | 100% | 102% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 |  |  |  |  |  |
| SVT12 |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Overall RGB |  |  |  |  |  |

Table 10: Results of option 2 on top of VTM-11.0 ( HLG  sequences)

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.98% | 1.04% | 1.04% | 101% | 99% |
| HLG422 | 0.94% | 0.99% | 0.99% | 100% | 99% |
| Overall | 0.96% | 1.02% | 1.01% | 100% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 |  |  |  |  |  |
| HLG422 |  |  |  |  |  |
| Overall |  |  |  |  |  |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 |  |  |  |  |  |
| HLG422 |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Overall HLG |  |  |  |  |  |

Table 11: Results of option 2 on top of VTM-11.0 ( PQ  sequences)

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.08% | 1.14% | 1.17% | 1.07% | 1.13% | 1.15% | 101% | 98% |
| PQ422 | 0.98% | 1.04% | 1.04% | 0.97% | 1.02% | 1.03% | 100% | 99% |
| Overall | 1.03% | 1.09% | 1.10% | 1.02% | 1.08% | 1.09% | 101% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.40% | 1.92% | 1.72% | 1.36% | 1.85% | 1.71% | 101% | 98% |
| PQ422 | 1.33% | 1.67% | 1.69% | 1.29% | 1.64% | 1.69% | 100% | 98% |
| Overall | 1.37% | 1.79% | 1.70% | 1.33% | 1.75% | 1.70% | 100% | 98% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-11.0 CTC |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.34% | 1.85% | 1.71% | 1.30% | 1.78% | 1.69% | 101% | 99% |
| PQ422 | 1.31% | 1.65% | 1.59% | 1.28% | 1.60% | 1.59% | 100% | 98% |
| Overall | 1.32% | 1.75% | 1.65% | 1.29% | 1.69% | 1.64% | 100% | 98% |
| Overall PQ | 1.24% | 1.54% | 1.49% | 1.21% | 1.50% | 1.48% | 101% | 99% |

The proposed methods are also implemented on top of CE-1.2.a [3]. Table 12, Table 13 and Table 14 show the results of option 1 of SVT, HLG, and PQ sequences, respectively as compared to CE-1.2.a. As shown in Table 12, the option 1 impacts the overall coding performance by 0.58% (G), 0.60%(B), and 0.60%(R) of SVT sequences, which is not significant in the given operating range. Although the performance impact is higher for HLG and PQ sequences, one can disable the proposed tool using the proposed high level flags for those cases.

Table 12: Results of option 1 on top of CE-1.2.a ( SVT sequences)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.44% | 0.48% | 0.48% | 99% | 91% |
| SVT12 | 0.52% | 0.56% | 0.57% | 100% | 99% |
| Overall | 0.48% | 0.52% | 0.52% | 99% | 95% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.55% | 0.53% | 0.54% | 100% | 91% |
| SVT12 | 0.70% | 0.73% | 0.73% | 100% | 99% |
| Overall | 0.62% | 0.63% | 0.63% | 100% | 95% |
|  |  |  | RA |  |  |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.55% | 0.54% | 0.54% | 100% | 92% |
| SVT12 | 0.70% | 0.74% | 0.74% | 100% | 99% |
| Overall | 0.62% | 0.64% | 0.64% | 100% | 95% |
| Overall RGB | 0.58% | 0.60% | 0.60% | 100% | 95% |

Table 13: Results of option 1 on top of CE-1.2.a ( HLG sequences)

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.98% | 1.05% | 1.05% | 100% | 99% |
| HLG422 | 0.94% | 0.99% | 1.00% | 100% | 99% |
| Overall | 0.96% | 1.02% | 1.02% | 100% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 1.47% | 1.44% | 1.47% | 100% | 99% |
| HLG422 | 1.46% | 1.43% | 1.48% | 100% | 98% |
| Overall | 1.47% | 1.44% | 1.47% | 100% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 |  |  |  |  |  |
| HLG422 |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Overall HLG |  |  |  |  |  |

Table 14: Results of option 1 on top of CE-1.2.a ( PQ sequences)

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1.2.a |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.08% | 1.14% | 1.15% | 1.07% | 1.13% | 1.13% | 101% | 100% |
| PQ422 | 0.98% | 1.06% | 1.06% | 0.97% | 1.04% | 1.04% | 100% | 100% |
| Overall | 1.03% | 1.10% | 1.10% | 1.02% | 1.08% | 1.09% | 101% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over CE-1.2.a |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.40% | 1.92% | 1.76% | 1.36% | 1.86% | 1.74% | 101% | 99% |
| PQ422 | 1.32% | 1.72% | 1.68% | 1.28% | 1.68% | 1.68% | 101% | 100% |
| Overall | 1.36% | 1.82% | 1.72% | 1.32% | 1.77% | 1.71% | 101% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over CE-1.2.a |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.34% | 1.87% | 1.71% | 1.30% | 1.80% | 1.69% | 101% | 99% |
| PQ422 | 1.31% | 1.65% | 1.60% | 1.28% | 1.62% | 1.60% | 101% | 99% |
| Overall | 1.32% | 1.76% | 1.66% | 1.29% | 1.71% | 1.65% | 101% | 99% |
| Overall PQ | 1.24% | 1.56% | 1.49% | 1.21% | 1.52% | 1.48% | 101% | 100% |

Table 15, Table 16 and Table 17 shows the results of option 2 of SVT, HLG, and PQ sequences, respectively. It is shown that the coding performance of option 1 and option 2 is similar, however, option 2 requires fewer number of alignments.

Table 15: Results of option 2 on top of CE-1.2.a ( SVT sequences)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.44% | 0.48% | 0.48% | 99% | 91% |
| SVT12 | 0.52% | 0.56% | 0.56% | 100% | 100% |
| Overall | 0.48% | 0.52% | 0.52% | 99% | 95% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.55% | 0.54% | 0.54% | 100% | 92% |
| SVT12 | 0.70% | 0.73% | 0.73% | 100% | 99% |
| Overall | 0.62% | 0.63% | 0.63% | 100% | 96% |
|  |  |  | RA |  |  |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.55% | 0.54% | 0.54% | 100% | 93% |
| SVT12 | 0.70% | 0.74% | 0.74% | 100% | 100% |
| Overall | 0.62% | 0.64% | 0.64% | 100% | 96% |
| Overall RGB | 0.58% | 0.60% | 0.60% | 100% | 96% |

Table 16: Results of option 2 on top of CE-1.2.a ( HLG sequences)

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.98% | 1.05% | 1.05% | 100% | 99% |
| HLG422 | 0.94% | 0.99% | 0.99% | 100% | 99% |
| Overall | 0.96% | 1.02% | 1.02% | 100% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 1.48% | 1.44% | 1.46% | 100% | 99% |
| HLG422 | 1.46% | 1.45% | 1.48% | 101% | 99% |
| Overall | 1.47% | 1.44% | 1.47% | 100% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over CE-1.2.a |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 |  |  |  |  |  |
| HLG422 |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Overall HLG |  |  |  |  |  |

Table 17: Results of option 2 on top of CE-1.2.a ( PQ sequences)

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1.2.a |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.08% | 1.15% | 1.15% | 1.07% | 1.14% | 1.14% | 101% | 100% |
| PQ422 | 0.98% | 1.05% | 1.05% | 0.97% | 1.03% | 1.03% | 100% | 100% |
| Overall | 1.03% | 1.10% | 1.10% | 1.02% | 1.08% | 1.08% | 101% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over CE-1.2.a |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.39% | 1.92% | 1.75% | 1.36% | 1.85% | 1.73% | 101% | 99% |
| PQ422 | 1.32% | 1.72% | 1.69% | 1.28% | 1.67% | 1.69% | 100% | 100% |
| Overall | 1.36% | 1.82% | 1.72% | 1.32% | 1.76% | 1.71% | 101% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over CE-1.2.a |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.33% | 1.86% | 1.71% | 1.30% | 1.80% | 1.70% | 101% | 99% |
| PQ422 | 1.32% | 1.65% | 1.61% | 1.28% | 1.62% | 1.61% | 100% | 99% |
| Overall | 1.32% | 1.75% | 1.66% | 1.29% | 1.71% | 1.65% | 101% | 99% |
| Overall PQ | 1.24% | 1.56% | 1.49% | 1.21% | 1.52% | 1.48% | 101% | 99% |

# Conclusions

This contribution proposes two options of CABAC bypass coding alignment to improve the throughput of the CABAC in the high bit-rate applications. In the first option, only the bypass alignment is applied to coefficient coding within a coefficient group (CG), without affecting the coding of sb_coded_flag of a transform block (TB). In the second option, in addition to CABAC bypass alignment, it is also proposed to switch to bypass coding of sb_coded_flag after the limit of context coded bins has been reached for the current TB. The number of alignments are reduced in the second option as compared to the first option.  In both options since the bypass bins are directly visible in ivlOffset, N bypass coded bins can be decoded just by looking at the N most-significant-but-one bits of ivlOffset concatenated with the bit-stream which ultimately allows decoding of multiple bypass coded bins in parallel.

# References

K. Sharman, N. Saunders, J. Gamei, “AHG 5 and 18: Entropy Coding Throughput for High Bit Depths”, JCTVC-N0190, July 2013.

A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-T2018, Oct 2020.

A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, “CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-T2022, Oct 2020.

M. G. Sarwer, R. L. Liao, J. Luo, Y. Ye, “ Non-CE7: Signaling of coded_sub_block_flag”, JVET-P0370, Oct 2019.

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, Oct 2020.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).