| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21th Meeting: by teleconference, 6 – 15 Jan 2021 | Document: JVET-U0074 |
| --- | --- |

| Title: | AHG 11: Exploration Experiments of SSIM based CNN model for in-loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tong Ouyang Han Zhu Zhenzhong Chen Wuhan University Xiaozhong Xu Shan Liu Tencent | Tel: Email: | oyjiyu@whu.edu.cn zhuhanlyx@whu.edu.cn zzchen@whu.edu.cn xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of Exploration Experiment 1 on Neural Network based filtering for video coding of JVET-T0069. In the test, the impact of Squeeze-and-excitation network (SE block) and SSIM based loss functions are investigated. More specifically, several sets of experiments are set up to explore their impacts separately.

Due to the limitation of computer resources and time, four sets of experiments are set up, which are implemented on VTM-10.0. The experimental results under All Intra (AI) and Random Access (RA) configuration with 5 QPs recommended by AHG11 are reported in this contribution. From the results, the SE-block is proven to have more BD-Rate savings over PSNR as well as VMAF (Video Multimethod Assessment Fusion). SSIM based loss function shows superiority in preserving subjective quality in compressed video, while the model trained with L1 loss performs better in saving more BD-Rate over PSNR.

Introduction

The architecture of the proposed network is shown in Fig. 1. The network mainly consists of 8 residual units with some 3x3 convolutional layers. Apart from the last convolution layer that generates a 3-channel output, other convolutional layers generate the same amount of feature maps (denoted as M=64). Two SE blocks and a 3×3 convolution layer are appended in the last of the network to adaptively fuse the preceding feature maps and generate the final reconstructed images. Note that 1x1 convolutional layer is not employed in the first unit. The global identity skip connection further facilitates the flow of shallow features.

Fig. 1 The architecture of proposed network.

The architecture of SE block is shown in Fig.2. The feature maps are squeezed into channel-wise statistics, which is produced by aggregating feature maps across their spatial dimensions (H×W). The aggregation is followed by an excitation operation, which takes the form of a simple self-gating mechanism. It takes the channel descriptor as input and produces a collection of per-channel modulation weights. These weights are applied to the feature maps U the generate the output of the SE block which can be fed directly into subsequent layers of the network. SE block adaptively recalibrates channel-wise feature responses by explicitly modeling interdependencies between channels, which benefits the model in learning the compressed information.

Fig. 2 The architecture of SE block.

In the contribution JVET-T0069, Equation 1 is chosen as the loss function used in training a model, namely the SSIM based loss function, while in this contribution, the subjective and objective performance are evaluated the SSIM based function and L1 loss. The L1 loss function is also known as the least absolute deviation (LAD) and the least absolute error (LAE). In general, it minimizes the sum of the absolute difference between the target value and the estimated value.

|  | LsΘ=i=1n0.8×SSIMFXi;Θ,Yi+0.2×FXi;Θ-Yi1, | (1) |
| --- | --- | --- |

Where Θ denotes the parameters of proposed network, Yi indicates the ground truth of the compressed image patch Xi.

Test Descriptions

Tests

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 | The proposed network trained with different loss functions (L1 loss) | Tong Ouyang, Xiaozhong Xu. |  |
| 2 | The proposed network without using SE block | Tong Ouyang, Xiaozhong Xu. |  |
| 3 | The proposed network trained with L1 loss, without using SE block | Tong Ouyang, Xiaozhong Xu. |  |

Based on the original scheme (Jvet-T0069), the network structure and loss functions are modified to explore their impact on performance respectively. 4 groups of experiments including the original scheme are set up. The specific settings of each group of experiments are shown below.

- Original scheme: The network structure is shown in Figure 1, which contains SE blocks. The SSIM based loss is used for model training, as shown in Equation (1). The scheme is retested with QPs = {22, 27, 32, 37, 42} under AI and RA configurations. (Results in Section 3.1)

- Test 1: The architecture is the same as Figure 1, while L1 loss was used during training. (Results in Section 3.2)

- Test 2: Every SE block is removed from the network architecture and the model is trained using SSIM based losses. (Results in Section 3.3)

- Test 3: All the SE blocks are removed from the original network, L1 loss is used as loss function for training. (Results in Section 3.4)

Simulation results

Original scheme

The experimental results of improving the subjective as well as the objective quality are reported in the flowing tables.

Table 1 Experimental results (All Intra)

|  | BD-rate Over |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | VMAF | EncT | DecT | bit DIFF |
| Class A1 | -2.13% | -5.09% | -4.97% | -6.43% | 182% | 44696% | 0% |
| Class A2 | -2.60% | -11.23% | -13.32% | -8.14% | 155% | 28308% | 0% |
| Class B | -1.60% | -11.71% | -8.69% | -7.12% | 134% | 26136% | 0% |
| Class C | -2.89% | -12.02% | -10.63% | -8.79% | 115% | 24669% | 0% |
| Class E | -4.13% | -8.67% | -8.10% | -9.25% | 142% | 36764% | 0% |
| Overall | -2.56% | -10.09% | -9.17% | -7.90% | 141% | #DIV/0! | 0% |
| Class D | -3.50% | -10.84% | -11.87% | -8.08% | 107% | 20776% | 0% |
| Class F | -1.11% | -5.77% | -4.44% | -4.52% | 109% | 22041% | 0% |

Table 2 Experimental results (Random Access)

|  | BD-rate Over |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | VMAF | EncT | DecT | bit DIFF |
| Class A1 | -0.63% | 0.98% | 3.81% | -5.02% | 152% | 54625% | 0% |
| Class A2 | -1.35% | -10.64% | -13.84% | -5.74% | 147% | 27710% | 0% |
| Class B | -0.61% | -11.30% | -6.57% | -5.77% | 150% | 16891% | 0% |
| Class C | -0.66% | -8.25% | -6.32% | -6.30% | 130% | 18818% | 0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.78% | -7.90% | -5.88% | -5.76% | 144% | 24272% | 0% |
| Class D | -0.39% | -7.35% | -7.00% | -5.36% | 127% | 31258% | 1% |
| Class F | 0.51% | -2.73% | -1.73% | -1.68% | 168% | 34482% | 1% |

Test 1 (Jvet-T0069 trained with L1 loss)

Table 9 Experimental results (All Intra)

|  | BD-rate Over |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | VMAF | EncT | DecT | bit DIFF |
| Class A1 | -2.47% | -7.75% | -8.36% | -4.82% | 172% | 32705% | 0% |
| Class A2 | -2.68% | -10.22% | -8.06% | -4.03% | 145% | 37858% | 0% |
| Class B | -2.22% | -10.08% | -9.68% | -3.32% | 130% | 32593% | 0% |
| Class C | -3.32% | -10.50% | -10.41% | -4.04% | 111% | 22771% | 0% |
| Class E | -4.04% | -6.98% | -8.36% | -4.78% | 142% | 39898% | 0% |
| Overall | -2.89% | -9.29% | -9.14% | -4.09% | 136% | 31933% | 0% |
| Class D | -4.20% | -8.82% | -11.42% | -4.50% | 106% | 20655% | 0% |
| Class F | -1.33% | -5.66% | -4.86% | -2.06% | 108% | 16565% | 0% |

Table 10 Experimental results (Random Access)

|  | BD-rate Over |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | VMAF | EncT | DecT | bit DIFF |
| Class A1 | -2.68% | -8.73% | -7.84% | -4.00% | 148% | 43337% | 0% |
| Class A2 | -2.68% | -12.57% | -9.06% | -2.77% | 146% | 25576% | 0% |
| Class B | -1.91% | -10.92% | -9.32% | -2.53% | 144% | 28465% | 0% |
| Class C | -1.93% | -9.31% | -8.05% | -2.31% | 126% | 23402% | 0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -2.23% | -10.38% | -8.63% | -2.81% | 140% | 28764% | 0% |
| Class D | -2.37% | -7.95% | -8.93% | -2.90% | 120% | 14513% | 0% |
| Class F | -0.55% | -3.71% | -3.27% | -0.82% | 165% | 7841% | 0% |

Test 2 (Jvet-T0069 remove SE-block)

Table 6 Experimental results (All Intra)

|  | BD-rate Over |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | VMAF | EncT | DecT | bit DIFF |
| Class A1 | -1.54% | -2.94% | -4.87% | -6.32% | 185% | 43613% | 0% |
| Class A2 | -1.39% | -7.50% | -5.52% | -7.59% | 144% | 25101% | 0% |
| Class B | -0.53% | -6.43% | -5.68% | -6.21% | 129% | 21291% | 0% |
| Class C | -1.44% | -5.81% | -6.31% | -7.69% | 111% | 20608% | 0% |
| Class E | -2.38% | -4.98% | -5.94% | -9.14% | 138% | 36121% | 0% |
| Overall | -1.35% | -5.65% | -5.70% | -7.28% | 137% | 26738% | 0% |
| Class D | -1.36% | -1.61% | -4.76% | -7.05% | 105% | 19461% | 0% |
| Class F | -0.58% | -3.71% | -3.41% | -4.26% | 109% | 22137% | 0% |

Table 7 Experimental results (Random Access)

|  | BD-rate Over |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | VMAF | EncT | DecT | bit DIFF |
| Class A1 | -0.41% | 4.52% | 8.45% | -5.80% | 148% | 52080% | 1% |
| Class A2 | 0.22% | -7.74% | -4.86% | -7.12% | 145% | 26559% | 0% |
| Class B | 0.22% | -5.91% | -4.04% | -5.29% | 144% | 14098% | 0% |
| Class C | 0.20% | -4.15% | -3.94% | -5.47% | 123% | 16106% | 0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.09% | -3.72% | -1.68% | -5.80% | 139% | 21532% | 0% |
| Class D | 0.87% | -0.36% | -1.97% | -4.46% | 118% | 25381% | 1% |
| Class F | 0.79% | -1.57% | -0.41% | -1.91% | 162% | 31049% | 1% |

Test 3 (Jvet-T0069 trained with L1 loss and remove SE-blocks)

Table 12 Experimental results (All intra)

|  | BD-rate Over |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | VMAF | EncT | DecT | bit DIFF |
| Class A1 | -2.02% | -5.36% | -6.26% | -4.54% | 167% | 17794% | 0% |
| Class A2 | -2.25% | -7.31% | -6.17% | -3.82% | 136% | 24026% | 0% |
| Class B | -1.86% | -7.42% | -8.01% | -3.38% | 123% | 18686% | 0% |
| Class C | -2.88% | -9.43% | -10.16% | -4.24% | 107% | 19755% | 0% |
| Class E | -3.78% | -6.33% | -7.03% | -5.39% | 135% | 35234% | 0% |
| Overall | -2.50% | -7.32% | -7.73% | -4.17% | 130% | 21749% | 0% |
| Class D | -3.17% | -7.48% | -10.04% | -4.66% | 103% | 18352% | 0% |
| Class F | -1.41% | -4.79% | -5.18% | -2.04% | 105% | 10646% | 0% |

Table 13 Experimental results (Random Access)

|  | BD-rate Over |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | VMAF | EncT | DecT | bit DIFF |
| Class A1 | -2.48% | -6.23% | -6.28% | -3.96% | 145% | 36254% | 0% |
| Class A2 | -2.43% | -10.56% | -7.84% | -3.30% | 144% | 23107% | 0% |
| Class B | -1.70% | -8.35% | -7.76% | -2.61% | 142% | 23751% | 0% |
| Class C | -1.77% | -8.04% | -8.10% | -2.38% | 125% | 20262% | 0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -2.02% | -8.29% | -7.57% | -2.96% | 138% | 24640% | 0% |
| Class D | -1.81% | -6.57% | -8.26% | -2.87% | 119% | 11842% | 0% |
| Class F | -0.69% | -3.36% | -3.84% | -0.67% | 157% | 6963% | 0% |

Evaluation tools

In experimental results, additional VMAF score is provided for evaluating the subjective quality of compressed video. In this section, the information of tools used to calculate VMAF score is described in detail, the download links are also provided.

- ffmpeg:

ffmpeg version N-54088-gb0203fa72b-static. https://johnvansickle.com/ffmpeg/

- VMAF model:

The version of model used for calculating VMAF scores is v0.6.1. https://github.com/Netflix/vmaf/blob/v1.5.3/model/vmaf_v0.6.1.pkl

- Command line:

ffmpeg -s 1920x1080 -pix_fmt yuv420p10le -i MarketPlace_intra.yuv -s 1920x1080 -pix_fmt yuv420p10le -i MarketPlace.yuv -lavfi libvmaf="model_path=vmaf_v0.6.1.pkl:log_path= MarketPlace.log" -f null -

Training stage

The proposed network is trained with DIV2K dataset, there are 800 images in its training set and 100 images in its validation set. All the images are compressed under the common test conditions (AI configuration and QP= {22, 27, 32, 37, 42}) with VTM-10.0. More information related with the network architecture are reported in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | CPU: Intel® Xeon® CPU E5-2637 v4 @ 3.50GHz x8, 64GB GPU: GTX 1080ti x8 x 11GB |
|  | SW environment: | OS: CentOS Linux Release 7.4 GPU: CUDA 9.0, cuDNN 7.1.4, nvidia driver 396.37 |
|  | Framework: | Pytorch v0.4 |
|  | Epoch: | 300 |
|  | Batch size: | 2Kx16 |
|  | Training time: | 57h |
|  | Training data information: | DIV2K (800 images for training and 100 for validation) |
|  | Configurations for generating compressed training data (if different to VTM CTC): | QP= {22,27,32,37,42}, AI. |
| Optional | Patch size | 128x128 |
|  | Learning rate: | 5e-4 |
|  | Optimizer: | ADAM |
|  | Loss function: | 0.8×SSIM+0.2×L1 |
|  | Preprocessing: | Mean shift |
|  | Other information: | NO |

Inference stage

The proposed model is tested with 4 different QPs= {22, 27, 32, 37, 42} under AI and RA configuration.

Table 2. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | CPU: Intel® Xeon® CPU E5-2637 v4 @ 3.50GHz x8, 64GB GPU: NO |
|  | SW environment: | OS: CentOS Linux Release 7.4 GPU: NO |
|  | Framework: | Pytorch v0.4 |
|  | Total Conv. Layers | 45 |
|  | Total FC Layers | 0 |
|  | Total Parameter Number | 131299 |
|  | Parameter Precision | 32(F) |
|  | Memory Parameter (MB) | 0.50 |
|  | Memory Temp (MB) | 32399.36 (input size: 3840x2160) |
|  | MAC (Giga) | 988 Giga (input size: 3840x2160) |
| Optional | Preprocessing: | Mean shift |
|  | Other information: | NO |

Conclusion

By comparing the results between section 3.1 and 3.3, it can be seen that SE blocks play a significant role in BD-Rate saving in both subjective and objective aspects. When SE blocks are removed from the network, the BD-Rate savings decrease to different extents no matter what loss function is used to train the model.

In the results in section 3.2 and 3.4, it’s noticed that removing SE blocks would reduce the BD-rate savings on PSNR, but bring a higher BD-rate savings on VMAF, when the network is trained with L1 loss. Such results are related to the mechanism of SE block, which adds channel attention to the features to make the output closer to its ground truth. However, it should be noted that higher PSNR does not mean higher visual quality. The network without using SE block is not trained to improve the objective quality simply, so it’s reasonable that it has better performance in the improvement of subjective quality.

By Comparing the results between Section 3.1 and 3.2, 3.2 and 3.4, the SSIM based loss function does not show superiority in BD-Rate saving over PSNR than the L1 loss, but it is superior than the L1 loss in subjective quality preservation, which is reflected in the VMAF index.

References

- Vmaf, https://github.com/Netflix/vmaf.

- J. Hu, L. Shen and G. Sun, “Squeeze-and-Excitation Networks,” 2018 IEEE/CVF Conference on Computer Vision and Pattern Recognition, Salt Lake City, UT, 2018, pp. 7132-7141.

- JVET-K0391, “AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao (Tencent), Ljubljana, SI, 10–18 July 2018

- JVET-L0242, “AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao (Tencent), Macao, CN, 03–12 Oct. 2018

- JVET-M0508, “AHG9: Test Results of Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent), Marrakech, MA, 9–18 Jan. 2019

- JVET-N0254, “CE13: Dense Residual Convolutional Neural Network based In-Loop Filter (Test 2.2 and 2.3)”, Y. Wang, Z. Chen, Y. Li, L. Zhao, S. Liu, X. Li, Geneva CH, 19–27 March 2019

- JVET-O0101, “CE10: Dense Residual Convolutional Neural Network based In-Loop Filter (Tests 2.5 and 2.7)”, Y. Wang, T. Ouyang, C. Zou, Y. Li, Z. Chen (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent), Gothenburg, SE, 3–12 July 2019

- JVET-T0041, “Methodology and reporting template for neural network coding tool testing”, S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois, teleconference, 7-16 Oct. 2020

- Tong Ouyang, Feiyang Liu, Han Zhu, Zhenzhong Chen, Xiaozhong Xu, Shan Liu, “AHG 11: SSIM based CNN model for in-loop filtering”, JVET-T0069, Oct. 2020, teleconference

Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.