| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0078 |
| --- | --- |

| Title: | AHG9: Out-of-loop luma mapping with chroma scaling using APS or SEI message parameters signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Philippe de Lagrange, Fabrice Le Léannec | Tel: Email: | edouard.francois@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes a variant of LMCS, where it can be applied out of the decoding loop, instead of being applied inside the decoding loop as done in VTM11. Prior to the encoding process, luma mapping and cross-component chroma scaling are applied to the input video. After the decoding process, the decoded video is post-processed by an inverse luma mapping and inverse cross-component chroma scaling. The luma mapping function is built using the same process as in VTM. The cross-component chroma scaling function is a piece-wise linear function built from the VTM chroma scaling LUT values. In a first option, the out-of-loop LMCS parameters are signaled in the APS, as in current VVC specification, and a new SPS flag is added to indicate if LMCS is in-loop or out-of-loop. BD-rate variations compared to the VTM, for the CTCs HDR PQ content, are reported in the table below, for AI, RA and LB* configurations.

| Config. | DE100 | PSNRL100 | wPSNRY | wPSNRU | wPSNRV | PSNRY | PSNRU | PSNRV | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AI | -1.01% | -0.81% | -0.55% | -5.94% | -8.04% | -0.28% | -3.11% | -4.22% | 99% | 94% |
| RA | -2.06% | -1.62% | -1.12% | -5.89% | -8.98% | -0.86% | -2.08% | -3.69% | 96% | 84% |
| LB | -6.97% | -2.61% | -1.54% | -11.45% | -16.60% | -1.30% | -1.85% | -7.17% | 98% | 96% |

*LB configuration is not part of the HDR CTCs.

In another option, it is proposed to signal the out-of-loop LMCS parameters in a new SEI message (“Colour Transform Information”), inspired from the Colour Remapping Information SEI message specified in AVC and HEVC.

# Introduction

LMCS is a flexible in-loop filtering process that allows adapting the signal dynamically depending on its codewords statistics. In-loop LMCS allows storing in the DPB the pictures in their original (non-mapped) format. In the VTM, for SDR and HLG content, the luma mapping function is derived adaptively from the signal analysis. In addition, LMCS can be adaptively enabled or enabled depending on the picture type. On the contrary, for HDR HLG and PQ content, the luma mapping function is fixed.

In this contribution, it is proposed to enable LMCS to be performed out-of-loop. This feature can be of interest for some use cases or content types. In particular, for HDR PQ cases, where the VTM uses a fixed luma mapping function, it is observed that out-of-loop LMCS leads to compression gains compared to in-loop LMCS.

# Proposal

## Out-of-loop LMCS process overview

The VTM11 code is modified to enable LMCS to be applied out-of-loop. The input luma and chroma signals are mapped prior to being encoded by a pre-process. After decoding, the decoded pictures are processed by the inverse mapping. The luma component is mapped using the luma mapping function as defined in the VTM11. The chroma components are mapped by a cross-component scaling process defined as follows.

The scaling applied in the pre-processing step is directly applied to the each chroma sample Cin(p) at location p in the picture by the following process (shown with floating-point representation):

Cscale(p) = Clip3(0,2B-1, int(.5 + 2048./scale(Ycoloc)*(Cin(p) – 2B-1) + 2B-1 )

where scale(Ycoloc) is the scaling factor (or inverse scaling factor) derived from the collocated luma sample Ycoloc. The inverse scaling applies after decoding to the decoded sample Cdec(p) as follows:

Cscale(p) = Clip3(0,2B-1, ( scale(Ycoloc)*(Cdec(p) – 2B-1) + 2B-1+11 + 210 ) >> 11 )

The chroma scaling function scale() is derived from the luma mapping parameters defined in the LMCS APS as described in the next sub-section.

## Modified chroma scaling function

In the VVC11 LMCS design, the cross-component chroma scaling function is defined as successive fixed values per interval, which leads to a non-continuous function, as illustrated in the figure below (blue curve).

When the cross-component chroma scaling is applied out-of-loop, these discontinuities may lead to coding efficiency losses. Indeed, a slight variation of the luma value may lead to strong differences in the chroma scaling values. Because the luma signal is subject to compression errors, these differences may frequently happen.

To avoid this drawback, the chroma scaling function for the cross-component dcaling is derived as a piece-wise linear function with pivot points chrPivot[i], i=0 to 16, derived from the table ChromaScaleCoeff[i] (see VVC specification), as follows:

for i = 1..15

chrPivot[i] = (ChromaScaleCoeff[i] + ChromaScaleCoeff[i - 1] + 1) / 2

chrPivot[0] = ChromaScaleCoeff[0]

chrPivot[16] = ChromaScaleCoeff[15]

The new function scale() derived from the pivot points chrPivot[i] is depicted in the figure below (orange curve).

Figure 1. Chroma scaling function in VTM and in the proposal.

## Signalling

In a first option, it is proposed to keep the existing LMCS parameters signaling in the APS, and to add a flag in the SPS to indicate if LMCS is to be applied in the decoding loop, or after the decoding process.

The syntax change is indicated in yellow in the SPS syntax table below.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| if(sps_lmcs_enabled_flag) |  |
| sps_lmcs_out_of_loop_flag | u(1) |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The semantics of sps_lmcs_out_of_loop_flag is defined as follows.

sps_lmcs_out_of_loop_flag equal to 1 specifies that LMCS is performed inside the decoding loop.  sps_lmcs_out_of_loop_flag equal to 0 specifies that LMCS is performed after the decoding loop to the reconstructed samples.

# Simulation results

The proposal is implemented in VTM11.0. The corresponding patch is attached to the contribution. Out-of-loop LMCS is enabled by setting the macro LMCS_OLF to 1. For the proposal, a new parameter, LMCSOutOfLoop, is added. Simulations for the proposal are made with LMCSOffset set to 0 and LMCSOutOfLoop set to 1.

Out-of-loop LMCS has been tested against in-loop LMCS for HDR PQ content, using the fixed luma mapping function as derived in the VTM11.0. Simulations have been performed for the AI, RA configurations as specified in the HDR CTCs (JVET-U2011). LB configuration has also been tested.

Results are summarized in the table below. Detailed results are reported in the attached excel file. Gains are observed for all metrics and all configurations.

|  |  | wPSNR |  |  | PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| All Intra |  |  |  |  |  |  |  |  |  |
| -1.01% | -0.81% | -0.55% | -5.94% | -8.04% | -0.28% | -3.11% | -4.22% | 99% | 94% |
| Random Access Gop 32 |  |  |  |  |  |  |  |  |  |
| -2.06% | -1.62% | -1.12% | -5.89% | -8.98% | -0.86% | -2.08% | -3.69% | 96% | 84% |
| Low Delay B |  |  |  |  |  |  |  |  |  |
| -6.97% | -2.61% | -1.54% | -11.45% | -16.60% | -1.30% | -1.85% | -7.17% | 98% | 96% |

# Alternate option: Colour Transform Information SEI message

Alternatively, instead of using the APS signalling, it is proposed to define a new SEI message, named Colour Transform Information SEI message, inspired from the Colour Remapping Information (CRI) SEI message defined in HEVC and AVC.

The CRI defines the characteristics of the post-processed signal (bit-depth, colour primaries, transfer function, matrix coefficients). It also defines the parameters of three transforms, made of 3 pre-LUTs, applying to each colour component, a 3x3 matrix, applying after the application of the pre-LUTs, and 3 post-LUTs, applying to each colour component.

The main differences of the new SEI message, compared to the CRI are:

- A syntax element “colour_transform_pre_lut_cross_component_flag” is added to indicate the chroma mapping mode using the pre-LUTs, which can be either an intra-component mapping, or a cross-component scaling.

- A syntax element “colour_transform_pre_lut_cross_component_inferred_flag” is added to indicate if the chroma pre-LUTs are inferred or not from the luma pre-LUTs when the cross-component scaling mode is used.

- A syntax element is added to indicate if the x-coordinates of the pivot points of the pre- and post-LUTs are equally distributed.

- The maximum number of pivot points of the pre- and post-LUTs is reduced to 32 (instead of 256).

A tentative syntax proposal is presented in the table below. Main changes compared to CRI syntax are highlighted in yellow.

| colour_transform_info( payloadSize ) { | Descriptor |
| --- | --- |
| colour_transform_id | ue(v) |
| colour_transform_cancel_flag | u(1) |
| if( !colour_transform_cancel_flag ) { |  |
| colour_transform_persistence_flag | u(1) |
| colour_transform_video_signal_info_present_flag | u(1) |
| if( colour_transform_video_signal_info_present_flag ) { |  |
| colour_transform_full_range_flag | u(1) |
| colour_transform_primaries | u(8) |
| colour_transform_transfer_function | u(8) |
| colour_transform_matrix_coefficients | u(8) |
| } |  |
| colour_transform_input_bit_depth | u(8) |
| colour_transform_output_bit_depth | u(8) |
| colour_transform_pre_lut_cross_component_flag | u(1) |
| if (colour_transform _pre_lut_cross_component_flag) { |  |
| colour_transform_pre_lut_cross_comp_inferred_flag | u(1) |
| } |  |
| for( c = 0; c < colourTransformPreLutNumber; c++ ) { |  |
| pre_lut_num_val_minus1[ c ] | u(5) |
| if( pre_lut_num_val_minus1[ c ] > 0 ) { |  |
| pre_lut_uniform_sampling_flag | u(1) |
| for( i = 0; i <= pre_lut_num_val_minus1[ c ]; i++ ) { |  |
| if( pre_lut_uniform_sampling_flag = = 0 ) |  |
| pre_lut_coded_value[ c ][ i ] | u(v) |
| pre_lut_target_value[ c ][ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| colour_transform_matrix_present_flag | u(1) |
| if( colour_transform_matrix_present_flag ) { |  |
| log2_matrix_denom | u(4) |
| for( c = 0; c < 3; c++ ) |  |
| for( i = 0; i < 3; i++ ) |  |
| colour_transform_coeffs[ c ][ i ] | se(v) |
| } |  |
| for( c = 0; c < 3; c++ ) { |  |
| post_lut_num_val_minus1[ c ] | u(5) |
| if( post_lut_num_val_minus1[ c ] > 0 ) { |  |
| post_lut_uniform_sampling_flag | u(1) |
| for( i = 0; i <= post_lut_num_val_minus1[ c ]; i++ ) { |  |
| if( post_lut_uniform_sampling_flag = = 0 ) |  |
| post_lut_coded_value[ c ][ i ] | u(v) |
| post_lut_target_value[ c ][ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

colourTransformPreLutNumber is set to 3 when colour_transform_pre_lut_cross_comp_inferred_flag is equal to 0. colourTransformPreLutNumber is set to 1 when colour_transform_pre_lut_cross_comp_inferred_flag is equal to 1.

In case the chroma scaling pre-LUTs are inferred from the luma pre-LUTs, the chroma scaling pre-LUTs are derived in a similar way as done in the VTM, from the slopes of the luma pre-LUT.

As in the LMCS APS signalling, the value signalled in pre_lut_target_value[ c ][ i ] can be a delta value relative to pre_lut_target_value[ c ][ i – 1 ] (and same for pre_lut_coded_value, post_lut_coded_value, post_lut_target_value). And the pivot points are deduced from these delta values.

# Conclusion

In this contribution the application of LMCS out-of-loop has been studied. For HDR PQ content, it has been shown that good coding efficiency can be obtained. One exemplary use case of the out-of-loop LMCS is in the context of the energy usage control by the end-user. Out-of-loop LMCS can be used to map the input HDR PQ video to a version close to SDR format. At decoder side, the end-user can choose to display the decoded video as is, with lower brightness and therefore lower energy consumption from the display, or the apply the inverse mapping process to go back to the HDR PQ version which may require higher energy consumption from the display.

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Acknowledgement: this work has been achieved in the context of the 3EMS project funded by the “Region Bretagne”.