| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0079-v1 |
| --- | --- |

| Title: | A DNN Architecture for Intra-Frame Coding in YUV 4:2:0 format with Cross-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Ankitesh K. Singh, Muhammed Coban, Marta Karczewicz | Email: | hegilmez@qti.qualcomm.com ankitesh@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Most existing deep neural network (DNN) based video coding architectures are designed to operate in non-subsampled input formats such as RGB or YUV 4:4:4. However, video coding standards such as HEVC and VVC are designed primarily for the YUV 4:2:0 colour format. In the 20th JVET meeting, JVET-T0123 introduced three distinct network architectures including a solution jointly coding luma and chroma components. This contribution document proposes two alternative joint coding designs for intra-frame coding in YUV 4:2:0 colour format. The experimental results demonstrate that the proposed methods outperform HM-16.20 about 12.5% in luma coding. Over the joint coding scheme in JVET-T0123, Method 1 provides 6.43% Y, 5.56% U, 6.76% V, and Method 2 achieves 6.64% Y, 6.97% U, 8.34% V coding improvements.

# Introduction

Recent studies in DNN-based compression have shown that competitive coding gains can be achieved as compared to existing standards in RGB colour format, where DNNs are trained and evaluated on RGB input frames. However, most practical codecs work with the subsampled YUV 4:2:0 input colour format, and there is very little or no work on DNN designs for YUV 4:2:0 in the literature. Although the same DNN architecture with different set of weights can be used to support non-subsampled colour formats such as RGB and YUV 4:4:4, handling of YUV 4:2:0 in DNNs is non-trivial. Moreover, it is needed for a proper comparison against existing standards (e.g., HEVC [1] and VVC [2]) that are primarily focusing on the YUV 4:2:0 input.

This input document introduces two front-end architectures by comparing against the solutions proposed in JVET-T0123. Note that the proposed front-end architectures accommodating the YUV 4:2:0 input format are applicable to many DNN-based compression architectures, including the ones in [3][4] originally designed for RGB input format. As in JVET-T0123, to fairly compare different architectures, this document also integrates the front-end architectures to the mean-scale hyperprior based DNN scheme in [4] where both training and testing are performed on the CTC videos in YUV 4:2:0 format.

- Front-end architectures in JVET-T0123

In JVET-T0123, three different methods for handling YUV 4:2:0 input are integrated to the mean-scale hyperprior based DNN scheme in [4]:

- Joint coding: In this approach luma and chroma channels go through separate convolution and GDN layers and combined before the second convolution layer as illustrated in Figure 1. This method introduces an additional GDN module compared to the original scheme in [4].

- Separate coding: In this approach, luma and chroma channels are coded separately using separate neural codec based on [4]. This method doubles the number of parameters of the original network and disconnects luma and chroma channels as shown in Figure 2.

- Six-channel coding: In this method, luma channel is subsampled by 2 in each dimension, resulting in 4 channels. The luma channels are stacked with 2 chroma channels (6-channel input) and processed by [4] as shown in Figure 3. This method has additional parameters in the first convolutional layer due to increase input channel size.

Figure 1: Illustration of joint coding architecture integrated to [4] that directly works with the 4:2:0 input (Y, U and V jointly).

Figure 2: Illustration of separate coding architecture integrated to [4] where separate models are trained and tested separately for luma (Y) and chroma channels (U and V) where K=5 for luma and K=3 for chroma channels.

Figure 3: Illustration of six-channel coding architecture integrated to [4] where 6 input channels obtained by subsampling Y component into 4 partitions matching dimensions of U and Y.

- DNN for joint coding with cross-component prediction

This document introduces two architectures illustrated in Figures 4 and 5, where Y and UV channels are fed into two separated layers initially and combined using a 1x1 convolutional layer (which combines values across channels). The 1x1 convolution performs a linear combination of across Y, U and V components and serves as a cross component prediction, and then a pReLU operator is used for nonlinear mapping. Note that Method 1 in Figure 4, PReLU is only used after 1x1 convolution, and Method 2 in Figure 5 completely replaces GDN layers with PReLU.

Although both methods are joint coding schemes, i.e., like the one in JVET-T0123 (see in Figure 1), the 1x1 convolutional layer with pReLU operator can provide significant coding gains over all three solutions in JVET-T0123 as will be demonstrated in Section 5.

Figure 4: (Method 1) Front-end architecture for joint coding with the 1x1 convolution layer and a single PReLU operator

Figure 5: (Method 2) Front-end architecture for joint coding with the 1x1 convolution layer where all GDN layers are replaced with PReLU.

- Training and Testing

The front-end architectures are integrated to [4] as discussed in Section 2, and the training and testing is performed on CompressAI framework [5]. For training mean-squared error (MSE) is used as the distortion metric, and the trained network is tested and evaluated using PSNR. Specifically, the distortion term in RD(β) = β×D+R is formulated as:

D = (8×MSEY + 2×MSEU + 2×MSEV)/12

which combines MSEs from Y, U and V components such that the weighted terms are proportional to pixels (i.e., the contribution of Y is 4 times that of U or V, being proportional to the size of each component in YUV 4:2:0 format).

For testing the bitstream generation is done by using the ANS entropy coding (implemented in compressAI) on latent (hyper) variable coefficients, and the bitrates are obtained from the compressed bitstream files.

The training is performed on the Vimeo 90K triplet data set [7] where the RGB images are converted to YUV 4:2:0 using the method specified in JVET-T0041[9].The model was trained for five rate points for each architecture. The training details are summarized in Table 1.

Table 1: Neural network training parameters.

| Dataset | Vimeo90K (converted to YUV420p) |
| --- | --- |
| Input Size | 256x256(Y),128x128(UV) |
| Iterations | 4M |
| Batch Size | 8 |
| Optimizer | Adam |
| Learning rate | 10-4 (inital), 10-5(auxiliary) |
| Learning rate decay schedule (fine tuning) | the initial learning rate times 0.1 after 2M iteration |

The β values of {0.005, 0.01, 0.025, 0.1, 0.2} are used in compressAI. The associated rate points for these β parameters roughly correspond to bitrates for 5 QP parameters, 42, 37, 32, 27 and 22 in HEVC, respectively.

# Experimental Results

The DNN-based compression experiments are performed are performed on the CTC sequences in [6]. As suggested in JVET-T0041, the results are compared against HM-16.20 at 5 QP points (42, 37, 32, 27 and 22). For the quality metric only PSNR is used.

Tables 2 and 3 demonstrate the results of proposed methods with cross-component prediction support discussed in Section 3 over HM-16.20.

Table 2: Performance of Method 1 (with single PReLU) in this proposal.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over HM-16.20 |  |  |
|  | Y | U | V |
| Class A1 | -19.12% | 109.65% | -11.03% |
| Class A2 | -19.68% | 83.79% | 85.44% |
| Class B | -9.47% | 19.83% | -1.85% |
| Class C | -5.91% | 24.96% | 18.98% |
| Class E | -11.98% | 16.68% | -14.73% |
| Overall | -12.41% | 46.07% | 13.65% |

Table 3: Performance of Method 2 (removing all GDNs) in this proposal.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over HM-16.20 |  |  |
|  | Y | U | V |
| Class A1 | -18.12% | 74.46% | -10.32% |
| Class A2 | -19.26% | 70.31% | 79.23% |
| Class B | -10.16% | 23.22% | -4.30% |
| Class C | -6.20% | 21.44% | 10.98% |
| Class E | -13.08% | 12.85% | -10.92% |
| Overall | -12.61% | 37.48% | 10.91% |

Table 4 shows the average BD-rate results from JVET-T0123 (used as anchors in Tables 5, 6 and 7) evaluated using 5 QP points according to JVET-T0041 against HM-16.20. Note that the results in JVET-T0123 are evaluated using 4 QP points according to [8].

Table 4: Overall performance of the approaches from JVET-T0123 over HM.16.20

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over HM-16.20 |  |  |
|  | Y | U | V |
| Joint | -6.20% | 69.29% | 20.76% |
| Separate | -6.25% | 73.80% | 25.78% |
| Six-channel | -5.48% | 70.53% | 16.96% |

Tables 5, 6 and 7 provide a summary of the overall coding gains of Methods 1 and 2 over the approaches in JVET-T0123.

Table 5: Overall performance of Methods 1 and 2 over joint coding from JVET-T0123

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over joint coding in JVET-T0123 |  |  |
|  | Y | U | V |
| Method 1 | -6.43% | -5.56% | -6.76% |
| Method 2 | -6.64% | -6.97% | -8.34% |

Table 6: Overall performance of Methods 1 and 2 over separate coding from JVET-T0123

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over separate coding in JVET-T0123 |  |  |
|  | Y | U | V |
| Method 1 | -6.57% | -4.51% | -7.57% |
| Method 2 | -6.81% | -6.04% | -9.07% |

Table 7: Overall performance of Methods 1 and 2 over six-channel coding from JVET-T0123

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over six-channel coding in JVET-T0123 |  |  |
|  | Y | U | V |
| Method 1 | -6.61% | -5.91% | -3.50% |
| Method 2 | -6.84% | -7.47% | -5.30% |

- Conclusion

This document proposes two different methods for the joint coding of luma and chroma components in a DNN-based architecture for coding YUV 4:2:0 videos. The performance evaluations show that significant coding gains can be achieved over the approaches in JVET-T0123. It is suggested to integrate both methods (Methods 1 and 2) in CompressAI framework to support YUV 4:2:0 for future work. A further study on balancing luma and chroma coding gains is also encouraged. JVET-U0080 demonstrates the feasibility of such balancing as an initial study.

- References

- HEVC Test Model (HM) 16.20, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/ HM-16.20/.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10),” JVET-S2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 19th Meeting, 22 June–1 July 2020.

- J. Ballé, D. Minnen, S. Singh, S. J. Hwang, and N. Johnston, ‘Variational image compression with a scale hyperprior’, arXiv:1802.01436.

- D. Minnen, J. Ballé, and G. Toderici, ‘Joint Autoregressive and Hierarchical Priors for Learned Image Compression’, arXiv:1809.02736.

- J. Bégaint, F. Racapé, S. Feltman, A. Pushparaja, “CompressAI: A PyTorch library and evaluation platform for end-to-end compression research,” m54467, July 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- Vimeo 90K Triplet dataset, http://toflow.csail.mit.edu/index.html#triplet.

- Draft test conditions for DNNVC, w19583, ISO/IEC JTC 1/SC 29/WG 11, July 2020.

- S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois, “Methodology and reporting template for neural network coding tool testing”, JVET-T0041, October 2020.

- A. K. Singh, H. E. Egilmez, M. Coban, M. Karczewicz, “A study of handling YUV420 input format for DNN-based video coding”, JVET-T0123, October 2020.

- A. K. Singh, H. E. Egilmez, M. Coban, M. Karczewicz, “Balancing Luma-Chroma Channel Coding for DNN-based Intra-Frame Coding in JVET-U0079”, JVET-T0080, January 2021.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).