| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0082-v1 |
| --- | --- |

| Title: | AHG9: Scalability dimension SEI message and three HEVC SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang, Li Zhang, Kai Zhang, Zhipin Deng, Yang Wang (Bytedance) Anthony Vetro (MERL) Marta Mrak, Saverio Blasi (BBC) | Tel: Email: | yekui.wang@bytedance.com avetro@merl.com marta.mrak@bbc.co.uk |
| Source: | Bytedance Inc., Mitsubishi Electric Research Laboratories, BBC R&D |  |  |

_____________________________

# Abstract

This contribution reproposes the scalability dimension SEI message proposed in JVET-T0070, for signalling of scalability dimension information for a VVC bitstream, including 1) whether the bitstream is a multiview bitstream, and if yes, the view ID of each layer, and 2) whether one or more layers in the bitstream represent auxiliary information such as alpha, depth, etc., and if yes, which layers represent what.

Furthermore, this contribution also proposes to include three HEVC SEI messages into VSEI/VVC: the multiview acquisition information SEI message, the depth representation information SEI message, and the alpha channel information SEI message.

# Introduction

JVET-T0070 proposed the scalability dimension SEI message. The JVET meeting minutes includes the following notes on JVET-T0070:

"Additional information would be needed for complete interpretation such as camera parameters, depth channel interpretation, alpha channel interpretation specification.

... It was commented that the SEI message approach seems like a good one for handling this sort of information.

Further study was encouraged toward development of a more complete approach."

VVC supports multi-layer scalabilities. However, given a VVC multi-layer bitstream, it is unknown whether the bitstream of an OLS is a multiview bitstream or just a bitstream consisting of multiple layers with SNR and/or spatial scalabilities. Furthermore, given a VVC multi-layer bitstream, it is unknown whether there is one or more layers representing auxiliary information such as alpha, depth, etc., and if yes, which layers represent what. To address these issues, an SEI message, named scalability dimension SEI message, is proposed.

For sufficient support of multiview and applications involving depth and alpha signals, we also propose the following three HEVC SEI messages: the multiview acquisition information SEI message, which carries camera parameters, the depth representation information SEI message, and the alpha channel information SEI message.

# Proposal

The syntax and semantics of the proposed SEI messages are provided below, same as in JVET-T0070 and HEVC, respectively.

## Scalability dimension SEI message

| scalability_dimension( payloadSize ) { | Descriptor |
| --- | --- |
| sd_max_layers_minus1 | u(6) |
| sd_multiview_info_flag | u(1) |
| sd_auxilary_info_flag | u(1) |
| if( sd_multiview_info_flag ¦ ¦ sd_auxilary_info_flag ) { |  |
| if( sd_multiview_info_flag ) |  |
| sd_view_id_len | u(4) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| if( sd_multiview_info_flag ) |  |
| sd_view_id_val[ i ] | u(v) |
| if( sd_auxilary_info_flag ) |  |
| sd_aux_id[ i ] | u(8) |
| } |  |
| } |  |
| } |  |

The scalability dimension SEI message provides the scalability dimension information for each layer in bitstreamInScope (defined below), such as 1) when bitstreamInScope may be a multiview bitstream, the view ID of each layer; and 2) when there may be auxilary information (such as deppth or alpha) carried by one or more layers in bitstreamInScope, the auxilary ID of each layer.

The bitstreamInScope is the sequence of AUs that consists, in decoding order, of the AU containing the current scalability dimension SEI message, followed by zero or more AUs, including all subsequent AUs up to but not including any subsequent AU that contains a scalability dimension SEI message.

sd_max_layers_minus1 plus 1 indicates the maximum number of layers in bitstreamInScope.

sd_multiview_info_flag equal to 1 indicates that bitstreamInScope may be a multiview bitstream and the sd_view_id_val[ ] syntax elements are present in the scalability dimension SEI message. sd_multiview_flag equal to 0 indicates that bitstreamInScope is not a multiview bitstream and the sd_view_id_val[ ] syntax elements are not present in the scalability dimension SEI message.

sd_auxilary_info_flag equal to 1 indicates that there may be auxilary information carried by one or more layers in bitstreamInScope and the sd_aux_id[ ] syntax elements are present in the scalability dimension SEI message. sd_auxilary_info_flag equal to 0 indicates that there is no auxilary information carried by one or more layers in bitstreamInScope and the sd_aux_id[ ] syntax elements are not present in the scalability dimension SEI message.

sd_view_id_len specifies the length, in bits, of the sd_view_id_val[ i ] syntax element.

sd_view_id_val[ i ] specifies the view ID of the i-th layer in bitstreamInScope. The length of the sd_view_id_val[ i ] syntax element is sd_view_id_len bits. When not present, the value of sd_view_id_val[ i ] is inferred to be equal to 0.

sd_aux_id[ i ] equal to 0 indicates that the i-th layer in bitstreamInScope does not contain auxilary pictures. sd_aux_id[ i ] greater than 0 indicates the type of auxiliary pictures in the i-th layer in bitstreamInScope as specified in Table 1.

Table 1 – Mapping of sd_aux_id[ i ] to the type of auxiliary pictures

| sd_aux_id[ i ] | Name | Type of auxiliary pictures |
| --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3..127 |  | Reserved |
| 128..159 |  | Unspecified |
| 160..255 |  | Reserved |

NOTE 1 – The interpretation of auxiliary pictures associated with sd_aux_id in the range of 128 to 159, inclusive, is specified through means other than the sd_aux_id value.

sd_aux_id[ i ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, for bitstreams conforming to this version of this Specification. Although the value of sd_aux_id[ i ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, in this version of this Specification, decoders shall allow values of sd_aux_id[ i ] in the range of 0 to 255, inclusive.

## Multiview acquisition information SEI message

| multiview_acquisition_info( payloadSize ) { | Descriptor |
| --- | --- |
| intrinsic_param_flag | u(1) |
| extrinsic_param_flag | u(1) |
| if( intrinsic_param_flag ) { |  |
| intrinsic_params_equal_flag | u(1) |
| prec_focal_length | ue(v) |
| prec_principal_point | ue(v) |
| prec_skew_factor | ue(v) |
| for( i = 0; i <= intrinsic_params_equal_flag ? 0 : numViewsMinus1; i++ ) { |  |
| sign_focal_length_x[ i ] | u(1) |
| exponent_focal_length_x[ i ] | u(6) |
| mantissa_focal_length_x[ i ] | u(v) |
| sign_focal_length_y[ i ] | u(1) |
| exponent_focal_length_y[ i ] | u(6) |
| mantissa_focal_length_y[ i ] | u(v) |
| sign_principal_point_x[ i ] | u(1) |
| exponent_principal_point_x[ i ] | u(6) |
| mantissa_principal_point_x[ i ] | u(v) |
| sign_principal_point_y[ i ] | u(1) |
| exponent_principal_point_y[ i ] | u(6) |
| mantissa_principal_point_y[ i ] | u(v) |
| sign_skew_factor[ i ] | u(1) |
| exponent_skew_factor[ i ] | u(6) |
| mantissa_skew_factor[ i ] | u(v) |
| } |  |
| } |  |
| if( extrinsic_param_flag ) { |  |
| prec_rotation_param | ue(v) |
| prec_translation_param | ue(v) |
| for( i = 0; i <= numViewsMinus1; i++ ) |  |
| for( j = 0; j < 3; j++ ) { /* row */ |  |
| for( k = 0; k < 3; k++ ) { /* column */ |  |
| sign_r[ i ][ j ][ k ] | u(1) |
| exponent_r[ i ][ j ][ k ] | u(6) |
| mantissa_r[ i ][ j ][ k ] | u(v) |
| } |  |
| sign_t[ i ][ j ] | u(1) |
| exponent_t[ i ][ j ] | u(6) |
| mantissa_t[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |

The multiview acquisition information SEI message specifies various parameters of the acquisition environment. Specifically, intrinsic and extrinsic camera parameters are specified. These parameters could be used for processing the decoded views prior to rendering on a 3D display.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the multiview acquisition information SEI message applies as specified in clause D.3.1.

When present, the multiview acquisition information SEI message that applies to the current layer shall be included in an access unit that contains an IRAP picture that is the first picture of a CLVS of the current layer. The information signalled in the SEI message applies to the CLVS.

When the multiview acquisition information SEI message is included in a scalable nesting SEI message, the syntax elements bitstream_subset_flag, nesting_op_flag and all_layers_flag in the scalable nesting SEI message shall be equal to 0.

The variable numViewsMinus1 is derived as follows:

- If the multiview acquisition information SEI message is not included in a scalable nesting SEI message, numViewsMinus1 is set equal to 0.

- Otherwise (the multiview acquisition information SEI message is included in a scalable nesting SEI message), numViewsMinus1 is set equal to nesting_num_layers_minus1.

Some of the views for which the multiview acquisition information is included in a multiview acquisition information SEI message may not be present.

In the semantics below, index i refers to the syntax elements and variables that apply to the layer with nuh_layer_id equal to nestingLayerIdList[ 0 ][ i ].

The extrinsic camera parameters are specified according to a right-handed coordinate system, where the upper left corner of the image is the origin, i.e., the ( 0, 0 ) coordinate, with the other corners of the image having non-negative coordinates. With these specifications, a 3-dimensional world point, wP = [ x y z ] is mapped to a 2-dimensional camera point, cP[ i ] = [ u v 1 ], for the i-th camera according to:

s * cP[ i ] = A[ i ] * R−1[ i ] * ( wP − T[ i ] )	(X)

where A[ i ] denotes the intrinsic camera parameter matrix, R−1[ i ] denotes the inverse of the rotation matrix R[ i ], T[ i ] denotes the translation vector and s (a scalar value) is an arbitrary scale factor chosen to make the third coordinate of cP[ i ] equal to 1. The elements of A[ i ], R[ i ] and T[ i ] are determined according to the syntax elements signalled in this SEI message and as specified below.

intrinsic_param_flag equal to 1 indicates the presence of intrinsic camera parameters. intrinsic_param_flag equal to 0 indicates the absence of intrinsic camera parameters.

extrinsic_param_flag equal to 1 indicates the presence of extrinsic camera parameters. extrinsic_param_flag equal to 0 indicates the absence of extrinsic camera parameters.

intrinsic_params_equal_flag equal to 1 indicates that the intrinsic camera parameters are equal for all cameras and only one set of intrinsic camera parameters are present. intrinsic_params_equal_flag equal to 0 indicates that the intrinsic camera parameters are different for each camera and that a set of intrinsic camera parameters are present for each camera.

prec_focal_length specifies the exponent of the maximum allowable truncation error for focal_length_x[ i ] and focal_length_y[ i ] as given by 2−prec_focal_length. The value of prec_focal_length shall be in the range of 0 to 31, inclusive.

prec_principal_point specifies the exponent of the maximum allowable truncation error for principal_point_x[ i ] and principal_point_y[ i ] as given by 2−prec_principal_point. The value of prec_principal_point shall be in the range of 0 to 31, inclusive.

prec_skew_factor specifies the exponent of the maximum allowable truncation error for skew factor as given by 2−prec_skew_factor. The value of prec_skew_factor shall be in the range of 0 to 31, inclusive.

sign_focal_length_x[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the horizontal direction is positive. sign_focal_length_x[ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_x[ i ] specifies the exponent part of the focal length of the i-th camera in the horizontal direction. The value of exponent_focal_length_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length.

mantissa_focal_length_x[ i ] specifies the mantissa part of the focal length of the i-th camera in the horizontal direction. The length of the mantissa_focal_length_x[ i ] syntax element is variable and determined as follows:

- If exponent_focal_length_x[ i ] is equal to 0, the length is Max( 0, prec_focal_length − 30 ).

- Otherwise (exponent_focal_length_x[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_focal_length_x[ i ] + prec_focal_length − 31 ).

sign_focal_length_y[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the vertical direction is positive. sign_focal_length_y[ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_y[ i ] specifies the exponent part of the focal length of the i-th camera in the vertical direction. The value of exponent_focal_length_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length.

mantissa_focal_length_y[ i ] specifies the mantissa part of the focal length of the i-th camera in the vertical direction.

The length of the mantissa_focal_length_y[ i ] syntax element is variable and determined as follows:

- If exponent_focal_length_y[ i ] is equal to 0, the length is Max( 0, prec_focal_length − 30 ).

- Otherwise (exponent_focal_length_y[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_focal_length_y[ i ] + prec_focal_length − 31 ).

sign_principal_point_x[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the horizontal direction is positive. sign_principal_point_x[ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_x[ i ] specifies the exponent part of the principal point of the i-th camera in the horizontal direction. The value of exponent_principal_point_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point.

mantissa_principal_point_x[ i ] specifies the mantissa part of the principal point of the i-th camera in the horizontal direction. The length of the mantissa_principal_point_x[ i ] syntax element in units of bits is variable and is determined as follows:

- If exponent_principal_point_x[ i ] is equal to 0, the length is Max( 0, prec_principal_point − 30 ).

- Otherwise (exponent_principal_point_x[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_principal_point_x[ i ] + prec_principal_point − 31 ).

sign_principal_point_y[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the vertical direction is positive. sign_principal_point_y[ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_y[ i ] specifies the exponent part of the principal point of the i-th camera in the vertical direction. The value of exponent_principal_point_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point.

mantissa_principal_point_y[ i ] specifies the mantissa part of the principal point of the i-th camera in the vertical direction. The length of the mantissa_principal_point_y[ i ] syntax element in units of bits is variable and is determined as follows:

- If exponent_principal_point_y[ i ] is equal to 0, the length is Max( 0, prec_principal_point − 30 ).

- Otherwise (exponent_principal_point_y[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_principal_point_y[ i ] + prec_principal_point − 31 ).

sign_skew_factor[ i ] equal to 0 indicates that the sign of the skew factor of the i-th camera is positive.

sign_skew_factor[ i ] equal to 1 indicates that the sign is negative.

exponent_skew_factor[ i ] specifies the exponent part of the skew factor of the i-th camera. The value of exponent_skew_factor[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified skew factor.

mantissa_skew_factor[ i ] specifies the mantissa part of the skew factor of the i-th camera. The length of the mantissa_skew_factor[ i ] syntax element is variable and determined as follows:

- If exponent_skew_factor[ i ] is equal to 0, the length is Max( 0, prec_skew_factor − 30 ).

- Otherwise (exponent_skew_factor[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_skew_factor[ i ] + prec_skew_factor − 31 ).

The intrinsic matrix A[ i ] for i-th camera is represented by

(X)

prec_rotation_param specifies the exponent of the maximum allowable truncation error for r[ i ][ j ][ k ] as given by 2−prec_rotation_param. The value of prec_rotation_param shall be in the range of 0 to 31, inclusive.

prec_translation_param specifies the exponent of the maximum allowable truncation error for t[ i ][ j ] as given by 2−prec_translation_param. The value of prec_translation_param shall be in the range of 0 to 31, inclusive.

sign_r[ i ][ j ][ k ] equal to 0 indicates that the sign of ( j, k ) component of the rotation matrix for the i-th camera is positive. sign_r[ i ][ j ][ k ] equal to 1 indicates that the sign is negative.

exponent_r[ i ][ j ][ k ] specifies the exponent part of ( j, k ) component of the rotation matrix for the i-th camera. The value of exponent_r[ i ][ j ][ k ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified rotation matrix.

mantissa_r[ i ][ j ][ k ] specifies the mantissa part of ( j, k ) component of the rotation matrix for the i-th camera. The length of the mantissa_r[ i ][ j ][ k ] syntax element in units of bits is variable and determined as follows:

- If exponent_r[ i ] is equal to 0, the length is Max( 0, prec_rotation_param − 30 ).

- Otherwise (exponent_r[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_r[ i ] + prec_rotation_param − 31 ).

The rotation matrix R[ i ] for i-th camera is represented as follows:

(X)

sign_t[ i ][ j ] equal to 0 indicates that the sign of the j-th component of the translation vector for the i-th camera is positive. sign_t[ i ][ j ] equal to 1 indicates that the sign is negative.

exponent_t[ i ][ j ] specifies the exponent part of the j-th component of the translation vector for the i-th camera. The value of exponent_t[ i ][ j ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified translation vector.

mantissa_t[ i ][ j ] specifies the mantissa part of the j-th component of the translation vector for the i-th camera. The length v of the mantissa_t[ i ][ j ] syntax element in units of bits is variable and is determined as follows:

- If exponent_t[ i ] is equal to 0, the length v is set equal to Max( 0, prec_translation_param − 30 ).

- Otherwise (0 < exponent_t[ i ] < 63), the length v is set equal to Max( 0, exponent_t[ i ] + prec_translation_param − 31 ).

The translation vector T[ i ] for the i-th camera is represented by:

(X)

The association between the camera parameter variables and corresponding syntax elements is specified by Table ZZ. Each component of the intrinsic and rotation matrices and the translation vector is obtained from the variables specified in Table ZZ as the variable x computed as follows:

- If e is in the range of 0 to 63, exclusive, x is set equal to ( −1 )s * 2e − 31 * ( 1 + n ÷ 2v ).

- Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989.

Table ZZ – Association between camera parameter variables and syntax elements.

| x | s | e | n |
| --- | --- | --- | --- |
| focalLengthX[ i ] | sign_focal_length_x[ i ] | exponent_focal_length_x[ i ] | mantissa_focal_length_x[ i ] |
| focalLengthY[ i ] | sign_focal_length_y[ i ] | exponent_focal_length_y[ i ] | mantissa_focal_length_y[ i ] |
| principalPointX[ i ] | sign_principal_point_x[ i ] | exponent_principal_point_x[ i ] | mantissa_principal_point_x[ i ] |
| principalPointY[ i ] | sign_principal_point_y[ i ] | exponent_principal_point_y[ i ] | mantissa_principal_point_y[ i ] |
| skewFactor[ i ] | sign_skew_factor[ i ] | exponent_skew_factor[ i ] | mantissa_skew_factor[ i ] |
| rE[ i ][ j ][ k ] | sign_r[ i ][ j ][ k ] | exponent_r[ i ][ j ][ k ] | mantissa_r[ i ][ j ][ k ] |
| tE[ i ][ j ] | sign_t[ i ][ j ] | exponent_t[ i ][ j ] | mantissa_t[ i ][ j ] |

## Depth representation information SEI message

| depth_representation_info( payloadSize ) { | Descriptor |
| --- | --- |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( d_min_flag ¦ ¦ d_max_flag ) |  |
| disparity_ref_view_id | ue(v) |
| if( z_near_flag ) |  |
| depth_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| depth_rep_info_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| depth_rep_info_element( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| depth_rep_info_element( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] |  |
| } |  |
| } |  |

The syntax elements in the depth representation information SEI message specify various parameters for auxiliary pictures of type AUX_DEPTH for the purpose of processing decoded primary and auxiliary pictures prior to rendering on a 3D display, such as view synthesis. Specifically, depth or disparity ranges for depth pictures are specified.

When present, the depth representation information SEI message shall be associated with one or more layers with AuxId value equal to AUX_DEPTH. The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the depth representation information SEI message applies.

When present, the depth representation information SEI message may be included in any access unit. It is recommended that, when present, the SEI message is included for the purpose of random access in an access unit in which the coded picture with nuh_layer_id equal to targetLayerId is an IRAP picture.

For an auxiliary picture with AuxId[ targetLayerId ] equal to AUX_DEPTH, an associated primary picture, if any, is a picture in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ targetLayerId ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] for all values of j in the range of 0 to 2, inclusive, and 4 to 15, inclusive.

The information indicated in the SEI message applies to all the pictures with nuh_layer_id equal to targetLayerId from the access unit containing the SEI message up to but excluding the next picture, in decoding order, associated with a depth representation information SEI message applicable to targetLayerId or to the end of the CLVS of the nuh_layer_id equal to targetLayerId, whichever is earlier in decoding order.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary pictures as specified in Table Y1. In Table Y1, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

The variable maxVal is set equal to ( 1  <<  ( 8 + bit_depth_luma_minus8 ) ) − 1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Table Y1 – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an auxiliary picture represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of an auxiliary picture represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| 2 | Each decoded luma sample value of an auxiliary picture represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 3 | Each decoded luma sample value of an auxiliary picture represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[ i ]. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

disparity_ref_view_id specifies the ViewId value against which the disparity values are derived.

NOTE 1 – disparity_ref_view_id is present only if d_min_flag is equal to 1 or d_max_flag is equal to 1 and is useful for depth_representation_type values equal to 1 and 3.

The variables in the x column of Table Y2 are derived from the respective variables in the s, e, n and v columns of Table Y2 as follows:

–	If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s * 2e − 31 * ( 1 + n ÷ 2v ).

–	Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 1 – The above specification is similar to that found in IEC 60559:1989.

Table Y2 – Association between depth parameter variables and syntax elements

| x | S | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture with ViewId equal to ViewId of the auxiliary picture.

The units for the ZNear and ZFar values, when present, are identical but unspecified.

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piece-wise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piece-wise linear segments for mapping of decoded luma sample values of an auxiliary picture to a scale that is uniformly quantized in terms of disparity. The values of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 2 – When depth_representation_type is equal to 3, an auxiliary picture contains nonlinearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform decoded depth sample values from the nonlinear representation to the linear representation, i.e., uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first ( 0, 0 ) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k  <=  depth_nonlinear_representation_num_minus1 + 1; k++ ) {
	pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )
	dev1 = depth_nonlinear_representation_model[ k ]
	pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )
	dev2 = depth_nonlinear_representation_model[ k + 1 ]	(X)

	x1 = pos1 − dev1
	y1 = pos1 + dev1
	x2 = pos2 − dev2
	y2 = pos2 + dev2

	for( x = Max( x1, 0 ); x  <=  Min( x2, maxVal ); x++ )
			DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )
}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

| depth_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

The syntax structure specifies the value of an element in the depth representation information SEI message.

The syntax structure sets the values of the OutSign, OutExp, OutMantissa and OutManLen variables that represent a floating-point value. When the syntax structure is included in another syntax structure, the variable names OutSign, OutExp, OutMantissa and OutManLen are to be interpreted as being replaced by the variable names used when the syntax structure is included.

da_sign_flag equal to 0 indicates that the sign of the floating-point value is positive. da_sign_flag equal to 1 indicates that the sign is negative. The variable OutSign is set equal to da_sign_flag.

da_exponent specifies the exponent of the floating-point value. The value of da_exponent shall be in the range of 0 to 27 − 2, inclusive. The value 27 − 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 27 − 1 as indicating an unspecified value. The variable OutExp is set equal to da_exponent.

da_mantissa_len_minus1 plus 1 specifies the number of bits in the da_mantissa syntax element. The value of da_mantissa_len_minus1 shall be in the range of 0 to 31, inclusive. The variable OutManLen is set equal to da_mantissa_len_minus1 + 1.

da_mantissa specifies the mantissa of the floating-point value. The variable OutMantissa is set equal to da_mantissa.

## Alpha channel information SEI message

| alpha_channel_info( payloadSize ) { | Descriptor |
| --- | --- |
| alpha_channel_cancel_flag | u(1) |
| if( !alpha_channel_cancel_flag ) { |  |
| alpha_channel_use_idc | u(3) |
| alpha_channel_bit_depth_minus8 | u(3) |
| alpha_transparent_value | u(v) |
| alpha_opaque_value | u(v) |
| alpha_channel_incr_flag | u(1) |
| alpha_channel_clip_flag | u(1) |
| if( alpha_channel_clip_flag ) |  |
| alpha_channel_clip_type_flag | u(1) |
| } |  |
| } |  |

The alpha channel information SEI message provides information about alpha channel sample values and post-processing applied to the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA and one or more associated primary pictures.

For an auxiliary picture with nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to AUX_ALPHA, an associated primary picture, if any, is a picture in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ nuhLayerIdA ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] for all values of j in the range of 0 to 2, inclusive, and 4 to 15, inclusive.

When an access unit contains an auxiliary picture picA with nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to AUX_ALPHA, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the alpha channel information SEI message applies.

alpha_channel_cancel_flag equal to 1 indicates that the alpha channel information SEI message cancels the persistence of any previous alpha channel information SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that alpha channel information follows.

Let currPic be the picture that the alpha channel information SEI message is associated with. The semantics of alpha channel information SEI message persist for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB with nuh_layer_id equal to targetLayerId in an access unit containing an alpha channel information SEI message with nuh_layer_id equal to targetLayerId is output having PicOrderCnt( picB ) greater than PicOrderCnt( currPic ), where PicOrderCnt( picB ) and PicOrderCnt( currPic ) are the PicOrderCntVal values of picB and currPic, respectively, immediately after the invocation of the decoding process for picture order count for picB.

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. Values greater than 2 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2.

alpha_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary picture. alpha_channel_bit_depth_minus8 shall be in the range 0 to 7 inclusive. alpha_channel_bit_depth_minus8 shall be equal to bit_depth_luma_minus8 of the associated primary picture.

alpha_transparent_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered transparent for purposes of alpha blending. The number of bits used for the representation of the alpha_transparent_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_opaque_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered opaque for purposes of alpha blending. The number of bits used for the representation of the alpha_opaque_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_channel_incr_flag equal to 0 indicates that the interpretation sample value for each decoded auxiliary picture luma sample value is equal to the decoded auxiliary picture sample value for purposes of alpha blending. alpha_channel_incr_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample value that is greater than Min( alpha_opaque_value, alpha_transparent_value ) should be increased by one to obtain the interpretation sample value for the auxiliary picture sample and any auxiliary picture luma sample value that is less than or equal to Min( alpha_opaque_value, alpha_transparent_value ) should be used, without alteration, as the interpretation sample value for the decoded auxiliary picture sample value. When not present, the value of alpha_channel_incr_flag is inferred to be equal to 0.

alpha_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded auxiliary picture. alpha_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded auxiliary picture are altered according to the clipping process described by the alpha_channel_clip_type_flag syntax element. When not present, the value of alpha_channel_clip_flag is inferred to be equal to 0.

alpha_channel_clip_type_flag equal to 0 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than ( alpha_opaque_value − alpha_transparent_value ) / 2 is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less or equal than ( alpha_opaque_value − alpha_transparent_value ) / 2 is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample. alpha_channel_clip_type_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than alpha_opaque_value is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less than or equal to alpha_transparent_value is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample.

NOTE – When both alpha_channel_incr_flag and alpha_channel_clip_flag are equal to one, the clipping operation specified by alpha_channel_clip_type_flag should be applied first followed by the alteration specified by alpha_channel_incr_flag to obtain the interpretation sample value for the auxiliary picture luma sample.

# Patent rights declaration(s)

Bytedance Inc., Mitsubishi Electric Research Laboratories, and the British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).