| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0085-v1 |
| --- | --- |

| Title: | Some VVC errata items |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Editorial input |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2, Yang Wang2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com wangyang.cs@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports some errata items in VVC.

# The errata items

- In clause D.2.2, the list VclAssociatedSeiList should include the value 132 (decoded picture hash). In HEVC, VclAssociatedSeiList includes the value 132.

- In clause D.2.2, the user_data_registered_itu_t_t35 and user_data_unregistered SEI messages should be handled the same way as 133 (scalable nesting) in the following constraint:

When there are multiple SEI messages with a particular value of payloadType not equal to 133 that are associated with a particular AU or DU and apply to a particular OLS or layer, regardless of whether some or all of these SEI messages are scalable-nested, the SEI messages shall have the same SEI payload content.

I.e., change it to be as follows (i.e., add the yellow-highlighted texts), such that specific rules are not specified for user-defined SEI messages:

When there are multiple SEI messages with a particular value of payloadType not equal to 4, 5, or 133 that are associated with a particular AU or DU and apply to a particular OLS or layer, regardless of whether some or all of these SEI messages are scalable-nested, the SEI messages shall have the same SEI payload content.

- Search "with sn_ols_flag is equal to 0" and remove "is".

- Search "recovery point point" and replace with "recovery point picture".

- Add the following clarification regarding SEI messages allowed to be nested in a scalable nesting SEI message in a suffix SEI NAL unit, e.g., as a NOTE in clause D.2.2:

Only filler payload, decoded picture hash, and scalable nesting SEI messages may be included in a suffix SEI NAL unit; all other SEI messages are not allowed to be included in a suffix SEI NAL unit. When there is a scalable nesting SEI message included in a suffix SEI NAL unit, it is only allowed to contain those SEI messages that are allowed to be included in a suffix NAL unit. When there is a scalable nesting SEI message included in a prefix SEI NAL unit, it is only allowed to contain those SEI messages that are allowed to be included in a prefix NAL unit. (Gary's comment: This should be body text instead of NOTE.)

- Make the following changes (i.e., add the yellow-highlighted texts):

It is a requirement of bitstream conformance that, when an SLI SEI message applicable to an OLS is present for a CVS, for all the SPSs referenced by the pictures in the multiSubpicLayers in the OLS, the value of sps_num_subpics_minus1 shall be the same and the value of sps_subpic_treated_as_pic_flag[ i ] shall be equal to 1 for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive.