| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0087 |
| --- | --- |

| Title: | AHG11: Updated information on inter-prediction coding tool with deep neural network |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zeqiang Li, Byeongdoo Choi, Wei Wang, Wei Jiang, Xiaozhong Xu, Shan Liu | Email: | {zeqiangli, bdchoi }@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This informative contribution reports the updated results of deep neural network (DNN) utilization for inter-prediction with newly designed and trained models. The basic framework is the same as JVET-T0058  whose idea is inserting of a new (virtual) reference picture, which is generated by inference processes of trained networks, into reference picture list (RPL). The new network consists of several sub-network models for flow estimation/compensation and detail enhancement. The new models show better coding gains for classes C & D (2.80%, 4.63% respectively in luma for RA) than other classes.

# Introduction

This informative contribution is the updated report of JVET-T0058 [1]. As a follow-work, we re-designed and trained new models with various training sequences. Compared to the previous model of JVET-T0058, the size and complexity of model is significantly reduced, so that the number of parameters becomes a quarter. Whereas the coding efficiency is improved up to 0.6% in small resolution. This contribution document consists of several sections with the introduction of the proposed DNN-based inter-prediction framework, neural network models, simulation results with coding performance and complexity analysis, and finally discussion on future directions.

# Framework of inter-prediction with neural networks

The basic framework is the same as JVET-T0058. Fig. 1 shows a basic inter-prediction framework with neural network models.

Figure 1. Proposed inter-prediction framework with neural network process

When multiple decoded pictures are stored in decoded picture buffer (DPB) as reference pictures for motion-compensated prediction, one or more forward and backward reference pictures may be available according to the GOP structure. By default, one nearest forward reference picture and one nearest backward one that have minimum POC distances from the current picture are used for generating the virtual reference picture. For example, as shown in Fig.1, when the POC value of the current picture is equal to 5 with a given GOP structure, the decoded reference pictures with POC values equal to 4 or 6 are used for generating a virtual reference picture of the current picture, as input data of network inference process. If the virtual reference picture is generated through multiple network inference processes including optical flow estimation-compensation and detail enhancement, the virtual reference picture that is synthesized to be similar to the current picture is stored in DPB as another reference picture and used for motion-compensated prediction. The POC value of the virtual reference picture is set equal to the POC value of the current picture (POC 5 in the example in Fig. 1). To avoid any conflict with temporal motion vector prediction or POC-based MV scaling, the values of MVs of the virtual reference picture are set equal to zero by default and the virtual picture is marked as “long-term reference”. Ideally, the virtual reference picture for en-/decoding the current picture can be used for en-/decoding the following picture. However, in our current implementation, the virtual reference picture is removed from DPB, when decoding process of the current picture is completed.

For coding large sequences (e.g. 4K or 8K), network inference process cannot be applied to the entire picture due to resource limits (e.g. memory). In that case, the virtual reference picture is hypothetically divided into multiple virtual regions and each region is independently generated through network inference process. The independently generated virtual reference regions are used for composition of the entire virtual reference or used for region-wise inter-prediction process. The following section have more details on network models for generation of virtual picture or region.

# Updated neural network models

For spatial alignment between multiple sequential pictures, usually optical flow estimation and compensation with inference process of pre-trained neural network are utilized in motion-compensated frame interpolation. As most existing methods usually estimate the bi-directional optical flows for two sequences, and then use linear algorithms to mix and combine them to approximate intermediate flows. From our research on frame interpolation or rate up-conversion based on neural network, however, we observed that the single flow estimation model can directly estimate the intermediate flows from images. Also, with the more precise flows, artifacts on motion boundaries can be significantly reduced. After calculating the intermediate flows from center reference frame to the other two frames, we use all original input frames and warped frames for fusion process to synthesize intermediate frame.

To further improve the frame’s quality, after getting the virtual reference frame from the frame generation model, all three frames will be fed into a detail enhancement module to strengthen the frame details based on the spatial and temporal information. To overcome the limits of the conventional approaches and generate the high-quality reference picture for better coding efficiency, we proposed three step (optical flow estimation, frame generation and detail enhancement) network inference processes with several pre-trained models, as shown in Fig. 2.

Figure 2. Proposed neural network process for generation of virtual reference picture

After calculating the intermediate flows from center reference frame to the other two frames (optical flow estimation), the warped frames can be generated with backward warping and then all be fed into fusion process (frame generation). The fusion process takes the original input frames, the warped frames, and the intermediate flows as inputs. Inside this module, a fusion map will be estimated along with another residual map, then the warped frames are linearly combined according to the fusion result and added with the residual map to do reconstruction to get intermediate frame.

Once an intermediate virtual reference is obtained, the image quality of the virtual reference picture may be enhanced with a post-processing, utilizing two input reference frames. The detail enhancement module uses enhanced deformable convolutional networks to collect information from two reference frames in order to estimate the offset flow for convolutional operation. It consists of two parts, PCD (Pyramid, Cascading and Deformable) alignment module for spatial and temporal optimization and TSA (Temporal and Spatial Attention) fusion module to apply attention to emphasize important features for subsequent restoration.

# Simulation results

The below tables show the complexity analysis and the coding performance based on [2]. Compared to JVET-T0058, the total parameter number and model size is reduced by 75%, and the coding gains are improved upto 0.6% and 0.7% in classes C & D, respectively.  [Simulation results will be updated]

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | CPU: Intel(R) Xeon(R) CPU E5-2698 v4 @ 2.20GHz GPU: Tesla V100-DGXS-32GB |
|  | SW environment: | OS: Ubuntu 18.04.2 LTS GPU driver: Nvidia driver 418.67 |
|  | Framework: | Pytorch 1.14 + CUDA 10.1 + cuDNN 7.6.4 |
|  | Epoch: | 50 |
|  | Batch size: | 4 |
|  | Training time: | 10 days |
|  | Training data information: | BVI-DVC [3] |
|  | Configurations for generating compressed training data (if different to VTM CTC): | Same as VTM CTC |

Table 2. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
|  | HW environment: | CPU: Intel Core i9-9900x CPU @ 3.60GHz, 32GB Memory GPU: RTX 2080ti-11GB |
|  | SW environment: | OS: Ubuntu 20.04.3 GPU driver: Nvidia driver 400 |
|  | Framework: | Pytorch 1.14 + CUDA 10.1 + cuDNN 7.6.4 |
|  | Total Parameter Number | 30, 384,025 |
|  | Model Size | 121.8 MB |
|  | MAC (Giga) | 100 ~ 200 GMac (unable to calculate, rough number) |

Table 3. BD-rate and Coding Time for NN-based Video Coding Tool Testing in Inference Stage 
(Tested 33frames with GOP=32 and patch size 256x256)

|  | Random Access Over VTM-10.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -2.80% | -5.65% | -4.45% | 1735.92% | 4345947.72% |  |  |
| Class E | O | O | O | O | O |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -4.63% | -3.55% | -3.55% | 607.50% | 3525513.58% |  |  |
| Class F |  |  |  |  |  |  |  |

# Discussion and conclusion

The model size and parameter number are significantly reduced by redesigning network models and the coding efficiency is considerably reduced. For improvement of coding efficiency, interactions with CU-level inter-prediction tools would be further studied. Most inter-prediction coding tools such as Merge, sub-CU inter-prediction (e.g affine mode), DMVR, and BIO are affected by the insertion of virtual reference picture. Those inter-prediction coding tools can be normatively or non-normatively optimized for better interactions with the proposed neural network based inter prediction.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Choi, Z. Li, W. Wang, W. Jiang, X. Xu, S. Liu, “AHG11: Information on inter-prediction coding tool with deep neural network”, ISO/IEC JTC1/SC29 JVET-T0058.

[2] S. Liu, A. Segall, E. Alshina, R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, ISO/IEC JTC1/SC29 JVET-T2006.

[3] Fan Zhang, Di Ma, David Bull: BVI-DVC. https://doi.org/10.5523/bris.3hj4t64fkbrgn2ghwp9en4vhtn