| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0089 |
| --- | --- |

| Title: | 8-bit profiles for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. Ye, G. Wu, L. Wang, J. Chen (Alibaba) L. Zhang, Y.-K. Wang, K. Zhang (Bytedance) M. Karczewicz (Qualcomm) Y.-W. Huang, S.-M. Lei (MediaTek) X. Wang (Kwai) D. Wang (OPPO) Wenpeng Ding (Baidu) Yi-Pin Hsiao (Vivo) Ping Wu (ZTE) M-L Champel (Xiaomi) Tarek Amata (Twitch) | Tel: Email: | yan.ye@alibaba-inc.com lizhang.idm@bytedance.com martak@qti.qualcomm.com yuwen.huang@mediatek.com xianglinwang@kwai.com wangdong7@oppo.com dingwenpeng@baidu.com yhsiao@vivo.com ping.wu@zte.com.cn champelmaryluc@xiaomi.com amatarek@twitch.tv |
| Source: | Alibaba, Bytedance, Qualcomm, MediaTek, Kwai, OPPO, Baidu, Vivo, ZTE, Xiaomi, Twitch |  |  |

_____________________________

# Abstract

This contribution proposes to establish Main 8 and Main 8 Still Picture profiles for VVC. It is asserted that today there are many video applications that still solely rely on video content in 8-bit 4:2:0 format. It is asserted that the proposed 8-bit profiles make VVC more accessible and friendlier to these video applications by establishing an interoperability point that is more relevant to them. It is claimed that with the proposed 8-bit profiles, VVC gains the opportunity for wider and swifter commercial adoption, and brings much needed bandwidth savings to these applications at a more favorable price performance ratio, thus creating a win-win situation. Further, along with the two 8-bit profiles aforementioned, this contribution encourages discussion of whether to also include 8-bit profiles for 4:4:4 format (i.e. Main 8 4:4:4 and Main 8 4:4:4 Still Picture profiles) as in HEVC.

# Introduction

In recent years, HDR and 10-bit video has seen significant market adoption and become widely used in consumer video applications such as streaming. Nevertheless, many popular video applications today still rely solely on 8-bit 4:2:0 video content. Examples of such applications include e-commerce video such as Taobao Live, short video platforms such as Bytedance’s Douyin, Kwai’s Kuaishou, Baidu’s Baidu app and Haokan, and Xiaomi’s Mi Live. These video apps are very popular among consumers, and billions of daily active users (DAUs) consume hours of video each day on these platforms. The DAUs of Douyin, Kuaishou, Baidu app, and Taobao have already exceeded 600 millions, 300 millions, 200 millions and 300 millions respectively, and these users collectively spend an average of more than 80 minutes a day on video consumption of various forms including short videos and live shows. Live streaming of gaming and gamers is another popular application with millions of DAUs and still growing fast. Just in December 2020, more than nine million people spent on average two hours a day on Twitch. Successful live streaming applications all enjoy very large base of concurrent users, which in term translates to significant bandwidth usage. Bandwidth cost constitutes a significant portion of business cost on these platforms, and they all stand to benefit significantly from VVC’s stellar compression capability.

Another important business consideration for these platforms is the fact that they all have to deal heavily with mobile devices of all tiers, ranging from high-end mobile phones that costs more than $1000 and equipped with the highest hardware configuration, to the most affordable smartphone models that are less than $100 and often lack the latest hardware capability. High level of device diversity means that well-defined interoperability as well as favorable price-performance ratio are crucial. In other words, to use VVC, these video applications need to have a reasonable profile (i.e. conformance point) that is the most relevant to them in order to ensure customers from the highest and the lowest tiers can interoperate efficiently and seamlessly with each other. As the vast majority of these applications rely on 8-bit 4:2:0 video, the most relevant conformance point for them is indeed an 8-bit 4:2:0 profile.

In terms of encoder and decoder implementations, we first consider hardware implementations, which provide the most power-efficient solution. We note that the proponents include two largest mobile phone chip providers, and it is in their expertise to suggest that an 8-bit profile incurs little, if any, additional cost to hardware encoder and decoder implementations. From software encoder/decoder point of view, we suggest that 8-bit software implementations are lighter and faster than 10-bit software implementations, mostly because of the significant reduction in memory cost (e.g. due to the storage of reference pictures) but also because of the improved ability to parallelize using SIMD instructions. The over-the-top internet companies in the proponent list suggest that having an 8-bit profile makes VVC more accessible and friendlier to their applications.

Last but not the least, we address the concern that having too many profiles in VVC may potentially cause market “confusion.” We first cite the fact that H.264/AVC, the most successful and most widely adopted video standard in the history of the video industry, defines a multitude of 8-bit profiles, a lot of which continue to be widely used in different types of real-world applications to this day, 17 years after its first version was finalized. Deducing from the H.264/AVC anecdote, we claim that having more profiles does not necessarily confuse the market and hinder commercial adoption. We then note that the proponents constitute a large span of the video industry, from internet companies to hardware encoder/decoder implementers to device and phone manufacturers. Collectively we do not expect the presence of 8-bit VVC profiles to cause significant market confusion and hinder VVC’s commercialization. Quite to the contrary, it is the belief of the proponents that VVC 8-bit profiles will foster wider and more rapid VVC commercial adoption.

# The proposed Main 8 and Main 8 Still Picture profiles

Text changes corresponding to the inclusion of these two 8-bit 4:2:0 profiles are provided in the zip package. Two options are provided, one with each profile using its own value of profile indicator (similarly as existing profiles in VVC v1), and another with some profiles sharing the same value of profile indicator (similarly as in HEVC).

The proponents suggest that there are two factors favoring the first option where each profile uses its own profile indicator value:

- It allows the player to more quickly configure the appropriate decoder conformance requirements;

- It is consistent with how existing VVC profiles are defined.

# 8-bit profiles for 4:4:4 format

Along with the proposed 8-bit profiles for 4:2:0 format, the proponents would like to encourage discussion among JVET experts regarding whether to include 8-bit profiles for 4:4:4 format as well. The proponents do not have a strong view on this matter, but would like to share the following observations for discussion:

- Main 8 4:4:4 profile may be useful for some applications, such as video conferencing, particularly when screen sharing; and if Main 8 4:4:4 is included, it is only natural to include Main 8 4:4:4 Still Picture;

- HEVC defines these two profiles.

Text changes corresponding to the inclusion of these 8-bit 4:4:4 profiles are also provided in the zip package for both options, and can be identified easily from the Main 8 and Main 8 Still Picture profiles.

# Patent rights declaration(s)

Alibaba, Bytedance, Qualcomm, MediaTek, Kwai, OPPO, Baidu, Vivo, ZTE, Xiaomi, and Twitch may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).