| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0091 |
| --- | --- |

| Title: | AHG9/AHG11: SEI message for carriage of neural network information for post filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi, Zeqiang Li, Wei Wang, Wei Jiang, Xiaozhong Xu, Stephan Wenger, Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes an SEI message design for carriage of a neural network topology and parameters that are utilized for post filtering with neural network models. The proposed straw-man design of high level syntax structure and elements includes both approaches; 1) internal carriage of description of topology information and compressed network parameters in SEI message, for simple convolutional neural network (CNN) use cases, and 2) external linkage information to provide where the network topology information and corresponding parameters are present in external files or remote locations, in case that the data size of neural network information is too huge to be contained in SEI messages. The goal of this proposal initiates the discussion on how to specify neural-network based post/in-loop filtering in VSEI, because the development of CNN-based post/in-loop filtering shows the good progress. The design of the proposed syntax structure is aimed to be specified in VSEI as codec-agnostic approach, but potentially the similar mechanism can be specified in parameter sets targeting VVC/HEVC-extensions.

# Introduction

This contribution is a straw-man proposal for aiming to draft a SEI message specification for signaling a neural network topology and trained parameters, which are potentially used to process network inference designed for post/in-loop filtering, super-resolution processing, virtual reference generation or any other applications in video CODEC. Although the information on network topology and parameters can be explicitly specified in VSEI spec with fixed parameters, having a more flexible mechanism of network representation with trained parameters of users, which can be used for post-processing or side-operation to improve the quality of output video, should be useful for various applications, without any harm on the video CODEC core design.

The basic idea is to have two mechanisms in terms of signaling neural network model information; The first one is signaling explicitly one or more topology information as well as the corresponding parameters trained with specific syntax elements defined in VSEI, the other one is providing an external linkage information which indicates where the corresponding topology information and network parameters are present.

For signaling network topologies and parameters, referencing the existing formats that have been developed for representation of network. One good example could be Neural Network Exchange Format (NNEF) that is a generalized neural network exchanging format developed by Khronos [1]. Other possible examples are Open Neural Network Exchange (ONNX)[2] and MPEG NNR[3], which is the new format for coded representation of neural network.

Ideally, any neural network models can be exported to NNEF and other formats, and network accelerator and libraries can consume data in the formats without compatibility issue with any network framework. As a simplest and practical method, it would be good to directly reference outside files or bitstream with URI information. However, it is also desired to have a lightweight syntax design to represent video coding specific network for VVC or HEVC-extension with novel neural-network based video coding tools, because a generic representation of network model may be bulky to be used for the compressed video format. Since mostly used network models for video compression are based on convolutional neural network (CNN), having a compact representation of CNN in SEI message is expected to be helpful to reduce the total bitrate as well as to enable easy access to the network model data.

As a starting point to this goal, the following sections figure out what things to be specified and proposes a straw-man syntax design. Although the proposed syntax structure and elements in this document are specified in SEI message, but potentially can be specified in any parameter sets with consideration of VVC/HEVC extensions.

# Things to be specified

Generally, a neural network model can be represented by a computational graph, which is a directed graph with multiple nodes. The nodes consist of operation nodes and data, such as tensor. According to applications, various network topologies are designed and used. However, for post-/in-loop filtering for video processing, CNN-based simple models are popularly adopted. In the case, a simple multi-layered feedforward network like CNN can be represented by a linear graph that starts from the input data, where each operation node in a layer produces an intermediately processed data. Finally, the output data is generated through multiple layers.

Figure 1. An example of computational graph for convolutional neural network.

Fig.1 illustrate an example of linear computational graph for CNN. When the input tensor data are fed into an operation node, the operation node processes the input data with and the pretrained constants and/or variables and outputs the intermediate tensor data. When the operation nodes are executed, actual data is consumed by operation node. Usually for CNN, the weighted summation of input data with trained constants and/or updated variables is the output of each operation node.

In terms of specification of the network graph, once a specific operation node is specified as a single step, the same operation node can be used iteratively. We believe such simple network topologies can be described by some well-designed syntax elements in SEI message. In case that more complicated model designs are required, external formats like NNEF or ONNX can be used. 
For delivering network parameters, usually the bit size of trained network parameters is too huge to be contained in an SEI message. To reduce the data size, MPEG-NNR format can be used for compressing parameters and can be partitioned into multiple data chunk. Each chunk of compressed parameters can be contained in an SEI message or a separate data file, which is potentially delivered in the same bitstream or stored in a remote server. When decoded, all concatenated chunk data in SEI messages for representation of neural network are spliced and consumed by neural network library or decoders.

# Example of SEI message for carriage of post-filtering NN info

## Neural-network based post-filtering SEI message

### Neural-network based post-filtering SEI message syntax

To be updated

### Neural-network based post-filtering SEI message semantics

To be updated

# Discussion and suggestion

In this proposal, it is asserted that the SEI message specification for carriage of neural network topology information and corresponding (trained) parameters is useful for various application that needs post-processing with neural network. Several syntax design choices need to be further discussed: 1) linkage to external neural network information, 2) lightweight description of network information in SEI message, 3) partitioning of network information into multiple SEI messages. Further details can be filled out through group activities with experts. On the other hand, at least, we believe that the proposed design is a good starting point as a straw-man design for further studies. Therefore, it is proposed to start the specification work on SEI message for post filtering with neural network, based on this contribution.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Neural Network Exchange Format, The Khronos NNEF Working Group, Version 1.0.3, 2020-06-12

(https://www.khronos.org/registry/NNEF/specs/1.0/nnef-1.0.3.pdf)

[2] Open Neural Network Exchange, VERSION 6, 2019-09-19

(https://github.com/onnx/onnx/blob/master/onnx/onnx.proto)

[3] “Text of ISO/IEC DIS 15938-17 Compression of Neural Networks for Multimedia Content Description and Analysis”, ISO/IEC JTC 1/SC29/WG11 Documents, MDS19764, Online, Oct 2020.

[4] IETF RFC 1950, ZLIB Compressed Data Format Specification version 3.3, 1996.