| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0093 |
| --- | --- |

| Title: | YCgCo-R: Additional Experiments on the new representation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Derek Buitenhuis VideoLAN. Alexis Michael Tourapis Apple Inc. One Apple Park Way Cupertino, California, 95014 United States | Tel: Email: | derek@videolan.org atourapis@apple.com |
| Source: | VideoLAN, Apple Inc. |  |  |

_____________________________

# Abstract

This contribution presents additional information related to the new proposed YCgCo-R representation that was discussed in contribution JVET-T0111. The information includes lossless and lossy coding experiments using encoder implementations of the AVC video coding specification, and the JPEG 2000, JPEG-XL, and JPEG-XR still image video coding specifications. Clarifications with regards to this representation, i.e. that there are implications to also the final bit-depth representation of the reconstructed RGB signal, are also made.

# Introduction

During the 20th JVET meeting a new YCgCo-R representation was proposed to be included in the CICP specification, and in consequence in the MPEG-4 AVC, HEVC, EVC, and VVC video coding specifications. The corresponding contribution highlighted an issue that many video coding specifications, including EVC and VVC, only allow the same bit depth for their luma and chroma components, restricting essentially the support of the YCgCo-R representation, which could allow improved coding performance, especially for lossless applications, compared to the RGB representation, especially if no internal transformation is supported. It was also pointed out that many HW encoders of standards that may support different bitdepth between luma and chroma, such as AVC and HEVC, commonly are also limited in supporting equal bit depths for all planes. The proposed representation allowed for a more efficient, coding-wise, packing of the YCgCo-R data, essentially allowing the impacted specifications or encoder implementations to achieve similar coding benefits as intended with the original method.

In particular, the YCgCo-R representation is defined as a lossless transformation between the original RGB representation and YCgCo using a lifting scheme transformation method defined as follows:

Co = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )

t = Round( B ) + ( ( Co − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

Cg = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )

Y = t + ( ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

where R, G, and B are the red, green, and blue input colour component values, Co, Cg, and Y are the chroma orange-difference, chroma green-difference, and luma output colour component values, and BitDepthC corresponds to the output bit depth of the two chroma components. BitDepthC is required to be equal to the bit depth of the luma component, and consequently the red, green, and blue components, plus 1.

Unfortunately, since VVC cannot support different luma and chroma bitdepths the actual transformation supported is as follows:

Y = Round( 0.5 * G + 0.25 * ( R + B ) )

Cg = Round( 0.5 * G − 0.25 * ( R + B ) ) + ( 1 << ( BitDepthC − 1 ) )

Co = Round( 0.5 * (R − B ) ) + ( 1 << ( BitDepthC − 1 ) )

In order for this transformation to be lossless BitDepthC, and essentially the bit depth of all colour components, needs to be higher of that of the original input R, G, and B signals. This, in effect, results in the luma component being MSB aligned in the resulting representation. Unfortunately, experiments have shown that this particular representation is not as efficient as expected in the context of such applications, primarily because of inefficiencies in the corresponding coding specifications of handling the expanded bit depth of the luma component.

# Proposed improvement to the YCgCo-R representation

To avoid the above stated problem, JVET-T0111 proposed adding additional YCgCo based code points, i.e. new matrix coefficients entries, which would enforce an LSB alignment of the luma values in the luma component according to the YCgCo transformation process. In particular, it was suggested introducing at least one more matrix coefficients code point in the CICP specification, which would permit the same transformation as the current YCgCo matrix coefficients entry (value 8) specified for when BitDepthC != BitDepthY, i.e.:

Co = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )

t = Round( B ) + ( ( Co − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

Cg = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )

Y = t + ( ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

with however the following differences in the inverse process. In particular, in the first code point the inverse process would be recommended to be performed as follows:

maxValue = ( 1 << (BitDepthY – bitOffset) ) – 1

t = Y − ( ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

G = Clip3( 0, maxValue, t + ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) )

B = Clip3( 0, maxValue, t − ( ( Co − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )

R = Clip3(0, maxValue, B + ( Co − ( 1 << ( BitDepthC − 1 ) ) ) )

where the value of bitOffset is set equal to 2. The intent of this value is to allow for an "even" bit depth RGB representation, e.g. 8 bits, to be efficiently represented in a 10 bit input representation to the video encoder, or in general for applications where both the original RGB and intended input to the encoder representations are of odd or evn precision. For the second code point, the bitOffset is instead set to 1. This second code point is intended for applications where the RGB and input representations are of different bit depth parity, i.e. even vs odd representations. However, since odd bit-depth representations are relatively uncommon, this second code value may be omitted. Alternatively, it might be possible to reuse the "full range" flag indication to support both modes.

One point that was not highlighted in JVET-T0111 is that also, with these representations, the bit depth of the RGB components is also reduced according to the value of bitOffset. In particular, the bit depth of the red, green, and blue components is equal to a new variable bitDepth, which is computed as follows:

bitDepth = BitDepthY – bitOffset

Consequently, a video range representation RGB signal would be generated as:

R = Clip1Y( ( 1 << ( bitDepth − 8 ) ) * ( 219 * E′R + 16 ) )

G = Clip1Y( ( 1 << ( bitDepth − 8 ) ) * ( 219 * E′G + 16 ) )

B = Clip1Y( ( 1 << ( bitDepth − 8 ) ) * ( 219 * E′B + 16 ) )

while a full range representation RGB signal would be generated as:

R = Clip1Y( ( ( 1 << bitDepth) − 1 ) * E′R )

G = Clip1Y( ( ( 1 << bitDepth) − 1 ) * E′G )

B = Clip1Y( ( ( 1 << bitDepth) − 1 ) * E′B )

# Additional Experiments

In JVET-T0111 experiments using the VVC and HEVC reference encoder implementations were conducted. We have verified that our experiments used the recommended configuration parameters for the specific encoders, however it is still not clear if there are any implementation limitations in the corresponding software and especially for the VTM.

In this contribution we include additional experimental results using the JM reference software implementation of MPEG-4 AVC, as well as its popular x264 open source encoder implementation. Additional results using encoder implementations of JPEG 2000, JPEG XR, and JPEG XL are also included.

Table 1 through Table 4 include the results using the VTM software that were presented in JVET-T0111.

|  | RGB |  |  | YCoCg-R |  |
| --- | --- | --- | --- | --- | --- |
|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| Mean | 100.00% | 100.98% | 101.52% | 123.27% | 97.20% |
| StdDev | 0.00% | 0.85% | 0.81% | 3.64% | 2.86% |
| Best | 100.00% | 99.63% | 100.04% | 111.51% | 87.58% |
| Worst | 100.00% | 103.69% | 104.18% | 131.11% | 103.19% |

Table 1: Lossless coding performance for subset1 using the VTM

|  | RGB |  |  | YCoCg-R |  |
| --- | --- | --- | --- | --- | --- |
|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| Mean | 100.00% | 100.94% | 101.49% | 123.26% | 97.19% |
| StdDev | 0.00% | 0.88% | 0.86% | 2.95% | 2.13% |
| Best | 100.00% | 99.08% | 99.93% | 117.01% | 91.07% |
| Worst | 100.00% | 103.17% | 103.70% | 131.53% | 101.11% |

Table 2: Lossless coding performance for subset2 using the VTM

|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 101.12% | 101.40% | 119.75% | 95.69% |
| StdDev | 0.00% | 1.02% | 0.94% | 6.27% | 4.17% |
| Best | 100.00% | 95.18% | 96.22% | 90.85% | 74.00% |
| Worst | 100.00% | 108.66% | 105.58% | 153.45% | 115.25% |

Table 3: Lossless coding performance for subset3 using the VTM

|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 101.23% | 101.53% | 119.82% | 95.67% |
| StdDev | 0.00% | 1.14% | 1.00% | 6.23% | 4.00% |
| Best | 100.00% | 92.66% | 91.67% | 84.26% | 67.66% |
| Worst | 100.00% | 111.75% | 105.72% | 146.40% | 114.84% |

Table 4: Lossless coding performance for subset4 using the VTM

Table 5 through Table 8 showcase the performance of the proposed representations using the JM reference SW of MPEG-4 AVC, while Table 9 through Table 12 provide additional information using the x264 encoder implementation of the same standard. It should be noted that x264 is not currently capable of encoding data with different bit depths between luma and chroma or 9 bit representations.

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R LSB 9-bit | YCoCg-R 9-bit Co/Cg, 8-bit Y | YCoCg-R MSB 9-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 91.64% | 91.94% | 91.97% | 100.33% | 119.31% |
| StdDev | 0.00% | 9.24% | 9.32% | 9.35% | 9.88% | 13.23% |
| Best | 100.00% | 73.94% | 74.46% | 74.63% | 67.44% | 81.13% |
| Worst | 100.00% | 109.35% | 109.96% | 110.01% | 124.23% | 145.81% |

Table 5: Lossless coding performance for subset1 using the JM

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R LSB 9-bit | YCoCg-R 9-bit Co/Cg, 8-bit Y | YCoCg-R MSB 9-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 90.99% | 90.92% | 90.96% | 100.85% | 119.84% |
| StdDev | 0.00% | 10.64% | 10.58% | 10.46% | 10.06% | 14.96% |
| Best | 100.00% | 64.14% | 64.14% | 64.14% | 73.06% | 79.69% |
| Worst | 100.00% | 114.34% | 114.85% | 111.57% | 120.80% | 152.42% |

Table 6: Lossless coding performance for subset2 using the JM

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R LSB 9-bit | YCoCg-R 9-bit Co/Cg, 8-bit Y | YCoCg-R MSB 9-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 88.16% | 88.35% | 88.41% | 98.36% | 120.42% |
| StdDev | 0.00% | 11.01% | 11.00% | 11.00% | 10.25% | 15.99% |
| Best | 100.00% | 32.11% | 32.11% | 32.10% | 47.66% | 69.36% |
| Worst | 100.00% | 135.79% | 136.98% | 137.20% | 150.46% | 208.96% |

Table 7: Lossless coding performance for subset3 using the JM

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R LSB 9-bit | YCoCg-R 9-bit Co/Cg, 8-bit Y | YCoCg-R MSB 9-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 87.56% | 87.75% | 87.79% | 97.41% | 120.44% |
| StdDev | 0.00% | 10.13% | 10.14% | 10.14% | 9.46% | 14.69% |
| Best | 100.00% | 40.98% | 41.61% | 41.81% | 56.74% | 73.10% |
| Worst | 100.00% | 138.54% | 138.55% | 138.53% | 147.33% | 177.08% |

Table 8: Lossless coding performance for subset4 using the JM

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- |
| Mean | 100.00% | 85.73% | 116.30% |
| StdDev | 0.00% | 8.45% | 10.98% |
| Best | 100.00% | 58.94% | 81.47% |
| Worst | 100.00% | 103.35% | 141.55% |

Table 9: Lossless coding performance for subset1 using x264

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- |
| Mean | 100.00% | 84.84% | 115.24% |
| StdDev | 0.00% | 8.21% | 11.06% |
| Best | 100.00% | 55.34% | 75.42% |
| Worst | 100.00% | 100.59% | 136.37% |

Table 10: Lossless coding performance for subset2 using x264

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- |
| Mean | 100.00% | 85.10% | 114.77% |
| StdDev | 0.00% | 8.44% | 10.79% |
| Best | 100.00% | 33.78% | 51.98% |
| Worst | 100.00% | 115.60% | 154.56% |

Table 11: Lossless coding performance for subset3 using x264

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- |
| Mean | 100.00% | 84.84% | 114.52% |
| StdDev | 0.00% | 7.66% | 9.89% |
| Best | 100.00% | 34.26% | 51.93% |
| Worst | 100.00% | 120.34% | 156.60% |

Table 12: Lossless coding performance for subset4 using x264

It can be seen from the JM results that the proposed representations allow for similar coding performance as that of the original YCgCo-R representation. Coding benefits seem to be achieved almost always compard to an MSB aligned representation.

# Conclusion

It is proposed that one or more new matrix coefficients values are introduced in the CICP specification and consequently any video specification that utilize such code points.

# Patent rights declaration(s)

VideoLAN may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).