| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0094-v1 |
| --- | --- |

| Title: | EE: Tests on Neural Network-based In-Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang Marta Karczewicz Jianle Chen Anand Meher Kotra | Email: | hongtaow@qti.qualcomm.commartak@qti.qualcomm.com cjianle@qti.qualcomm.com akotra@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the EE test results of JVET-T0079. In the proposed Neural Network-based method, one NN-based filter can be selected out of X candidates as an in-loop filter for each picture. Scaling factors are derived and signaled at picture level for each color component to further improve the quality of the filter output. In this EE, several aspects of the proposed NN filter, including different numbers of candidate filters (X=4 or 12), model sizes (1M or 510K), residual scaling on and off, and positions of the filter (placed after ALF or after deblocking ) are tested.

# Introduction

The NN filter originally proposed in JVET-T0079 [1] is illustrated in Figure 1. It contains 14 layers, including N=12 hidden layers with 96 channels in each layer. Each hidden layer consists of a 3x3 convolutional layer and a Leaky RELU. There are about one million model parameters in total. It is also tested in this EE that the number of hidden layers N=6, in this case, the total number of parameters of each model is about 510K.

Figure 1 Proposed network architecture

The inputs of the NN filtering process are one 128x128 size luma block and two 64x64 chroma blocks. The luma samples are interleaved into four 64x64 size blocks before being used as inputs of the filtering process as proposed in [2]. Several sub-tests in this EE (subtest 1, 2 and 3) combined training data of multiple QPs to reduce the number of candidate models, in this case, the QP is converted into an additional input to the NN filter.

Multiple models are trained based on different QPs and encoder configurations (AI, RA) and one model is selected at the picture level to be used an additional loop filter. The index of the selected model is signaled in picture header.

Besides the filter index, one scaling factor is signaled for each color component at the picture level so that the decoder can adjust the NN filter output to achieve better rate-distortion performance (residual scaling).

In this EE, 5 sub-tests are performed regarding to the following aspects of the proposed NN network design:

- Number of hidden layers (N=12, 6 are tested).

- Number of candidate models (X=4, 12 are tested).

- Performance comparison of residual scaling on and off.

- Location of the NN filter (before SAO or after ALF).

The detailed description of each subtest can be found in section 2.

# Test descriptions

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 | Number of hidden layers N=12, number of models X=4, located before SAO, using QP as an additional input. | Hongtao Wang |  |
| 2 | Number of hidden layers N=6, number of models X = 4, located before SAO, using QP as an additional input. | Hongtao Wang |  |
| 3 | subtest #1 without residual scaling | Hongtao Wang |  |
| 4 | Number of hidden layers N=12, number of models X=12, located before SAO | Hongtao Wang |  |
| 5 | Number of hidden layers N=12, number of models X=12, located after ALF | Hongtao Wang |  |

# Simulation Results

Simulations are performed based on the common test conditions for neural network-based video coding technology [4]. The simulation results of each subtest are reported in the following sub-sections.

## Subtest 1

Table 1 Simulation results of sub-test 1

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -5.42% | -9.39% | -10.52% | 142% | 15121% | 0% |
| Class A2 | -5.60% | -15.12% | -12.05% | 139% | 13821% | 0% |
| Class B | -5.13% | -13.58% | -13.32% | 143% | 15448% | 0% |
| Class C | -5.28% | -13.95% | -14.98% | 135% | 14767% | 0% |
| Class E |  |  |  |  |  |  |
| Overall | -5.32% | -13.15% | -12.95% | 140% | 14864% | 0% |
| Class D | -6.49% | -15.27% | -16.56% | 137% | 16110% | 1% |
| Class F | -2.41% | -8.55% | -8.81% | 177% | 19768% | 0% |
| Class H | -4.74% | -8.80% | -11.71% | 187% | 19940% | 0% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.62% | -10.36% | -10.84% | 147% | 15659% | 1% |
| Class A2 | -4.33% | -15.19% | -12.02% | 144% | 14190% | 1% |
| Class B | -4.32% | -13.43% | -13.66% | 147% | 16375% | 1% |
| Class C | -4.95% | -15.89% | -17.12% | 135% | 15436% | 1% |
| Class E | -4.42% | -8.82% | -9.03% | 211% | 26899% | 2% |
| Overall | -4.55% | -14.52% | -15.19% | 153% | 17014% | 1% |
| Class D | -6.14% | -18.21% | -18.44% | 132% | 15417% | 2% |
| Class F | -2.43% | -9.30% | -9.59% | 179% | 24082% | 1% |
| Class H | -4.02% | -11.32% | -13.65% | 171% | 24377% | 1% |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.25% | -7.40% | -9.39% | 147% | 10838% | 0% |
| Class A2 | -4.46% | -10.31% | -8.63% | 127% | 8742% | 0% |
| Class B | -4.30% | -10.32% | -10.61% | 121% | 8616% | 0% |
| Class C | -4.93% | -11.79% | -13.22% | 113% | 6061% | 0% |
| Class E | -6.54% | -9.90% | -10.54% | 128% | 9845% | 0% |
| Overall | -4.83% | -10.09% | -10.65% | 125% | 8515% | 0% |
| Class D | -5.31% | -11.04% | -14.34% | 103% | 6214% | 0% |
| Class F | -3.51% | -9.26% | -9.34% | 108% | 7891% | 0% |
| Class H | -4.98% | -6.59% | -7.43% | 132% | 8100% | 0% |

## Subtest 2

Table 2  Simulation results of sub-test 2

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.68% | -8.29% | -9.20% | 123% | 7901% | 0% |
| Class A2 | -4.90% | -13.06% | -10.48% | 121% | 7182% | 0% |
| Class B | -4.33% | -12.09% | -12.09% | 124% | 8139% | 0% |
| Class C | -4.55% | -13.02% | -13.73% | 121% | 7811% | 0% |
| Class E |  |  |  |  |  |  |
| Overall | -4.57% | -11.77% | -11.62% | 122% | 7805% | 0% |
| Class D | -5.78% | -14.27% | -15.38% | 121% | 8501% | 1% |
| Class F | -2.03% | -8.35% | -7.82% | 143% | 10385% | 0% |
| Class H | -4.08% | -8.25% | -10.81% | 150% | 10476% | 0% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -3.85% | -9.84% | -8.64% | 127% | 8180% | 1% |
| Class A2 | -3.57% | -12.64% | -10.14% | 125% | 7486% | 1% |
| Class B | -3.64% | -11.72% | -11.53% | 127% | 8627% | 1% |
| Class C | -4.20% | -15.20% | -14.68% | 121% | 8115% | 1% |
| Class E | -3.53% | -8.16% | -8.42% | 162% | 14036% | 1% |
| Overall | -3.80% | -13.27% | -12.93% | 130% | 8934% | 1% |
| Class D | -5.46% | -18.43% | -16.74% | 117% | 8223% | 1% |
| Class F | -2.14% | -10.92% | -6.65% | 145% | 12670% | 1% |
| Class H | -3.28% | -9.75% | -10.93% | 139% | 12818% | 1% |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.00% | -6.62% | -8.70% | 126% | 5721% | 0% |
| Class A2 | -3.86% | -8.49% | -7.81% | 114% | 4636% | 0% |
| Class B | -3.66% | -8.81% | -9.51% | 110% | 4533% | 0% |
| Class C | -4.15% | -10.75% | -12.36% | 107% | 3219% | 0% |
| Class E | -5.72% | -8.49% | -10.68% | 117% | 5140% | 0% |
| Overall | -4.20% | -8.77% | -9.92% | 114% | 4511% | 0% |
| Class D | -4.65% | -9.78% | -12.94% | 97% | 3287% | 0% |
| Class F | -2.99% | -8.43% | -8.10% | 105% | 4203% | 0% |
| Class H | -4.28% | -4.87% | -6.62% | 118% | 4301% | 0% |

## Subtest 3

Table 3 Simulation results of sub-test 3

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -5.22% | -8.37% | -10.62% | 142% | 15065% | 0% |
| Class A2 | -5.07% | -13.93% | -10.98% | 138% | 13786% | 0% |
| Class B | -4.55% | -12.29% | -12.29% | 142% | 15450% | 0% |
| Class C | -4.53% | -11.82% | -13.20% | 135% | 14772% | 0% |
| Class E |  |  |  |  |  |  |
| Overall | -4.78% | -11.71% | -11.94% | 139% | 14847% | 0% |
| Class D | -6.17% | -13.00% | -14.85% | 137% | 16103% | 1% |
| Class F | -1.96% | -7.23% | -7.41% | 176% | 19751% | 0% |
| Class H | -4.52% | -7.94% | -11.77% | 187% | 19940% | 0% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.33% | -11.05% | -11.07% | 148% | 15632% | 1% |
| Class A2 | -3.74% | -14.86% | -11.57% | 144% | 14217% | 1% |
| Class B | -3.71% | -12.85% | -14.35% | 146% | 16387% | 1% |
| Class C | -4.24% | -14.73% | -15.87% | 135% | 15349% | 1% |
| Class E | -5.13% | -10.05% | -13.22% | 212% | 26975% | 1% |
| Overall | -4.24% | -13.69% | -15.03% | 153% | 17005% | 1% |
| Class D | -5.49% | -15.81% | -16.63% | 132% | 15457% | 1% |
| Class F | -2.42% | -8.95% | -8.66% | 179% | 24092% | 1% |
| Class H | -3.66% | -13.31% | -16.86% | 171% | 24416% | 0% |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.09% | -7.02% | -9.52% | 145% | 10758% | 0% |
| Class A2 | -4.35% | -9.94% | -8.22% | 125% | 8754% | 0% |
| Class B | -3.98% | -9.93% | -10.63% | 119% | 8582% | 0% |
| Class C | -4.67% | -10.27% | -11.69% | 111% | 6017% | 0% |
| Class E | -6.29% | -9.32% | -11.10% | 125% | 9790% | 0% |
| Overall | -4.60% | -9.42% | -10.36% | 123% | 8475% | 0% |
| Class D | -5.24% | -9.52% | -13.20% | 101% | 6201% | 0% |
| Class F | -3.02% | -8.14% | -8.22% | 108% | 7844% | 0% |
| Class H | -4.90% | -6.48% | -8.14% | 131% | 8046% | 0% |

## Subtest 4

Table 4 Simulation results of sub-test 4

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.99% | -8.81% | -10.81% | 218% | 15147% | 0% |
| Class A2 | -5.14% | -14.10% | -11.58% | 207% | 13748% | 0% |
| Class B | -4.80% | -13.25% | -13.10% | 215% | 15256% | 0% |
| Class C | -4.96% | -13.99% | -13.99% | 191% | 14917% | 0% |
| Class E |  |  |  |  |  |  |
| Overall | -4.95% | -12.73% | -12.58% | 208% | 14831% | 0% |
| Class D | -6.39% | -15.49% | -15.02% | 200% | 16635% | 1% |
| Class F | -2.18% | -9.44% | -8.70% | 305% | 19764% | 0% |
| Class H | -4.29% | -7.17% | -10.59% | 332% | 19897% | 0% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.47% | -10.15% | -10.24% | 223% | 15312% | 1% |
| Class A2 | -4.03% | -14.34% | -12.06% | 217% | 13946% | 1% |
| Class B | -4.21% | -14.13% | -14.95% | 223% | 16211% | 1% |
| Class C | -4.84% | -16.59% | -16.72% | 194% | 15696% | 1% |
| Class E | -4.34% | -10.58% | -9.43% | 414% | 27206% | 2% |
| Overall | -4.45% | -15.22% | -15.74% | 239% | 16949% | 1% |
| Class D | -6.21% | -19.28% | -18.65% | 193% | 15548% | 2% |
| Class F | -2.48% | -12.32% | -10.78% | 310% | 24254% | 1% |
| Class H | -3.70% | -10.41% | -12.94% | 296% | 24526% | 1% |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.03% | -7.41% | -9.47% | 224% | 10617% | 0% |
| Class A2 | -3.96% | -9.85% | -8.11% | 168% | 8703% | 0% |
| Class B | -3.87% | -10.03% | -10.98% | 154% | 8534% | 0% |
| Class C | -4.44% | -11.43% | -13.08% | 131% | 6141% | 0% |
| Class E | -5.98% | -10.42% | -10.71% | 170% | 9767% | 0% |
| Overall | -4.39% | -9.94% | -10.67% | 163% | 8490% | 0% |
| Class D | -4.86% | -11.09% | -13.93% | 120% | 6441% | 0% |
| Class F | -3.09% | -8.89% | -8.46% | 125% | 7921% | 0% |
| Class H | -4.41% | -5.93% | -7.11% | 188% | 8098% | 0% |

## Subtest 5

Table 5 Simulation results of sub-test 5

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.31% | -7.87% | -9.74% | 218% | 15095% | 0% |
| Class A2 | -4.40% | -13.83% | -13.35% | 207% | 13701% | 0% |
| Class B | -3.77% | -12.92% | -12.31% | 215% | 15253% | 0% |
| Class C | -3.83% | -12.88% | -13.24% | 192% | 14900% | 0% |
| Class E |  |  |  |  |  |  |
| Overall | -4.02% | -12.08% | -12.25% | 208% | 14805% | 0% |
| Class D | -4.92% | -13.64% | -14.92% | 203% | 16635% | 1% |
| Class F | -1.30% | -7.82% | -6.88% | 304% | 19721% | 0% |
| Class H | -3.52% | -7.69% | -10.97% | 333% | 19839% | 0% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -3.75% | -9.34% | -9.62% | 222% | 15323% | 1% |
| Class A2 | -3.66% | -14.46% | -13.46% | 216% | 13968% | 1% |
| Class B | -3.49% | -14.83% | -15.52% | 224% | 16169% | 1% |
| Class C | -3.99% | -16.57% | -15.85% | 195% | 15680% | 1% |
| Class E | -3.43% | -10.68% | -9.95% | 415% | 26969% | 1% |
| Overall | -3.64% | -15.60% | -15.67% | 239% | 16915% | 1% |
| Class D | -5.12% | -17.43% | -17.40% | 197% | 15655% | 1% |
| Class F | -1.58% | -11.52% | -9.62% | 311% | 24232% | 1% |
| Class H | -3.20% | -10.99% | -13.95% | 297% | 24544% | 0% |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -3.49% | -5.66% | -8.89% | 222% | 10492% | 0% |
| Class A2 | -3.04% | -8.17% | -8.31% | 168% | 8790% | 0% |
| Class B | -2.94% | -8.75% | -9.10% | 154% | 8516% | 0% |
| Class C | -2.79% | -9.93% | -10.39% | 130% | 6135% | 0% |
| Class E | -4.39% | -8.06% | -9.49% | 171% | 9768% | 0% |
| Overall | -3.26% | -8.28% | -9.28% | 163% | 8480% | 0% |
| Class D | -2.90% | -8.98% | -10.69% | 119% | 6344% | 0% |
| Class F | -1.79% | -7.04% | -5.77% | 125% | 7905% | 0% |
| Class H | -3.63% | -5.58% | -7.25% | 190% | 8065% | 0% |

# Training and inference information

All the models reported in this EE are trained based on BVI-DVC data set[3], which contains 4 different resolutions of 200 video sequences. All resolutions of 20 randomly selected sequences are used and validation set and the remaining 180 sequences are used as training set. More information related to the training & validation of one model used in sub-test 1 (1M * 4 models) are reported below. Models of other subtests are designed in the same way and the details can be found in the results excel sheets.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla V100-SXM2-32GB |
|  | Framework: | TF v2.2.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 250 |
|  | Batch size: | 10Kx64 |
|  | Training time: | 104h |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP=20,25,30,35,40,45 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 8.00E-04 |
|  | Optimizer: | ADAM |
|  | Loss function: | Weighted L2 |
|  | Preprocessing: | Convert 128x128 YUV420 signal to 6 64x64 blocks. Normalize to 0~1 |
|  | Other information: |  |

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | TF v2.2.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1007814 x 4Models |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 3.84 x 4 models |
|  | MAC (Giga) | 1.57e-4 Giga/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 14 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage | 1.5MB (input size 128x128) |
|  | Other information: |  |

# References

- AHG11: Neural Network-based In-Loop Filter, JVET-T0079, H. Wang, M. Karczewicz, J. Chen, A.M. Kotra (Qualcomm)

- AHG9: Convolutional neural network loop filter, JVET-M0159, Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn ), Di Ma, Fan Zhang, David R. Bull

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-T2006, S. Liu, A. Segall, E. Alshina, R.-L. Liao,

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).