| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0098 |
| --- | --- |

| Title: | AHG9: Composite Picture Information (CPI) SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Hyeongmun Jang Seung Hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com hm.jang@lge.com seunghwan3.kim@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes a design for an SEI message to support generating composite pictures from pictures in a multi-layer VVC bitstream.

It is asserted that composite picture information (CPI) SEI message provides more advantages when used for implementing the given use-cases (i.e., broadcasting / streaming online game program and video conferencing) compared to using subpictures. The asserted advantages are:

- Better coding efficiency as the approach based on CPI SEI message does not require IRAP subpicture whenever there is change to the display layout.

- Better support for personalization of the layout for each bitstream receiver. Changing the display layout is as simple as replacing the SEI message without having to alter the coded pictures.

It is also asserted that the proposed feature has more benefits when specified as SEI message rather than as descriptor at system level for at least the following reasons:

- The feature would be available to any system that uses VVC, rather than only to system that specified it.

- It allows content creators to provide recommended display layout for their contents without having to worry about which system will be used to transport their contents to receivers.

# Introduction

## Use-cases

### Use-case 1

Figure 1 below illustrates the use of composite picture broadcasting / streaming an online gaming program. For the program, viewer can view each gamer screen, highlighted gamer screen, scoreboard, and also summary of the game in one view using composite display. In this use-case, all part of a picture from each layer is used in the composite picture.

Figure 1 – Use of composite picture in broadcasting / streaming online game program

### Use-case 2

Figure 2 below illustrates an example of layout of video conferencing. Participant that is speaking is shown in the bigger portion in the display whereas the rest of the participants are shown on the side.

Figure 2 – Example of display layout for video conferencing

## Composite picture SEI message approach vs. subpicture approach

Although the use-cases described above may be implemented by using subpicture, implementing them using composite picture SEI message has more advantages. When there are frequent change of speakers during the video conferencing, which is a very common case, in which the speaker is shown in the bigger part of the display, using subpicture would require the subpicture corresponding to the participants whose position in the display are changed to be coded as IRAP. This would cause degradation of coding efficiency and possibly spike in bitrate. On the other hand, implementing the same use-case using multilayer and composite picture SEI does not require IRAP picture to be used in such event. This is because each layer may be coded independently and the displayed is independent from the coding itself.

Another advantage of using the SEI message compared to subpictures for implementing the above use-cases is that the SEI message supports more personalization of the layout for each bitstream receiver. Changing the display layout is as simple as replacing the SEI message without having to alter the coded pictures. On the other hand, solution using subpictures would require modifications to the coded pictures which may be less desirable.

In authors’ opinion, while subpicture is an excellent feature for cases involving extraction and merging such as for region of interest (ROI) related use-cases, it has limitation in the use-cases where more flexible layout is needed.

## Composite picture as SEI message or system descriptor

During discussion of JVET-T0049 in the 20th JVET meeting, it was mentioned that the feature enabled by the proposed composite picture SEI message could be specified at system level. However, in authors’ opinion, inclusion of this feature at bitstream level as an SEI message provides more benefits as follows:

- The feature would be available for any system implemented on top of VVC, rather than to each individual system that specifies this feature.

- Being system agnostic allows content creators to provide recommended display layout for their contents without having to worry about which system will be used to transport their contents to receivers.

# Proposed design of CPI SEI

For supporting composite picture based for multilayer bitstream, we propose the following SEI message design:

- A composite picture is generated from pictures in an access unit.

- A composite picture contains one or more rectangular shaped patches.

- For each patch in a composite picture, the following information is signalled:

  - The location of the patch in the composite picture. This information can be signalled as the position of the top-left corner of the patch in the composite picture.

  - The size (i.e., width and height) of the patch in the composite picture.

  - The source of the pels of the patch, which is the layer id of the picture.

- The size of a patch in a composite picture shall be greater than 0 and the associated area (i.e., the source of the pels of the patch) in the associated picture shall be greater than 0.

- There can be gap between two neighboring patches in a composite picture. In other words, there may be area in the composite picture that is not covered by any patch.

- There can be overlap of patches in a composite picture. In other words, there may be area in the composite picture that is covered by two or more patches.

- When there is overlap between two patches, the patch with higher index is rendered after the patch with lower index such that in the overlapped area between the two patches, the pels come from the patch with higher index.

- There shall be no two patches with the same size that covered the same area in a composite picture. This is intended to avoid signalling redundant patch(es) as there is only one will be displayed / visible.

- A patch in a composite picture shall correspond an area in a picture from an output layer only.

- It is possible that a patch in a composite picture corresponds to an area in a picture from an output layer but the picture is not outputted by decoder (e.g., the PictureOutputFlag of the picture is derived to be equal to 0 due to certain condition). In such situation, implementer of the SEI message may skip copying the pels from the source area to the patch in the composite picture (e.g., a gray patch is shown in the composite picture).

- The Composite Picture Information (CPI) SEI message describes composition for its associated OLS. The association is established by allowing CPI SEI message to be carried in a scalable nesting SEI message.

  - When a scalable nesting SEI message contains CPI SEI message(s), the value of the value of sn_ols_flag shall be equal to 1.

  - When a CPI SEI message is not present in a scalable nesting SEI message, it applies to the targetOLS (i.e., to the whole bitstream being decoded).

- The CPI SEI message applies to the AU auA that contains the SEI and all AUs that follow auA in output order until one of the following is true:

  - The end of the bitstream.

  - The next AU contains a new CPI SEI message that applies to the same OLS.

- There shall be no two CPI SEI messages associated with the same OLS but contain different content in an AU. This would create problem for implementer to choose which one is the correct one. Thus, it is constrained that when two CPI SEI message are present in an AU and they are associated with the same OLS, they shall have the same content.

- When present, CPI SEI message is present in AU with TemporalId equal to 0 to avoid the SEI being removed during sublayer extraction.

- When present in a CVS, the first of the CPI SEI message to be present in the CVSS AU.

# Text for composite picture information SEI message

Syntax and Semantics of CPI SEI message

| composite_picture_info( ) { | Descriptor |
| --- | --- |
| cpi_position_and_size_bit_len | u(16) |
| cpi_num_patches_minus1 | u(14) |
| cpi_scaling_allowed_flag | u(1) |
| cpi_gap_allowed_flag | u(1) |
| cpi_unit_size_hor | u(16) |
| cpi_unit_size_hor | u(16) |
| cpi_width_minus1 | u(v) |
| cpi_height_minus1 | u(v) |
| for ( i = 0; i <= cpi_num_patches_minus1; i++ ) { |  |
| cpi_patch_layer_id[ i ] | u(6) |
| cpi_patch_top_left_x[ i ] | u(v) |
| cpi_patch_top_left_y[ i ] | u(v) |
| if( cpi_scaling_flag) { |  |
| cpi_patch_width_minus1[ i ] | u(v) |
| cpi_patch_height_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

A composite picture info (CPI) SEI message provides information for constructing composite picture from one or more patches originated from pictures of an access unit.

When one or more CPI SEI message is present in a bitstream, the first CPI SEI message shall be present in a CVSS AU. When two or more CPI SEI message are present in an AU, the content of the CPI SEI messages shall be the same.

When a scalable nesting SEI message contains a CPI SEI message, the value of sn_ols_flag shall be equal to 1.

A CPI SEI message applies to an AU auA that contains the SEI message and all AUs that follow auA in output order until one of the following applies:

- The end of the bitstream.

- The next AU contains a new CPI SEI message that applies to the target OLS.

When present, CPI SEI message or scalable nesting SEI message that contains CPI SEI message, shall be contained in an access unit with temporal Id equal to 0. The temporal Id of the CPI SEI message or the scalable nesting SEI message that contains the CPI SEI message shall be equal to 0.

cpi_position_and_size_bit_len specifies the number of bits for signalling of syntax elements cpi_patch_top_left_x[ i ], cpi_patch_top_left_y[ i ], cpi_patch_unit_width_minus1[ i ], and cpi_patch__unit_height_minus1[ i ].

cpi_num_patches_minus1 plus 1 specifies the number of patches of each composite picture created from AUs in which the SEI applies.

cpi_scaling_allowed_flag equal to 1 specifies that the size of the i-th patch may be a scaled version of its original picture size in the source layer. cpi_scaling_allowed_flag equal to 0 specifies that the size of the i-th patch is the same as the picture size in the source layer.

cpi_gap_allowed_flag equal to 1 specifies that the union of all patches may not cover all area in the composite picture. cpi_gap_allowed_flag equal to 0 specifies that the union of all patches covers all area in the composite picture.

NOTE: When cpi_gap_allowed_flag is equal to 1, pels in the area that is not covered by any patch need to be initialized with a valid pel value.

cpi_unit_size_hor specifies the unit size of syntax elements cpi_width_minus1, cpi_patch_top_left_x[ i ] and cpi_patch_height_minus1[ i ].

cpi_unit_size_ver specifies the unit size of syntax elements cpi_height_minus1, cpi_patch_top_left_y[ i ] and cpi_patch_height_minus1[ i ].

cpi_width_minus1[ i ] plus 1 specifies width of the composite picture. The length of the cpi_width_minus1[ i ] syntax element is cpi_position_and_size_bit_len bits.

The variable CompositePictureWidth is derived as ( cpi_width_minus1[ i ] + 1 ) * cpi_unit_size_hor.

cpi_height_minus1[ i ] plus 1 specifies height of the composite picture. The length of the cpi_patch_height_minus1[ i ] syntax element is cpi_position_and_size_bit_len bits.

The variable CompositePictureHeight is derived as ( cpi_height_minus1[ i ] + 1 ) * cpi_unit_size_ver.

cpi_patch_layer_id[ i ] specifies the layer Id of the source picture for the i-th patch.

It is constrained that when CPI SEI message is contained in a scalable nesting SEI message, the value of  cpi_patch_layer_id[ i ] for i in the ranges from 0 to cpi_num_patches_minus1, inclusive, shall be equal to one of layer included in the OLS associated with the SEI message.

It is constrained that when CPI SEI message is not contained in a scalable nesting SEI message, the value of  cpi_patch_layer_id[ i ] for i in the ranges from 0 to cpi_num_patches_minus1, inclusive, shall be equal to one of layer that is present in the bitstream.

It is constrained that the layer with layer Id equal to cpi_patch_layer_id[ i ] for i in the ranges from 0 to cpi_num_patches_minus1, inclusive, shall be an output layer in the output layer set associated with the SEI message.

Picture with layer Id equal to cpi_patch_layer_id[ i ] for i in the ranges from 0 to cpi_num_patches_minus1, inclusive, may not be present in the AU in which the SEI applies. When The AU has no picture in cpi_patch_layer_id[ i ], application that generate the composite picture based on the SEI may assign any valid pel value for the i-th patch in the composite picture. Furthermore, in such situation, application may further assign the same pel values for the entire area covered the i-th patch.

cpi_patch_top_left_x[ i ] specifies horizontal position of top left pel in the composite picture for the i-th patch. The length of the cpi_patch_top_left_x[ i ] syntax element is cpi_position_and_size_bit_len bits.

The variable cpiPatchTopLeftX[ i ] is derived as cpi_patch_top_left_x[ i ] * cpi_unit_size_hor.

cpi_patch_top_left_y[ i ] specifies vertical position of top left pel in the composite picture for the i-th patch. The length of the cpi_patch_top_left_y[ i ] syntax element is cpi_position_and_size_bit_len bits.

The variable cpiPatchTopLeftY[ i ] is derived as cpi_patch_top_left_y[ i ] * cpi_unit_size_ver.

cpi_patch_width_minus1[ i ] plus 1 specifies width of the area in the composite picture for the i-th patch. When not present, the value of cpi_patch_width_minus1[ i ] is inferred to be equal to the width of the picture in luma samples of the layer cpi_patch_layer_id[ i ]. The length of the cpi_patch_width_minus1[ i ] syntax element is cpi_position_and_size_bit_len bits.

The variable cpiPatchWidth[ i ] is derived as ( cpi_patch_width_minus1[ i ] + 1 ) * cpi_unit_size_hor.

cpi_patch_height_minus1[ i ] plus 1 specifies height of the area in the composite picture for the i-th patch. When not present, the value of cpi_patch_height_minus1[ i ] is inferred to be equal to the height of the picture in luma samples of the layer cpi_patch_layer_id[ i ]. The length of the cpi_patch_height_minus1[ i ] syntax element is cpi_position_and_size_bit_len bits.

The variable cpiPatchHeight[ i ] is derived as ( cpi_patch_height_minus1[ i ] + 1 ) * cpi_unit_size_ver.

For area that is covered by two or more patches, the value of each pel in such area shall be from the patch with the highest i-th index among those patches.

It is constrained that there shall be no two patches in the composite picture, i-th patch and j-th patch, such that the values of cpi_patch_top_left_x[ i ], cpi_patch_top_left_y[ i ], cpi_patch_width_minus1[ i ], and cpi_patch_height_minus1[ i ] are equal to cpi_patch_top_left_x[ j ], cpi_patch_top_left_y[ j ], cpi_patch_width_minus1[ j ], and cpi_patch_height_minus1[ j ], respectively.

It is constrained that when two patches in the composite picture, i-th patch and the j-th patch, have the values of cpi_patch_top_left_x[ i ] and cpi_patch_top_left_y[ i ] equal to cpi_patch_top_left_x[ j ] and cpi_patch_top_left_y[ j ], respectively, and the value of cpi_patch_width[ i ] * cpi_patch_height[ i ] are greater than cpi_patch_width[ j ] * cpi_patch_height[ j ], respectively, then j shall be greater than i.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).