| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21th Meeting: by teleconference, 6 – 15 Jan 2021 | Document: JVET-U0104-v1 |
| --- | --- |

| Title: | AHG11: In-loop filtering with convolutional neural network and large activation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Hongtao Wang Anand Meher Kotra Marta Karczewicz | Email: | cjianle@qti.qualcomm.com hongtaow@qti.qualcomm.com akotra@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes a Neural Network-based in-loop filter (NN filter). A CNN with large activation layer is employed and the filter is applied after deblocking filter. The proposed filters achieve:

x.xx%, xx % and xx% BD rate saving for RA and 3.45%, 6.54%, 6.84% BD rate saving for AI, for Y, Cb and Cr components respectively, with ~138K model parameters.

x.xx%, xx % and xx% BD rate saving for RA and 4.02%, 9.00% and 9.77% BD rate saving for AI, for Y, Cb and Cr components respectively, with ~270K model parameters.

x.xx%, xx % and xx% BD rate saving for RA and 4.50%, 10.40%, 11.23% BD rate saving for AI, for Y, Cb and Cr components respectively, with ~510K model parameters.

# Introduction

Convolutional Neural Networks (CNN)-based methods recently showed lot of potential in artifact removal and objective quality improvement. A Neural Network based In-Loop Filter is proposed as an additional in-loop filter stage is being tested in the EE contribution : JVET-U0094 [1]. In this contribution, a different NN structure with similar or less parameter amount is studied,

# Proposed NN Filter

The proposed NN filter is illustrated in Figure 1. The hidden layer consist of the filter block with each filter block consists of two 1x1 convolution with a leaky ReLU as activation layer between them, and followed by a 3x3 convolution layer, where the value of M is set larger than the value of K. The inputs of the NN filtering process are a 128x128 size luma block and 2 64x64 chroma block. The luma samples are interleaved into four 64x64 size blocks before used as inputs of the filtering process. As done in [1], the normalized QP_step is also used as the input of the NN filter.

Figure 1. Proposed network architecture

## Training process

The training process was same as done in [1].The BVI-DVC dataset [2] was used in our training. The raw images are compressed by VTM software with AI and RA configurations, and the reconstructed frames are obtained as the training dataset. The QPs used for training process are 20, 25, 30, 35, 40 and 45. During the training, the reconstructed pictures were split into 128x128 luma and 64x64 chorma blocks. The batch size is set to 64.

## Implementation in VTM software

The filtering process of the proposed method was same as done in [1]. The proposed NN filter is incorporated into VTM-10.0 as in-loop filter after deblocking, with CTU level switching (on/off) flags. Scaling of NN filter residues is also applied. As done subtest 1 of [1], 4 models are trained to cover different QP range and picture types, and the model selection is done at picture level.

# Experimental results

The common test conditions defined in [3] are used to evaluate the proposed NN filter. The results are summarized in Table 1~Table 3 It is observed that averagely 3.20%, 12.25 % and 11.39 % BD rate saving for RA and 3.63%, 9.47 % and 9.98 % BD rate saving for AI, for Y, Cb and Cr components respectively, are achieved.

The average encoding time and decoding running are 118% and 13967 % for RA, and 119% and 7853 % for AI.

Table 1 Simulation results of NN model with ~138K parameters

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-master |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -3.96% | -5.88% | -6.68% | 136% | 3973% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -3.63% | -9.28% | -8.33% | 132% | 4065% |
| Class C | -3.72% | -9.93% | -9.74% | 130% | 3827% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -4.76% | -11.11% | -11.84% | 130% | 4592% |
| Class F | -1.57% | -6.68% | -5.83% | 139% | 6200% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over VTM-master |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -3.02% | -4.54% | -6.34% | 129% | 2826% |
| Class A2 | -3.21% | -5.69% | -5.03% | 126% | 2362% |
| Class B | -3.04% | -6.71% | -6.29% | 120% | 2295% |
| Class C | -3.43% | -8.58% | -8.82% | 116% | 1632% |
| Class E | -4.84% | -6.37% | -7.41% | 123% | 2593% |
| Overall | -3.45% | -6.54% | -6.84% | 122% | 2259% |
| Class D | -4.02% | -7.86% | -9.84% | 108% | 1695% |
| Class F | -2.47% | -6.78% | -6.22% | 115% | 2114% |

Table 2 Simulation results of NN model with ~270K parameters

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-master |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -4.32% | -13.04% | -12.72% | 133% | 6365% |
| Class C | -4.49% | -13.05% | -13.87% | 118% | 5744% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -5.57% | -14.53% | -16.27% | 110% | 5933% |
| Class F | -2.06% | -8.67% | -8.31% | 127% | 7090% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over VTM-master |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -3.53% | -6.46% | -9.16% | 133% | 3836% |
| Class A2 | -3.63% | -8.58% | -7.46% | 129% | 3191% |
| Class B | -3.52% | -9.34% | -9.51% | 122% | 3128% |
| Class C | -4.12% | -10.71% | -12.22% | 118% | 2287% |
| Class E | -5.63% | -9.15% | -9.83% | 125% | 3732% |
| Overall | -4.02% | -9.00% | -9.77% | 124% | 3119% |
| Class D | -4.67% | -10.03% | -13.29% | 108% | 2320% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 3 Simulation results of NN model with ~510K parameters

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-master |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -4.97% | -15.79% | -14.72% | 146% | 9858% |
| Class C | -5.18% | -15.63% | -16.13% | 142% | 9189% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -6.41% | -17.28% | -17.92% | 144% | 11828% |
| Class F | -2.37% | -10.09% | -9.38% | 176% | 14824% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over VTM-master |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -3.98% | -7.41% | -10.12% | 171% | 9695% |
| Class A2 | -4.11% | -9.94% | -8.79% | 145% | 6885% |
| Class B | -3.99% | -10.89% | -11.14% | 134% | 6346% |
| Class C | -4.54% | -12.06% | -13.65% | 134% | 5500% |
| Class E | -6.23% | -10.85% | -11.73% | 138% | 7403% |
| Overall | -4.50% | -10.40% | -11.23% | 142% | 6862% |
| Class D | -4.99% | -11.28% | -13.95% | 116% | 4822% |
| Class F | -3.32% | -9.85% | -9.37% | 124% | 5691% |

# References

- EE: Tests on Neural Network-based In-Loop Filter, JVET-U0094, Hongtao Wang, Marta Karczewicz, Jianle Chen, Anand Meher Kotra

- BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn ), Di Ma, Fan Zhang, David R. Bull

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-T2006, S. Liu, A. Segall, E. Alshina, R.-L. Liao

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).