| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0114-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-U0069 (AHG8: CABAC-bypass alignment for high bit-depth coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Reported in this document is the cross-check results for signalling and encoder optimization for CABAC-bypass alignment for high bit-depth coding as reported in JVET-U0069. The cross-check partial results match the coding performance reported by the proponent.

# Results

The source code for JVET-U0069 was provided by the proponent. The software is built based on VTM-11.0rc1 and VTM-11.0. The crosscheck results are illustrated as following tables. The detailed results could be seen in the attached excel files.

## Results of option 1 on top of VTM-11.0

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.08% | 1.15% | 1.20% | 1.07% | 1.13% | 1.19% | 102% | 100% |
| PQ422 | 0.98% | 1.05% | 1.05% | 0.97% | 1.03% | 1.03% | 103% | 101% |
| Overall | 1.03% | 1.10% | 1.12% | 1.02% | 1.08% | 1.11% | 102% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.40% | 1.92% | 1.74% | 1.37% | 1.85% | 1.73% | 100% | 100% |
| PQ422 | 1.32% | 1.69% | 1.69% | 1.29% | 1.65% | 1.68% | 99% | 101% |
| Overall | 1.36% | 1.80% | 1.72% | 1.33% | 1.75% | 1.70% | 99% | 100% |

|  |  |  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.34% | 1.85% | 1.71% | 1.30% | 1.79% | 1.69% | 102% | 98% |
| PQ422 | 1.32% | 1.64% | 1.58% | 1.28% | 1.60% | 1.58% | 102% | 97% |
| Overall | 1.33% | 1.74% | 1.65% | 1.29% | 1.69% | 1.64% | 102% | 98% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.98% | 1.04% | 1.04% | 101% | 103% |
| HLG422 | 0.94% | 0.99% | 0.99% | 102% | 101% |
| Overall | 0.96% | 1.02% | 1.01% | 102% | 102% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.24% | 0.29% | 0.29% | 100% | 103% |
| SVT12 | 0.48% | 0.54% | 0.54% | 97% | 99% |
| Overall | 0.36% | 0.41% | 0.41% | 98% | 101% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| SVT12 | 0.71% | 0.73% | 0.73% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Results of option 2 on top of VTM-11.0

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.08% | 1.14% | 1.17% | 1.07% | 1.13% | 1.15% | 106% | 102% |
| PQ422 | 0.98% | 1.04% | 1.04% | 0.97% | 1.02% | 1.03% | 99% | 96% |
| Overall | 1.03% | 1.09% | 1.10% | 1.02% | 1.08% | 1.09% | 102% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.40% | 1.92% | 1.72% | 1.36% | 1.85% | 1.71% | 102% | 102% |
| PQ422 | 1.33% | 1.67% | 1.69% | 1.29% | 1.64% | 1.69% | 103% | 103% |
| Overall | 1.37% | 1.79% | 1.70% | 1.33% | 1.75% | 1.70% | 102% | 102% |

|  |  |  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.34% | 1.85% | 1.71% | 1.30% | 1.78% | 1.69% | 106% | 98% |
| PQ422 | 1.31% | 1.65% | 1.59% | 1.28% | 1.60% | 1.59% | 99% | 99% |
| Overall | 1.32% | 1.75% | 1.65% | 1.29% | 1.69% | 1.64% | 102% | 98% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.98% | 1.04% | 1.04% | 102% | 103% |
| HLG422 | 0.94% | 0.99% | 0.99% | 101% | 100% |
| Overall | 0.96% | 1.02% | 1.01% | 102% | 101% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.24% | 0.29% | 0.29% | 96% | 104% |
| SVT12 | 0.48% | 0.54% | 0.54% | 96% | 98% |
| Overall | 0.36% | 0.41% | 0.41% | 96% | 101% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.42% | 0.42% | 0.43% | 100% | 102% |
| SVT12 | 0.71% | 0.73% | 0.73% | 96% | 96% |
| Overall | 0.57% | 0.58% | 0.58% | 98% | 99% |

## Results of option 1 on top of CE-1.2.a

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.08% | 1.14% | 1.15% | 1.07% | 1.13% | 1.13% | 100% | 100% |
| PQ422 | 0.98% | 1.06% | 1.06% | 0.97% | 1.04% | 1.04% | 100% | 100% |
| Overall | 1.03% | 1.10% | 1.10% | 1.02% | 1.08% | 1.09% | 100% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.40% | 1.92% | 1.76% | 1.36% | 1.86% | 1.74% | 100% | 99% |
| PQ422 | 1.32% | 1.72% | 1.68% | 1.28% | 1.68% | 1.68% | 100% | 100% |
| Overall | 1.36% | 1.82% | 1.72% | 1.32% | 1.77% | 1.71% | 100% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.34% | 1.87% | 1.71% | 1.30% | 1.80% | 1.69% | 100% | 100% |
| PQ422 | 1.31% | 1.65% | 1.60% | 1.28% | 1.62% | 1.60% | 100% | 100% |
| Overall | 1.32% | 1.76% | 1.66% | 1.29% | 1.71% | 1.65% | 100% | 100% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.98% | 1.05% | 1.05% | 100% | 100% |
| HLG422 | 0.94% | 0.99% | 1.00% | 99% | 101% |
| Overall | 0.96% | 1.02% | 1.02% | 100% | 100% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.44% | 0.48% | 0.48% | 100% | 99% |
| SVT12 | 0.52% | 0.56% | 0.57% | 100% | 100% |
| Overall | 0.48% | 0.52% | 0.52% | 100% | 99% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.55% | 0.53% | 0.54% | 100% | 100% |
| SVT12 | 0.70% | 0.73% | 0.73% | 100% | 100% |
| Overall | 0.62% | 0.63% | 0.63% | 100% | 100% |

## Results of option 2 on top of CE-1.2.a

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.08% | 1.15% | 1.15% | 1.07% | 1.14% | 1.14% | 97% | 98% |
| PQ422 | 0.98% | 1.05% | 1.05% | 0.97% | 1.03% | 1.03% | 101% | 100% |
| Overall | 1.03% | 1.10% | 1.10% | 1.02% | 1.08% | 1.08% | 99% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.39% | 1.92% | 1.75% | 1.36% | 1.85% | 1.73% | 102% | 104% |
| PQ422 | 1.32% | 1.72% | 1.69% | 1.28% | 1.67% | 1.69% | 100% | 103% |
| Overall | 1.36% | 1.82% | 1.72% | 1.32% | 1.76% | 1.71% | 101% | 103% |

|  |  |  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.33% | 1.86% | 1.71% | 1.30% | 1.80% | 1.70% | 98% | 98% |
| PQ422 | 1.32% | 1.65% | 1.61% | 1.28% | 1.62% | 1.61% | 98% | 96% |
| Overall | 1.32% | 1.75% | 1.66% | 1.29% | 1.71% | 1.65% | 98% | 97% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.44% | 0.48% | 0.48% | 100% | 99% |
| SVT12 | 0.52% | 0.56% | 0.56% | 99% | 100% |
| Overall | 0.48% | 0.52% | 0.52% | 100% | 99% |

# Conclusion

The partial results obtained in the cross-check match the results presented in JVET-U0069.