| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0127 |
| --- | --- |

| Title: | Coding results of DERF-TWITCH game sequences for the SDR HD low delay VVC verification test |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fabrice Le Léannec Gaëlle Martin-Cocher Edouard François 975 Avenue des Champs Blancs, 35510 Cesson-Sévigné, France Mathias Wien Lehrstuhl für Bildverarbeitung, RWTH Aachen University 52056 Aachen | Tel: Email: | fabrice.leleannec@interdigital.com wien@lfb.rwth-aachen.de |
| Source: | InterDigital, RWTH |  |  |

_____________________________

# Abstract

Contribution JVET-U0043 proposed three game video sequences for the SDR HD low delay VVC verification tests and was discussed during the AHG4 meeting of December 9, 2020.

Following this meeting, a number of chunks in game sequences, freely available on the DERF-TWITCH site, have been pre-selected and coded at various QPs. The goal is to provide candidate sequences to be used for the SDR HD low delay VVC verification tests.

This informational contribution describes the selected pieces of DERF-TWITCH sequences. It reports the HM-16.22 and VTM-11.0 rate distortion points that have been generated for each of the selected sequences in low delay configuration. BD-rate performances of VTM-11.0 over HM-16.22 with default CTC QPs are also provided.

The coded bit-streams are available on the JVET ftp site for further use in the VVC verification test, SDR HD low-delay.

# DERF-TWITCH game sequences

## Repository

The sequences that have been pre-selected can be downloaded in y4m format at following URL: https://media.xiph.org/video/derf/twitch/y4m/

and can be converted into YUV format with ffmpeg by using the following exemplary command line:

ffmpeg -i input.y4m -pix_fmt output.yuv

## Copyright information

The copyright under which the video sequences from DERF-TWITCH is made available on the following link:

https://media.xiph.org/video/derf/twitch/copyright.txt

which states that “These video game recordings are placed in the public domain. The recordings may contain copyrighted work which can only be used in compliance with fair use.”

## Sequences characteristics

The test sequences provided by DERF-TWITCH have the following properties:

- Resolution: 1920 x1080

- Scan: Progressive

- Frame rate: 60 fps

- Bit depth: 8-bit

- Length: 3600 frames (60s)

- Color format: 4:2:0

- Colour Components: Y’CbCr

- Colour space: ITU-R BT.709

# Pre-selected sequence chunks

Some 10 seconds picture intervals have been selected in a subset of the DERF-TWITCH set. They have been selected to avoid scene cut and too fast motion, so that the sub-sequences are suitable for visual assessment. The selected sequences and intervals are listed in the Table 1.

Table 1: DERF-TWITCH game sequences and associated picture intervals used for the reported coding results

| Sequence name | Snapshot | Considered picture interval |
| --- | --- | --- |
| CSGO |  | 1470 - 2069 |
| DOTA2 |  | 0940-1539 |
| EuroTruckSimulator2 |  | 3000-3599 |
| GTAV |  | 0091-0690 |
| Hearthstone |  | 0000-0599 |
| RUST |  | 0360-0959 |
| STARCRAFT2 |  | 2463-2062 |
| WITCHER3 |  | 0-599 |

# Rate distortion performances of HM-16.22 and VTM-11.0

The sequences in Table 1 have been coded with HM-16.22 and VTM-11.0, according to the JVET default test conditions, with a GOP size equal to 8. All QPs comprised in the [20,46] have been tested.

This section reports the obtained objective results. The Figure 1 shows all the generated RD points.

Figure 1: RD points obtained with all QP in interval [20,46] with HM-16.22 and VTM-11.0. Top: linear bitrate scale. Bottom: logarithmic bitrate scale

The Table 2 provides the comparative BD-PSNR performances of the VTM-11.0 and HM-16.22, when considering CTC QP values 22, 27, 32, and 37.

Table 2: VTM-11.0 rate distortion performances compared to HM-16.22, with default JVET CTC QP values.

# Conclusion

This contribution describes a set of pre-selected game sequences, as candidates for subjective assessment in the SDR HD low-delay VVC verification test.

Rate distortion points are provided in the attached Excel file, and objective results of the VTM-11.0 relative to HM-16.22 are provided.

The bit-streams generated during these encodings are available on the JVET ftp site, in directory /JVET-U_VT/sdr-hd-ld/gaming, for further use in the SDR HD low-delay VVC verification test.

# References

- M. Wien and V. Baroncini, “AHG4: Agenda and report of the AHG meeting on the SDR HD Verification Tests on 2020-09-03,” JVET-T0045, Sep. 2020.

- F. Le Léannec, G. Martin-Cocher, “Game video sequences proposal for the SDR HD low delay VVC verification test,” JVET-U0043, Jan. 2021.

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.