| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0137 |
| --- | --- |

| Title: | Side activity on preparation NN based video coding technologies viewing preparation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Wien, A.Segall, E. Alshina | Email: | wien@lfb.rwth-aachen.de asegall@sharplabs.com elena.alshina@huawei.com |
| Source: | AhG 11 |  |  |

_____________________________

# Abstract

This document summarizes activity on preparation viewing for NN based video coding technologies.

# Introduction

The goal of the viewing under preparation is to understand

- if BD-rate gain of 5-6% in PSNR or VMAF metric is noticeable,

- if artifacts generated by NN based algorithms look different from traditional artifacts generated by hybrid video codec,

Also it will be verified how successful selection of content and bit-rates for viewing was.

Comparison of different proposals in NN based video coding category is not the goal of the viewing. EE proposal will be viewed in comparison with VTM10.0 anchor (not against each other).

At this stage up to 1920×1080 videos have been selected for viewing. This upper limit for vide resolution should simplify viewing.

# Selection of materials for viewing.

Only proposals participated in EE conducted between JVET-U and JVET-U meetings considered for the viewing, since group has got better understanding of those technologies.

The majority of EE tests was about CNN based in-loop filters technologies. One test (JVET-U0096 [1]) included more than enchantment filter. It is expected that visual quality of this proposal would deviate from VTM anchor the most.

Proponents in EE reported PSNR and VMAF quality metrics. Also NN based filters were optimized using MSE and combination of SSIM and L1 norm is loss function. Two different loss functions used in a training are also expected to provide difference in visual quality. Selected EE tests are “original design” of JVET-U0074 [2] and “test 1” of JVET-U0094 [3].

Table 1. Selected candidates for viewing in Random Access configuration.

| Sequence | QP | bitrate, kbps |  | BD-rate, VMAF | bitrate, kbps |  | BD-rate, PSNR Y |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VTM10.0 | JVET-U0074 (original design) |  | VTM10.0 | JVET-U0094 (test 1) |  |
|  |  | GOP=16 | GOP=16 |  | GOP=32 | GOP=32 |  |
| BasketballDrive | 37 | 1071.9 | 1075.6 | -7.0% | 1080.9 | 1074.3 | -5.4% |
|  | 42 | 550.1 | 554.0 |  | 550.9 | 549.7 |  |
| BQTerrace | 37 | 746.9 | 750.3 | -8.5% | 754.5 | 750.8 | -6.0% |
|  | 42 | 365.0 | 366.8 |  | 367.1 | 367.0 |  |

Table 2. Selected candidates for viewing in Low delay -P configuration.

| Sequence | QP | VTM10.0 | JVET-U0096 | BD-rate, PSNR Y |
| --- | --- | --- | --- | --- |
| Cactus | 37 | 831.2 | 718.2 | -15.7% |
|  | 42 | 400.7 | 342.7 |  |
| SlideEditing | 37 | 69.5 | 64.3 | 52.3% |
|  | 42 | 50.5 | 46.1 |  |

Reconstructed 10 bits video for each of this test were lightly compressed using ffmpeg. Example of command line can be found below:

ffmpeg -s:v 1920x1080 -c:v rawvideo -pix_fmt yuv420p10le -r 60 -i ra_42_BQTerrace_1920x1080_60fps_10bit_420.yuv  -c:v libx265 -crf 10 ra_gop16_42_BQTerrace_1920x1080_60fps_10bit_420.mp4

Materials for viewing in mp4 format are uploaded to ftp://ftp.ient.rwth-aachen.de/JVET-U_DNN-viewing/.

# Instructions for viewing participants

TBA

# References

[1] JVET-U0096 EE: Tests on Decomposition, Compression and Synthesis (DCS)-based Technology

[2] JVET-U0074, EE: SSIM based CNN model for in-loop filtering

[3] JVET-U0094 EE: Tests on Neural Network-based In-Loop Filter