| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0138_v2 |
| --- | --- |

| Title: | Information on VVC coding efficiency in high bit depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-50-5213-6337 ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The contribution reports an experimental result comparing VTM vs HM in light of high bit depth coding. Compared to HM RExt coding,

In 12 bit case (overall AI/RA/LDB in HLG444/422)

- VTM shows 7.7 % gain while CE-1.3 RRC and TR fix shows 9.3 % gain in bdrateY

In 16 bit case (AI in SVT16)

- VTM show 25.1 % loss while CE-1.3 RRC and TR fix shows 1.4 % gain in bdrateG/B/R ave.

- One of the best performing RRC of CE-1.4 shows 2.7 % gain

It is noted that the gain and loss are QP and content dependent. It is the current CTC based result. PQ case is skipped for comparison since some coding tools in VTM shows loss.

Observation in overall AI/RA/LDB in 12 bit HLG444/422 reported as follows:

- Baseline VTM gain is 7.7 %

- RRC of CE-1.3 and TSRC of CE-2.1, gain is 7.8 % and 7.7 % respectively

- One of the best performing RRC of CE-1.4, gain is 7.9 %

- TR extension in JVET-U0052, gain is 9.1 %

- RRC of CE-1.3 and TR fix combination, gain becomes 9.3 % (one of the best case)

- 422 gain is bigger than 444 gain

Observation in AI in 16 bit SVT is reported as follows:

- Baseline VTM loss is 34.9 %

- RRC of CE-1.3 and TSRC of CE-2.1, loss reduces to 0.4 % and 5.1 % respectively

- One of the best performing RRC of CE-1.4, gain becomes 1.4 % (one of the best case)

- RRC of CE-1.3 and TSRC of CE-2.1 combination, loss reduces to 0.1 %

- TR fix in JVET-U0052, loss reduces to 26.7 %

- RRC of CE-1.3 and TR fix combination, gain is 0.5 %

V2 fixed SVT HM results

Summary of results

Summary of overall (AI+RA+LDB)

|  |  | psnrY | psnrU | psnrV |  |  | psnrG | psnrB | psnrR | average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HBT CTC | Overall HLG | -7.71% | -12.97% | -12.71% |  | Overall RGB | 11.54% | 12.80% | 12.77% | 12.4% |
| Rice RRC (CE-1.3) | Overall HLG | -7.88% | -13.09% | -12.83% |  | Overall RGB | -1.90% | -0.43% | -0.46% | -0.9% |
| Rice TS (CE-2.1) | Overall HLG | -7.74% | -12.99% | -12.73% |  | Overall RGB | 0.54% | 1.88% | 1.89% | 1.4% |
| Rice RRCTS (CE-3.1) | Overall HLG | -7.89% | -13.09% | -12.83% |  | Overall RGB | -2.01% | -0.51% | -0.54% | -1.0% |
| TR fix/ext (U0052) | Overall HLG | -9.14% | -14.99% | -14.87% |  | Overall RGB | 8.37% | 10.09% | 10.07% | 9.5% |
| Rice RRC+TR fix | Overall HLG | -9.26% | -15.07% | -14.95% |  | Overall RGB | -2.58% | -1.07% | -1.10% | -1.6% |
| Rice RRC (CE-1.4) | Overall HLG | -7.94% | -13.15% | -12.89% |  | Overall RGB | -2.82% | -1.12% | -1.14% | -1.7% |

Summary of AI

| HBT CTC | HLG444 | -5.43% | -5.88% | -6.49% |  | SVT16 | 36.24% | 34.30% | 34.24% | 34.9% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rice RRC (CE-1.3) | HLG444 | -5.88% | -6.26% | -6.86% |  | SVT16 | 0.54% | 0.30% | 0.26% | 0.4% |
| Rice TS (CE-2.1) | HLG444 | -5.52% | -5.92% | -6.52% |  | SVT16 | 5.24% | 5.03% | 5.03% | 5.1% |
| Rice RRCTS (CE-3.1) | HLG444 | -5.92% | -6.27% | -6.86% |  | SVT16 | 0.18% | 0.14% | 0.10% | 0.1% |
| TR fix*1 (U0052) | HLG444 | -7.04% | -8.64% | -9.47% |  | SVT16 | 26.22% | 26.91% | 26.85% | 26.7% |
| Rice+TR fix | HLG444 | -7.47% | -8.98% | -9.80% |  | SVT16 | -0.80% | -0.27% | -0.34% | -0.5% |
| Rice RRC (CE-1.4) | HLG444 | -5.96% | -6.28% | -6.88% |  | SVT16 | -2.07% | -1.11% | -1.14% | -1.4% |

Experiment results

VTM CTC anchor

| HDR HLG |  |  | AI |  |  |  | SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -5.43% | -5.88% | -6.49% | 3963% | 211% |  | SVT16 | 36.24% | 34.30% | 34.24% | 5312% | 159% |
| HLG422 | -6.93% | -8.74% | -9.16% | 3388% | 202% |  | SVT12 | 0.40% | 1.46% | 1.37% | 5646% | 156% |
| Overall | -6.18% | -7.31% | -7.83% | 3676% | 206% |  | Overall | 18.32% | 17.88% | 17.80% | 5479% | 157% |
|  |  |  | LDB |  |  |  |  |  |  | LDB |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -6.00% | -8.76% | -9.09% | 629% | 203% |  | SVT16 | 20.03% | 21.17% | 21.22% | 778% | 164% |
| HLG422 | -8.70% | -12.84% | -12.48% | 773% | 214% |  | SVT12 | -2.76% | -0.39% | -0.39% | 818% | 160% |
| Overall | -7.35% | -10.80% | -10.79% | 701% | 209% |  | Overall | 8.63% | 10.39% | 10.41% | 798% | 162% |
|  |  |  | RA |  |  |  |  |  |  | RA |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -6.21% | -8.77% | -9.09% | 707% | #DIV/0! |  | SVT16 | 18.19% | 20.83% | 20.75% | 1056% | 176% |
| HLG422 | -12.99% | -32.81% | -29.94% | 931% | 229% |  | SVT12 | -2.84% | -0.55% | -0.56% | 977% | 167% |
| Overall | -9.60% | -20.79% | -19.51% | 819% | #DIV/0! |  | Overall | 7.68% | 10.14% | 10.10% | 1016% | 171% |
| Overall HLG | -7.71% | -12.97% | -12.71% | 1732% | #DIV/0! |  | Overall RGB | 11.54% | 12.80% | 12.77% | 2431% | 163% |

Rice in RRC, CE1.3 (JVET-U0050)

| HDR HLG |  |  | AI |  |  |  | SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -5.88% | -6.26% | -6.86% | 4052% | 210% |  | SVT16 | 0.54% | 0.30% | 0.26% | 5450% | 120% |
| HLG422 | -7.38% | -9.11% | -9.54% | 3445% | 199% |  | SVT12 | -2.76% | -1.55% | -1.63% | 5758% | 152% |
| Overall | -6.63% | -7.68% | -8.20% | 3748% | 204% |  | Overall | -1.11% | -0.62% | -0.69% | 5604% | 136% |
|  |  |  | LDB |  |  |  |  |  |  | LDB |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -6.00% | -8.75% | -9.07% | 635% | 198% |  | SVT16 | -1.14% | 0.26% | 0.25% | 813% | 140% |
| HLG422 | -8.74% | -12.84% | -12.47% | 779% | 213% |  | SVT12 | -3.23% | -0.91% | -0.92% | 846% | 165% |
| Overall | -7.37% | -10.79% | -10.77% | 707% | 206% |  | Overall | -2.18% | -0.33% | -0.33% | 830% | 152% |
|  |  |  | RA |  |  |  |  |  |  | RA |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -6.23% | -8.77% | -9.08% | 772% | #DIV/0! |  | SVT16 | -1.38% | 0.43% | 0.37% | 971% | 144% |
| HLG422 | -13.03% | -32.81% | -29.95% | 1011% | 246% |  | SVT12 | -3.42% | -1.09% | -1.10% | 1023% | 158% |
| Overall | -9.63% | -20.79% | -19.51% | 892% | #DIV/0! |  | Overall | -2.40% | -0.33% | -0.37% | 997% | 151% |
| Overall HLG | -7.88% | -13.09% | -12.83% | 1782% | #DIV/0! |  | Overall RGB | -1.90% | -0.43% | -0.46% | 2477% | 146% |

Rice in TS, CE-2.1 (JVET-U0072)

| HDR HLG |  |  | AI |  |  |  | SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -5.52% | -5.92% | -6.52% | #NUM! | #NUM! |  | SVT16 | 5.24% | 5.03% | 5.03% | #NUM! | #NUM! |
| HLG422 | -7.03% | -8.80% | -9.23% | #NUM! | #NUM! |  | SVT12 | -1.04% | -0.10% | -0.14% | #NUM! | #NUM! |
| Overall | -6.28% | -7.36% | -7.87% | #NUM! | #NUM! |  | Overall | 2.10% | 2.47% | 2.45% | #NUM! | #NUM! |
|  |  |  | LDB |  |  |  |  |  |  | LDB |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -6.01% | -8.77% | -9.10% | #NUM! | #NUM! |  | SVT16 | 2.83% | 4.19% | 4.22% | #NUM! | #NUM! |
| HLG422 | -8.70% | -12.85% | -12.49% | #NUM! | #NUM! |  | SVT12 | -3.07% | -0.73% | -0.71% | #NUM! | #NUM! |
| Overall | -7.35% | -10.81% | -10.80% | #NUM! | #NUM! |  | Overall | -0.12% | 1.73% | 1.76% | #NUM! | #NUM! |
|  |  |  | RA |  |  |  |  |  |  | RA |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -6.21% | -8.77% | -9.09% | #NUM! | #DIV/0! |  | SVT16 | 2.52% | 3.78% | 3.81% | #NUM! | #NUM! |
| HLG422 | -12.99% | -32.81% | -29.94% | #NUM! | #NUM! |  | SVT12 | -3.24% | -0.89% | -0.87% | #NUM! | #NUM! |
| Overall | -9.60% | -20.79% | -19.51% | #NUM! | #DIV/0! |  | Overall | -0.36% | 1.44% | 1.47% | #NUM! | #NUM! |
| Overall HLG | -7.74% | -12.99% | -12.73% | #NUM! | #NUM! |  | Overall RGB | 0.54% | 1.88% | 1.89% | #NUM! | #NUM! |

Rice in RRC & TS, CE-3.1 (JVET-U0050)

| HDR HLG |  |  | AI |  |  |  | SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -5.92% | -6.27% | -6.86% | 4079% | 207% |  | SVT16 | 0.18% | 0.14% | 0.10% | 5500% | 122% |
| HLG422 | -7.43% | -9.14% | -9.57% | 3458% | 198% |  | SVT12 | -2.80% | -1.65% | -1.71% | 5827% | 155% |
| Overall | -6.68% | -7.70% | -8.22% | 3769% | 202% |  | Overall | -1.31% | -0.76% | -0.81% | 5664% | 139% |
|  |  |  | LDB |  |  |  |  |  |  | LDB |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -6.00% | -8.74% | -9.08% | 657% | 205% |  | SVT16 | -1.22% | 0.24% | 0.23% | 857% | 147% |
| HLG422 | -8.73% | -12.83% | -12.48% | 787% | 212% |  | SVT12 | -3.28% | -0.95% | -0.95% | 851% | 165% |
| Overall | -7.37% | -10.79% | -10.78% | 722% | 209% |  | Overall | -2.25% | -0.36% | -0.36% | 854% | 156% |
|  |  |  | RA |  |  |  |  |  |  | RA |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -6.22% | -8.77% | -9.08% | 716% | #DIV/0! |  | SVT16 | -1.45% | 0.26% | 0.22% | 974% | 140% |
| HLG422 | -13.03% | -32.81% | -29.94% | 945% | 224% |  | SVT12 | -3.50% | -1.12% | -1.12% | 1018% | 158% |
| Overall | -9.63% | -20.79% | -19.51% | 830% | #DIV/0! |  | Overall | -2.48% | -0.43% | -0.45% | 996% | 149% |
| Overall HLG | -7.89% | -13.09% | -12.83% | 1774% | #DIV/0! |  | Overall RGB | -2.01% | -0.51% | -0.54% | 2505% | 148% |

Transform fix (JVET-U0052)

| HDR HLG |  |  | AI |  |  |  | SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -7.04% | -8.64% | -9.47% | 4182% | 194% |  | SVT16 | 26.22% | 26.91% | 26.85% | 5971% | 160% |
| HLG422 | -7.98% | -10.43% | -10.95% | 3494% | 182% |  | SVT12 | -0.16% | 0.43% | 0.34% | 5625% | 145% |
| Overall | -7.51% | -9.53% | -10.21% | 3838% | 188% |  | Overall | 13.03% | 13.67% | 13.59% | 5798% | 152% |
|  |  |  | LDB |  |  |  |  |  |  | LDB |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -8.11% | -11.31% | -11.79% | 624% | 200% |  | SVT16 | 16.10% | 18.40% | 18.44% | 854% | 169% |
| HLG422 | -9.70% | -14.31% | -14.03% | 756% | 199% |  | SVT12 | -3.53% | -1.14% | -1.14% | 822% | 160% |
| Overall | -8.91% | -12.81% | -12.91% | 690% | 200% |  | Overall | 6.29% | 8.63% | 8.65% | 838% | 164% |
|  |  |  | RA |  |  |  |  |  |  | RA |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -8.20% | -11.34% | -11.81% | 691% | #DIV/0! |  | SVT16 | 15.12% | 17.18% | 17.19% | 999% | 170% |
| HLG422 | -13.81% | -33.89% | -31.17% | 877% | 213% |  | SVT12 | -3.57% | -1.23% | -1.24% | 990% | 155% |
| Overall | -11.01% | -22.61% | -21.49% | 784% | #DIV/0! |  | Overall | 5.78% | 7.97% | 7.98% | 995% | 163% |
| Overall HLG | -9.14% | -14.99% | -14.87% | 1771% | #DIV/0! |  | Overall RGB | 8.37% | 10.09% | 10.07% | 2543% | 160% |

Rice CE1.3 (JVET-U0050) + Transform fix (JVET-U0052)

| HDR HLG |  |  | AI |  |  |  | SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -7.47% | -8.98% | -9.80% | 4029% | 183% |  | SVT16 | -0.80% | -0.27% | -0.34% | 6374% | 121% |
| HLG422 | -8.40% | -10.77% | -11.29% | 3373% | 173% |  | SVT12 | -3.56% | -2.77% | -2.86% | 5485% | 131% |
| Overall | -7.93% | -9.88% | -10.55% | 3701% | 178% |  | Overall | -2.18% | -1.52% | -1.60% | 5929% | 126% |
|  |  |  | LDB |  |  |  |  |  |  | LDB |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -8.06% | -11.26% | -11.73% | 590% | 184% |  | SVT16 | -1.54% | 0.05% | 0.06% | 944% | 149% |
| HLG422 | -9.68% | -14.25% | -13.98% | 721% | 191% |  | SVT12 | -3.93% | -1.55% | -1.56% | 835% | 152% |
| Overall | -8.87% | -12.76% | -12.85% | 656% | 187% |  | Overall | -2.73% | -0.75% | -0.75% | 890% | 150% |
|  |  |  | RA |  |  |  |  |  |  | RA |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -8.16% | -11.30% | -11.76% | 654% | #DIV/0! |  | SVT16 | -1.62% | -0.16% | -0.16% | 1102% | 146% |
| HLG422 | -13.79% | -33.86% | -31.13% | 849% | 206% |  | SVT12 | -4.06% | -1.72% | -1.74% | 994% | 146% |
| Overall | -10.97% | -22.58% | -21.45% | 752% | #DIV/0! |  | Overall | -2.84% | -0.94% | -0.95% | 1048% | 146% |
| Overall HLG | -9.26% | -15.07% | -14.95% | 1703% | #DIV/0! |  | Overall RGB | -2.58% | -1.07% | -1.10% | 2622% | 141% |

Rice in RRC, CE1.4 (JVET-U0057)

| HDR HLG |  |  | AI |  |  |  | SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -5.96% | -6.28% | -6.88% | #NUM! | #NUM! |  | SVT16 | -2.07% | -1.11% | -1.14% | #NUM! | #NUM! |
| HLG422 | -7.49% | -9.17% | -9.61% | #NUM! | #NUM! |  | SVT12 | -3.75% | -2.29% | -2.38% | #NUM! | #NUM! |
| Overall | -6.73% | -7.73% | -8.24% | #NUM! | #NUM! |  | Overall | -2.91% | -1.70% | -1.76% | #NUM! | #NUM! |
|  |  |  | LDB |  |  |  |  |  |  | LDB |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -6.06% | -8.82% | -9.14% | #NUM! | #NUM! |  | SVT16 | -1.65% | -0.22% | -0.21% | #NUM! | #NUM! |
| HLG422 | -8.78% | -12.92% | -12.56% | #NUM! | #NUM! |  | SVT12 | -3.65% | -1.31% | -1.31% | #NUM! | #NUM! |
| Overall | -7.42% | -10.87% | -10.85% | #NUM! | #NUM! |  | Overall | -2.65% | -0.76% | -0.76% | #NUM! | #NUM! |
|  |  |  | RA |  |  |  |  |  |  | RA |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |  |  | psnrG | psnrB | psnrR | EncT | DecT |
| HLG444 | -6.28% | -8.84% | -9.15% | #NUM! | #NUM! |  | SVT16 | -1.88% | -0.28% | -0.28% | #NUM! | #NUM! |
| HLG422 | -13.06% | -32.86% | -29.99% | #NUM! | #NUM! |  | SVT12 | -3.90% | -1.50% | -1.51% | #NUM! | #NUM! |
| Overall | -9.67% | -20.85% | -19.57% | #NUM! | #NUM! |  | Overall | -2.89% | -0.89% | -0.90% | #NUM! | #NUM! |
| Overall HLG | -7.94% | -13.15% | -12.89% | #NUM! | #NUM! |  | Overall RGB | -2.82% | -1.12% | -1.14% | #NUM! | #NUM! |

References

- JVET-U0050, “CE-1.3 and CE-3.1: Rice parameter derivation for high bit-depth coding”, T. Hashimoto, T. Ikai (Sharp)

- JVET-U0057, “CE-1.4, CE-1.5, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths”, A. Browne, S. Keating, K. Sharman (Sony)

- JVET-U0075, “CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang (Kwai Inc.)

- JVET-U0052, “AHG8: Transform coefficients range extension for high bit-depth coding”, T. Zhou, T. Chujoh, E. Sasaki, T. Ikai (Sharp)

- JVET-T2018, “Common test conditions for high bit depth and high bit rate video coding”, A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).