| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0139 |
| --- | --- |

| Title: | BoG report on high bit rate / high bit depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne | Email: | adrian.browne@sony.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This is a report of activities from the BoG on high bit rate / high bit depth coding. The BoG held a meeting at the following time during the 21st JVET meeting:

- January 13th 13:10 – 15:15

The BoG recommends the following with respect to adding HDR CRC based higher QP tests to the high bit depth / high bit rate CTC (JVET-T2018):

- Consider choice of either QP range 10-25 or 22-37

- InternalBitDepth=12 but otherwise HDR CTC settings in addition to existing HBD CTC

- 420 content only – no 422/444

The BoG recommends the following with respect to the existing tests in the high bit depth CTC:

- use 2K/HD HLG sequences.

- ask for contributions of new 12/16 bit RGB sequences

- disable LMCS for PQ/HLG

- consider disabling: BCW, MMVD, SMVD, Geo, BIO, DMVR, Affine, ISP

- to include JVET-U0103 in VTM 12.0 (SIMD enhancements)

- include FireEater sequence in high bit depth CTC

Additionally, the BoG recommends the following with respect to the high bit depth CTC:

- request 12 bit screen content to include in HBD CTC and opinions on suitable QPs for this content

The BoG recommends the following with respect to the next round of the CE:

- use VTM 12.0 with SIMD modifications (JVET-U0103)

- use CE-1.1 for RRC and CE-2.1 for TSRC as anchor for 16 bit testing in CE

# Mandates

The BoG was established with the following mandates:

- to propose the plan for the next round of the CE on entropy coding for high bit depths.

- identify which of the current proposals should be further investigated.

- discuss modifications of test conditions

  - sequences

  - QP ranges

  - anchor for 12/16 bit

  - additional HEVC RExt anchor

# Contributions

JVET-U0138, “Information on VVC coding efficiency in high bit depth coding”, T. Ikai (Sharp)

There was no time to review this contribution.

# Common Test Conditions

Greater range of QPs – including higher QPs normally associated with consumer applications

- Using modified HDR CTC settings

  - What QPs?

    - 12-27

  - 420 / 422?

  - Sequences from HBD CTC?

One expert considered testing at such high QPs already handled by previous experiments.

One experts QPs at 10 – 25 to enable simpler comparison between 10 and 12 bit

Another expert backs the range of 10-25 to enable evaluation of other 12 bit tools

One expert suggests existing range 22-37 for comparison with 10 bit HDR CTC and potentially suitable for premium consumer level applications

One expert suggests the decision should depend on the expected bit rate increase and this is a good way of thinking about how much QPs can be adjusted

One expert suggests that the bit rate in a 12 bit consumer profile/use-case will be close to that for a 10 bit profile

Recommendation: Consider choice of either QP range 10-25 or 22-37

Recommendation: InternalBitDepth=12 but otherwise HDR CTC settings in addition to existing HBD CTC

Recommendation: HBD 10-25 range 420 content only – no 422/444

Note: need to create 420 versions of HBD CTC sequences

One expert suggested only including All Intra and Random Access – not Low Delay B/P - recommended

One expert would like to include some screen content sequences – this could be optional

The lack of 12 bit screen content is an issue that needs to be resolved

Recommendation: Request 12 bit screen content to include in HBD CTC and opinions on suitable QPs for this content

Replace 4K HLG sequences with HD HLG sequences to reduce simulation times (need HDRTools for conversion)

Recommendation: use 2K/HD HLG sequences.

Alternatives to SVT 12/16 bit RGB sequences

Recommendation: ask for contributions of new 12/16 bit RGB sequences

Disable tools (not for 10-25 420 QP tests) above? (JVET-U0072 tool off) (JVET-U0103 SIMD)

- LMCS for PQ / HLG

- BCW

- MMVD

- SMVD

- Geo

- BIO

- DMVR

- Affine

- ISP

- LFNST (already disabled)

One expert suggested keeping the tools enabled to allow comparison once CE is complete

Several experts suggested disabling tools was useful for encode run time reduction

Recommendation: consider disabling tools above.

VTM 12.0 anticipated 3 weeks after the meeting – end first week Feb

Recommendation: to include JVET-U0103 in VTM 12.0

Recommendation: include FireEater sequence in high bit depth CTC

# CE

Experiments

CE-1 – RRC

CE-1.1

CE-1.2a

CE-1.2b

CE-1.3

CE-1.5

CE-3.5

CE-3.6

JVET-U0062

One expert suggested not having a combination phase in this CE – combinations of techniques can be CE-related or non-CE contributions

One expert suggested that the agenda for the CE is not clear – how to evaluate the combination of transform coefficient clipping and rice coding modifications

CE-2 – TSRC?

CE-2.1

CE-2.3

One expert suggested combinations of 1 technique from either CE-1 and CE-2 with CE-3 would be useful as a combination not further combinations of multiple CE-1 / CE-2 tests -this is better as a CE-related / non-CE contribution

One expert suggested that any combinations only be allowed in the CE if defined in the initial CE drafting phase.

CE-3 – Transform coefficient range clipping (JVET-U0052)

Tested clipping to 20 – have other values been checked e.g. 18, 19, 20, 21, 22 for both 12 and 16 bit cases

Code base

Recommendation: VTM 12.0 with SIMD modifications

Anchors

VTM with rice coding tool for assessing 16 bit SVT

One expert suggested CE-2.1 for TSRC

One expert suggested highest performance for 16 bit configuration

Several experts suggested using simplest would be appropriate

Several expert suggested CE-1.1 for RRC

Recommendation: use CE-1.1 for RRC and CE-2.1 for TSRC as anchor for 16 bit testing in CE (all proponents of CE-1 and CE-2 content)

One expert emphasized that tests for CE-1.1 and CE-2.1 would still be required against VTM

One expert suggested full agreed anchor for VTM still required

HM 16.22

Timetable (what is deadline for next meeting)