| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U1004-v1 |
| --- | --- |

| Title: | Errata report items for VVC, VSEI, HEVC, AVC, Video CICP, and CP Usage TR |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Errata report |  |  |
| Author(s) or Contact(s): | Chris Rosewarne Gary Sullivan Yasser Syed Ye-Kui Wang | Tel: Email: | chris.rosewarne@canon.com.au garysull@miscrosoft.com yasser_syed@comcast.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains a list of reported errata items for VVC, VSEI, HEVC, AVC, Video CICP, and the TR on usage of video signal type code points, for tracking purposes. Some of the items have been confirmed by the JVET and have been agreed to require fixing, while some other items have not yet been confirmed. This document also provides publication status backgrounds of these standards.

Incorporated items at the JVET-U meeting:

- For VVC (the changes are included in an attachment to this document):

  - Some minor editorial corrections and improvements (JVET-U0073, JVET-U0085, and one item from Hendry)

  - Fixes for tickets #1416, #1428, #1432, #1454, and #1469

- For AVC: Added a reference to JVET-T1006-v1 for the errata changes, and removed the actual errata text changes from this document. (JVET-U0049)

# General aspects and issues affecting multiple standards

## General and minor matters

Please note that the alignment between the text published by ITU-T and that published by ISO/IEC should also be checked. Minor editorial issues and aspects that are highlighted for potential further checking include the following:

- (for HEVC only, and for the ITU-T text only; for the ISO/IEC text, this has been corrected in w18277_FDIS_23008-2_4thEd_plusAmd1_w18881) In the semantics of matrix_coeffs, there seems to be a problem in the HEVC spec (the AVC text is OK), saying "one or more" instead of "both" regarding the following constraint:

matrix_coeffs shall not be equal to 0 unless both of the following conditions are true:

–	BitDepthC is equal to BitDepthY.

–	chroma_format_idc is equal to 3 (the 4:4:4 chroma format).

- Updating the reference to Rec. ITU-R BT.2100 (the current version being BT.2100-2) and the associated equations for the ICTCP matrix coefficients interpretation for HLG (esp. check Video CICP; this is correct in Rec. ITU-T H.265 2019-06).

- (for Video CICP only, in the ISO/IEC version only) Logical indentation nesting in the ISO/IEC version of the Video CICP colour interpretation was inadvertently removed in the publication process.

- (for Video CICP only) The range of values for the extended aspect ratio indicator in Video CICP is not clearly specified and may implicitly be interpreted as inadequate to cover the range of values expressed in the video coding standards.

## sYCC colour indicator interpretation

### Status

This item has been confirmed by the JCT-VC and resolved for the ITU-T text, but has not yet been resolved for the ISO/IEC text. It affects multiple standards: HEVC, AVC, and Video CICP (and JPEG XR). For background, see JCTVC-AJ0023.

### Description of the issue

This issue relates to the transfer characteristics and matrix coefficients indicators for the sYCC colour representation specified in IEC 61966-2-1. The the semantics of transfer characteristics (Table E.4 of HEVC), and matrix coefficients (Table E.5 of HEVC) need correction to address the issue.

The issue is a bit complicated because the same transfer characteristics indicator value is used for both sRGB and sYCC, but IEC 61966-2-1 actually indicates that the transfer characteristics function should be somewhat different for the two cases. In the sRGB case, the range of the input value is constrained to be from 0 to 1, but in the sYCC case, this constraint should not apply.

The agreed correction for this aspect is to condition the interpretation of the transfer_characteristics syntax element for the value 13 on the value of the matrix_coeffs syntax element (since that value would differ between sYCC and sRGB).

Additionally, the the informative remark relating to the matrix coefficients indicator value for IEC 61966-2-1 sYCC should be changed to indicate that sYCC should be indicated with the matrix coefficients indicator equal to 5 (as for Rec. ITU R BT.601) rather than 1 (as for Rec. ITU-R BT.709).

### Description of the proposed fix

In E.3.1 (VUI semantics), in Table E.4 (Transfer characteristics interpretation using the transfer_characteristics syntax element) replace the row for the value 13 with:

| 13 | – If matrix_coeffs is equal to 0 V = α * Lc( 1 ÷ 2.4 ) − ( α − 1 ) for 1 >= Lc >= β V = 12.92 * Lc for β > Lc >= 0 – Otherwise V = α * Lc( 1 ÷ 2.4 ) − ( α − 1 ) for Lc >= β V = 12.92 * Lc for β > Lc > −β V = − α * ( −Lc )( 1 ÷ 2.4 ) + ( α − 1 ) for −β >= Lc | IEC 61966-2-1 sRGB (with matrix_coeffs equal to 0) IEC 61966-2-1 sYCC (with matrix_coeffs equal to 5) |
| --- | --- | --- |

In E.3.1 (VUI semantics), after Equation E-3, replace the next paragraph and associated bullet points with:

In this case, the range of E′R, E′G, and E′B is specified as follows:

–	If transfer_characteristics is equal to 11 or 12, or transfer_characteristics is equal to 13 and matrix_coeffs is not equal to 0, E′R, E′G, and E′B are real numbers with values that have a larger range than the range of 0 to 1, inclusive, and their range is not specified in this Specification.

–	Otherwise, E′R, E′G and E′B are real numbers in the range of 0 to 1.

In E.3.1 (VUI semantics), in Table E.5 (Matrix coefficients interpretation using the matrix_coeffs syntax element), move “IEC 61966-2-1 sYCC” from the row for the value 1 to the row for the value 5.

# Reported errata items for VVC

Some editorial errata items for VVC are included in an attachment to this document. JVET-U2005 includes other errata changes, particularly some technical corrections (in addition to some amendments to VVC).

## Publication status background

Rec. ITU-T H.266 (https://www.itu.int/rec/T-REC-H.266/en)

- (08/2020, Edition 1) Approved 2020-08-29, published 2020-11-10

ISO/IEC 23090-3 (https://www.iso.org/standard/73022.html)

- ISO/IEC 23090-3:2021 (Edition 1); published 2021-02

# Reported errata items for VSEI

JVET-U2006 includes some errata changes for VSEI (in addition to some amendments).

## Publication status background

Rec. ITU-T H.274 (https://www.itu.int/rec/T-REC-H.274/en)

- (08/2020, Edition 1) Approved 2020-08-29, published 2020-11-10

ISO/IEC 23002-7 (https://www.iso.org/standard/79112.html)

- ISO/IEC 23002-7:2021 (Edition 1); published 2021-01

# Reported errata items for HEVC

See also Section 1.

## Publication status background

Rec. ITU-T H.265

- (02/18, Edition 5) Approved 2018-02-13, published 2018-05-11

- (06/19, Edition 6) Approved 2019-06-29, published 2019-09-23

- (10/19, Edition 7) Approved 2019-11-29, published 2020-01-10

ISO/IEC 23008-2

- ISO/IEC 23008-2:2017 (Edition 3); published 2017-10

- ISO/IEC 23008-2:2017/Amd 1:2018 (Additional colour representation code point); published 2018-03

- ISO/IEC 23008-2:2017/Amd 2:2018 (Main 10 still picture profile); published 2018-03

- ISO/IEC 23008-2:2017/Amd 3:2018 (Additional supplemental enhancement information); published 2018-07

- Work items merged for FDIS ballot as publication office action; Stage 50.00 Final text received or FDIS registered for formal approval as of 2020-03-19

  - ISO/IEC DIS 23008-2:201x (Edition 4); DIS ballot closed 2018-10-06; stage 40.99, Full report circulated: DIS approved for registration as FDIS 2019-02-18; Pending FDIS ballot based on WG 11 N 18277

  - ISO/IEC 23008-2:201x (Edition 4)/DAmd 1:201x (Additional supplemental enhancement information); DAM ballot started 2019-07-10, closed 2019-10-02

## On the general decoding process

### Status

This item applies to both ITU-T and ISO/IEC texts.

This item has been confirmed by the JCT-VC, and the proposed fix has been agreed in spirit. For background, see JCTVC-AI0022 and JCTVC-AJ0021. However, the specific text for the fix is not yet available and hence this has not yet been resolved.

### Description of the issue

The general decoding process is specified in a CVS-by-CVS manner, because the sub-bitstream extraction process relies on the SPS syntax element sps_max_sub_layers_minus1, while the active SPS can change across CVSs.

For each CVS of the input bitstream, the sub-bitstream extraction process is applied first with the output being BitstreamToDecode. Therefore, BitstreamToDecode contains one CVS. After that, the decoding process for a coded picture specified in clause 8.1.3 is repeatedly invoked for each coded picture in BitstreamToDecode.

The following is stated in clause 8.1.3:

The decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7. When interpreting the semantics of each syntax element in each NAL unit, the term "the bitstream" (or part thereof, e.g., a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

From the yellow-highlighted wording, it is clear that the term "the bitstream" mentioned above was intended to mean something that can contain more than one CVS.

Furthermore, there are many places related to "interpreting the semantics of each syntax element in each NAL unit" that have the wording "the first picture in the bitstream", particularly when used to determine whether a CRA picture starts a CVS. In my understanding, in those places, the intent was that the term "the bitstream" mentioned above was intended to mean something that can contain more than one CVS.

Lastly, at the beginning of the general decoding process for the input bitstream, strictly speaking, determining the CVSs in the input bitstream would not be clear before determination of the variable NoRaslOutputFlag for each IRAP picture, which currently is only part of the decoding process of a CVS.

### Description of the proposed fix

It is proposed to change the description of the general decoding process as follows:

- Firstly the CVSs in the input bitstream are identified.

- Sub-bitstream extraction is applied for each CVS.

- The extracted results of all the CVSs are concatenated to BitstreamToDecode.

- BitstreamToDecode is decoded CVS by CVS, and each CVS picture by picture.

## On HRD for splicing

### Status

This item has been confirmed by the JCT-VC and resolved for the ITU-T text, but has not yet been resolved for the ISO/IEC text. For background, see JCTVC-AK0027.

### Description of the issue

It is reported that the current HEVC specification determines, when concatenationFlag is set to 1, that the AuNominalRemovalTime for the splicing point under some circumstances (non-seamless splicing) may be greater than AuNominalRemovalTime[ prevNonDiscardablePic ] + ( auCpbRemovalDelayDeltaMinus1 + 1 ).

The text in HEVC specifies the following:

if( !concatenationFlag ) {
	baseTime = AuNominalRemovalTime[ firstPicInPrevBuffPeriod ]
	tmpCpbRemovalDelay = AuCpbRemovalDelayVal
} else {
	baseTime = AuNominalRemovalTime[ prevNonDiscardablePic ]
	tmpCpbRemovalDelay =
		Max( ( auCpbRemovalDelayDeltaMinus1 + 1 ),	(C-10)
			Ceil( ( InitCpbRemovalDelay[ SchedSelIdx ] ÷ 90000 +
				AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] ) ÷ ClockTick ) )
}
AuNominalRemovalTime[ n ] = baseTime + ClockTick * ( tmpCpbRemovalDelay − CpbDelayOffset )

In the following figure, a splicing operation is shown that reportedly does not allow for seamless splicing since the InitCpbRemovalDelay[ SchedSelIdx ] imposes a later removal time after splicing than equidistant CPB removal times as present in seamless playout.

If instead of deriving the removal time of AU 6 from the AU n-1 (i.e. AU 5) as indicated above in the figure, we use the last non-discardable picture (AU 4), this would reportedly result in the following:

taf6= trm4 + ∆ >= trm6 (CPB underflow resulting in non-seamless splicing)

trm6= taf5+InitCpbRemovalDelay

∆ =taf6 – trm4= Size / Bitrate – ( trm4 – taf5 )

trm4 + Size / Bitrate – ( trm4 – taf5 )  >=  taf5 + InitCpbRemovalDelay

Size / Bitrate – ( trm4 – taf5 )  >=  ( taf5 – trm4 )+ InitCpbRemovalDelay

taf5 + Size / Bitrate = taf6  >=  trm4 + ( taf5 – trm4 ) + InitCpbRemovalDelay

In the derived formula, the part marked in yellow highlight and boldface in Equation C-10 above is reportedly not correct.

### Description of the proposed fix

The proposed fix is to change Equation C-10 to the following:

if( !concatenationFlag ) {
	baseTime = AuNominalRemovalTime[ firstPicInPrevBuffPeriod ]
	tmpCpbRemovalDelay = AuCpbRemovalDelayVal
} else {
	baseTime1 = AuNominalRemovalTime[ prevNonDiscardablePic ]
	tmpCpbRemovalDelay1 = ( auCpbRemovalDelayDeltaMinus1 + 1 )
	baseTime2 = AuNominalRemovalTime[ n − 1 ]
	tmpCpbRemovalDelay2 = 
		Ceil( ( InitCpbRemovalDelay[ SchedSelIdx ] ÷ 90000 +
			AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] ) ÷ ClockTick )	(C-10)
	if( baseTime1 + ClockTick * tmpCpbRemovalDelay1 < 
		baseTime2 + ClockTick * tmpCpbRemovalDelay2 ) {
		baseTime = baseTime2
		tmpCpbRemovalDelay = tmpCpbRemovalDelay2
	} else {
		baseTime = baseTime1
		tmpCpbRemovalDelay = tmpCpbRemovalDelay1
	}
}
AuNominalRemovalTime[ n ] = baseTime + ClockTick * ( tmpCpbRemovalDelay − CpbDelayOffset )

## On filtering process for chroma block edges

### Status

This item has been confirmed by the JCT-VC and resolved for the ITU-T text, but has not yet been resolved for the ISO/IEC text.

### Bug fix

In 8.7.2.5.5 (Filtering process for chroma block edges), change the following:

If ChromaArrayType is equal to 1, the variable QpC is determined as specified in Table ‎8-10 based on the index qPi derived as follows:

qPi = ( ( QpQ + QpP + 1 )  >>  1 ) + cQpPicOffset	(‎8-384)

Otherwise (ChromaArrayType is greater than 1), the variable QpC is set equal to Min( qPi, 51 ).

to

The index qPi derived as follows:

qPi = ( ( QpQ + QpP + 1 )  >>  1 ) + cQpPicOffset	(8-384)

The variable QpC is derived as follows:

–	If ChromaArrayType is equal to 1, the variable QpC is determined based on qPi as specified in Table 8-10.

–	Otherwise (ChromaArrayType is greater than 1), the variable QpC is set equal to Min( qPi, 51 ).

## Absence of “persistence flag” for the annotated regions SEI message

This item applies to both ITU-T and ISO/IEC texts.

In all SEI messages that have a "cancel flag", there is also a corresponding "persistence flag", except in the case of the newly adopted annotated regions SEI message. Without this flag, it is not possible to indicate that the annotated regions information applies only to the current picture. Due to the extensibility provision that is built into the SEI message syntax, it would be possible to append a persistence flag at the end of the syntax to correct this likely oversight. The presence of the extra flag could be optional (conditioned on whether there is more data in the SEI message), so that encoders built for the original syntax would still be conforming. When the flag is not present, it would be inferred to be equal to 1.

## On the film grain characteristics (FGC) SEI message

This item came from JCTVC-AN0021 and was refined in JVET-T0075-v2, and is likely applicable to both ITU-T and ISO/IEC texts.

Note that the refined text changes in JVET-T0075-v2 are equivalent to the semantics of the FGC SEI message in JVET-S2007-v7.

The changes are as follows:

### D.3.13	Film grain characteristics SEI message semantics

This SEI message provides the decoder with a parameterized model for film grain synthesis.

NOTE 1 – For example, an encoder could use the film grain characteristics SEI message to characterize film grain that was present in the original source video material and was removed by pre-processing filtering techniques. Synthesis of simulated film grain on the decoded images for the display process is optional and does not need to exactly follow the specified semantics of the film grain characteristics SEI message. When synthesis of simulated film grain on the decoded images for the display process is performed, there is no requirement that the method by which the synthesis is performed be the same as the parameterized model for the film grain as provided in the film grain characteristics SEI message.

NOTE 2 – The display process is not specified in this Specification.

NOTE 3 – SMPTE RDD 5 specifies a film grain simulator based on the information provided in the film grain characteristics SEI message.

The film grain models specified in the film grain characteristics SEI message are expressed for application to decoded pictures that have 4:4:4 colour format with luma and chroma bit depths corresponding to the luma and chroma bit depths of the film grain model and use the same colour representation domain as the identified film grain model. When the colour format of the decoded video is not 4:4:4 or the decoded video uses a different luma or chroma bit depth from that of the film grain model or uses a different colour representation domain from that of the identified film grain model, an unspecified conversion process is expected to be applied to convert the decoded pictures to the form that is expressed for application of the film grain model.

NOTE 4 – Because the use of a specific method is not required for performing the film grain generation function used by the display process, a decoder could, if desired, down-convert the model information for chroma in order to simulate film grain for other chroma formats (4:2:0 or 4:2:2) rather than up-converting the decoded video (using a method not specified in this Specification) before performing film grain generation.

film_grain_characteristics_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous film grain characteristics SEI message in output order that applies to the current layer. film_grain_characteristics_cancel_flag equal to 0 indicates that film grain modelling information follows.

film_grain_model_id identifies the film grain simulation model as specified in Table D.4. The value of film_grain_model_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for film_grain_model_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with film_grain_model_id equal to 2 or 3.

Table D.4 – film_grain_model_id values

| Value | Description |
| --- | --- |
| 0 | Frequency filtering |
| 1 | Auto-regression |

separate_colour_description_present_flag equal to 1 indicates that a distinct combination of luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients for the film grain characteristics specified in the SEI message is present in the film grain characteristics SEI message syntax. separate_colour_description_present_flag equal to 0 indicates that the combination of luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients for the film grain characteristics specified in the SEI message are the same as indicated in VUI parameters for the CVS.

NOTE 5 – When separate_colour_description_present_flag is equal to 1, any of the luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients specified for the film grain characteristics specified in the SEI message could differ from that for the pictures in the CVS.

When VUI parameters are not present for the CVS or the value of colour_description_present_flag is equal to 0, and equivalent information to that conveyed when colour_description_present_flag is equal to 1 is not conveyed by external means, separate_colour_description_present_flag shall be equal to 1.

The decoded image Idecoded used in the equations in this clause is in the same colour representation domain as the simulated film grain signal. Therefore, when any of these parameters does differ from that for the pictures in the CVS, the decoded image Idecoded used in the equations in this clause would be in a different colour representation domain than that for the pictures in the CVS. For example, when the value of film_grain_bit_depth_luma_minus8 + 8 is greater than the bit depth of the luma component of the pictures in the CVS, the bit depth of Idecoded used in the equations in this clause is also greater than the bit depth of the luma component of the pictures in the CVS. In such a case, the decoded image Idecoded corresponding to an actual decoded picture would be generated by converting the actual decoded picture to be in the same colour representation domain as the the simulated film grain signal. The process for converting an actual decoded pictures to the 4:4:4 colour format with same colour representation domain as the the simulated film grain signal is not specified in this Specification.

film_grain_bit_depth_luma_minus8 plus 8 specifies the bit depth used for the luma component of the film grain characteristics specified in the SEI message. When film_grain_bit_depth_luma_minus8 is not present in the film grain characteristics SEI message, the value of film_grain_bit_depth_luma_minus8 is inferred to be equal to bit_depth_luma_minus8.

The value of filmGrainBitDepth[ 0 ] is derived as follows:

filmGrainBitDepth[ 0 ] = film_grain_bit_depth_luma_minus8 + 8	(D-3)

film_grain_bit_depth_chroma_minus8 plus 8 specifies the bit depth used for the Cb and Cr components of the film grain characteristics specified in the SEI message. When film_grain_bit_depth_chroma_minus8 is not present in the film grain characteristics SEI message, the value of film_grain_bit_depth_chroma_minus8 is inferred to be equal to bit_depth_chroma_minus8.

The value of filmGrainBitDepth[ c ] for c = 1 and 2 is derived as follows:

filmGrainBitDepth[ c ] = film_grain_bit_depth_chroma_minus8 + 8, with c = 1, 2	(D-4)

film_grain_full_range_flag has the same semantics as specified in clause E.3.1 for the video_full_range_flag syntax element, except as follows:

–	film_grain_full_range_flag specifies the video full range flag of the film grain characteristics specified in the SEI message, rather than the video full range flag used for the CVS.

–	When film_grain_full_range_flag is not present in the film grain characteristics SEI message, the value of film_grain_full_range_flag is inferred to be equal to video_full_range_flag.

film_grain_colour_primaries has the same semantics as specified in clause E.3.1 for the colour_primaries syntax element, except as follows:

–	film_grain_colour_primaries specifies the colour primaries of the film grain characteristics specified in the SEI message, rather than the colour primaries used for the CVS.

–	When film_grain_colour_primaries is not present in the film grain characteristics SEI message, the value of film_grain_colour_primaries is inferred to be equal to colour_primaries.

film_grain_transfer_characteristics has the same semantics as specified in clause E.3.1 for the transfer_characteristics syntax element, except as follows:

–	film_grain_transfer_characteristics specifies the transfer characteristics of the film grain characteristics specified in the SEI message, rather than the transfer characteristics used for the CVS.

–	When film_grain_transfer_characteristics is not present in the film grain characteristics SEI message, the value of film_grain_transfer_characteristics is inferred to be equal to transfer_characteristics.

film_grain_matrix_coeffs has the same semantics as specified in clause E.3.1 for the matrix_coeffs syntax element, except as follows:

–	film_grain_matrix_coeffs specifies the matrix coefficients of the film grain characteristics specified in the SEI message, rather than the matrix coefficients used for the CVS.

–	When film_grain_matrix_coeffs is not present in the film grain characteristics SEI message, the value of film_grain_matrix_coeffs is inferred to be equal to matrix_coeffs.

–	The values allowed for film_grain_matrix_coeffs are not constrained by the chroma_format_idc of the decoded pictures that is indicated by the value of chroma_format_idc for the semantics of the VUI parameters.

blending_mode_id identifies the blending mode used to blend the simulated film grain with the decoded images as specified in Table D.5. blending_mode_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for blending_mode_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with blending_mode_id equal to 2 or 3.

Table D.5 – blending_mode_id values

| Value | Description |
| --- | --- |
| 0 | Additive |
| 1 | Multiplicative |

Depending on the value of blending_mode_id, the blending mode is specified as follows:

–	If blending_mode_id is equal to 0, the blending mode is additive as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  filmGrainBitDepth[ c ] ) − 1, Idecoded[ c ][ x ][ y ] + G[ c ][ x ][ y ] )	(D-5)

–	Otherwise (blending_mode_id is equal to 1), the blending mode is multiplicative as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  filmGrainBitDepth[ c ] ) − 1, Idecoded[ c ][ x ][ y ] +	(D-6)
			Round( ( Idecoded[ c ][ x ][ y ] * G[ c ][ x ][ y ] ) ÷ ( ( 1  <<  bitDepth[ c ] ) − 1 ) ) )

where Idecoded[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the decoded image Idecoded, G[ c ][ x ][ y ] is the simulated film grain value at the same position and colour component, and filmGrainBitDepth[ c ] is the number of bits used for each sample in a fixed-length unsigned binary representation of the arrays Igrain[ c ][ x ][ y ], Idecoded[ c ][ x ][ y ], and G[ c ][ x ][ y ], where c = 0..2, x = 0..pic_width_in_luma_samples − 1, and y = 0..pic_height_in_luma_samples – 1

log2_scale_factor specifies a scale factor used in the film grain characterization equations.

comp_model_present_flag[ c ] equal to 0 indicates that film grain is not modelled on the c-th colour component, where c equal to 0 refers to the luma component, c equal to 1 refers to the Cb component, and c equal to 2 refers to the Cr component. comp_model_present_flag[ c ] equal to 1 indicates that syntax elements specifying modelling of film grain on colour component c are present in the SEI message.

When separate_colour_description_present_flag is equal to 0 and chroma_format_idc is equal to 0, the value of comp_model_present_flag[ 1 ] and comp_model_present_flag[ 2 ] shall be equal to 0.

num_intensity_intervals_minus1[ c ] plus 1 specifies the number of intensity intervals for which a specific set of model values has been estimated.

NOTE 6 – The intensity intervals could overlap in order to simulate multi-generational film grain.

num_model_values_minus1[ c ] plus 1 specifies the number of model values present for each intensity interval in which the film grain has been modelled. The value of num_model_values_minus1[ c ] shall be in the range of 0 to 5, inclusive.

intensity_interval_lower_bound[ c ][ i ] specifies the lower bound of the i-th intensity interval for which the set of model values applies.

intensity_interval_upper_bound[ c ][ i ] specifies the upper bound of the i-th intensity interval for which the set of model values applies.

The variable intensityIntervalIdx[ c ][ x ][ y ][ j ] represents the j-th index to the list of intensity intervals selected for the sample value Idecoded[ c ][ x ][ y ] for c = 0..2, x = 0..pic_width_in_luma_samples − 1, y = 0..pic_height_in_luma_samples − 1, and j = 0..numApplicableIntensityIntervals[ c ][ x ][ y ] − 1, where numApplicableIntensityIntervals[ c ][ x ][ y ] is derived below.

Depending on the value of film_grain_model_id, the selection of one or more intensity intervals for the sample value Idecoded[ c ][ x ][ y ] is specified as follows:

–	The variable numApplicableIntensityIntervals[ c ][ x ][ y ] is initially set equal to 0.

–	If film_grain_model_id is equal to 0, the following applies:

- The top-left sample location ( xB, yB ) of the current 8x8 block b that contains the sample value Idecoded[ c ][ x ][ y ] is derived as ( xB, yB ) = ( x / 8, y / 8 ).

- The average value bavg of the current 8x8 block is derived as follows:

sum8x8 = 0
for( i = 0; i  < 8; i++ )
	for( j = 0; j < 8; j++ )
	     sum8x8  +=  Idecoded[ xB * 8 + I, yB * 8 + j, c ]	(D-7)
bavg = Clip3( 0, 255, ( sum8x8 + ( 1  <<  ( filmGrainBitDepth[ c ] − 3 ) ) )  >>  ( filmGrainBitDepth[ c ] − 2 ) )

- The value of intensityIntervalIdx[ c ][ x ][ y ][ j ] is derived as follows:

for( i = 0, j = 0; i  <=  num_intensity_intervals_minus1[ c ]; i++ )
	if( bavg  >=  intensity_interval_lower_bound[ c ][ i ]  
			&&  bavg  <=  intensity_interval_upper_bound[ c ][ i ] ) {
		intensityIntervalIdx[ c ][ x ][ y ][ j ] = i		(D-8)
		j++
	}
numApplicableIntensityIntervals[ c ][ x ][ y ] = j

–	Otherwise (film_grain_model_id is equal to 1), the value of intensityIntervalIdx[ c ][ x ][ y ][ j ] is derived as follows:

I8[ c ][ x ][ y ] = ( filmGrainBitDepth[ c ]  = =  8 ) ? ( Idecoded[ c ][ x ][ y ] :
		Clip3( 0, 255, ( Idecoded[ c ][ x ][ y ] + ( 1  <<  ( filmGrainBitDepth[ c ] − 9 ) ) )  >>  ( filmGrainBitDepth[ c ] − 8 ) )
for( i = 0, j = 0; i  <=  num_intensity_intervals_minus1[ c ]; i++ )
	if( I8[ c ][ x ][ y ]  >=  intensity_interval_lower_bound[ c ][ i ]  &&  
			I8[ c ][ x ][ y ]  <=  intensity_interval_upper_bound[ c ][ i ] ) {	(D-9)
		intensityIntervalIdx[ c ][ x ][ y ][ j ] = i
		j++
	}
numApplicableIntensityIntervals[ c ][ x ][ y ] = j

Samples that do not fall into any of the defined intervals (i.e., those samples for which the value of numApplicableIntensityIntervals[ c ][ x ][ y ] is equal to 0) are not modified by the grain generation function. Samples that fall into more than one interval (i.e., those samples for which the value of numApplicableIntensityIntervals[ c ][ x ][ y ] is greater than 1) will originate multi-generation grain. Multi-generation grain results from adding the grain computed independently for each ot the applicable intensity intervals.

In the equations in the remainder of this clause, the variable sj in each instance of the list comp_model_value[ c ][ sj ] is the value of intensityIntervalIdx[ c ][ x ][ y ][ j ] derived for the sample value Idecoded[ c ][ x ][ y ].

comp_model_value[ c ][ i ][ j ] specifies the j-th model value present for the colour component c and the i-th intensity interval. The set of model values has different meaning depending on the value of film_grain_model_id.

The value of comp_model_value[ c ][ i ][ j ] is constrained as follows, and could be additionally constrained as specified elsewhere in this clause:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ j ] shall be in the range of 0 to 2filmGrainBitDepth[ c ] − 1, inclusive.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ j ] shall be in the range of −2( filmGrainBitDepth[ c ] − 1 ) to 2( filmGrainBitDepth[ c ] − 1 ) − 1, inclusive.

Depending on the value of film_grain_model_id, the synthesis of the film grain is modelled as follows:

–	If film_grain_model_id is equal to 0, a frequency filtering model enables simulating the original film grain for c = 0..2, x = 0..pic_width_in_luma_samples − 1 and y = 0..pic_height_in_luma_samples − 1 as specified by:

G[ c ][ x ][ y ] = ( comp_model_value[ c ][ sj ][ 0 ] * Q[ c ][ x ][ y ] + comp_model_value[ c ][ sj ][ 5 ] *
			G[ c − 1 ][ x ][ y ] )  >>  log2_scale_factor	(D-10)

where Q[ c ] is a two-dimensional random process generated by filtering 16x16 blocks gaussRv with random-value elements gaussRvij generated with a normalized Gaussian distribution (independent and identically distributed Gaussian random variable samples with zero mean and unity variance) and where the value of an element G[ c − 1 ][ x ][ y ] used in the right-hand side of the equation is inferred to be equal to 0 when c − 1 is less than 0.

NOTE 7 – A normalized Gaussian random variable can be generated from two independent, uniformly distributed random values over the interval from 0 to 1 (and not equal to 0), denoted as uRv0 and uRv1, using the Box-Muller transformation specified by:

gaussRvi,j = Sqrt( −2 * Ln( uRv0 ) ) * Cos( 2 * π * uRv1 )	(D-11)

where  is Archimedes' constant 3.141 592 653 589 793....

The band-pass filtering of blocks gaussRv can be performed in the discrete cosine transform (DCT) domain as follows:

for( y = 0; y < 16; y++ )
	for( x = 0; x < 16; x++ )
		if( ( x < comp_model_value[ c ][ sj ][ 3 ]  &&  y < comp_model_value[ c ][ sj ][ 4 ] )  | |	(D-12)
				x > comp_model_value[ c ][ sj ][ 1 ]  | |  y > comp_model_value[ c ][ sj ][ 2 ] )
			gaussRv[ x ][ y ] = 0
filteredRv = IDCT16x16( gaussRv )

where IDCT16x16( z ) refers to a unitary inverse discrete cosine transformation (IDCT) operating on a 16x16 matrix argument z as specified by:

IDCT16x16( z ) = r * z * rT		(D-13)

where the superscript T indicates a matrix transposition and r is the 16x16 matrix with elements rij specified by:

ri,j = (  i  = =  0  ? 1  : Sqrt( 2 )4* Cos i *  2 * j + 1   * π32 	(D-14)

where  is Archimedes' constant 3.141 592 653 589 793....

Q[ c ] is formed by the frequency-filtered blocks filteredRv.

NOTE 8 – Coded model values are based on blocks of size 16x16, but a decoder implementation could use other block sizes. For example, decoders implementing the IDCT on 8x8 blocks could down-convert by a factor of two the set of coded model values comp_model_value[ c ][ sj ][ i ] for i equal to 1..4.

NOTE 9 – To reduce the degree of visible blocks can result from mosaicking the frequency-filtered blocks filteredRv, decoders could apply a low-pass filter to the boundaries between frequency-filtered blocks.

–	Otherwise (film_grain_model_id is equal to 1), an auto-regression model enables simulating the original film grain for c = 0..2, x = 0..pic_width_in_luma_samples – 1, and y = 0..pic_height_in_luma_samples − 1 as specified by:

G[ c ][ x ][ y ] = ( comp_model_value[ c ][ sj ][ 0 ] * n[ c ][ x ][ y ] +
comp_model_value[ c ][ sj ][ 1 ] * ( G[ c ][ x − 1 ][ y ] + ( ( comp_model_value[ c ][ sj ][ 4 ] * G[ c ][ x ][ y − 1 ] )  >>
		log2_scale_factor ) ) +
comp_model_value[ c ][ sj ][ 3 ] * ( ( ( comp_model_value[ c ][ sj ][ 4 ] * G[ c ][ x − 1 ][ y − 1 ] )  >>
		log2_scale_factor ) + G[ c ][ x + 1 ][ y − 1 ] ) +
comp_model_value[ c ][ sj ][ 5 ] * ( G[ c ][ x − 2 ][ y ] +
		( ( comp_model_value[ c ][ sj ][ 4 ] * comp_model_value[ c ][ sj ][ 4 ] * G[ c ][ x ][ y − 2 ] )  >>
			( 2 * log2_scale_factor ) ) ) +
	comp_model_value[ c ][ sj ][ 2 ] * G[ c − 1 ][ x ][ y ] )  >>  log2_scale_factor	(D-15)

where n[ c ][ x ][ y ] is a random value with normalized Gaussian distribution (independent and identically distributed Gaussian random variable samples with zero mean and unity variance for each value of x, y and c) and where the value of an element G[ c ][ x ][ y ] used in the right-hand side of the equation is inferred to be equal to 0 when any of the following conditions are true:

–	x is less than 0,

–	y is less than 0,

–	c is less than 0.

...

## On SEI syntax extension in the buffering period SEI message

From JVET-T0048. Agreed at the 20th JVET meeting (i.e., the JVET-T meeting).

Change the syntax of the buffering period SEI message as follows:

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( more_data_in_payload( ) ) |  |
| if( payload_extension_present( ) ) |  |
| use_alt_cpb_params_flag | u(1) |
| } |  |

Change the semantics of the buffering period SEI message as follows:

...

use_alt_cpb_params_flag may be used to derive the value of UseAltCpbParamsFlag. When irap_cpb_params_present_flag is equal to 0, use_alt_cpb_params_flag shall not be equal to 1. When use_alt_cpb_params_flag is not present, it is inferred to be equal to 0.

NOTE 5 – The syntax element use_alt_cpb_params_flag may be present in the payload extension of the buffering period SEI message. Decoders conforming to profiles specified in Annex A may ingore ignore this syntax element.

It is a requirement of bitstream conformance that when use_alt_cpb_params_flag is present in the buffering perid SEI message, the return value of the more_data_in_payload( ) function in the sei_payload( ) syntax structure containing the buffering period SEI message shall be equal to 1.

## Miscellaneous items for the published ITU-T H.265 version 7 (potentially also applicable to the ISO/IEC 23008-2 text)

- In clause 5.10, replace "a mixture of lower case and upper case letter" with " a mixture of lower case and upper case letters" (missing 's' in the last word).

- In clause 6.3.3, replace "The following divisions of processing elements of this Specification form spatial or component-wise partitioning:" with "The following divisions of processing elements of this Specification form spatial or component-wise partitionings:" (missing 's' in the last word)? Note that in the ISO version there is the 's'. Which one is (more) correct?

- In clause 7.2, definition of next_bits( n ), replace "Provides a look at the next n bits in the bitstream with n being its argument." with "It provides a look at the next n bits in the bitstream with n being its argument."

- In clause 7.3.3, in the syntax condition for general_max_14bit_constraint_flag, replace the "Enter" with "shift+Enter".

- In clause 7.3.3, in the syntax condition for sub_layer_max_14bit_constraint_flag[ i ], replace two instances of "Enter" with "shift+Enter".

- In clause 7.3.3, resolving ticket #1504, in profile_tier_level syntax in tabular form remove an extra ‘)’ character after a condition check of general_profile_compatibility_flag[ 7 ]. .

- In clause 7.4.2.4.3, consider resolving ticket #1505 by making the following change suggested in the ticket:

It is a requirement of bitstream conformance that, when present, the next access unit after an access unit that contains an end of sequence NAL unit or an end of bitstream NAL unit shall be an IRAP access unit, which may be an IDR access unit, a BLA access unit, or a CRA access unit.

- In clause 7.4.4, remove the instance of "and " highlighted below:

When profilePresentFlag is equal to 1, general_profile_idc is not equal to 9 or 11 and is not in the range of 1 to 5, inclusive, and general_profile_compatibility_flag[ 9 ] is not equal to 1, general_profile_compatibility_flag[ 11 ] is not equal to 1, and general_profile_compatibility_flag[ j ] is not equal to 1 for any value of j in the range of 1 to 5, inclusive, the value of general_inbld_flag is inferred to be equal to 0.

- In clause 8.4.4.27, resolving ticket #1500, replace the following equations:

xL = palette_transpose_flag ? x * nSubHeight : x * nSubWidth	(8-69)

yL = palette_transpose_flag ? y * nSubWidth : y * nSubHeight	(8-70)

with the following:

xL = palette_transpose_flag ? y * nSubHeight : x * nSubWidth	(8-69)

yL = palette_transpose_flag ? x * nSubWidth : y * nSubHeight	(8-70)

- In clause 8.5.3.2.7, resolving ticket #1427, remove the following equations as shown:

refIdxB = RefIdxLX[ xNbBk ][ yNbBk ]	(8-185)

refIdxB = RefIdxLY[ xNbBk ][ yNbBk ]	(8-187)

- In clause 8.6.8.1, replace the following (ticket #1437):

- an array resSampleArrayCb specifying the chroma residual samples of the current block,

- an array resSampleArrayCr specifying the chroma residual samples of the current block.

with the following:

- an array resSampleArrayCb specifying the Cb residual samples of the current block,

- an array resSampleArrayCr specifying the Cr residual samples of the current block.

- In clause 9.3.3.1, resolving ticket #1498, change the following (note that it was said in the JCTVC-AL meeting minutes that this was confirmed present in H.265 (11/19), but it is not in the published ITU-T H.265/HEVC version 7):

cMax = ( PrefixOffset  <<  1 ) > PaletteMaxRunMinus1 ?
( PalletMaxRun − PrefixOffset ) : ( PrefixOffset − 1 )

to

cMax = ( PrefixOffset  <<  1 ) > PaletteMaxRunMinus1 ?
( PalleteMaxRunMinus1 − PrefixOffset ) : ( PrefixOffset − 1 )

- In clause 9.3.4.1, resolving ticket #1491, make the following changes:

The parsing of each bin is specified by the following two ordered steps:

1.	The derivation process for ctxTable, ctxIdx, and bypassFlag as specified in clause 9.3.4.2 is invoked with binIdx as input and ctxTable, ctxIdx and bypassFlag as outputs.

NOTE – As a consequence of invoking the process in clause 9.3.4.2, tThe arithmetic decoding process as specified in clause 9.3.4.3 is also invoked with ctxTable, ctxIdx and bypassFlag as inputs and the value of the bin as output.

- In Table 9-48, resolving ticket #1507, remove the last four rows of the table (i.e., for syntax elements cu_qp_delta_abs, cu_qp_delta_sign_flag, cu_chroma_qp_offset_flag, andcu_chroma_qp_offset_idx. These rows are duplicates of rows appearing earlier in the same table.

- In clause A.4.1, Table A.8, add "MaxSliceSegments" after "Max slice segments per picture".

- In clause A.4.2, Table A.10, some of the numbers, e.g., "xxxx" are not in the form of "x xxx". Make the style of all the numbers consistent.

- In clause D.3.1, replace the following:

The list SingleLayerSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 134 to 152, inclusive, 154 to 159, and 200 to 202, inclusive.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 159, and 200 to 202, inclusive.

with the following:

The list SingleLayerSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, and 200 to 202, inclusive.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 159, inclusive, and 200 to 202, inclusive.

- In clause D.3.32, NOTE 2, some of the numbers, e.g., "1000" are not in the form of "1 000". Make the style of all the numbers consistent.

- In clause D.3.39, check whether the values "0.0001" and "0.00002" should be in the forms of "0.000 1" and "0.000 02", respectively.

- In the ITU-T version of HEVC and the w18881 MPEG document for FDAM1, in the semantics of ar_object_label_update_flag, “will not” is repeated.

- In D.3.41.6.2 (Projection for one sample location), it says that there are inputs to the process that are “pictureWidth and pictureHeight, which are the width and height, respectively, of a monoscopic projected luma picture, in relative projected picture sample units” and “the centre point of a sample location (hPos, vPos) along the horizontal and vertical axes, respectively, in relative projected picture sample units, where hPos and vPos may have non-integer real values”. It does not seem clear what this means.

- The pictureWidth and pictureHeight variables are also modified internally in D.3.41.6.2, which seems potentially unwise.

- D.3.41.6.4 (Conversion of sample locations for rectangular region-wise packing) has inputs that are “offset values for the sampling position (offsetX, offsetY)”. D.3.41.6.5 says “offsetX is set equal to 0.5 and offsetY is set equal to 0.5.” Are those the only values that are ever used? If so, why define a variable that does not vary in value?

- The variable names offsetX and offset that are used in D.3.41.6.4 are also used in 8.5.3.2, but for a different purpose. It may be unwise to use the same variable names in Annex D.

- The first sentence of D.3.41.6.5 (Mapping of luma sample locations within a cropped decoded picture to sphere coordinates relative to the global coordinate axes) says “This clause specifies the semantics of luma sample locations within a cropped decoded picture to sphere coordinates relative to the global coordinate axes.” Here, “semantics” should probably be “mapping”.

- In clause D.3.41.7.2, Equation D-57, change the last row from "θ = Asin( z ÷ Sqrt( x2 + y2 + z2 ) ) ÷ π" to "θ = Asin( z ÷ Sqrt( x2 + y2 + z2 ) ) * 180 ÷ π".

- The indent distance for the titles of clauses D.3.41.7.3 to D.3.41.7.6 should be aligned to be with those for clauses D.3.41.7.1, D.3.41.7.2, and D.3.41.7.7.

- D.3.41.7.7 is only for the fisheye video information SEI message, which is handled somewhat separately and differently than other projection mappings; is this appropriate and clear enough?

- In clause E.3.1, check whether the values "0.3127" etc. should be in the forms of "0.312 7" etc.

- In clause E.3.1, replace "matrix_coeffs shall not be equal to 0 unless one or more of the following conditions are true:" with "matrix_coeffs shall not be equal to 0 unless both of the following conditions are true:".

- In clause E.3.1, Equations E60-E67, some numbers are in the forms of "0.986566" and "4096" etc. instead of in the forms of "0.986 566" and "4 096" etc.

- In clause E.3.1, Table E.5, some numbers are in the forms of "0.0593" etc. instead of in the forms of "0.059 3"etc.

- In clause E.3.1, NOTE 16, change "1920x540" to "1 920 x 540".

- In clause E.3.2, replace "The need for presence of buffering periods for NAL HRD operation to be present in the bitstream in buffering period SEI messages is determined by the application" with "The need for the presence of buffering period parameters for NAL HRD operation in the bitstream in buffering period SEI messages is determined by the application".

- In clause E.3.2, replace "The need for presence of buffering periods for VCL HRD operation to be present in the bitstream in buffering period SEI messages is determined by the application" with "The need for the presence of buffering period parameters for VCL HRD operation in the bitstream in buffering period SEI messages is determined by the application".

- In clause E.3.2, replace "The need for presence of CPB and DPB output delays to be present in the bitstream in picture timing SEI messages is determined by the application" with "The need for the presence of CPB and DPB output delays in the bitstream in picture timing SEI messages is determined by the application".

- In clause F.7.4.3.1.1, replace "1023" with "1 023".

- In clauses F.13.2.3 and F.13.4, replace "90000" with "90 000".

- In clause G.14.3.1, replace the following

The list VclAssociatedSeiList is set to consist of payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 161, 165, 167, 168, 177, 178, 179, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 168, inclusive, 176 to 180, inclusive, and 200 to 202, inclusive.

with the following:

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 161, 165, 167, 168, 177, 178, 179, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 168, inclusive, 176 to 180, inclusive, and 200 to 202, inclusive.

- In clause I.7.4.3.1.7, replace "65535" with "65 535".

- In clause I.7.4.3.2.3, replace "is not used the decoding process" with "is not used in the decoding process", and replace "is not present coding units" with "is not present in coding units".

- In clause I.14.3.1, replace the following:

The list VclAssociatedSeiList is set to consist of payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 161, 165, 167, 168, 177, 178, 179, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 168, inclusive, 176 to 181, inclusive, and 200 to 202, inclusive.

with the following:

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 161, 165, 167, 168, 177, 178, 179, and 200 to 202, inclusive.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 168, inclusive, 176 to 181, inclusive, and 200 to 202, inclusive.

## More miscellaneous items from JCTVC-AN0024

For both the ITU-T and the ISO texts:

- The semantics of the deblocking disable flag in the PPS should be updated to better reflect the behaviour.

- Corresponding to the following changes to VVC, an errata report for AVC and HEVC, to change the MinCr limit to be derived from the CPB size limit or add a note for cases where the CPB size imposes a tighter limit than the MinCr does:

MaxCPB = 80 000 for level 6, 120 000 for level 6.1, 180 000 for level 6.2, and change MinCrScaleFactor for the 4:4:4 profile to 0.75, and change MinCrBase to 8 for level 6.2.

- The following two bullet items on non-scalable-nested BP/PT/DUI SEI messages should be changed as shown below:

–	For a non-scalable-nested SEI message, when payloadType is equal to 0 (buffering period) or 130 (decoding unit information), the non-scalable-nested SEI message applies to the operation point that has OpTid equal to the greatest value of nuh_temporal_id_plus1 among all VCL NAL units in the bitstream, and that has OpLayerIdList containing all values of nuh_layer_id in all VCL units in the bitstream, and has only the base layer as the output layer.

–	For a non-scalable-nested SEI message, when payloadType is equal to 1 (picture timing), the frame field information carried in the syntax elements pic_struct, source_scan_type and duplicate_flag, when present, in the non-scalable-nested picture timing SEI message applies to the base layer only, while the picture timing information carried in other syntax elements, when present, in the non-scalable-nested picture timing SEI message applies to the operation point that has OpTid equal to the greatest value of nuh_temporal_id_plus1 among all VCL NAL units in the bitstream, and that has OpLayerIdList containing all values of nuh_layer_id in all VCL units in the bitstream, and has only the base layer as the output layer.

- In the semantics of the decoded picture hash SEI message, the specification that the colour component arrays use two’s complement representation was an error and needs to be corrected. Unsigned integers are used for the colour component arrays.

- Add derivation of TemporalId and reference pic_parameter_set_id to the decoding process for generating unavailable reference pictures (in order to enable checking of some constraints for them).

- In clause C.4 (Bitstream conformance), change the constraint on i.e., the number of all pictures n in the DPB for referencing or output after invocation of the process for removal of pictures from the DPB, change "CpbRemovalTime[ n ] less than CpbRemovalTime[ n ]" to "DpbOutputTime[ n ] greater than CpbRemovalTime[ currPic ]". This is asserted to be a bug, because all decoded pictures in the DPB are always decoded earlier than decoding of the current picture, and thus CpbRemovalTime[ n ] in the context is always less than CpbRemovalTime[ currPic ].

- Relax the semantics so that an extension_flag in VPS, SPS, or PPS equal to 1 specifies that specifies extension_data_flag syntax elements may be present. (Currently, it says these flags are present.)

- Fix an asserted bug in Equation C.10 as follows, coming from JVET-S0101:

if( !concatenationFlag ) {
	baseTime = AuNominalRemovalTime[ firstPicInPrevBuffPeriod ]
	tmpCpbRemovalDelay = AuCpbRemovalDelayVal
	tmpCpbDelayOffset = CpbDelayOffset
} else {
	baseTime1 = AuNominalRemovalTime[ prevNonDiscardablePic ]
	tmpCpbRemovalDelay1 = ( auCpbRemovalDelayDeltaMinus1 + 1 )
	baseTime2 = AuNominalRemovalTime[ n − 1 ]
	tmpCpbRemovalDelay2 =				(C.X)
			Ceil( ( InitCpbRemovalDelay[ Htid ][ ScIdx ] ÷ 90000 +
				AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] ) ÷ ClockTick )
	if( baseTime1 + ClockTick * tmpCpbRemovalDelay1 <
				baseTime2 + ClockTick * tmpCpbRemovalDelay2 ) {
		baseTime = baseTime2
		tmpCpbRemovalDelay = tmpCpbRemovalDelay2
	} else {
		baseTime = baseTime1
		tmpCpbRemovalDelay = tmpCpbRemovalDelay1
	}
	tmpCpbDelayOffset = 0
}
AuNominalRemovalTime[ n ] = baseTime + ( ClockTick * tmpCpbRemovalDelay − tmpCpbDelayOffset

- In the semantics of elemental_duration_in_tc_minus1[ i ], the syntax element fixed_pic_rate_general_flag[ i ] of both the first and second CVS to determine whether the fixed output rate applies also across CVSs. However, it is asserted that the value of fixed_pic_rate_general_flag[ i ] of the first CVS should not be taken into account for determining whether the fixed output rate applies across the two CVSs. Therefore, the first two instances of fixed_pic_rate_general_flag highlighted in green below should be fixed_pic_rate_within_cvs_flag instead:

elemental_duration_in_tc_minus1[ i ] plus 1 (when present) specifies, when Htid is equal to i, the temporal distance, in clock ticks, between the elemental units that specify the HRD output times of consecutive pictures in output order as specified below. The value of elemental_duration_in_tc_minus1[ i ] shall be in the range of 0 to 2047, inclusive.

When Htid is equal to i and fixed_pic_rate_general_flag[ i ] is equal to 1 for a CVS containing picture n, and picture n is a picture that is output and is not the last picture in the bitstream (in output order) that is output, the value of the variable DpbOutputElementalInterval[ n ] is specified by:

- DpbOutputElementalInterval[ n ] = DpbOutputInterval[ n ]  elementalOutputPeriods	(113)

where DpbOutputInterval[ n ] is specified in Equation C.16 and elementalOutputPeriods is specified as follows:

–	If a PT SEI message is present for picture n, elementalOutputPeriods is equal to the value of pt_display_elemental_periods_minus1 + 1.

–	Otherwise, elementalOutputPeriods is equal to 1.

When Htid is equal to i and fixed_pic_rate_general_flag[ i ] is equal to 1 for a CVS containing picture n, and picture n is a picture that is output and is not the last picture in the bitstream (in output order) that is output, the value computed for DpbOutputElementalInterval[ n ] shall be equal to ClockTick * ( elemental_duration_in_tc_minus1[ i ] + 1 ), wherein ClockTick is as specified in Equation C.X (using the value of ClockTick for the CVS containing picture n) when one of the following conditions is true for the following picture in output order nextPicInOutputOrder that is specified for use in Equation C.X:

–	picture nextPicInOutputOrder is in the same CVS as picture n.

–	picture nextPicInOutputOrder is in a different CVS and fixed_pic_rate_general_flag[ i ] is equal to 1 in the CVS containing picture nextPicInOutputOrder, the value of ClockTick is the same for both CVSs, and the value of elemental_duration_in_tc_minus1[ i ] is the same for both CVSs.

When Htid is equal to i and fixed_pic_rate_within_cvs_flag[ i ] is equal to 1 for a CVS containing picture n, and picture n is a picture that is output and is not the last picture in the CVS (in output order) that is output, the value computed for DpbOutputElementalInterval[ n ] shall be equal to ClockTick * ( elemental_duration_in_tc_minus1[ i ] + 1 ), wherein ClockTick is as specified in Equation C.X (using the value of ClockTick for the CVS containing picture n) when the following picture in output order nextPicInOutputOrder that is specified for use in Equation C.X is in the same CVS as picture n.

- In clause 8.1.1, add the derivation of the variables DuHrdPreferredFlag and DecodingUnitHrdFlag, similarly as in HEVC.

- Add "The variable DuHrdPreferredFlag is either specified by external means, or when not specified by external means, set equal to 0." to clause 8.1.2.

- Fix the description of the recovery point picture to consider the case with a recovery POC distance of 0.

## More miscellaneous items from JVET-T0054

Agreed at the 20th JVET meeting (i.e., the JVET-T meeting).

- Change the range notation from the following:

x = y..z	x takes on integer values starting from y to z, inclusive, with x, y, and z being integer numbers and z being greater than y.

to be as follows (same as in VVCv1):

x = y..z	x takes on integer values starting from y to z, inclusive, with x, y, and z being integer numbers and z being greater than or equal to y.

- Search and replace "satisify" with "satisfy".

- In clause 9.3.3.6, fix the wrong indention for the paragraph starting with "Otherwise".

- Search and check "CVS", particularly in semantics, and fix to be CLVS as needed.

- Search and replace instances of "clasue" with "clause".

- The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are initially derived in clause C.2.1 and updated in clause C.2.3, and the second variable is not even used in clause C.2.3. However, currently, the wording in clause C.2.3 is that these variables are derived, which can confuse readers as to why the second variable is derived but not used in clause C.2.3 thus why not just remove the mentioning of the second variable from clause C.2.3.

Similar issue was identified during the development of VVCv1, and the final text in there was to use the phrase "updated" instead of "derived" for these two variables in clause C.2.3.

It is therefore proposed to change the following phrase in clause C.2.3:

The variables InitCpbRemovalDelay[ SchedSelIdx ], InitCpbRemovalDelayOffset[ SchedSelIdx ], CpbDelayOffset and DpbDelayOffset are derived as follows:

to be as follows:

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are updated, and the variables CpbDelayOffset and DpbDelayOffset are derived, as follows:

- Replace all the eight instances of "decoder conformance to profiles" with "the decoding process". These instances are as follows:

vps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annex A. Decoders conforming to a profile specified in Annex A but not supporting the INBLD capability specified in Annex F shall ignore all vps_extension_data_flag syntax elements.

sps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all sps_extension_data_flag syntax elements.

pps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all pps_extension_data_flag syntax elements.

slice_segment_header_extension_data_byte may have any value. Decoders shall ignore the value of slice_segment_header_extension_data_byte. Its value does not affect decoder conformance to profiles specified in Annex A.

vps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annexes A, G or H. Decoders conforming to a profile specified in Annexes A, G or H shall ignore all vps_extension_data_flag syntax elements.

vps_non_vui_extension_data_byte may have any value. Decoders shall ignore the value of vps_non_vui_extension_data_byte. Its value does not affect decoder conformance to profiles specified in this version of this Specification.

slice_segment_header_extension_data_bit may have any value. Decoders shall ignore the value of slice_segment_header_extension_data_bit. Its value does not affect decoder conformance to profiles specified in this version of this Specification.

vps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annexes A, G, H, or I. Decoders conforming to a profile specified in Annexes A, G, H, or I shall ignore all vps_extension_data_flag syntax elements.

- In the informative Table D.1, change the persistence scope for the SEI manifest SEI message and the SEI prefix indication SEI message from "The CLVS containing the SEI message" to "The CVS containing the SEI message", to be aligned with the semantics of the SEI messages. Note that this is not needed for AVC as AVC does not have such an informative table.

- Remove a tab from each of the two middle rows below:

| content_light_level_info( payloadSize ) { | Descriptor |
| --- | --- |
| max_content_light_level | u(16) |
| max_pic_average_light_level | u(16) |
| } |  |

# Reported errata items for AVC

JVET-T1006-v1 includes some errata items for AVC (in addition to some AVC amendments). See also Section 1.

## Publication status background

Rec. ITU-T H.264

- (06/19, Edition 13) Approved 2019-06-13, published 2019-09-06

ISO/IEC 14496-10

- ISO/IEC 14496-10:2014 (Edition 8), published 2014-09

- ISO/IEC 14496-10:2014/Amd 1:2015 (Multi-resolution frame compatible stereoscopic video with depth maps, additional supplemental enhancement information and video usability information), published 2015-11

- ISO/IEC 14496-10:2014/FDAMD 2 (Additional Levels and Supplemental Enhancement Information); stage 50.98, deleted in preparation for Edition 9

- ISO/IEC 14496-10:2014/Amd 3:2016 (Additional supplemental enhancement information); published 2016-12, published 2016-12

- ISO/IEC DIS 14496-10:201x (Edition 9); stage 40.99 Full report circulated: DIS approved for registration as FDIS as of 2018-01-31

-

-

--

-

--

-

ri,j = (  i  = =  0  ? 1  : Sqrt( 2 )4* Cos i *  2 * j + 1   * π32

# Reported errata items for Video CICP

See also Section 1.

## Publication status background

Rec. ITU-T H.273

- (02/16, Edition 1) Approved 2016-12-22, published 2017-04-27

ISO/IEC 23091-2 (previously part of ISO/IEC 23001-8)

- ISO/IEC 23091-2:2019, published 2019-07

## Errata items

- There is an error in the range of values for the sample aspect ratio width and height.

- The CICP revision should account for the various errata issues previously noted in AL1004. It was confirmed that the BT.2100 reference needs updating (to -2).

- The equations for ICtCp with HLG have not been updated and need be corrected.

# Reported errata items for Codepoint Usage TR

## Publication status background

Sup. ITU-T H.Sup19

- (03/19, Edition 1) Approved 2019-03, published 2019-04-30

- (10/19, Edition 2) Approved 2019-10, published 2019-11-14

ISO/IEC 23091-4

- ISO/IEC 23091-4:2019, published 2019-08

- ISO/IEC 23091-4:202x, stage 60.00 Under publication since 2020-04-05

## Miscellaneous items

- In a liaison letter sent to ITU-T SG16, SMPTE identified an anomaly in Table 7 of the 1st edition, as follows:

| Mastering Display Minimum Luminance | Registration identifier | was 060e2b34.0101010e.04200401.01030000 should be urn:smpte:ul:060e2b34.0101010e.04200401.01040000 see MasteringDisplayMaximumLuminance at https://registry.smpte-ra.org/view/published/elements_by_group_view.html |
| --- | --- | --- |

- SMPTE also recommended using urn notation for writing labels within a text “urn:smpte:ul:”, as they said this significantly increases the ability to machine parse and process documents. The correction to the supplied document was provided in the urn notation above.

- The registers can be checked online at this URL: https://registry.smpte-ra.org/apps/pages/published/. This could be mentioned in the text.

- For 3.14 of the ISO/IEC pre-publication text, an editor suggested that the definition of “narrow range” does not describe narrow range sufficiently, and suggested defining it as “range in a fixed-point (integer) representation where active video does not span the full range of values that could be expressed with that bit depth but does use the remaining range for under/overshoot (processing artifacts) and sync”

- In clause 4 of the ISO/IEC pre-publication text, an editor suggested that UHD now informally includes 1080P with HDR and WCG, and proposed the following “UHD: informally used with several resolutions (3840 × 2160 for television or 4096 × 2160 for film with or without HDR/WCG); (7680 × 4320 or 8192 × 4320 with or without HDR/WCG); (1080P WITH HDR/WCG)”.

- In the last paragraph of clause 6 of the ISO/IEC pre-publication text, an editor suggested to add MaxFall/MaxCLL to make a sentence more complete, as in “The approved colour volume, which may be smaller than the container volume, is often indicated with SMPTE ST 2086 including MaxFall/MaxCLL metadata. Over time, it is expected that WCG and/or high dynamic range (HDR) applications will evolve to use more of the available container colour volume.”

- For 7.1 of the ISO/IEC pre-publication text, an editor suggested to add facts: that MP4 containers (and QuickTime) use color atoms utilizing MPEG CICP values to indicate Color Gamut/Transfer Function/Matrix Coefficients, as in “MP4 (and QuickTime) wrappers utilize MPEG CICP integer values shown in Table 4 to indicate Color Gamut/Transfer Function/Matrix Coefficients. These values are stored in color “atoms”.”

- For an Annex C of the ISO/IEC pre-publication text, an editor suggested to consider adding a reference to RP 214 for carriage of dynamic HDR metadata, saying that RP 214 is being considered for storage of dynamic HDR metadata like ST.2094 and that dynamic metadata can carry more information about content within a stream which assists in remapping HDR video images to a specific displays capabilities. RP 214 describes a means for packing SMPTE KLV encoded metadata and/or essence into SMPTE 291M ancillary data packets. The editor said there is strong interest in carrying audio/visual data, metadata, and/or essence, within the same digital stream and said that the use of one data stream facilitates delivery of the overall multimedia presentation. Metadata is classified as information about the essence.

- Check for P3D65x1000n0005 vs P3D65x1000n005

- Check for P3D65x4000n0005 vs P3D65x4000n005

- Check for “specs"” (ending in a quote mark)

- Review the phrase “and the combination is specified in ITU-R and/or SMPTE specifications”

- 7.2.3 note below table 5 add “(Rec. ITU-R BT.709)” to the sentence: "ATSC specifications list use of the transfer characteristics value of 1 for SDR NCG  (Rec. ITU-R BT.709) video"

- A small punctuation suggestion for 7.2.4 (adding a comma):

“It is important for tools to process video according to the colour volume it is operating in to make sure the conversion is consistent.”

Change to:

“It is important for tools to process video according to the colour volume it is operating in, to ensure the conversion is consistent.”

- A slightly awkward sentence in the same note area in 7.2.4:

"ARIB STD B32 lists use of the transfer characteristics value 1 for HD and 14 for UHD for SDR WCG video.”

Change to:

"ARIB STD B32 lists use of the transfer characteristics value 1 for HD (Rec. ITU-R BT.709) and 14 for UHD SDR WCG (Rec. ITU-R BT.2020) video.”

- To the last sentence in 7.2.4, add as in

“The indicated chroma sample location alignment is only applicable for 4:2:0 chroma sampling. ChromaLocType (the generic label used in this document for the HEVC and AVC bitstream syntax elements: chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field), listed in Tables 1 and 3 of this document, indicates the 4:2:0 chroma sample position alignment.”

Change to:

“The indicated chroma sample location alignment is only applicable for 4:2:0 chroma sampling. ChromaLocType (the generic label used in this document for the HEVC and AVC bitstream syntax elements: chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field), listed in Tables 1 and 3 of this document, indicates the 4:2:0 chroma sample position alignment.  The Rec. ITU-R program signal exchange series (BT.601, BT.709, BT.2020, BT.2100) consider 4:2:2 and 4:4:4 chroma samples to be co-sited with luma.”

- Also insert parenthetic details in Annex A

“The transfer characteristics indicator values of 1, 6, 14, and 15 are functionally the same. Blu-ray BD-ROM 3.1 (“4K”) and the DVB UHD specifications list use of the transfer characteristics value of 14 for SDR/WCG (Rec. ITU-R BT.2020) video. ATSC specifications list use of the transfer characteristics value of 1 for SDR video. ARIB STD B32 lists use of the transfer characteristics value 1 for HD and 14 for UHD for SDR WCG video.”

Change to:

“The transfer characteristics indicator values of 1, 6, 14, and 15 are functionally the same. Blu-ray BD-ROM 3.1 (“4K”) and the DVB UHD specifications list use of the transfer characteristics value of 14 for SDR/WCG (Rec. ITU-R BT.2020) video. ATSC specifications list use of the transfer characteristics value of 1 for SDR NCG (Rec. ITU-R BT.709) and SDR WCG (Rec. ITU-R BT.2020) video. ARIB STD B32 lists use of the transfer characteristics value 1 for HD (Rec. ITU-R BT.709) and 14 for UHD SDR WCG (Rec. ITU-R BT.2020) video.”

- General: Review parentheticals

- General: Review/rephrase instances of “and that”.

## Miscellaneous items from JVET-T0113

- Ballot comments: Removal of version numbers: Agreed, and similarly remove version numbers from BT.656-5, BT.1120-9, BT.2077-2 (Tables 8 & 9).

- Change “Interstitial” to “Vertically interstitial”.

- Typo: “ST.2036-4”

- Copy-paste typo: Bottom line of Table 3 “ChromaLocType = 0” should be “ChromaLocType = 2”

- Left alignment on bibliographic entry for SMPTE RA.