| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U2012 |
| --- | --- |

| Title: | JVET common test conditions and evaluation procedures for 360° video |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Y. He J. Boyce K. Choi J.-L. Lin | Email: | yuwen.he@bytedance.com jill.boyce@intel.com |
| Source: | JVET |  |  |

_____________________________

# Abstract

This document defines common test conditions (CTC), conversion practices, and software reference configurations to be used in the context of 360° video ad-hoc group (AHG) and core experiments (CE) conducted after the 21st JVET meeting. These common test conditions are also recommended for use in technical contributions to the 22nd and following JVET meetings, if applicable.

# Introduction

Common test conditions are desirable to conduct experiments in a well-defined environment and ease the comparison of the outcome of experiments.

This document defines 4 test conditions, reflecting intra-only, random-access, and low-delay settings:

- Intra, 10 bit

- Random access, 10 bit

- Low delay, 10 bit

- Low delay, P slices only, 10 bit

A subset of these test conditions might be used for a particular experiment. For example, when testing an intra coding tool, only intra configurations might be used.

Version 12.0 of the VTM software is expected to be used for most experiments. Version 12.0 of the 360Lib software should be used for conversions and quality measurement. More recent versions are encouraged where applicable.

The following sections define testing procedure, test sequences, coding projection formats, quantization parameter values, evaluation procedure, configuration files, and compile-time options to be used.

# Testing procedure

Figure 1 depicts the testing procedure. For a given input 360° video, conversion is applied first to convert source projection format into coding projection format. The coding projection format is in lower resolution than that of the source projection format to reduce the bias towards the native projection format when comparing different projection formats and their impact on 360° video compression. For example, 8K source video is coded in 4K resolution. After coding, the reconstructed signal in coding projection format is converted back to the source projection format at the source resolution.

Figure 1. 360° video testing procedure

# Test sequences

Table 1 defines the set of test sequences to be used for intra, random-access, and low-delay conditions. All frames (as defined by frame count in the table) shall be encoded for all sequences and test cases described below.

Test sequences are available on ftp://jvet@ftp.ient.rwth-aachen.de/testsequences/testset360 and ftp://jvet@ftp.hhi.fraunhofer.de/testsequences/testset360

Test sequence are only available to qualified JVET participants. Qualified participants may contact the JCT-VC/JVET chairs for login information.

Table 1. Test sequences

| Class | Sequence name | Width | Height | Frame count | Frame rate | Bit depth | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | SkateboardInLot | 8192 | 4096 | 300 | 30 fps | 10 | O | M | O |
| S1 | ChairLift | 8192 | 4096 | 300 | 30 fps | 10 | O | M | O |
| S1 | KiteFlite | 8192 | 4096 | 300 | 30 fps | 8 | O | M | O |
| S1 | Harbor | 8192 | 4096 | 300 | 30 fps | 8 | O | M | O |
| S1 | Trolley | 8192 | 4096 | 300 | 30 fps | 8 | O | M | O |
| S1 | GasLamp | 8192 | 4096 | 300 | 30 fps | 8 | O | M | O |
| S2 | Balboa | 6144 | 3072 | 600 | 60 fps | 8 | O | M | O |
| S2 | Broadway | 6144 | 3072 | 600 | 60 fps | 8 | O | M | O |
| S2 | Landing2 | 6144 | 3072 | 300 | 30 fps | 8 | O | M | O |
| S2 | BranCastle2 | 6144 | 3072 | 300 | 30 fps | 8 | O | M | O |

Notes:

- When the encoder operates in 8-bit mode (InternalBitDepth=8) for a 10-bit source, each 10-bit source sample x is converted prior to encoding to an 8-bit value (x+2) / 4 clipped to the [0,255] range. Similarly, when the encoder operates in 10-bit mode (InternalBitDepth=10) for an 8-bit source, each 8-bit source sample x is converted prior to encoding to a 10-bit value 4*x. This behavior is built into the reference encoder and no external conversion program is required.

- M and O are for mandatory and optional, respectively.

Table 2. MD5 checksum of test sequences

| Sequence name | MD5 checksum |
| --- | --- |
| SkateboardInLot | e8eae04c43e959060f641fec4892fced |
| ChairLift | 9126f753bb216a73ec7573ecc4a280c3 |
| KiteFlite | 18c0ea199b143a2952cf5433e8199248 |
| Harbor | aa827fdd01a58d26904d1dbdbd91a105 |
| Trolley | 25c1082d1e572421da2b16530718156d |
| GasLamp | 3417d0b862ffb0fd34f65c3bc810d25c |
| Balboa | 1457bb109ae0d47265f5a589cb3464d7 |
| Broadway | 1baf6667ef84bc3534809463faee8e82 |
| Landing2 | d1b064b272d82d4de6f853a760c2f75e |
| BranCastle2 | 9b411717977f6c50aadb458f52b96951 |

# Coding projection format

Prior to encoding, the high resolution equi-rectangular projection (ERP) [1] input shall be converted to low resolution coding projection format at resolutions specified in Table 3. For new projection format proposals, the number of coded samples should be within ±3% of the number of coded samples in ERP coded format. For all conversions between projection formats prior to the encoding and after decoding, the conversion shall use 6-taps Lanczos filter for luma and 4-taps Lanczos filter for chroma with 10 bits internal bit-depth.

Table 3. Frame sizes for different projections for 8K and 6K input resolution

| Format | 8K and 6K Input ERP Resolution |  |  |  |
| --- | --- | --- | --- | --- |
|  | faceW | faceH | W | H |
| ERP (coded) | 4432 | 2216 | 4432 | 2216 |
| Padded ERP | 4432 | 2216 | 4448 | 2216 |
| CMP [1] | 1280 | 1280 | 3840 | 2560 |
| ACP [1] | 1280 | 1280 | 3840 | 2560 |
| EAC [1] | 1280 | 1280 | 3840 | 2560 |
| GCMP [1] | 1280 | 1280 | 3856 | 2592 |
| RSP [1] | 1280 | 1280 | 3840 | 2560 |

# Quantization parameter values

The same quantization parameter values as in the SDR CTC [2] are to be used for each video sequence.

# Anchor

The anchor is provided for the test model (VTM) encoded in the padded generalized cubemap (GCMP) format with 8 samples padding around face row and blending. Config files are provided, as described in section 8.

# Evaluation

## Reported metrics

The attached Excel sheet contains a reporting template in which bitrate, end-to-end WS-PSNR, end-to-end S-PSNR-NN, codec WS-PSNR, codec S-PSNR-NN, cross-format S-PSNR-NN, cross-format CPP-PSNR, viewport PSNR, coding time, and BD-rate results are reported for the tested configuration against the VTM anchor in the PHEC format.

For the purpose of reporting encoding and decoding running times, the anchor and test should be simulated on the same platform, e.g. similar CPU cores, to have reliable time comparison.

In the attached Excel sheet, BD-rate is calculated using piece-wise cubic interpolation.

Results reported shall include the following metrics:

- End-to-end WS-PSNR for Y, Cb and Cr

- End-to-end S-PSNR-NN for Y, Cb and Cr

- Viewport PSNR for Y, Cb and Cr

For coding tool proposals, results reported shall include the following metrics:

- Codec WS-PSNR for Y, Cb and Cr

- Codec S-PSNR-NN for Y, Cb and Cr

Additionally, results reported may include the following metrics:

- Cross-format S-PSNR-NN for Y, Cb and Cr

- Cross-format CPP-PSNR for Y, Cb and Cr

The objective metrics should be calculated using the 360Lib. All metrics shall use the sequence data provided as input to the encoder as the original sequence. When post-processing, e.g., blending, is used, the same post-processing shall be applied prior to calculation of the cross-format and end-to-end metrics. For CPP-PSNR, the metric is calculated at the coded resolution (i.e., resolution of the reconstructed coding projection format). Additional information for calculating the metrics is provided in [1].

In addition, for each projection format, an end-to-end WS-PSNR calculation of the projection mapping process without compression should also be provided, calculated between the original and reconstructed video, but skipping the Enc/Dec step in Figure 1. These results will be referred to as conversion-only.

For coding tool proposals, tool-on/tool-off tests are mandatory to be reported for the same coding projection format. Test results for other projection formats are optional.

For viewport PSNR calculations, two dynamic viewports shall be generated for each test sequence. Viewport parameters for each test sequence are listed in Table 4. A range of (yaw, pitch) offset positions are provided for the first and last pictures of the sequence, with linear interpolation of the offset position for the other pictures in the sequence. The 360Lib should be used to generate the dynamic viewports. The viewport shall be generated by rectilinear projection using bilinear interpolation filters [1]. The viewport shall have a field of view of 78.1×49.1 degrees and a resolution of 1920×1080. The viewport shall use 4:2:0 colour sampling with 10 bits per sample.

Table 4. Dynamic viewport parameters

| Class | Sequence name | V1 start (yaw, pitch) | V1 end (yaw, pitch) | V2 start (yaw, pitch) | V2 end (yaw, pitch) |
| --- | --- | --- | --- | --- | --- |
| S1 | SkateboardInLot | -180, 35 | -270, 5 | -65, -5 | 25, 25 |
| S1 | ChairliftRide | -45, -15 | 45, 15 | -201, -73 | -111, -43 |
| S1 | KiteFlite | -114, -53 | -24, -23 | 13, -41 | 103, -11 |
| S1 | Harbor | -83, -56 | 7, -26 | -140, 12 | -50, 42 |
| S1 | Trolley | 210, -18 | 300, 12 | 30, -44 | 120, -14 |
| S1 | GasLamp | -51, -11 | 39, 19 | 61, -15 | 151, 15 |
| S2 | Balboa | -4, 3 | 86, 33 | -116, -24 | -26, 6 |
| S2 | Broadway | 50, -20 | 140, 10 | -111, -41 | -21, -11 |
| S2 | Landing2 | -45, 0 | 45, 30 | -225, 0 | -135, 30 |
| S2 | BranCastle2 | 116, -15 | 206, 15 | -145, 0 | -55, 30 |

## Subjective quality check using “evil viewports”

For each projection format, a subjective quality check test should be performed to evaluate the impact of discontinuous edges, as described in [3]. Bitstreams or decoded video should be provided for viewing, according to the following procedure.

Two vertex locations will be identified for each projection format, as indicated in Table 5. The first vertex will be along the picture border at a discontinuous edge. The second vertex will be within the picture, not along the picture border, at a position with a maximum number of discontinuous edges. For ERP, the second “vertex” is at the “north” pole.

Figure 2. 360° video subjective testing procedure with “evil viewports”

Table 5. Vertex positions for “evil viewports”

| Format | Vertex 1 (yaw, pitch) (in degrees) | Vertex 2 (yaw, pitch) (in degrees) |
| --- | --- | --- |
| ERP | -180, 0 | 0, 90 |
| CMP | -135, 35 | 45, -35 |
| ACP | -135, 35 | 45, -35 |
| EAC | -135, 35 | 45, -35 |
| GCMP | -135, 35 | 45, -35 |
| RSP | -135, 0 | 0, -45 |

Table 6. Static viewport center before rotation

| Class | Sequence name | Static viewport 1 center (yaw, pitch) | Static viewport 2 center (yaw, pitch) |
| --- | --- | --- | --- |
| S1 | SkateboardInLot | -225, 20 | -20, 10 |
| S1 | ChairLift | 0, 0 | -156, -58 |
| S1 | KiteFlite | -69, -38 | 58, -26 |
| S1 | Harbor | -38, -41 | -95, 27 |
| S1 | Trolley | 255, -3 | 75, -29 |
| S1 | GasLamp | -6, 4 | 106, 0 |
| S2 | Balboa | 41, 18 | -71, -9 |
| S2 | Broadway | 95, -5 | -66, -26 |
| S2 | Landing2 | 0, 15 | -180, 15 |
| S2 | BranCastle2 | 161, 0 | -110, 15 |

As the sequence is rotated prior to encoding, as illustrated in Figure 2, it requires a separate encoding at the lowest rate point for each identified vertex and each identified static viewport. Note that the vertex indicated in Table 5 is the destination viewport center in the rotated projection format, and the static viewport center defined in Table 6 is the source viewport center in the original picture before rotation. After decoding, static viewports centered at the vertex defined in Table 5 should be extracted for subjective quality check. For a particular projection format, four decoded evil viewports shall be provided for each sequence, including (a) Static viewport 1 centered at Vertex 1, (b) Static viewport 1 centered at Vertex 2, (c) Static viewport 2 centered at Vertex 1, and (d) Static viewport 2 centered at Vertex 2. To force the identified vertex to be located in the center of the identified viewport, two steps of rotations are needed by using the 360Lib software. Specifically, for a given vertex in (dst_yaw, dst_pitch) Table 5 and a given source viewport center in (src_yaw, src_pitch) Table 6, firstly, the input ERP is rotated to let the source viewport center to be located at the (0, dst_pitch) center of the picture, secondly, it is further rotated to let the (0, dst_pitch) center of the picture to the identified vertex (dst_yaw, dst_pitch). The first step is a pure ERP-to-ERP rotation and the output frame size is the same with the input. The second step is done together with the encoding and format conversion (down-sampling).

# Configuration files

The same encoder configuration files as in the SDR CTC [2] are to be used for each test case. The temporal pre-filtering (TemporalFilter configuration option) is disabled for 360° video. Sequence-specific parameters are to be found in the cfg-360Lib/per-sequence/360/ folder. Projection format-specific parameters are to be found in the cfg-360Lib/ and cfg-360Lib/360Lib/ folders.

# Subsampling for “All Intra”

The same temporal subsampling as in the SDR CTC [2] is to be used for the “All Intra” configurations.

# Parallel encoding/decoding

The same parallel encoding/decoding as in the SDR CTC [2] may be applied for RA configurations.

# Compile-time settings

The same compile-time settings as in the SDR CTC [2] are to be used.

# References

- Y. Ye, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12)”, Joint Video Exploration Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-T2004, December 2020, by teleconference.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-T2010, October 2020, by teleconference.

- J. Boyce, Z. Deng, “AHG8: Subjective testing of 360º video projection/packing formats”, Joint Video Exploration Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-F0021, 6th Meeting, Apr. 2017.