| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U2018-v2 |
| --- | --- |

| Title: | Common test conditions for high bit depth and high bit rate video coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document from JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomohiro Ikai Dmytro Rusanovskyy Mohammed Golam Sarwer Xiaoyu Xiu | Tel: Email: | adrian.browne@sony.com ikai.tomohiro@sharp.co.jp dmytror@qti.qualcomm.com m.sarwer@alibaba-inc.com xiaoyuxiu@kwai.com |
| Source: | JVET |  |  |

_____________________________

# Abstract

This document proposes common test conditions (CTC) and software reference configurations to be used in the context of high bit depth and high bit rate video coding experiments after the 21st JVET meeting. These common test conditions are recommended for use in technical contributions to the 22nd and following JVET meetings, as applicable.

# Introduction

Common test conditions (CTC) are desirable to conduct experiments in a well-defined environment and to ease the comparison of the outcome of experiments.

This document defines three test conditions for high bit depth and high bit rate content, reflecting intra-only, random-access, and low-delay settings:

- Intra, 12 and 16 bit

- Random access, 12 and 16 bit

- Low delay, 12 and 16 bit

A subset of these test conditions might be used for a particular experiment. For example, when testing an intra coding tool, only intra configurations might be used.

The test conditions define tests at bit depths of 12 and 16 bits. For 12 bit tests, PQ, HLG and RGB sequences have been defined. For 16 bit tests, only RGB sequences are used.

Version 12.0 or later of the VTM software is expected to be used for most experiments. More recent versions are encouraged where applicable.

Any input contributions to JVET related to high bit depth, high bit rate or high frame rate video coding should provide a set of results using these test conditions that is as complete as possible.

The following sections define the software configuration, test sequences, quantization parameter values, evaluation processes, and encoder configuration files to be used.

# Software configuration

Version 12.0 (or later if available) of VTM shall be configured for high bit depth processing by defining the following macro in the source code (TypeDef.h)

#define JVET_R0351_HIGH_BIT_DEPTH_ENABLED			1

In addition, VTM shall be enabled with HDRLib functionality. For more information see the HDR CTC[2].

The macro of the SIMD modification of JVET-U0103 shall be enabled as follows

#define JVET_U0103_HIGH_BIT_DEPTH_SIMD                  1

It is noted that the above software configuration shall be applied in both low QP range and normal QP range defined in this document.

# Quantization parameter values

Two sets of quantization parameters shall be used: low QP range and normal QP range.

## Low QP range for both 12 and 16 bit

Results shall be provided using two sets of six quantization parameter values, one for 12-bit test cases and the other for 16-bit test cases as follows:

- 12 bit: 12, 7, 2, -3, -8 and -13 (mandatory)

- 16 bit: -8, -13, -18, -23, -28, -33 (mandatory)

- 16 bit: 12, 7, 2 and -3 (optional)

These values define the initial QP values that are used to derive respective QP for I- and B-frames in a sequence.

The method for calculating six point BD-rates described in JVET-H0030[3] should be used.

## Normal QP range for 12 bit YUV 420 content

Results shall be provided using two sets of four quantization parameter values, one for 12-bit test cases as follows:

- 12 bit: 22, 27, 32, and 37 (mandatory)

These values define the initial QP values that are used to derive respective QP for I- and B-frames in a sequence.

The method for calculating four points BD-rates described in [4] should be used.

# Test sequences

## Test sequences for low QP range

Table 1, Table 2, Table 3, define the sets of test sequences to be used for intra, random-access, and low-delay in both lossy and lossless experiment conditions in case of low QP range. All frames (as defined by frame count in the table) shall be encoded for all sequences in the random-access and low-delay test cases described below. For the intra configuration, the first of every eight frames shall be encoded starting with the first frame in the sequence for all sequences, and bit rates shall be calculated using the sequence frame rate (as defined by frame rate in the table) divided by eight. Please note that temporal sub-sampling may be enabled in the VTM software using the parameter TemporalSubsampleRatio. For random access and low delay configurations, if there is no number in the “encoded frame count” column for the configuration, the test case is not requested in the CTC.

Table 1: SVT RGB 4:4:4 content

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| SVT (HD) | OldTownCross | 50 | 100 | 97 | 50 | 16 | 12 and 16 |
|  | CrowdRun | 50 | 100 | 97 | 50 | 16 | 12 and 16 |
|  | ParkJoy | 50 | 100 |  |  | 16 | 12 and 16 |
|  | DucksTakeOff | 50 | 100 |  |  | 16 | 12 and 16 |
|  | InToTree | 50 | 100 |  |  | 16 | 12 and 16 |

RGB sequences are mandatory in both 12 and 16 bit tests.

Table 2: PQ YUV 4:4:4 and 4:2:2 content

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| PQ (HD) | BallonFestival | 24 | 100 | 97 | 50 | 12 | 12 |
|  | EBU_04_Hurdles | 50 | 100 | 97 | 50 | 12 | 12 |
|  | Market3Clip4000 | 50 | 100 |  |  | 12 | 12 |
|  | EBU_06_Starting | 50 | 100 |  |  | 12 | 12 |
|  | FireEater2Clip4000r1 | 25 | 100 |  |  | 12 | 12 |

PQ sequences are in both YUV 4:4:4 and YUV 4:2:2 format and are mandatory in 12 bit tests.

Table 3: HLG YUV 4:4:4 and 4:2:2 content

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| HLG (HD) | DayStreet | 60 | 100 | 97 | 50 | 12 | 12 |
|  | PeopleInShoppingCenter | 60 | 100 | 97 | 50 | 12 | 12 |
|  | NightStreet | 60 | 100 |  |  | 12 | 12 |
|  | StainedGlass | 60 | 100 |  |  | 12 | 12 |

HLG sequences are in both YUV 4:4:4 and YUV 4:2:2 format and are mandatory in 12 bit tests.

Table 4 and Table 5 indicate which test configurations are required for each type of content. The column corresponding to each configuration is interpreted as follows:

- “M” indicates that the test sequence is mandatory in the CTC for the given configuration

- “O” indicates that the test sequence is optional (but encouraged) in the CTC for the given configuration

- “-” indicates that the test sequence is not requested in the CTC for the given configuration

Table 4: Required test conditions for lossy coding

| Class | Chroma sub-sampling | 12 bit |  |  | 16 bit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LDB | AI | RA | LDB |
| SVT | 4:4:4 | M | M | M | M | O | M |
| PQ | 4:4:4 | M | M | M | - | - | - |
|  | 4:2:2 | M | O | O | - | - | - |
| HLG | 4:4:4 | M | M | M | - | - | - |
|  | 4:2:2 | M | O | O | - | - | - |

Table 5: Required test conditions for lossless coding

| Class | Chroma sub-sampling | 12 bit |  |  | 16 bit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LDB | AI | RA | LDB |
| SVT | 4:4:4 | O | O | O | O | O | O |
| PQ | 4:4:4 | O | O | O | - | - | - |
|  | 4:2:2 | O | O | O | - | - | - |
| HLG | 4:4:4 | O | O | O | - | - | - |
|  | 4:2:2 | O | O | O | - | - | - |

Original versions of the test sequences in Table 1, 2 and 3 are available on ftp://jvet@ftp.ient.rwth-aachen.de/ctc/hbd/.  (Properly qualified participants of JVET may please contact the JVET chairs for login information).

## Test sequences for normal QP range

For normal QP tests, YUV 4:2:0 version of the PQ and HLG test sequences described in Table 2 and 3 will be used. The YUV 4:2:0 version of the PQ and HLG sequences are also be available to download from the JVET ftp site ( ftp://jvet@ftp.ient.rwth-aachen.de/ctc/hbd/ ).

Table 6: PQ YUV 4:2:0 content

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| PQ (HD) | BallonFestival | 24 | 240 | 240 |  | 12 | 12 |
|  | EBU_04_Hurdles | 50 | 500 | 500 |  | 12 | 12 |
|  | Market3Clip4000 | 50 | 400 | 400 |  | 12 | 12 |
|  | EBU_06_Starting | 50 | 500 | 500 |  | 12 | 12 |

Table 7: HLG YUV 4:2:0 content

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| HLG (HD) | DayStreet | 60 | 300 | 300 |  | 12 | 12 |
|  | PeopleInShoppingCenter | 60 | 300 | 300 |  | 12 | 12 |
|  | NightStreet | 60 | 300 | 300 |  | 12 | 12 |
|  | StainedGlass | 60 | 300 | 300 |  | 12 | 12 |

Table 8: Required test conditions for lossy coding

| Class | Chroma sub-sampling | 12 bit |  |  |
| --- | --- | --- | --- | --- |
|  |  | AI | RA | LDB |
| PQ (HD) | 4:2:0 | M | M | O |
| HLG (HD) | 4:2:0 | M | M | O |

# Configuration

## Configuration for low QP range

Configuration files for the three ruested test conditions are provided with the VTM software in the cfg directory and shall be used. The three configurations provided are as follows:

- “All Intra” (AI): encoder_intra_vtm.cfg

- “Random access” (RA): encoder_randomaccess_vtm_gop16.cfg

- “Low-delay B” (LDB): encoder_lowdelay_vtm.cfg

In addition to the three test conditions, format and class specific parameter files are also provided with the VTM software and shall be used as follows:

SVT: cfg/per-class/formatRGB.cfg

PQ (both 4:4:4 and 4:2:2): cfg/per-class/classH1.cfg

HLG (both 4:4:4 and 4:2:2): cfg/per-class/classH2.cfg

To reduce simulation times and use the consistent configuration, the high bit depth and high bit rate CTC also introduce some configuration changes for low QP range to enable / disable certain tools and limit the maximum size of partitioning. Following tools will be disabled for low QP range test only.

- Disable LFNST, LMCS, BCW, MMVD, SMVD, Geo, BIO, DMVR, Affine, ISP, but encourage testing of tool-on configurations in the CE / other contributions.

- Disable temporal filtering.

These changes are included in the file cfg/hbd/hbd.cfg which shall be used.

Additional configuration, including the setting for the extended precision flag, is required separately for 12 bit and 16 bit test cases. The specific changes for different bit depths described are included in the two configuration files cfg/hbd/12-bit.cfg and cfg/hbd/16-bit.cfg and shall be used for 12 and 16 bit test cases respectively.

For lossless simulations, a further series of configurations is additionally required. For SVT content the settings contained in cfg/lossless/lossless.cfg and cfg/lossless/losslessRGB.cfg shall be used. The lossless configuration files set the internal bit depth to 0. For 12 bit SVT simulations only, this shall be overwritten by 12 by applying the configuration file cfg/hbd/12-bit.cfg after cfg/lossless/lossless.cfg. For PQ and HLG content the settings contained in cfg/lossless/lossless.cfg and cfg/lossless/lossless444.cfg shall be used.

Finally, sequence specific parameters are provided with the VTM software in the cfg/per-sequence-HBD directory and shall used.

## Configuration for normal QP range

Only AI and RA configurations provided are as follows:

- “All Intra” (AI): encoder_intra_vtm.cfg

- “Random access” (RA): encoder_randomaccess_vtm.cfg

The detail tool settings from 10-bit HDR CTC (JVET-T2011) will be used with exception of following

- InternalBitDepth=12

# HM Anchor

For the purposes of comparison an HM anchor can be created using the latest version of HM (16.22 or later) built with the following macro defined:

#define RExt__HIGH_BIT_DEPTH_SUPPORT		1

When creating an HM anchor the following configuration files shall be used for each test case:

- “All Intra” (AI): encoder_intra_main_rext.cfg

- “Random access” (RA): encoder_randomaccess_main_rext.cfg

- “Low-delay B” (LDB): encoder_lowdelay_main_rext_vtm.cfg

These files shall be used in conjuction with the files cfg/12-bit.cfg and cfg/16-bit.cfg for 12 and 16 bit test cases respectively.

# Evaluation

Results reported for PQ sequences shall include the following metrics:

- PSNR for Y′, Cb and Cr

- wPSNR for Y’, Cb and Cr (for more information see the HDR CTC[2])

Results reported for the HLG and SVT sequences class shall include the following metrics:

- PSNR for Y’, Cb and Cr

Three Excel templates for reporting test results are provided alongside this document.

- Template for normal QP test (Anchor is VTM-12.0 or later if available)

- Template for low QP test as VTM anchor (Anchor is VTM-12.0 or later if available)

- Template for low QP test as HM anchor (Anchor is HM-16.22 or later if available)

# MD5

## SVT

6004ff26cafcf2b066faa00c935a7621  CrowdRun_1920x1080_50_16bit_444.rgb

2ab943d7b1c6e3b26a2c6da416b69828  ParkJoy_1920x1080_50_16bit_444.rgb

d882efad39ae3d360896ec09d9213424  DucksTakeOff_1920x1080_50_16bit_444.rgb

6f7d413472f856741e95543217f9795d  InToTree_1920x1080_50_16bit_444.rgb

ae5bab60c3dfb16399971b9afff120da  OldTownCross_1920x1080_50_16bit_444.rgb

## PQ

### PQ 4:2:0

38216309e091dafabea8be6b4393fd7d  BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_420.yuv

3ea0125f8d34fd0e4ba1ed95c5a8cc01  Market3Clip4000r2_1920x1080p_50_12b_pq _709_ct2020_420.yuv

8836277e0e14e1ae86436cb84c2fe6da  EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_420.yuv

26773f9e00132ca318a4b45339ca02ee EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_420.yuv

### PQ 4:2:2

b029dace9f10af78aa6c3363531b1768  BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_422.yuv

716c1ec421a72c46011f31754077eec4  Market3Clip4000r2_1920x1080p_50_12b_pq _709_ct2020_422.yuv

b9f22e80fa8eb33e3c00633e6824844f  EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_422.yuv

a480ed6fc9e9f4e6b84b1ab6e7bc4f1b  EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_422.yuv

### PQ 4:4:4

1664a322cf91934872afd07035819a05  BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_444.yuv

564904c4001957472483a3d79b55f17d  Market3Clip4000r2_1920x1080p_50_12b_pq_709_ct2020_444.yuv

544b1b8816d0003e69ce6a28bbd01810  EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_444.yuv

f6761039392e76336493effdc257bf1a  EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_444.yuv

## HLG

### HLG 4:2:0

ae1af2b4d594315089186d0e6650aaca  DayStreet_1920x1080_60p_12bit_420_hlg.yuv

2c3059fbcb4b2711500f905b4c49df23 PeopleInShoppingCenter_1920x1080_60p_12bit_420_hlg.yuv

c8c3d00e7cc9f3a06b60d13522bf4de0  NightStreet_1920x1080_60p_12bit_420_hlg.yuv

803b5fda1d99c1c236cb67fef68a6ac6  StainedGlass_1920x1080_60p_12bit_420_hlg.yuv

### HLG 4:2:2

ba3c3b0576f98bc923773d48db3f4416  DayStreet_1920x1080_60p_12bit_422_hlg.yuv

1dbc59e747b08e0abe4d7a7c574c691b  PeopleInShoppingCenter_1920x1080_60p_12bit_422_hlg.yuv

271ae806359e070d1347750299dba4cb  NightStreet_1920x1080_60p_12bit_422_hlg.yuv

dde95ad0188511e050b9cc23a5491f26  StainedGlass_1920x1080_60p_12bit_422_hlg.yuv

### HLG 4:4:4

1e9702b1aea62a4a00224bd0398238ca  DayStreet_1920x1080_60p_12bit_444_hlg.yuv

2fab7d501cd6232f0e9e80b21ebb1eb8  PeopleInShoppingCenter_1920x1080_60p_12bit_444_hlg.yuv

d54aa384b79b1b10f8160b63529d1df2 NightStreet_1920x1080_60p_12bit_444_hlg.yuv

daf169ed0811a929fbc1927a0a6a9709  StainedGlass_1920x1080_60p_12bit_444_hlg.yuv

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-T2010, JVET, 20th Meeting: by teleconference, 7-16 Oct. 2020

[2] A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy , “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-T2011, JVET, 20th Meeting: by teleconference, 7-16 Oct. 2020

[3] A. M. Tourapis, D. Singer, Y. Su, K. Mammou, “BD-Rate/BD-PSNR Excel extensions”, JVET-H0030, JVET, 8th Meeting: Macao, CN, 18–25 Oct. 2017

[4] K. Andersson, F. Bossen, J.-R. Ohm, A. Segall, R. Sjöberg, J. Ström, G. J. Sullivan, A. Tourapis “Summary information on BD-rate experiment evaluation practices,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 output document JVET-R2016, 18th JVET meeting, by teleconference, April 2020 (available online at https://jvet-experts.org/).