| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U2022 |
| --- | --- |

| Title: | CE on entropy coding for high bit depth and high bit rate coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomonori Hashimoto Hong-Jheng Jhu Dmytro Rusanovskyy Kei Kawamura Tianyang Zhou | Email: | adrian.browne@sony.com tomonori.hashimoto@sharp.co.jp jhuhong-jheng@kwai.com dmytror@qti.qualcomm.com ki-kawamura@kddi.com zhou.tianyang@sharp.co.jp |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to conduct a study of entropy coding for VVC which will consider  Rice parameter derivation and transform coefficient clipping proposals submitted to the U meeting of JVET.

Participants in this activity are Kwai, Qualcomm, Sharp, Sony and KDDI.

The software basis for this CE is VTM-12. For the test sequences, configurations and test conditions, the High Bit-depth CTC described in JVET-U2018 is used, unless otherwise specified in the CE description.

# Participants

| Nr. | Name | Company | Email |
| --- | --- | --- | --- |
|  | Dmytro Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com |
|  | Luong Pham Van | Qualcomm | lphamvan@qti.qualcomm.com |
|  | Xiaoyu Xiu | Kwai | xiaoyuxiu@kwai.com |
|  | Hong-Jheng Jhu | Kwai | jhuhong-jheng@kwai.com |
|  | Tomonori Hashimoto | Sharp | tomonori.hashimoto@sharp.co.jp |
|  | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |
|  | Tianyang Zhou | Sharp | zhou.tianyang@sharp.co.jp |
|  | Adrian Browne | Sony | adrian.browne@sony.com |
|  | Steve Keating | Sony | steve.keating@sony.com |
|  | Kenji Kondo | Sony | kenji.kondo@sony.com |
|  | Mohammed Sarwer | Alibaba | m.sarwer@alibaba-inc.com |
|  | Kei Kawamura | KDDI | kei@kddi-research.jp |

# Test conditions and evaluation criteria

The proposals will be tested under the High Bit-Depth CTC specified in [1] in both lossy and lossless configurations. An updated spreadsheet containing the additional sequences will be made available at the same time as the final CE text.

Planned tests in the CE shall be implemented on, and compared with, VTM-12.0 with the modification described in JVET-U2018[1] to enable high bit depth processing. In addition, for testing of SVT content, the software base of the CE will be augmented with the rice coding modifications described in CE-1.1 and CE-2.1 of the CE from the U meeting[2]. For 16 bit testing, extended precision processing will be enabled, and all results will be compared with an anchor with the same setting and code base. For 12 bit testing, extended precision will be disabled and the results will be compared with an anchor with extended precision disabled.

Transform skip settings will follow CTC settings for all tests.

Proposals will be compared with respect to bit rate, objective quality and complexity. To provide an indication of complexity, comparative run-times for encoding and decoding will be used. In addition, throughput issues should be considered, in particular any change to the bin to bit ratio.

If a proposal changes coding for 8 or 10 bit sources, additional VVC 10 bit CTC results shall be provided.

# Proposals descriptions

## Rice parameter selection for high bit depths (JVET-U0057 and CE-1.5/CE-2.3 of JVET-T0022)

The proposal JVET-U0057 describes a simplification of a modification previously reported in JVET-T0072 to the VVC Rice parameter derivation method for both regular residual coding (RRC) and transform skip residual coding (TSRC). The modification extends the existing VVC Rice parameter deriviation method by adding an adaption technique based on selecting one of a series of counters. These counters generate a prediction of the log2 size of each coefficients being coded once for each sub-TU coded. The counters are incremented whenever a coefficient on the leading diagonal of the TU is coded which is greater than predicted and decremented whenever a coefficient is less than predicted. When coding a coefficient, the log2 predicted size is then used to derive a suitable number of bits to shift right the computed value of locSumAbs. This size of this shift is then added to the rice value derived from the lookup table g_auiGoRiceParsCoeff. TSRC is modified to use this log2 predicted size to update the Rice parameter directly as TSRC does not use the locSumAbs mechanism.

## Rice parameter derivation for high bit depths (JVET-U0050 and CE-1.3 of JVET-T0022)

The proposal of JVET-U0050/JVET-T0085 introduces a modification to the VVC Rice parameters derivation method for regular residual coding (RRC). It is proposed to use a formula instead of conventional look-up table. Specifically, the rice parameter value is predicted based on an adjusted value of the locSumAbs using linear prediction with log2 operation. The linear prediction calicurate floorLog2(a * locSumAbs + b) + c. The linear prediction parameter (a, b and c) depend on syntax (i.e. abs_reminder or dec_abs_level). An enablied flag for the proposed method is signalled in SPS. This flag doesn’t affect the process if bitdepth is equal or less than 10.

## Slice based Rice parameter selection for transform skip residual coding (JVET-U0075)

The proposal of JVET-U0075 introduces a modification to the VVC Rice parameters derivation method for transform skip residual coding (TSRC). It is proposed to explicitly signal the Rice parameter for each slice to indicate the Rice parameter for the binary codewords of abs_remainder. In the proposed method, one control flag is signaled in sequence parameter set to indicate the Rice parameter used for the TSRC is allowed to be adjusted for each slice separately. When the flag is enabled, another syntax element is further signaled for each slice to indicate the Rice parameter used for the TSRC of that slice. Otherwise (the flag is disabled), no further syntax element is signaled at slice level and the default Rice parameter (e.g. 1) is used for all the slices.

## On the Rice parameter derivation for high bit-depth coding (JVET-T0105)

The proposal of JVET-T0105 introduces a modification to the VVC Rice parameters derivation method for regular residual coding (RRC). It is proposed for high bitdepth coding to derive rice parameters based on a adjusted value of the locSumAbs. Firstly, the locSumAbs is scalled/normalized by a scalling factor which derived from signalled syntax ellements and local coefficient activity. Secondly, adjusted localSumAbs value is used to derive Rice parameter via look-up table (Table 128) as in VVC. And finally, Rice parameter is updated using the scaling factor. Parameters controlling the locSumAbs scaling/normalization and Rice parameter update are expressed through thresholds against which locSumAbs is compared and scaling factors/Rice update value. Amount of adjustment (adjustment parameters) can be determined either globally, outside of the currently coded block, directly from signalled parameters, bitdepth, or being locally adaptive, depending on locSumAbs value within currently coded block.

## On Rice Parameter Derivation with Content Adaptation (JVET-U0062)

The proposal of JVET-U0062 introduces a rice parameter selection method of regular residual coding. It is proposed for high bit depth coding to decomposes the locSumAbs value into shift parameter and index parameter of the pre-defined look-up table in VVC version 1. When the internal bit-depth is greater than 10 bit, the proposed selection is always applied. No slice level signalling is required to adjust the shift parameter.

## Transform coefficients range extension for high bit-depth coding (JVET-U0052)

The proposal of JVET-U0052 intoduces an extended coefficient precision for high bit-depth coding for VVC. This proposal provides two methods as follow:

Method 1:

ExtendedPrecisionFlag = extended_precision_processing_flag && bitDepth > 10

Log2TransformRange = ExtendedPrecisionFlag ? Max( 15, Min(20, BitDepthY + 6) ) : 15

bdShift1 = BitDepthY + rectNonTsFlag +	( ( Log2 ( nTbW ) + Log2 ( nTbH ) ) / 2 ) + 10 − Log2TransformRange+ sh_dep_quant_used_flag

bdShift2 = ( nTbH > 1  &&  nTbW > 1 ) ? 5 + Log2TransformRange – BitDepth : 6 + Log2TransformRange – BitDepth

truncSuffixLen = Log2TransformRange

maxPreExtLen = 26 – Log2TransformRange

Method 2:

Log2TransformRange = Max( 15, Min(20, BitDepthY + 5 ) )

bdShift1 = BitDepthY + rectNonTsFlag +	( ( Log2 ( nTbW ) + Log2 ( nTbH ) ) / 2 ) + 10 − Log2TransformRange+ sh_dep_quant_used_flag

bdShift2 = ( nTbH > 1  &&  nTbW > 1 ) ? 5 + Log2TransformRange – BitDepth : 6 + Log2TransformRange – BitDepth

truncSuffixLen = Log2TransformRange

maxPreExtLen = 26 – Log2TransformRange

For method 1 an enabled flag is signaled in SPS extension. For method 2 derives without ExtendedPrecisionFlag. Both methods use clipping 20 for the coefficient range and do not change 8 to 10 bit case (also plan to test different clipping values e.g. 17, 18, 19).

## RRC rice derivation methods of JVET-T0105 and CE1.1 and CE1.2.a of JVET-U0064

Two method of RRC rice derivation process were proposed in JVET-T0105 with globally and locally derived adjusment for rice parameter derivation. In the first method (earlier tested as CE1.1 in JVET-U0064), to extend the range of the Rice parameters, it is proposed to scale values locSumAbs with a right shift, if locSumAbs exceed certain threshold, such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with shift value if scaling process was taken a place for a given locSumAbs value.In the second method, (earlier tested as CE1.2.a in JVET-U0064), locally adaptive adjusment for Rice parameter derivation is being evaluated. Similarly to the the first method, to extend the range of the Rice parameters, it is proposed to scale values locSumAbs with a shift, if locSumAbs exceed certain threshold, such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with shift value if scaling process was taken a place for a given locSumAbs value.

## RRC rice derivation methods of 3.7 with history utilization (Methods of CE1.2.b of JVET-U0064 and CE3.5 of JVET-U0066)

Two methods of history utilization for improving performance of the RRC rice derivation were proposed in JVET-U0064 and JVET-U0066. The first method of history utilization was earlier tested in CE1.2.b of JVET-U0064). In this method, a history update was conducted in first TU and its history result was utilized in the following TU to replace template based Rice parameter derivation for certain transform coeffitients. The second method of history utilization was earlier tested in CE3.5 of JVET-U0066). In this method, a history update process was identical to method 1, with history result utilization being integrated to the template-based Rice derivation process.

# Planned tests

## Tests on proposed regular residual coding.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE-1.1 | Sony |  |
| CE-1.2 | Sony |  |
| CE-1.3 |  |  |
| CE-1.4 |  |  |
| CE-1.5 |  |  |

## Tests on proposed transform skip residual coding.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE-2.1 | Kwai |  |
| CE-2.2 | Sony |  |
| CE-2.3 | Sony |  |

## Tests on proposed transform coefficient clipping

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE-3.1 | TBD |  |
| CE-3.2 |  |  |
| CE-3.3 |  |  |

## Tests on rice coding combined with transform coefficient clipping

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE-4.1 | TBD |  |
| CE-4.2 |  |  |
| CE-4.3 |  |  |

# Tests description:

## CE-1.1: Method of CE-1.1 from JVET-U0064

In this test, the first method described in section 3.7 and proposed in CE-1.1 of JVET-U0064 to be tested.

## CE-1.x: Method of CE-1.2.a from JVET-U0064

In this test, the second method described in section 3.7 and proposed in CE-1.1 of JVET-U0064 to be tested.

## CE-1.x: History utilization method of JVET-U0064

In this test, the first history based method described in section 3.7 and earlier studied as CE1.2.b in JVET-U0064 is being tested.

## CE-1.x: Method of CE-1.2.a from JVET-U0064

In this test, the second history based method described in section 3.7 and earlier studied as CE3.5 in JVET-U0066 is being tested.

## CE-1.x: Method of CE-1.5 from JVET-U0057 with parameter set update once per sub-TU

In this test, the method proposed in CE-1.5 of JVET-U0057 is augmented by an SPS level flag which enables / disables the the method.

## CE-1.x: Method of CE-1.5 from JVET-U0057 with parameter set update once per TU

In this test, the method proposed in CE-1.5 of JVET-U0057 is modified to update the parameter set once per TU. In addition the method is augmented by an SPS level flag which enables / disables the the method.

## CE-2.1: Method of JVET-U0075

Method proposed in JVET-U0075, it is proposed to explicitly signal the Rice parameter for each slice to indicate the Rice parameter for the binary codewords of abs_remainder. In the proposed method, one control flag is signaled in sequence parameter set to indicate the Rice parameter used for the TSRC is allowed to be adjusted for each slice separately. When the flag is enabled, another syntax element is further signaled for each slice to indicate the Rice parameter used for the TSRC of that slice. Otherwise (the flag is disabled), no further syntax element is signaled at slice level and the default Rice parameter (e.g. 1) is used for all the slices.

## CE-2.2: Method of CE-2.3 from JVET-U0057 with parameter set update once per sub-TU

In this test, the method proposed in CE-2.3 of JVET-U0057 is augmented by an SPS level flag which enables / disables the the method.

## CE-2.3: Method of CE-2.3 from JVET-U0057 with parameter set update once per TU

In this test, the method proposed in CE-2.3 of JVET-U0057 is modified to update the parameter set once per TU. In addition the method is augmented by an SPS level flag which enables / disables the the method.

The following tests requires updating…

## CE-1.2: Method of JVET-T0105 with local adaptivity

In this test, method proposed in JVET-T0105 with locally adaptive adjusment for Rice parameter derivation is being evaluated. Adjustment parameters are determined from signalled syntax elemments, and updated locally, depending on locSumAbs value within currently coded block.

## CE-1.3: Method of JVET-T0085.

For this experiment, method proposed in JVET-T0085, it is proposed to use a formula instead of conventional look-up table for RRC, is evaluated.

## CE-1.4: Method of JVET-T0072 with standard TSRC

The RRC component of the method proposed in JVET-T0072 with rice parameter adaption based on previously coded coefficients. For this experiment the TSRC component of the modification described in JVET-T0072 is disabled and standard VVC TSRC is used.

## CE-1.5: Method of JVET-T0072 with simplification and standard TSRC

The RRC component of the simplification proposed in JVET-T0072 of the technique described in CE-1.4. For this experiment the TSRC component of the modification described in JVET-T0072 is disabled and standard VVC TSRC is used.

## CE-1.6: Method of JVET-U0062

For this experiment, method proposed in JVET-U0062, it is proposed to decompose locSumAbs value into shift parameter and index parameter of the conventioan look-up table for RRC, is evaluated.

## CE-2.2: Method of JVET-T0072 with standard RRC

The TSRC component of the method proposed in JVET-T0072 with rice parameter adaption based on previously coded coefficients. For this experiment the RRC component of the modification described in JVET-T0072 is disabled and standard VVC RRC is used.

## CE-2.3: Method of JVET-T0072 with simplification and standard RRC

The TSRC component of the simplification proposed in JVET-T0072 of the technique described in CE-2.2. In addition, this experiment replaces the usage of locSumAbs and g_auiGoRiceParsCoeff with an offset derived from the value of the selected counter. For this experiment the RRC component of the modification described in JVET-T0072 is disabled and standard VVC RRC is used.

## CE-3.1: Combination of CE-1.3 and CE-2.1

This experiment will test the combination of CE-1.3 and CE-2.1.

## CE-3.2: Combination of CE-1.5 and CE-2.1

This experiment will blend the RRC coding modifications from CE-1.5 with the TSRC coding modifications in CE-2.1.

## CE-3.3: Combination of CE-1.5 and CE-2.3

This experiment will blend the RRC coding modifications from CE-1.5 with the TSRC coding modifications in CE-2.3. The counter update method will be taken from CE-2.2 (the counter update simplification in CE-2.3 will not be applied).

## CE-3.4: Combination of CE-1.2 and CE-2.1

This experiment will blend the RRC coding modifications from CE-1.2.a with the TSRC coding modifications in CE-2.1.

## CE-3.5: Combination of CE-1.2 and CE-1.4/1.5

This experiment will investigate integration of the classification ellements of CE-1.4/1.5 with rice parameter inheritance mechanism and the Rice parameter derivation of the CE-1.2.b.

## CE-3.6: Combination of CE-1.3 and CE-3.5

This experiment will investigate integration of Rice parameter derivation method of CE1.3 with with Rice parameter inheritance mechanism of the CE1.2.b modified with classification ellements of CE-1.4/1.5.

# Time-line and Responsibilities

T1: 2021-January-29:	   			Version of CE description with final descriptions for tests CE-1, CE-2 and CE-3 uploaded. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to marked as "withdrawn".

T2: VTM 12.0 + 1 week:				Integration of CE-1.1 and CE-2.1 into a separate CE branch of the VTM (CE anchor) is completed

T3: VTM 12.0 + 2 week (not later than February 19) :

Integration of all CE-1/CE-2/CE-3 tools into a separate CE branch of the VTM is completed and initial study by cross-checkers begins.

T4: T3 + 2 weeks: 	 				Final description of combined tests is provided.

T5: T3 + 2 weeks: 					Final version of CE-1/CE-2/CE-3 software and full results are provided, final cross-check begins.

T6: T3 + 4 weeks: 					Final version of combination software and full results are provided, final cross-check of combination begins.

T7: 2021-April-09: 			   		CE contribution documents including specification text and complete test results are uploaded to the JVET document repository.

# References

[1] A. Browne, T. Ikai, D.  Rusanovskyy, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-T2018, JVET, 20th Meeting: by teleconference, 7-16 Oct. 2020

[2] A.Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, “CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-U0022, JVET, 21st Meeting by teleconference, 6-15 Jan. 2021