| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0010 |
| --- | --- |

| Title: |  | JVET AHG report: Encoding algorithm optimization (AHG10) |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | AHG report input to JVET |  |  |
| Purpose: |  | AHG report |  |  |
| Author(s) or Contact(s): | Alberto Duenas Alexis Michael Tourapis Andrey Norkin Rickard Sjöberg |  | Email: | albertoduenas@fb.com atourapis@apple.com anorkin@netflix.com rickard.sjoberg@ericsson.com |  |
| Source: | AHG |  |  |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG10: Encoding algorithm optimization, between the 21st meeting (teleconference, 6-15 January 2021) and the 22nd meeting (teleconference, 20-28 April 2021).

# Mandates

At the 21st meeting of the ITU-T/ISO/IEC Joint Video Experts Team (JVET), an ad hoc group on Encoding algorithm optimization was established with the following mandates:

- Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization.

- Study the impact of non-normative techniques of pre processing for the benefit of encoder optimization.

- Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality.

- Consider neural network-based encoding optimization technologies for video coding standards.

- Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding.

- Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). No e-mail related to AHG10 activity was sent to the JVET reflector during the AHG period.

# Overview of input documents related to the AHG

The following input documents were identified to be related to this AHG:

## JVET-V0056 [AHG10] GOP-based temporal filter improvements

This contribution proposes a set of changes to the GOP-based temporal filter in the VTM that aim at improving the BD-rate performance. These changes were first proposed in JVET-U0056.

The average Y/U/V BD-rate improvements using the VTM-12 CTC are reported to be equal to −1.4%/−1.0%/−1.1% for RA for the 3 colour components respectively. All BDR numbers were computed using the unfiltered source sequences as the reference.

In addition, the same changes were implemented in the HM where the BD-rate improvements were found to be equal to -1.0%/-1.0%/-1.0% under the same conditions.

The VTM proposal itself is unchanged since JVET-U0056, except for fixing a bug regarding motion compensation of blocks along the edge a picture. The code for the original proposal has been available since 2021-01-22 on the VTM Github.

Subjective viewing has been carried out by the proponents, where the anchor and test were found to have similar quality for all viewed sequences.

Finally, an HM version of the algorithm has been implemented, with similar performance. While the algorithm is the same for the HM implementation, the multiplications for the weights have been slightly modified which is explained in the contribution.

We do observe a relationship of the QP used with both the performance of this scheme as well as its impact on encoder and decoder complexity. In particular, for the lower QP values, i.e. QP values of 22 and 27, quite often a considerable bitrate reduction is observed. For the higher QP values an increase in bitrate for some material was also observed. Although it is not clear how much such numbers could be trusted, at the higher QP values an increase in encoding and decoding time was also observed that might not be reflected in the averages.

### Results

The proposal was implemented on top of VTM-12.0. The method is applied for RA coding only. The BD-rate result relative to VTM-12.0 is reported below.

|  | Over VTM-12.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.83% | -1.86% | -2.55% | 93% | 95% |
| Class A2 | -1.07% | -0.78% | -0.79% | 97% | 104% |
| Class B | -1.40% | -0.95% | -0.98% | 95% | 93% |
| Class C | -1.19% | -0.57% | -0.30% | 98% | 109% |
| Class E |  |  |  |  |  |
| Overall | -1.36% | -1.00% | -1.08% | 96% | 100% |
| Class D | -1.48% | -0.09% | -0.15% | 99% | 106% |
| Class F | 0.00% | 0.00% | 0.00% | 96% | 119% |

In addition, these are the results when compared to VTM-12.0 without temporal filtering:

|  | Over VTM-12.0 with MCTF = 0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.54% | -8.89% | -8.66% | 90% | 81% |
| Class A2 | -6.21% | -10.83% | -10.59% | 92% | 88% |
| Class B | -7.66% | -9.17% | -9.39% | 94% | 86% |
| Class C | -2.82% | -4.37% | -3.86% | 98% | 95% |
| Class E |  |  |  |  |  |
| Overall | -5.45% | -8.17% | -8.01% | 94% | 88% |
| Class D | -0.81% | -3.53% | -3.47% | 98% | 92% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 100% |

Finally, the contribution presents also results using HM-16.23 compared to the CTC:

|  | Random Access Main 10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | -1.4% | -1.5% | -1.8% |
| Class A2 | -1.1% | -1.1% | -1.2% |
| Class B | -1.6% | -1.8% | -1.7% |
| Class C | -1.0% | -0.9% | -1.0% |
| Class D | -1.0% | -0.8% | -0.6% |
| Class E |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% |
| Overall | -1.0% | -1.0% | -1.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 100% |  |  |

It is interesting to note that the proposed method, and ignoring the per QP variations mentioned earlier, shows an average decoder complexity reduction on the results. Nevertheless, it is indicated in JVET-U0056 that there seems to be no discernible impact on encoding complexity.

## JVET-V0057 [AHG10] Block importance mapping

This contribution proposes a new algorithm for the VTM to signal CTU level QP delta values in pictures that will be used for reference. The QP value to use for each CTU is based on the estimated importance of a given CTU for future pictures and the QP selected is in the range of −2 to +2 relative to the picture QP.

The method was reportedly implemented and tested together with the temporal filter changes first presented in JVET-U0056 under the VTM-12.0 RA test configurations. The method is not proposed to be used for AI or LD configurations.

The average CTC RA Y/U/V BD-rate improvements for the proposed changes together with JVET-V0056 compared to VTM-12 with JVET-V0056 are reported to be −1.4%/−3.9%/−3.8% for RA. Comparing the proposed changes together with JVET-V0056 to the regular VTM-12, the average BD-rates are −2.8%/−5.0%/−4.9%.

It is proposed by the contribution to adopt the algorithm into the VTM software.

Version 2 of this document provides a statement that the results for the HM implementation will be added later on.

The estimation of the difference between pictures is based on calculations performed in the temporal filter. The QP is only modified for frames where the temporal filter is active, i.e., where the POC is divisible by eight.

The contribution proposes that rather than using a fixed QP for each temporal layer, individual QP values for each CTU is set based on how important it is calculated the CTU will be to future pictures. This was proposed to estimate this importance is estimated by measuring the difference between motion compensated pictures.

The contribution indicates that there seems to be no discernible impact on complexity.

### Results

The proposal was implemented on top of VTM-12.0 with JVET-V0056 [2]. The method is applied for RA coding only. The BD-rate result relative to VTM-12.0 with JVET-V0056 is reported below. The contribution says that encoding and decoding times are not accurate.

|  | Over VTM-12.0 with V0056 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.66% | -0.84% | -2.41% | 108% | 114% |
| Class A2 | -1.09% | -4.11% | -2.93% | 108% | 111% |
| Class B | -2.02% | -4.97% | -5.10% | 107% | 116% |
| Class C | -1.56% | -4.88% | -3.94% | 132% | 126% |
| Class E |  |  |  |  |  |
| Overall | -1.44% | -3.95% | -3.82% | 114% | 117% |
| Class D | -1.08% | -4.69% | -4.35% | 125% | 152% |
| Class F | -2.61% | -3.48% | -2.13% | 105% | 107% |

In addition, these are the results when comparing the proposal together with JVET-V0056 to VTM-12.0:

|  | Over VTM-12.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.52% | -2.76% | -4.95% | 101% | 109% |
| Class A2 | -2.22% | -4.98% | -3.80% | 105% | 116% |
| Class B | -3.47% | -5.95% | -6.11% | 101% | 108% |
| Class C | -2.76% | -5.50% | -4.30% | 129% | 138% |
| Class E |  |  |  |  |  |
| Overall | -2.84% | -5.00% | -4.94% | 109% | 117% |
| Class D | -2.61% | -4.81% | -4.53% | 123% | 161% |
| Class F | -2.61% | -3.48% | -2.13% | 101% | 114% |

The method was also tested on HM-16.23, results will be provided at a later stage.

## JVET-V0078 AHG 10: QP control for very smooth blocks

Large transforms are excellent in reducing spatial redundancy. However, when source samples are very smooth the subjective quality can drop very quickly in such areas with too coarse quantization. It is therefore suggested to reduce the QP according to a linear model for very smooth regions to prolong the breakdown of the subjective quality. Since smooth regions are relatively easy to code it is claimed that the overhead in bitrate can be manageable.

The impact on BDR has been tested on HDR:

VTM-12.0 HDR RA CTC (luma/Cb/Cr):              0.14%/0.14%/0.02%

HM-16.22 (TF on) HDR RA CTC (luma/Cb/Cr): 0.49%/-0.36%/-0.28%

It is asserted that the proposal can improve subjective quality of smooth regions for HDR and it is suggested to update the VTM and HM with this functionality. The proposal leads to an objective BDR loss by reducing the QP for smooth blocks. In contrast, it is indicating that  the subjective quality can be improved in smooth areas.

### Objective results

The method has been tested for HDR RA CTC. It can be noted that there is a drop in BDR by the proposed method and that the drop is slightly larger for the HM than for the VTM. The encoding and decoding times are not reliable.

Table 1: VTM-12.0 for CTC HDR RA

|  | PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class H1 | 0,17% | 0,19% | 0,07% | 100% | 102% |
| Class H2 | 0,09% | 0,04% | -0,08% | 104% | 122% |
| Overall | 0,14% | 0,14% | 0,02% | 101% | 109% |

Table 2: HM-16.22 (TF on) for CTC HDR RA

|  | PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class H1 | 0,71% | -0,48% | -0,23% | 85% | 85% |
| Class H2 | 0,11% | -0,15% | -0,38% | 84% | 84% |
| Overall | 0,49% | -0,36% | -0,28% | 85% | 85% |

No additional objective results requested for H1 sequence class on the HDR RA CTC were presented.

## Subjective results

Subjective video quality is best evaluated by playout on an HDR capable TV. To illustrate the subjective quality improvement by the proposed method the contribution contains a snapshot from the 4K HDR PQ Netflix sequence SparksTruckPack (H1 sequence class)  and SunsetBeach (H2 sequence class).

## JVET-V0095 AHG10: Using original samples for SAO and ALF optimization

In VTM-12.0, motion compensated temporal pre-filtering (MCTF) is used for the random access (RA) configuration. Samples filtered by the MCTF are used for sample adaptive offset (SAO) and adaptive loop filter (ALF) optimizations. This contribution proposes to use original samples for SAO and ALF optimization as an encoder only modification. Compared to VTM-12.0, the average Y/U/V BD-rates reportedly are -0.22%/-0.09%/-0.08% for RA with similar running time.

In VTM-12.0, the encoder minimizes the rate*lambda + distortion cost metric for the sample adaptive offset (SAO) and adaptive loop filter (ALF) methods. When motion compensated temporal pre-filtering (MCTF) is enabled for the random access (RA) configuration, it was observed that the SAO and ALF processes minimize the cost for samples filtered by the MCTF. On the other hand, the final PSNR computation that is also used for the BD-rate computation is calculated using original unfiltered input samples.

In the proposal, instead of using samples filtered by the MCTF, the original samples are used to calculate the distortion for SAO and ALF for parameters estimation.

### Results

The proposed method was implemented on top of VTM-12.0 and evaluated under the common test conditions for RA and LDB.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-12.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.02% | -0.17% | 101% | 101% |
| Class A2 | -0.02% | -0.04% | 0.15% | 100% | 99% |
| Class B | -0.10% | 0.18% | 0.12% | 100% | 100% |
| Class C | -0.59% | -0.51% | -0.43% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.09% | -0.08% | 100% | 100% |
| Class D | -0.90% | -0.65% | -0.52% | 98% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-12.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 102% |

## JVET-V0066 AHG8: Encoder improvements to palette coding for high bit depth

This contribution proposes two encoder modifications to palette coding for high bit depth:

- Proposal#1 uses on-the-fly bit calculation functions instead of pre-defined lookup tables to reduce memory usage and has no impact to coding performance and can reduce memory size.

- Proposal#2 fixes QP offset derivation used for an error limit to improve coding performance of palette coding.

Proposal#2, it is reported that average bd-rate changes for YUV at normal QP are (-8.70%, -10.86%, -11.66%) for AI and (-6.09%, -8.43%, -9.16%) for RA, and average bd-rate changes for RGB are (-8.51%, -8.65%, -8.32%) for AI and (-5.64%, -5.86%, -5.73%) for RA; average bd-rate changes for YUV at low QP are (-1.80%, -2.08%, -2.09%) for AI and (-1.34%, -1.27%, -1.31%) for RA, and average bd-rate changes for RGB are (-1.87%, -1.93%, -1.96%) for AI and (-1.19%, -1.33%, -1.27%) for RA, respectively.

At the encoder, to retrieve errorLimit from the lookup table, the QP needs to be adjusted to an 8-bit domain QP by a QP offset. However, in the VTM, the QP offset always takes a fixed value of “-12”. This contribution proposes to fix the QP offset based on BitDepth, i.e. considering BdOffset as below.

The QPs used for testing are listed as follow:

	Normal QP (QP=22, 27, 32, 37)

	Low QP (QP=-13, -8, -3, 2, 7, 12)

This contribution reports encoder improvements to palette coding by using on-the-fly bit calculation functions instead of the pre-defined lookup tables and fixing QP Offset derivation used for an error limit. It is recommended to adopt proposed software fixes to the next VTM.

### Results

Coding performance of Proposal#2 at normal QP (Test#2)

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT | G | B | R | EncT | DecT |
| TGM 1080p | -16.37% | -18.13% | -17.86% | 102% | 89% | -12.64% | -12.80% | -12.47% | 103% | 91% |
| TGM 720p | -8.90% | -10.50% | -12.64% | 101% | 95% | -10.96% | -10.82% | -10.48% | 100% | 95% |
| Animation | -2.27% | -4.13% | -4.50% | 100% | 99% | -4.95% | -4.97% | -4.57% | 97% | 98% |
| Mixed content | -4.63% | -8.39% | -9.26% | 100% | 97% | -3.28% | -3.90% | -3.68% | 100% | 98% |
| Overall | -8.70% | -10.86% | -11.66% | 101% | 95% | -8.51% | -8.65% | -8.32% | 100% | 95% |
|  | Random access |  |  |  |  |  |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT | G | B | R | EncT | DecT |
| TGM 1080p | -10.63% | -12.93% | -12.61% | 97% | 97% | -7.62% | -8.06% | -7.86% | 99% | 98% |
| TGM 720p | -8.26% | -10.66% | -13.06% | 98% | 100% | -8.84% | -9.13% | -9.15% | 99% | 99% |
| Animation | -0.29% | -1.48% | -1.66% | 98% | 100% | -3.05% | -2.60% | -2.40% | 99% | 99% |
| Mixed content | -2.96% | -6.42% | -6.83% | 98% | 99% | -1.34% | -1.85% | -1.65% | 101% | 100% |
| Overall | -6.09% | -8.43% | -9.16% | 98% | 99% | -5.64% | -5.86% | -5.73% | 99% | 99% |

Coding performance of Proposal#2 at low QP (Test#2)

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT | G | B | R | EncT | DecT |
| TGM 1080p | -2.72% | -2.87% | -2.87% | 98% | 97% | -1.87% | -1.94% | -1.92% | 101% | 97% |
| TGM 720p | -2.61% | -2.25% | -2.49% | 100% | 96% | -4.73% | -4.85% | -4.90% | 97% | 94% |
| Animation | -1.34% | -2.69% | -2.55% | 95% | 97% | -0.29% | -0.33% | -0.36% | 103% | 99% |
| Mixed content | -0.55% | -0.51% | -0.45% | 97% | 99% | -0.60% | -0.60% | -0.65% | 101% | 99% |
| Overall | -1.80% | -2.08% | -2.09% | 97% | 97% | -1.87% | -1.93% | -1.96% | 100% | 98% |
|  | Random access |  |  |  |  |  |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT | G | B | R | EncT | DecT |
| TGM 1080p | -2.34% | -2.48% | -2.47% | 98% | 99% | -1.51% | -1.52% | -1.52% | 98% | 99% |
| TGM 720p | -1.44% | -0.71% | -1.04% | 98% | 99% | -2.69% | -3.32% | -3.01% | 99% | 99% |
| Animation | -1.44% | -1.81% | -1.66% | 100% | 99% | -0.29% | -0.30% | -0.31% | 101% | 99% |
| Mixed content | -0.15% | -0.08% | -0.07% | 100% | 100% | -0.28% | -0.19% | -0.25% | 100% | 100% |
| Overall | -1.34% | -1.27% | -1.31% | 99% | 99% | -1.19% | -1.33% | -1.27% | 100% | 99% |

## JVET-V0127 An optimized VVC encoder implementation

This contribution reports the performance of an optimized VVC encoder, BVC, developed by Bytedance. A majority of the coding tools and features introduced in VVC have been integrated and optimized in the BVC encoder. In addition, it also includes real-world encoder features like scene change detection, rate control and multi-threading. When compared to the HM and the VTM, the corresponding global BD-rate gain and speedup factors under the Random Access configuration are summarized as follows:

- Fast preset (6 threads): 34.43%, 17x (HM), 131x (VTM)

- Slow preset (6 threads): Will be presented on an update of the contribution.

### Simulation Results

For those simulations, 6 threads are used for the BVC encoder. It could be seen that with the fast preset, the BD-rate gain over HM is still approx. 34.43% on average, but a speedup of 17 times and 131 times over the HM and the VTM, respectively is observed.

The slow preset, which is targeting at close performance to VTM, the BD-rate gain over HM is no yet available, but a speedup of more than 7 times over VTM was reported.

Coding performance of the BVC encoder @ fast preset

| Class | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR BD-rate relative to HM |  |  |  | Speedup |  |
|  | Y (%) | U (%) | V (%) | YUV (%) | vs HM | vs VTM |
| A1 | -35.76 | -43.56 | -48.57 | -38.33 | 22.36 | 148.45 |
| A2 | -36.93 | -45.53 | -45.83 | -39.12 | 18.40 | 133.37 |
| B | -34.12 | -54.24 | -50.68 | -38.70 | 18.05 | 130.91 |
| C | -20.04 | -30.53 | -30.50 | -22.66 | 11.78 | 118.43 |
| Overall | -31.26 | -44.04 | -43.91 | -34.43 | 16.88 | 131.20 |