| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0022r1 |
| --- | --- |

| Title: | CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomonori Hashimoto Hong-Jheng Jhu Dmytro Rusanovskyy Kei Kawamura Tianyang Zhou | Email: | adrian.browne@sony.com tomonori.hashimoto@sharp.co.jp jhuhong-jheng@kwai.com dmytror@qti.qualcomm.com ki-kawamura@kddi.com zhou.tianyang@sharp.co.jp |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report for the Core Experiment on entropy coding for high bit depth and high bit rate coding. A total of 20 tests have been completed within the CE between the JVET U and V meetings to study and evaluate technologies related to Rice parameter derivation. The scope of this experiment covered tests on changes to the regular residual coding (RRC) (8 tests), transform skip residual coding (TSRC) (3 tests) and transform clipping (2 tests). In addition 7 tests have been completed in a second stage on combinations of RRC/TSRC tests with transform clipping tests. Results have been integrated on and tested against the Anchor CE software, which comprised integration of low complexity methods proposed in the CE of the U meeting, namely the RRC method of CE1.1 and the TSRC method of CE2.1.

In this report, coding performance and complexity are reported and analyzed. Test results against CE   anchors are provided to show the coding efficiency and complexity trade-off of each tool. Where appropriate results are also provided for lossless conditions and screen content. Cross-check results for the performed tests are submitted as separate documents and their summaries are integrated in this contribution.

# Core Experiment Anchor and CTC

## CE Anchor

The source code anchor for the CE was VTM12.0 modified with the lowest complexity rice coding modifications proposed in the CE of the U meeting. These changes consisted of the RRC modifications of CE1.1 and the TSRC modifications of CE2.1.

The anchor was generated using the high bit depth (HBD), high bit rate (HBR), high frame rate (HFR) CTC (JVET-U2018), which includes simulations within a standard QP range (QPs 22,27,32,37 for 12 bit YUV 4:2:0) and a low QP range (QPs 12,7,2,-3,-8,-13 for 12 bit YUV 4:2:2/4:4:4, -8,-13,-18,-23,-28,-33 for 16 bit RGB).

In addition to testing against VTM12.0, the CE anchor was also tested against HM16.23 configured for lossy and lossless coding using RExt test conditions (JVET-U1100) adjusted for the content and low QP range of JVET-U2018.

## CE Anchor results

### CE Anchor results vs. VTM12.0

Table 1.1. Simulation results for CE Anchor and its RRC and TSRC components vs. VTM12.0, 12 bits data, HBD/HFR CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | CE Anchor | -0.49% | -0.49% | -0.52% | -0.44% | -0.35% | -0.36% | -2.68% | -2.73% | -2.67% | -2.64% |
|  | RRC | -0.50% | -0.53% | -0.56% | -0.33% | -0.33% | -0.34% | -2.53% | -2.62% | -2.48% | -2.48% |
|  | TSRC | -0.10% | -0.06% | -0.07% | -0.21% | -0.15% | -0.15% | -1.55% | -1.45% | -1.62% | -1.57% |
| LDB | CE Anchor | -0.07% | -0.06% | -0.05% | 0.02% | -0.02% | -0.06% | -0.50% | -0.45% | -0.53% | -0.53% |
|  | RRC | -0.07% | -0.08% | -0.08% | -0.03% | -0.02% | -0.05% | -0.37% | -0.31% | -0.40% | -0.40% |
|  | TSRC | 0.00% | 0.00% | -0.01% | 0.05% | 0.00% | -0.04% | -0.40% | -0.38% | -0.42% | -0.40% |
| RA | CE Anchor | -0.13% | -0.13% | -0.14% | -0.01% | -0.04% | -0.06% | -0.54% | -0.51% | -0.56% | -0.54% |
|  | RRC | -0.14% | -0.15% | -0.15% | -0.04% | -0.04% | -0.05% | -0.42% | -0.40% | -0.43% | -0.43% |
|  | TSRC | 0.00% | 0.00% | -0.01% | 0.03% | 0.00% | -0.02% | -0.39% | -0.37% | -0.41% | -0.38% |

Table 1.2. Simulation results for CE Anchor and its RRC and TSRC components vs. VTM12.0, 16 bits data, HBD/HFR CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE Anchor | -24.70% | -25.62% | -24.25% | -24.24% |
|  | AnchorRRC | -24.35% | -25.14% | -23.95% | -23.95% |
|  | AnchorTSRC | -21.64% | -22.27% | -21.36% | -21.30% |
| LDB | CE Anchor | -16.38% | -16.99% | -16.06% | -16.09% |
|  | RRC | -15.83% | -16.66% | -15.29% | -15.55% |
|  | TSRC | -13.60% | -13.97% | -13.41% | -13.41% |
| RA | CE Anchor | -15.56% | -15.70% | -15.50% | -15.48% |
|  | RRC | -15.15% | -15.26% | -15.04% | -15.14% |
|  | TSRC | -12.80% | -12.67% | -12.89% | -12.84% |

Table 1.3. Run time estimates for CE Anchor and its RRC and TSRC components vs. VTM12.0, HBD/HFR CTC, LowQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  | SVT RGB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec | Enc | Dec |
| AI | CE Anchor | 101% | 95% | 100% | 96% | 98% | 89% |
|  | RRC | 102% | 99% | 101% | 100% | 99% | 89% |
|  | TSRC | 100% | 99% | 99% | 99% | 101% | 98% |
| LDB | CE Anchor | 101% | 100% | 99% | 98% | 99% | 94% |
|  | RRC | 102% | 103% | 102% | 100% | 99% | 95% |
|  | TSRC | 100% | 100% | 100% | 100% | 99% | 99% |
| RA | CE Anchor | 99% | 101% | 100% | 100% | 95% | 89% |
|  | RRC | 102% | 103% | 101% | 99% | 99% | 95% |
|  | TSRC | 100% | 99% | 100% | 100% | 98% | 98% |

Table 1.4. Simulation results for CE Anchor and its RRC and TSRC components vs. VTM12.0, HBD/HFR CTC, NormQP test configuration.

|  | Test | HDR PQ |  |  |  |  | HDR HLG |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DE100 | PSNR-L100 | wY | wU | wV | Y | U | V |
| AI | CE Anchor | 0.02% | 0.00% | 0.00% | 0.01% | 0.04% | 0.01% | -0.02% | -0.06% |
|  | RRC | 0.02% | 0.00% | 0.00% | 0.01% | 0.04% | 0.00% | -0.02% | -0.06% |
|  | TSRC | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| RA | CE Anchor | -0.07% | 0.01% | 0.01% | -0.04% | -0.16% | 0.04% | -0.16% | 0.02% |
|  | RRC | -0.09% | -0.01% | -0.01% | -0.06% | -0.18% | 0.03% | -0.17% | 0.00% |
|  | TSRC | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% |

Table 1.5. Run time estimates for CE Anchor and its RRC and TSRC components vs. VTM12.0, HBD/HFR CTC, LowQP test configuration.

| NormQP | Test | HDR PQ |  | HDR HLG |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec |
| AI | CE Anchor | 100% | 99% | 99% | 100% |
|  | RRC | 100% | 98% | 101% | 101% |
|  | TSRC | 100% | 100% | 100% | 100% |
| RA | CE Anchor | 102% | 102% | 101% | 101% |
|  | RRC | 99% | 99% | 100% | 100% |
|  | TSRC | 100% | 100% | 100% | 101% |

### CE Anchor results vs. HM16.23

Table 1.7. Simulation results for CE Anchor vs. HM16.23, 12 bits data, HBD/HFR CTC for VTM , RExt CTC for HM, LowQP test configuration.

| Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | -7.24% | -9.64% | -10.23% | -4.09% | -6.39% | -6.53% | -2.17% | -2.54% | -1.95% | -2.03% |
| LDB | -6.68% | -7.73% | -8.82% | -5.12% | -8.20% | -7.06% | -1.79% | -3.31% | -1.05% | -1.02% |
| RA | -7.61% | -11.64% | -11.85% | -5.73% | -8.68% | -7.37% | -2.11% | -3.72% | -1.32% | -1.29% |

Table 1.8. Simulation results for CE Anchor vs. HM16.23, 16 bits data, HBD/HFR CTC for VTM, RExt CTC for HM, LowQP test configuration.

| Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- |
|  | Aver.GBR | G | B | R |
| AI | 0.77% | 0.70% | 0.82% | 0.79% |
| LDB | 0.58% | -0.52% | 1.13% | 1.13% |
| RA | 0.32% | -0.86% | 0.91% | 0.91% |

Table 1.9. Run time estimates for CE Anchor vs. HM16.23, HBD/HFR CTC for VTM, RExt CTC for HM, LowQP test configuration.

| Test | HDR PQ |  | HDR HLG |  | SVT RGB |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Enc | Dec | Enc | Dec | Enc | Dec |
| AI | 2815% | 172% | 3080% | 169% | 5047% | 128% |
| LDB | 349% | 164% | 454% | 162% | 609% | 137% |
| RA | 414% | 164% | 535% | 159% | 758% | 137% |

Table 1.10. Simulation results for CE Anchor vs. HM16.23, lossless configuration, HBD/HFR CTC for VTM, RExt CTC for HM, LowQP test configuration.

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  |
| PQ444 | 2.5 | 2.5 | 0.89% | 3.1 | 3.0 | 4.80% | 3.1 | 3.0 | 4.47% |
| PQ422 | 2.3 | 2.3 | -0.18% | 2.9 | 2.8 | 3.40% | 2.9 | 2.8 | 3.13% |
| Overall | 2.4 | 2.4 | 0.36% | 3.0 | 2.9 | 4.10% | 3.0 | 2.9 | 3.80% |
| Enc Time[%] | 7206% |  |  | 976% |  |  | 983% |  |  |
| Dec Time[%] | 152% |  |  | 147% |  |  | 149% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  |
| HLG444 | 1.8 | 1.7 | 3.27% | 1.9 | 1.9 | 4.25% | 2.0 | 1.9 | 4.08% |
| HLG422 | 1.7 | 1.6 | 2.37% | 1.9 | 1.9 | 3.66% | 1.9 | 1.9 | 3.49% |
| Overall | 1.7 | 1.7 | 2.82% | 1.9 | 1.9 | 3.96% | 1.9 | 1.9 | 3.79% |
| Enc Time[%] | 7782% |  |  | 817% |  |  | 767% |  |  |
| Dec Time[%] | 132% |  |  | 128% |  |  | 126% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  |
| SVT16 | 1.2 | 1.2 | 1.46% | 1.2 | 1.2 | 1.18% | 1.2 | 1.2 | 1.11% |
| SVT12 | 1.3 | 1.2 | 3.12% | 1.3 | 1.3 | 2.71% | 1.3 | 1.3 | 2.55% |
| Overall | 1.2 | 1.2 | 2.29% | 1.3 | 1.2 | 1.95% | 1.3 | 1.2 | 1.83% |

# Tests on Regular Residual Coding (RRC)

This test category includes 8 tests studying proposed changes in the Regular Residual Coding (RRC) method of VVC for purpose of High Bit Depth Coding.

| Test | Tester | Description Document | Cross checker | Cross-chek document |
| --- | --- | --- | --- | --- |
| CE-1.1 | Dmytro Rusanovskyy (Qualcomm) | JVET-V0052 | Mohammed Golam Sarwer (Alibaba) | JVET-V0156 |
| CE-1.2 | Dmytro Rusanovskyy (Qualcomm) | JVET-V0052 | Tomonori Hashimoto (Sharp) | JVET-V0138 |
| CE-1.3 | Widthrawn |  |  |  |
| CE-1.4 | Dmytro Rusanovskyy (Qualcomm) | JVET-V0052 | Adrian Browne (Sony) | JVET-V0134 |
| CE-1.5 | Adrian Browne (Sony) | JVET-V0050 | Dmytro Rusanovskyy (Qualcomm) | JVET-V0139 |
| CE-1.6 | Adrian Browne (Sony) | JVET-V0050 | Dmytro Rusanovskyy (Qualcomm) | JVET-V0139 |
| CE-1.7 | Tomonori Hashimoto (Sharp) | JVET-V0046 | Dmytro Rusanovskyy (Qualcomm) | JVET-V0140 |
| CE-1.8 | Widthrawn |  |  |  |

## Tests description

## CE-1.1: Method of CE-1.1 from JVET-U0064

In this test, improvements to the first method described in section 3.6 and tested in CE-1.1 of JVET-U0064 to be tested.

## CE-1.2: Method of CE-1.2.a from JVET-U0064

In this test, improvements to the second method described in section 3.6 and tested in CE-1.2 of JVET-U0064 to be tested.

## CE-1.3: History utilization method of JVET-U0064

In this test, improvements to the first history based method described in section 3.7 and earlier studied as CE1.2.b in JVET-U0064 is being tested.

## CE-1.4: Method of CE-1.2.a from JVET-U0064

In this test, improvements to the second history based method described in section 3.7 and earlier studied as CE3.5 in JVET-U0066 is being tested.

After T4 deadline, proponent of the CE1.4 tests identified an encoder issue and proposed a software update to address it. Proposed change is not-normative, encoder only and limited to RRC history update during the RDO process. Updated SW and results were made available in the CE1.4 branch on 03/23/2021. After discussion with the cross-checker and CE participants, it was agreed to include updated SW and results for CE1.4 evaluation.

## CE-1.5: Method of CE-1.5 from JVET-U0057 with parameter set update once per sub-TU

In this test, the method proposed in CE-1.5 of JVET-U0057 is augmented by an SPS level flag which enables / disables the the method.

## CE-1.6: Method of CE-1.5 from JVET-U0057 with parameter set update once per TU

In this test, the method proposed in CE-1.5 of JVET-U0057 is modified to update the parameter set once per TU. In addition the method is augmented by an SPS level flag which enables / disables the the method.

## CE-1.7: Method of CE-1.3 from JVET-U0050.

For this experiment, the method proposed in CE-1.3 in JVET-U0050, it is proposed to use a formula instead of conventional look-up table for RRC, is evaluated.

## CE-1.8: Method of JVET-U0062

For this experiment, method proposed in JVET-U0062, it is proposed to decompose locSumAbs value into shift parameter and index parameter of the conventioan look-up table for RRC, is evaluated.

## Simulation results

Simulation results for the tests in this test category, have been provided for HBD/HBR/HFR CTC in the lowQP, normal QP range and lossless configurations and reported in corresponding JVET input documents. Summary of the results form reporting template is provided below.

Table 2.2. Simulation results for RRC tests, 12 bits data, CE CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | CE1.1 | -0.04% | 0.00% | 0.00% | -0.04% | 0.07% | 0.07% | -0.24% | -0.35% | -0.19% | -0.19% |
|  | CE1.2 | -0.09% | -0.06% | -0.07% | -0.10% | -0.03% | -0.04% | -0.41% | -0.45% | -0.39% | -0.39% |
|  | CE1.3 |  |  |  |  |  |  |  |  |  |  |
|  | CE1.4-A | -0.33% | -0.24% | -0.24% | -0.50% | -0.30% | -0.32% | -1.43% | -1.64% | -1.32% | -1.34% |
|  | CE1.4-B | -0.33% | -0.23% | -0.25% | -0.50% | -0.30% | -0.32% | -1.43% | -1.64% | -1.32% | -1.33% |
|  | CE1.5 | -0.22% | -0.12% | -0.13% | -0.42% | -0.23% | -0.23% | -1.36% | -1.60% | -1.24% | -1.24% |
|  | CE1.6 | -0.22% | -0.11% | -0.12% | -0.41% | -0.22% | -0.22% | -1.34% | -1.58% | -1.21% | -1.22% |
|  | CE1.7 | -0.09% | -0.07% | -0.07% | -0.08% | -0.03% | -0.03% | -0.37% | -0.42% | -0.34% | -0.34% |
|  | CE1.8 |  |  |  |  |  |  |  |  |  |  |
| LDB | CE1.1 | -0.01% | -0.03% | -0.02% | -0.03% | -0.01% | 0.00% | -0.05% | -0.05% | -0.05% | -0.05% |
|  | CE1.2 | 0.03% | 0.03% | 0.02% | 0.02% | 0.05% | 0.06% | -0.19% | -0.20% | -0.19% | -0.19% |
|  | CE1.3 |  |  |  |  |  |  |  |  |  |  |
|  | CE1.4-A | -0.06% | -0.07% | -0.08% | -0.02% | -0.01% | 0.01% | -0.66% | -0.71% | -0.63% | -0.63% |
|  | CE1.4-B | -0.06% | -0.08% | -0.09% | -0.06% | -0.01% | 0.00% | -0.66% | -0.71% | -0.63% | -0.63% |
|  | CE1.5 | -0.02% | -0.03% | -0.06% | -0.03% | -0.01% | -0.01% | -0.62% | -0.69% | -0.58% | -0.58% |
|  | CE1.6 | -0.03% | -0.04% | -0.06% | -0.04% | -0.02% | -0.01% | -0.60% | -0.68% | -0.57% | -0.56% |
|  | CE1.7 | 0.01% | 0.00% | 0.02% | 0.02% | 0.04% | 0.06% | -0.15% | -0.16% | -0.14% | -0.14% |
|  | CE1.8 |  |  |  |  |  |  |  |  |  |  |
| RA | CE1.1 | -0.01% | -0.03% | -0.03% | -0.01% | 0.00% | 0.00% | -0.06% | -0.06% | -0.06% | -0.05% |
|  | CE1.2 | -0.01% | -0.01% | -0.02% | 0.01% | 0.03% | 0.03% | -0.17% | -0.19% | -0.15% | -0.16% |
|  | CE1.3 |  |  |  |  |  |  |  |  |  |  |
|  | CE1.4-A | -0.08% | -0.07% | -0.09% | -0.04% | -0.01% | -0.01% | -0.57% | -0.66% | -0.53% | -0.54% |
|  | CE1.4-B | -0.08% | -0.08% | -0.09% | -0.04% | -0.02% | -0.02% | -0.57% | -0.65% | -0.53% | -0.54% |
|  | CE1.5 | -0.04% | -0.03% | -0.04% | -0.04% | -0.03% | -0.03% | -0.57% | -0.67% | -0.51% | -0.52% |
|  | CE1.6 | -0.03% | -0.03% | -0.04% | -0.04% | -0.02% | -0.02% | -0.55% | -0.66% | -0.50% | -0.50% |
|  | CE1.7 | 0.00% | 0.00% | -0.01% | 0.01% | 0.03% | 0.05% | -0.12% | -0.15% | -0.10% | -0.10% |
|  | CE1.8 |  |  |  |  |  |  |  |  |  |  |

Table 2.3. Simulation results for RRC tests, 16 bits data, CE CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE1.1 | -0.71% | -0.67% | -0.72% | -0.73% |
|  | CE1.2 | -0.79% | -0.79% | -0.79% | -0.79% |
|  | CE1.3 |  |  |  |  |
|  | CE1.4-A | -2.30% | -2.72% | -2.09% | -2.08% |
|  | CE1.4-B | -2.27% | -2.70% | -2.05% | -2.04% |
|  | CE1.5 | -2.27% | -2.80% | -2.02% | -2.00% |
|  | CE1.6 | -2.26% | -2.78% | -2.00% | -1.98% |
|  | CE1.7 | -0.66% | -0.59% | -0.69% | -0.70% |
|  | CE1.8 |  |  |  |  |
| LDB | CE1.1 | -0.58% | -0.53% | -0.61% | -0.62% |
|  | CE1.2 | -0.66% | -0.61% | -0.69% | -0.70% |
|  | CE1.3 |  |  |  |  |
|  | CE1.4-A | -1.55% | -1.51% | -1.58% | -1.57% |
|  | CE1.4-B | -1.53% | -1.49% | -1.55% | -1.54% |
|  | CE1.5 | -1.33% | -1.30% | -1.35% | -1.34% |
|  | CE1.6 | -1.32% | -1.31% | -1.34% | -1.32% |
|  | CE1.7 | -0.73% | -0.67% | -0.75% | -0.76% |
|  | CE1.8 |  |  |  |  |
| RA | CE1.1 | -0.51% | -0.50% | -0.50% | -0.52% |
|  | CE1.2 | -0.60% | -0.61% | -0.58% | -0.60% |
|  | CE1.3 |  |  |  |  |
|  | CE1.4-A | -1.45% | -1.43% | -1.46% | -1.45% |
|  | CE1.4-B | -1.45% | -1.43% | -1.47% | -1.46% |
|  | CE1.5 | -1.23% | -1.24% | -1.24% | -1.23% |
|  | CE1.6 | -1.23% | -1.24% | -1.22% | -1.21% |
|  | CE1.7 | -0.66% | -0.67% | -0.64% | -0.66% |
|  | CE1.8 |  |  |  |  |

Table 2.4. Reported run-time estimates RRC tests, CE CTC, LowQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  | SVT12 RGB |  | SVT16 RGB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec | Enc | Dec | Enc | Dec |
| AI | CE1.1 | 101% | 100% | 100% | 100% | 103% | 103% | 104% | 102% |
|  | CE1.2 | 102% | 102% | 102% | 103% | 102% | 103% | 101% | 99% |
|  | CE1.3 |  |  |  |  |  |  |  |  |
|  | CE1.4-A | 102% | 101% | 103% | 101% | 106% | 108% | 109% | 121% |
|  | CE1.4-B | 103% | 102% | 104% | 103% | 107% | 108% | 109% | 124% |
|  | CE1.5 | 101% | 100% | 103% | 103% | 104% | 109% | 109% | 128% |
|  | CE1.6 | 102% | 100% | 103% | 102% | 105% | 109% | 111% | 128% |
|  | CE1.7 | 101% | 100% | 103% | 100% | 105% | 101% | 109% | 97% |
|  | CE1.8 |  |  |  |  |  |  |  |  |
| LDB | CE1.1 | 100% | 99% | 101% | 98% | 103% | 105% | 104% | 103% |
|  | CE1.2 | 100% | 100% | 103% | 101% | 101% | 101% | 100% | 99% |
|  | CE1.3 |  |  |  |  |  |  |  |  |
|  | CE1.4-A | 103% | 105% | 102% | 101% | 105% | 111% | 117% | 115% |
|  | CE1.4-B | 104% | 108% | 103% | 102% | 106% | 114% | 118% | 119% |
|  | CE1.5 | 101% | 101% | 100% | 101% | 101% | 107% | 117% | 119% |
|  | CE1.6 | 101% | 101% | 101% | 101% | 101% | 105% | 117% | 117% |
|  | CE1.7 | 101% | 100% | 101% | 100% | 103% | 101% | 106% | 99% |
|  | CE1.8 |  |  |  |  |  |  |  |  |
| RA | CE1.1 | 101% | 98% | 101% | 100% | 103% | 103% | 106% | 102% |
|  | CE1.2 | 104% | 103% | 102% | 101% | 102% | 103% | 102% | 99% |
|  | CE1.3 |  |  |  |  |  |  |  |  |
|  | CE1.4-A | 104% | 101% | 103% | 101% | 105% | 108% | 121% | 119% |
|  | CE1.4-B | 103% | 101% | 103% | 102% | 106% | 107% | 120% | 117% |
|  | CE1.5 | 101% | 100% | 101% | 101% | 100% | 106% | 114% | 117% |
|  | CE1.6 | 100% | 99% | 101% | 101% | 101% | 106% | 114% | 116% |
|  | CE1.7 | 101% | 100% | 101% | 100% | 103% | 101% | 106% | 99% |
|  | CE1.8 |  |  |  |  |  |  |  |  |

Table 2.5. Simulation results for RRC tests, HBD/HBR/HFR CTC, NormQP test configuration.

|  | Test | HDR PQ |  |  |  |  | HDR HLG |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DE100 | PSNR-L100 | wY | wU | wV | Y | U | V |
| AI | CE1.1 | -0.07% | -0.01% | -0.02% | 0.07% | -0.07% | -0.01% | -0.04% | 0.03% |
|  | CE1.2 | 0.01% | -0.03% | -0.03% | 0.09% | -0.08% | -0.01% | -0.10% | 0.05% |
|  | CE1.3 |  |  |  |  |  |  |  |  |
|  | CE1.4-A | 0.02% | -0.02% | -0.03% | 0.12% | -0.06% | -0.01% | -0.02% | 0.04% |
|  | CE1.4-B | 0.00% | -0.02% | -0.02% | 0.06% | -0.07% | -0.01% | 0.00% | -0.04% |
|  | CE1.5 | -0.01% | 0.00% | 0.00% | -0.05% | -0.06% | 0.00% | 0.00% | 0.10% |
|  | CE1.6 | -0.01% | 0.00% | 0.00% | -0.05% | -0.06% | 0.00% | 0.00% | 0.10% |
|  | CE1.7 | -0.03% | -0.02% | -0.02% | 0.11% | -0.02% | -0.01% | 0.03% | -0.03% |
|  | CE1.8 |  |  |  |  |  |  |  |  |
| RA | CE1.1 | -0.03% | 0.02% | 0.00% | -0.29% | -0.05% | 0.00% | -0.11% | -0.21% |
|  | CE1.2 | 0.09% | 0.03% | 0.02% | -0.14% | -0.22% | -0.01% | 0.11% | -0.30% |
|  | CE1.3 |  |  |  |  |  |  |  |  |
|  | CE1.4-A | 0.10% | -0.02% | -0.03% | -0.17% | -0.15% | 0.02% | -0.07% | 0.01% |
|  | CE1.4-B | -0.01% | -0.03% | -0.05% | -0.29% | -0.25% | 0.00% | -0.02% | -0.09% |
|  | CE1.5 | 0.25% | -0.01% | -0.03% | 0.12% | 0.20% | 0.02% | 0.10% | 0.14% |
|  | CE1.6 | 0.25% | -0.01% | -0.02% | 0.12% | 0.21% | 0.02% | 0.10% | 0.14% |
|  | CE1.7 | -0.16% | -0.06% | -0.04% | -0.24% | -0.04% | 0.00% | -0.03% | -0.06% |
|  | CE1.8 |  |  |  |  |  |  |  |  |

Table 2.6. Run-time estimates for RRC tests, CE CTC, NormQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec |
| AI | CE1.1 | 100% | 98% | 99% | 97% |
|  | CE1.2 | 100% | 99% | 100% | 101% |
|  | CE1.3 |  |  |  |  |
|  | CE1.4-A | 101% | 96% | 101% | 99% |
|  | CE1.4-B | 101% | 100% | 100% | 97% |
|  | CE1.5 | 98% | 99% | 98% | 99% |
|  | CE1.6 | 98% | 100% | 98% | 99% |
|  | CE1.7 | 101% | 100% | 102% | 100% |
|  | CE1.8 |  |  |  |  |
| RA | CE1.1 | 98% | 98% | 98% | 98% |
|  | CE1.2 | 100% | 101% | 100% | 100% |
|  | CE1.3 |  |  |  |  |
|  | CE1.4-A | 99% | 99% | 98% | 98% |
|  | CE1.4-B | 98% | 99% | 96% | 98% |
|  | CE1.5 | 101% | 101% | 102% | 102% |
|  | CE1.6 | 101% | 101% | 102% | 102% |
|  | CE1.7 | 100% | 100% | 100% | 100% |
|  | CE1.8 |  |  |  |  |

Table 2.7. Simulation results for RRC tests, CE CTC, lossless test configuration.

|  | Test | bit-rate saving |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | HDR PQ | HLG PQ | SVT12 | SVT16 |
| AI | CE1.1 | -0.98% | -1.50% | -1.38% | 0.10% |
|  | CE1.2 | -0.57% | -1.16% | -0.42% | -0.82% |
|  | CE1.3 |  |  |  |  |
|  | CE1.4-A | -2.31% | -3.49% | -2.88% | -1.38% |
|  | CE1.4-B | -2.03% | -3.06% | -2.84% | -1.38% |
|  | CE1.5 | -1.81% | -2.96% | -2.91% | -1.46% |
|  | CE1.6 | -1.82% | -2.97% | -2.92% | -1.47% |
|  | CE1.7 | -1.20% | -1.70% | -1.53% | -0.20% |
|  | CE1.8 |  |  |  |  |
| LDB | CE1.1 | -1.55% | -2.51% | -0.77% | 0.17% |
|  | CE1.2 | -1.03% | -1.78% | -0.59% | -0.52% |
|  | CE1.3 |  |  |  |  |
|  | CE1.4-A | -3.06% | -4.30% | -1.51% | -0.39% |
|  | CE1.4-B | -2.87% | -4.05% | -1.51% | -0.38% |
|  | CE1.5 | -2.29% | -3.36% | -1.56% | -0.40% |
|  | CE1.6 | -2.30% | -3.38% | -1.61% | -0.43% |
|  | CE1.7 | -1.96% | -3.02% | -1.45% | -0.09% |
|  | CE1.8 |  |  |  |  |
| RA | CE1.1 | -1.63% | -2.58% | -0.71% | 0.18% |
|  | CE1.2 | -0.99% | -1.74% | 0.60% | -0.44% |
|  | CE1.3 |  |  |  |  |
|  | CE1.4-A | -2.98% | -4.22% | -1.42% | -0.37% |
|  | CE1.4-B | -2.79% | -3.98% | -1.42% | -0.36% |
|  | CE1.5 | -2.22% | -3.29% | -1.49% | -0.39% |
|  | CE1.6 | -2.23% | -3.31% | -1.53% | -0.42% |
|  | CE1.7 | -2.00% | -3.05% | -1.39% | -0.08% |
|  | CE1.8 |  |  |  |  |

# Tests on Transform Skip Residual Coding (TSRC)

| Test | Proponent(s) | Description Document | Cross checker | Cross-chek document |
| --- | --- | --- | --- | --- |
| CE-2.2 | Sony | JVET-V0050 | Kwai | JVET-V0143 |
| CE-2.3 | Sony | JVET-V0050 | Kwai | JVET-V0143 |

## Test Description

## CE-2.1: Method of JVET-U0075

Method proposed in JVET-U0075, it is proposed to explicitly signal the Rice parameter for each slice to indicate the Rice parameter for the binary codewords of abs_remainder. In the proposed method, one control flag is signaled in sequence parameter set to indicate the Rice parameter used for the TSRC is allowed to be adjusted for each slice separately. When the flag is enabled, another syntax element is further signaled for each slice to indicate the Rice parameter used for the TSRC of that slice. Otherwise (the flag is disabled), no further syntax element is signaled at slice level and the default Rice parameter (e.g. 1) is used for all the slices.

## CE-2.2: Method of CE-2.3 from JVET-U0057 with parameter set update once per sub-TU

In this test, the method proposed in CE-2.3 of JVET-U0057 is augmented by an SPS level flag which enables / disables the the method.

## CE-2.3: Method of CE-2.3 from JVET-U0057 with parameter set update once per TU

In this test, the method proposed in CE-2.3 of JVET-U0057 is modified to update the parameter set once per TU. In addition the method is augmented by an SPS level flag which enables / disables the the method.

## Simulation results

Simulation results for the tests in this test category, have been provided for HBD/HBR/HFR CTC in the lowQP, normal QP range and lossless configurations and reported in corresponding JVET input documents. Summary of the results form reporting template is provided below.

Table 3.2. Simulation results for TSRC tests, 12 bits data, CE CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.RGB | G | B | R |
| AI | CE-2.2 | -0.02% | -0.02% | -0.03% | -0.07% | -0.09% | -0.09% | -0.14% | -0.13% | -0.14% | -0.13% |
|  | CE-2.3 | -0.02% | -0.03% | -0.03% | -0.06% | -0.09% | -0.09% | -0.12% | -0.12% | -0.12% | -0.12% |
| LDB | CE-2.2 | 0.01% | -0.02% | -0.03% | 0.01% | 0.00% | -0.01% | -0.09% | -0.11% | -0.07% | -0.08% |
|  | CE-2.3 | 0.01% | -0.03% | -0.03% | 0.01% | 0.00% | -0.01% | -0.07% | -0.09% | -0.05% | -0.06% |
| RA | CE-2.2 | 0.02% | 0.00% | -0.02% | 0.01% | 0.00% | -0.01% | -0.09% | -0.10% | -0.08% | -0.09% |
|  | CE-2.3 | 0.02% | 0.00% | -0.02% | 0.02% | 0.00% | 0.00% | -0.07% | -0.08% | -0.05% | -0.07% |

Table 3.3. Simulation results for TSRC tests, 16 bits data, CE CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.RGB | G | B | R |
| AI | CE-2.2 | -0.20% | -0.26% | -0.17% | -0.16% |
|  | CE-2.3 | -0.18% | -0.24% | -0.15% | -0.14% |
| LDB | CE-2.2 | -0.14% | -0.14% | -0.14% | -0.12% |
|  | CE-2.3 | -0.08% | -0.10% | -0.08% | -0.07% |
| RA | CE-2.2 | -0.15% | -0.13% | -0.16% | -0.15% |
|  | CE-2.3 | -0.10% | -0.10% | -0.10% | -0.10% |

Table 3.4. Reported run-time estimates TSRC tests, CE CTC, LowQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  | SVT12 RGB |  | SVT16 RGB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec | Enc | Dec | Enc | Dec |
| AI | CE-2.2 | 98% | 99% | 99% | 100% | 99% | 101% | 100% | 100% |
|  | CE-2.3 | 98% | 98% | 98% | 98% | 101% | 100% | 100% | 100% |
| LDB | CE-2.2 | 97% | 97% | 101% | 101% | 101% | 105% | 102% | 105% |
|  | CE-2.3 | 98% | 96% | 99% | 99% | 101% | 104% | 101% | 103% |
| RA | CE-2.2 | 101% | 102% | 100% | 101% | 99% | 103% | 101% | 104% |
|  | CE-2.3 | 101% | 102% | 100% | 100% | 99% | 102% | 100% | 103% |

Table 3.5. Simulation results for TSRC tests, HBD/HBR/HFR CTC, NormQP test configuration.

|  | Test | HDR PQ |  |  |  |  |  |  | HDR HLG |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dE100 | L100 | wY | wU | wV | Enc | Dec | Y | U | V | Enc | Dec |
| AI | CE-2.2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 97% | 98% | 0.00% | 0.00% | 0.00% | 97% | 98% |
|  | CE-2.3 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 99% | 0.00% | 0.00% | 0.00% | 97% | 98% |
| RA | CE-2.2 | -0.03% | -0.02% | -0.02% | -0.02% | -0.04% | 101% | 101% | -0.01% | -0.03% | 0.07% | 102% | 102% |
|  | CE-2.3 | -0.03% | -0.02% | -0.02% | -0.02% | -0.03% | 101% | 101% | -0.01% | -0.03% | 0.07% | 101% | 102% |

Table 2.6. CE2.1/CE2.2/CE2.3 simulation results, 12 bits data, SCC TGM, LowQP test configuration.(Compared to VTM12.0).

|  | Test | SCC TGM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Aver.RGB | G | B | R | Enc | Dec |  |
| AI | CEAnc (CE1.1+CE2.1) | -3.00% | -3.03% | -2.98% | -2.99% | 100% | 98% |  |
|  | CE2.1 encoder fix+CE1.1 | -3.06% | -3.11% | -3.03% | -3.04% | 101% | 96% |  |
|  | CE2.2+CE1.1 | -3.26% | -3.38% | -3.20% | -3.19% | 105% | 99% |  |
|  | CE2.3+CE1.1 | -3.23% | -3.37% | -3.15% | -3.17% | 105% | 99% |  |
| LDB | CEAnc (CE1.1+CE2.1) | -1.16% | -1.23% | -1.11% | -1.14% | 100% | 99% |  |
|  | CE2.1 encoder fix+CE1.1 | -1.54% | -1.46% | -1.57% | -1.58% | 101% | 99% |  |
|  | CE2.2+CE1.1 | -1.76% | -1.76% | -1.76% | -1.75% | 103% | 100% |  |
|  | CE2.3+CE1.1 | -1.68% | -1.41% | -2.25% | -1.40% | 103% | 100% |  |
| RA | CEAnc (CE1.1+CE2.1) | -1.13% | -0.93% | -1.48% | -1.00% | 101% | 99% |  |
|  | CE2.1 encoder fix+CE1.1 | -1.67% | -1.75% | -1.70% | -1.56% | 101% | 99% |  |
|  | CE2.2+CE1.1 | -2.78% | -2.51% | -2.74% | -3.11% | 105% | 101% |  |
|  | CE2.3+CE1.1 | -2.32% | -2.13% | -2.36% | -2.48% | 105% | 101% |  |

# Transform Coefficient Clipping

This category includes tests CE3.1 and CE3.2 that are studing methods proposed in JVET-U0052.

| Test | Proponent(s) | Document description | Cross-checker(s) | Cross-check document |
| --- | --- | --- | --- | --- |
| CE-3.1 | Sharp | JVET-V0047 | Sony | JVET-V0136 |
| CE-3.2 | Sharp | JVET-V0047 | Sony, Qualcomm | JVET-V0136 |

## Tests Description

It has been assessed, that there are two changes compared to the HBD VVC Anchor and HBD CE Anchor being incorporated in these tests:

- Enabling extended precision processing of transform coefficients for 12 bit data coding, using 18 bits instead of 15 bits in the anchor.

- Changes in the derivation of the Log2TransformRange variable (compare to Rext/VVC SW), which affects transform coefficients range and invocation parameters of k-th order Exp-Golomb binarization, clause 9.3.3.6.

The isolated impact of enabling extended precision for 12 bit data (with Rext/VVC SW Log2TransformRange derivation) can be assessed from the simulation results of CE3.1.

The isolated impact of the Log2TransformRange derivation change can be assessed from the 16 bit tests of CE3.1.

Test CE3.2 in addition, introduces changes to Log2TransformRange derivation for both, 12 bit and 16 bit data coding and isolated impact cannot be assessed. This section summary of the results reflecting given above assessment.

## CE-3.1: Method 1 of JVET-U0052

In this test, Method 1 proposed in JVET-U0052, transform coefficients range extension for high bit-depth coding, is tested.

ExtendedPrecisionFlag = extended_precision_processing_flag && bitDepth > 10

Log2TransformRange = ExtendedPrecisionFlag ? Max( 15, Min(20, BitDepthY + 6) ) : 15

In this test, the software base will follow the completion of the initial ce base, that CE-1.1 is used for RRC and CE-2.1 is used for TSRC.

## CE-3.2: Method 2 of JVET-U0052

In this test, Method 2 proposed in JVET-U0052, transform coefficients range extension for high bit-depth coding, is tested.

Log2TransformRange = Max( 15, Min(20, BitDepthY + 5 ) )

In this test, the software base will follow the completion of the initial ce base, that CE-1.1 is used for RRC and CE-2.1 is used for TSRC.

## Simulation results

Simulation results for the tests in Category 3, have been provided for HBD/HBR/HFR CTC in the lowQP, and normal QP range configurations and reported in corresponding JVET input documents. Summary of the results form reporting template is provided below.

Table 4.2. CE3.1/CE3.2 simulation results, 12 bits data, CE CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.RGB | G | B | R |
| AI | CE-3.1 | -1.23% | -1.69% | -1.82% | -0.47% | -0.83% | -0.86% | -1.11% | -0.80% | -1.25% | -1.27% |
|  | CE-3.2 | -1.12% | -1.55% | -1.67% | -0.43% | -0.76% | -0.79% | -0.92% | -0.67% | -1.04% | -1.06% |
| LDB | CE-3.1 | -0.67% | -0.65% | -0.72% | -0.62% | -0.80% | -0.84% | -0.70% | -0.79% | -0.65% | -0.66% |
|  | CE-3.2 | -0.64% | -0.61% | -0.68% | -0.58% | -0.74% | -0.78% | -0.61% | -0.69% | -0.57% | -0.58% |
| RA | CE-3.1 | -0.60% | -0.63% | -0.69% | -0.50% | -0.73% | -0.78% | -0.63% | -0.61% | -0.64% | -0.65% |
|  | CE-3.2 | -0.57% | -0.59% | -0.66% | -0.47% | -0.67% | -0.73% | -0.56% | -0.53% | -0.57% | -0.58% |

Table 4.3. CE3.1/CE3.2 simulation results, 16 bits data, CE CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.RGB | G | B | R |
| AI | CE-3.1 | -6.53% | -6.82% | -6.39% | -6.37% |
|  | CE-3.2 | -0.53% | -0.69% | -0.45% | -0.45% |
| LDB | CE-3.1 | -2.92% | -2.98% | -2.91% | -2.88% |
|  | CE-3.2 | -0.22% | -0.20% | -0.23% | -0.22% |
| RA | CE-3.1 | -2.77% | -2.60% | -2.88% | -2.83% |
|  | CE-3.2 | -0.22% | -0.05% | -0.32% | -0.30% |

Table 4.4. Reported run-time estimates RRC tests, CE CTC, LowQP test configuration.

|  | Test | HDR PQ |  | HDR HLG |  | SVT12 RGB |  | SVT16 RGB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Enc | Dec | Enc | Dec | Enc | Dec | Enc | Dec |
| AI | CE-3.1 | 101% | 94% | 108% | 93% | 95% | 87% | 107% | 98% |
|  | CE-3.2 | 101% | 95% | 107% | 94% | 95% | 90% | 95% | 90% |
| LDB | CE-3.1 | 100% | 98% | 102% | 95% | 94% | 93% | 103% | 100% |
|  | CE-3.2 | 100% | 98% | 101% | 96% | 100% | 93% | 103% | 100% |
| RA | CE-3.1 | 100% | 99% | 102% | 97% | 98% | 96% | 102% | 99% |
|  | CE-3.2 | 100% | 94% | 101% | 90% | 99% | 83% | 102% | 89% |

Table 4.5. CE3.1/CE3.2 simulation results, 12 bits data, HBD/HBR/HFR CTC, NormQP test configuration.

|  | Test | HDR PQ |  |  |  |  |  |  | HDR HLG |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dE100 | L100 | wY | wU | wV | Enc | Dec | Y | U | V | Enc | Dec |
| AI | CE3.1 | 0.02% | 0.02% | 0.01% | 0.04% | -0.01% | 100% | 100% | 0.01% | -0.11% | -0.14% | 102% | 100% |
|  | CE3.2 | -0.07% | -0.01% | -0.01% | 0.12% | 0.01% | 100% | 100% | 0.00% | -0.05% | 0.00% | 101% | 100% |
| RA | CE3.1 | 0.00% | 0.02% | 0.02% | -0.23% | -0.30% | 101% | 100% | 0.01% | -0.06% | 0.20% | 106% | 100% |
|  | CE3.2 | -0.05% | -0.02% | 0.00% | -0.40% | -0.41% | 101% | 99% | 0.03% | -0.23% | 0.15% | 106% | 102% |

To assess performance of the coding with extended precision of transform coefficients (18 bits instead of 15 bits of anchor) for 12 bit data coding, coding gain improvement reported by CE3.1 over different codebase is summarized in the Table 4.6. Test CE3.1 was selected for this summarization, since it does not change Log2TransformRange derivation for 12 bits data compared to the respective anchors (e.g. VTM12.0).

Results of CE3.1-VTM provides coding gain achieved by CE3.1 being integratated in the VTM12.0 and tested against the corresponding VTM12.0 anchor. Results of CE3.1-CE provides coding gain achieved by CE3.1 being integratated in the VTM12.0 and tested against the corresponding VTM12.0 anchor.

Table 4.6. CE3.1 simulation results, 12 bits data, HBD/HBR/HFR CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | R | G | B | Aver.RGB |
| AI | CE3.1-VTM | -1.71% | -2.18% | -2.34% | -0.90% | -1.18% | -1.21% | -3.50% | -3.86% | -3.86% | -3.74% |
|  | CE3.1-CE | -1.23% | -1.69% | -1.82% | -0.47% | -0.83% | -0.86% | -0.80% | -1.25% | -1.27% | -1.11% |
| LDB | CE3.1-VTM | -0.74% | -0.71% | -0.78% | -0.60% | -0.82% | -0.90% | -1.22% | -1.15% | -1.16% | -1.18% |
|  | CE3.1-CE | -0.67% | -0.65% | -0.72% | -0.62% | -0.80% | -0.84% | -0.79% | -0.65% | -0.66% | -0.70% |
| RA | CE3.1-VTM | -0.73% | -0.76% | -0.83% | -0.51% | -0.77% | -0.84% | -1.11% | -1.15% | -1.16% | -1.14% |
|  | CE3.1-CE | -0.60% | -0.63% | -0.69% | -0.50% | -0.73% | -0.78% | -0.61% | -0.64% | -0.65% | -0.63% |

# Combination tests

| Test | Tester(s) |  | Cross-checker(s) |  |
| --- | --- | --- | --- | --- |
| CE-4.1 | Sharp | JVET-V0048 | Kwai | JVET-V0142 |
| CE-4.2 | Sharp | JVET-V0049 | Qualcomm | JVET-V0140 |
| CE-4.3 | Sony | JVET-V0051 | Qualcomm | JVET-V0139 |
| CE-4.4 | Sony | JVET-V0051 | Kwai | JVET-V0144 |
| CE-4.5 | Qualcomm | JVET-V0053 | Sony | JVET-V0135 |
| CE-4.6 | Qualcomm | JVET-V0053 | Sharp | JVET-V0145 |
| CE-4.7 | Qualcomm | JVET-V0053 | Sony | JVET-V0135 |

## Test Description

## CE-4.1: Combination test of CE-1.5, CE-2.1 and method 1 in JVET-U0052

In this test, state base rice paramter method and transform coefficient clipping is tested as follow:

The combination of CE-1.5 for RRC, CE-2.1 for TSRC and the method 1 describes JVET-U0052 will be tested.

This is a additional test to compare with the CE-3.1 which tested method 1 in JVET-U0052 on CE base (CE-1.1 and CE-2.1).

## CE-4.2: Combination test of CE-1.5, CE-2.1 and method 2 in JVET-U0052

In this test, state base rice paramter method and transform coefficient clipping is tested as follow:

The combination of CE-1.5 for RRC, CE-2.1 for TSRC and the method 2 describes JVET-U0052 will be tested.

This is a additional test to compare with the CE-3.2 which tested method 2 in JVET-U0052 on CE base (CE-1.1 and CE-2.1).

## CE-4.3: Combination test of CE-1.6 and CE-3.2

This test combines the RRC Rice coding modifications of CE-1.6 with the transform coefficient range extension modification of CE-3.2. For this test the TSRC method will be CE-2.1 in line with the CE anchor.

## CE-4.4: Combination test of CE-2.3 and CE-3.2

This test combines the TSRC Rice coding modifications of CE-2.3 with the transform coefficient range extension modification of CE-3.2. For this test the RRC method will be CE-1.1 in line with the CE anchor.

## CE-4.5: Combination test of CE-1.1, CE-2.1 and method 1 in JVET-U0052

In this test, state base rice paramter method and transform coefficient clipping is tested as follow:

The optimized integration of CE-1.1 for RRC, CE-2.1 for TSRC and transform coefficient range extension modification of CE-3.2.

## CE-4.6: Combination test of CE-1.2, CE-2.1 and method 1 in JVET-U0052

In this test, state base rice paramter method and transform coefficient clipping is tested as follow:

The optimized integration of CE-1.2 for RRC, CE-2.1 for TSRC and transform coefficient range extension modification of CE-3.2.

## CE-4.7: Combination test of CE-1.4, CE-2.1 and method 1 in JVET-U0052

In this test, state base rice paramter method and transform coefficient clipping is tested as follow:

The optimized integration of CE-1.4 for RRC, CE-2.1 for TSRC and transform coefficient range extension modification of CE-3.2.

## Simulation Results

Table 5.2. Simulation results for combined tests, 12 bits data, CE CTC, LowQP test configuration.

|  | Test | HDR PQ |  |  | HDR HLG |  |  | SVT12 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | wY | wU | wV | Y | U | V | Aver.GBR | G | B | R |
| AI | CE4.1 | -1.42% | -1.80% | -1.93% | -0.89% | -1.01% | -1.04% | -1.81% | -1.86% | -1.77% | -1.79% |
|  | CE4.2 | -1.32% | -1.66% | -1.78% | -0.85% | -0.96% | -0.99% | -1.78% | -1.85% | -1.73% | -1.75% |
|  | CE4.3 | -1.31% | -1.65% | -1.77% | -0.84% | -0.95% | -0.97% | -1.75% | -1.82% | -1.71% | -1.73% |
|  | CE4.4 | -1.14% | -1.57% | -1.68% | -0.47% | -0.81% | -0.83% | -0.93% | -0.68% | -1.05% | -1.07% |
|  | CE4.5-CE3.1 | -1.24% | -1.63% | -1.76% | -0.49% | -0.71% | -0.74% | -1.04% | -0.87% | -1.11% | -1.13% |
|  | CE4.5-CE3.2 | -1.14% | -1.51% | -1.62% | -0.45% | -0.66% | -0.69% | -0.95% | -0.83% | -1.00% | -1.03% |
|  | CE4.6-CE3.1 | -1.30% | -1.72% | -1.86% | -0.54% | -0.85% | -0.88% | -1.40% | -1.16% | -1.51% | -1.54% |
|  | CE4.6-CE3.2 | -1.20% | -1.59% | -1.71% | -0.51% | -0.80% | -0.82% | -1.26% | -1.07% | -1.35% | -1.37% |
|  | CE4.7-A-CE3.1 | -1.52% | -1.89% | -2.02% | -0.95% | -1.07% | -1.10% | -1.93% | -1.89% | -1.94% | -1.97% |
|  | CE4.7-A-CE3.2 | -1.42% | -1.75% | -1.87% | -0.91% | -1.02% | -1.05% | -1.89% | -1.88% | -1.88% | -1.90% |
|  | CE4.7-B-CE3.1 | -1.52% | -1.88% | -2.02% | -0.95% | -1.07% | -1.11% | -1.92% | -1.89% | -1.93% | -1.95% |
|  | CE4.7-B-CE3.2 | -1.42% | -1.74% | -1.88% | -0.91% | -1.02% | -1.05% | -1.88% | -1.88% | -1.87% | -1.89% |
| LDB | CE4.1 | -0.70% | -0.69% | -0.74% | -0.65% | -0.82% | -0.87% | -1.07% | -1.20% | -1.00% | -1.01% |
|  | CE4.2 | -0.66% | -0.65% | -0.72% | -0.61% | -0.77% | -0.81% | -1.04% | -1.16% | -0.98% | -0.99% |
|  | CE4.3 | -0.66% | -0.64% | -0.71% | -0.61% | -0.78% | -0.80% | -1.03% | -1.15% | -0.97% | -0.98% |
|  | CE4.4 | -0.63% | -0.63% | -0.70% | -0.57% | -0.75% | -0.79% | -0.62% | -0.70% | -0.58% | -0.59% |
|  | CE4.5-CE3.1 | -0.69% | -0.67% | -0.73% | -0.63% | -0.81% | -0.84% | -0.79% | -0.88% | -0.74% | -0.75% |
|  | CE4.5-CE3.2 | -0.65% | -0.63% | -0.69% | -0.59% | -0.76% | -0.80% | -0.70% | -0.78% | -0.65% | -0.66% |
|  | CE4.6-CE3.1 | -0.64% | -0.63% | -0.68% | -0.56% | -0.73% | -0.75% | -0.83% | -0.92% | -0.77% | -0.79% |
|  | CE4.6-CE3.2 | -0.62% | -0.59% | -0.65% | -0.53% | -0.69% | -0.72% | -0.76% | -0.84% | -0.71% | -0.72% |
|  | CE4.7-A-CE3.1 | -0.72% | -0.70% | -0.76% | -0.63% | -0.80% | -0.83% | -1.10% | -1.19% | -1.04% | -1.06% |
|  | CE4.7-A-CE3.2 | -0.69% | -0.67% | -0.74% | -0.59% | -0.74% | -0.79% | -1.06% | -1.15% | -1.01% | -1.02% |
|  | CE4.7-B-CE3.1 | -0.72% | -0.70% | -0.75% | -0.63% | -0.78% | -0.83% | -1.09% | -1.18% | -1.04% | -1.05% |
|  | CE4.7-B-CE3.2 | -0.69% | -0.67% | -0.74% | -0.59% | -0.74% | -0.77% | -1.06% | -1.14% | -1.01% | -1.02% |
| RA | CE4.1 | -0.63% | -0.65% | -0.72% | -0.55% | -0.75% | -0.81% | -0.99% | -1.05% | -0.95% | -0.96% |
|  | CE4.2 | -0.60% | -0.62% | -0.68% | -0.51% | -0.70% | -0.85% | -0.96% | -1.02% | -0.92% | -0.94% |
|  | CE4.3 | -0.60% | -0.63% | -0.69% | -0.50% | -0.69% | -0.75% | -0.95% | -1.02% | -0.91% | -0.93% |
|  | CE4.4 | -0.56% | -0.58% | -0.65% | -0.45% | -0.67% | -0.72% | -0.57% | -0.54% | -0.58% | -0.60% |
|  | CE4.5-CE3.1 | -0.61% | -0.64% | -0.71% | -0.51% | -0.73% | -0.80% | -0.73% | -0.71% | -0.74% | -0.75% |
|  | CE4.5-CE3.2 | -0.58% | -0.61% | -0.68% | -0.48% | -0.68% | -0.73% | -0.65% | -0.62% | -0.66% | -0.68% |
|  | CE4.6-CE3.1 | -0.60% | -0.63% | -0.69% | -0.47% | -0.69% | -0.74% | -0.76% | -0.75% | -0.76% | -0.78% |
|  | CE4.6-CE3.2 | -0.57% | -0.60% | -0.67% | -0.44% | -0.64% | -0.68% | -0.70% | -0.69% | -0.70% | -0.72% |
|  | CE4.7-A-CE3.1 | -0.67% | -0.70% | -0.77% | -0.53% | -0.73% | -0.78% | -0.98% | -1.01% | -0.96% | -0.98% |
|  | CE4.7-A-CE3.2 | -0.64% | -0.66% | -0.73% | -0.49% | -0.68% | -0.74% | -0.95% | -0.98% | -0.93% | -0.95% |
|  | CE4.7-B-CE3.1 | -0.68% | -0.69% | -0.76% | -0.53% | -0.73% | -0.79% | -0.98% | -1.01% | -0.96% | -0.98% |
|  | CE4.7-B-CE3.2 | -0.65% | -0.67% | -0.73% | -0.50% | -0.68% | -0.74% | -0.95% | -0.97% | -0.93% | -0.95% |

Table 5.3. Simulation results for combined tests, 16 bits data, CE CTC, LowQP test configuration.

|  | Test | SVT16 RGB |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aver.GBR | G | B | R |
| AI | CE4.1 | -2.45% | -2.96% | -2.21% | -2.18% |
|  | CE4.2 | -2.45% | -2.96% | -2.21% | -2.18% |
|  | CE4.3 | -2.44% | -2.94% | -2.20% | -2.17% |
|  | CE4.4 | -0.63% | -0.80% | -0.54% | -0.54% |
|  | CE4.5-CE3.1 | -1.21% | -1.39% | -1.12% | -1.13% |
|  | CE4.5-CE3.2 | -1.21% | -1.39% | -1.12% | -1.13% |
|  | CE4.6-CE3.1 | -1.24% | -1.40% | -1.16% | -1.17% |
|  | CE4.6-CE3.2 | -1.24% | -1.40% | -1.16% | -1.17% |
|  | CE4.7-A-CE3.1 | -2.48% | -2.90% | -2.27% | -2.26% |
|  | CE4.7-A-CE3.2 | -2.48% | -2.90% | -2.27% | -2.26% |
|  | CE4.7-B-CE3.1 | -2.45% | -2.88% | -2.24% | -2.22% |
|  | CE4.7-B-CE3.2 | -2.45% | -2.88% | -2.24% | -2.22% |
| LDB | CE4.1 | -1.40% | -1.39% | -1.41% | -1.40% |
|  | CE4.2 | -1.40% | -1.39% | -1.41% | -1.40% |
|  | CE4.3 | -1.39% | -1.40% | -1.40% | -1.38% |
|  | CE4.4 | -0.27% | -0.26% | -0.28% | -0.27% |
|  | CE4.5-CE3.1 | -0.80% | -0.78% | -0.82% | -0.81% |
|  | CE4.5-CE3.2 | -0.80% | -0.78% | -0.82% | -0.81% |
|  | CE4.6-CE3.1 | -0.87% | -0.86% | -0.88% | -0.87% |
|  | CE4.6-CE3.2 | -0.87% | -0.86% | -0.88% | -0.87% |
|  | CE4.7-A-CE3.1 | -1.62% | -1.60% | -1.64% | -1.63% |
|  | CE4.7-A-CE3.2 | -1.62% | -1.60% | -1.64% | -1.63% |
|  | CE4.7-B-CE3.1 | -1.60% | -1.57% | -1.62% | -1.60% |
|  | CE4.7-B-CE3.2 | -1.60% | -1.57% | -1.62% | -1.60% |
| RA | CE4.1 | -1.31% | -1.22% | -1.36% | -1.35% |
|  | CE4.2 | -1.31% | -1.22% | -1.36% | -1.35% |
|  | CE4.3 | -1.32% | -1.23% | -1.35% | -1.38% |
|  | CE4.4 | -0.27% | -0.10% | -0.36% | -0.35% |
|  | CE4.5-CE3.1 | -0.77% | -0.59% | -0.86% | -0.85% |
|  | CE4.5-CE3.2 | -0.77% | -0.59% | -0.86% | -0.85% |
|  | CE4.6-CE3.1 | -0.85% | -0.68% | -0.94% | -0.93% |
|  | CE4.6-CE3.2 | -0.85% | -0.68% | -0.94% | -0.93% |
|  | CE4.7-A-CE3.1 | -1.52% | -1.41% | -1.58% | -1.57% |
|  | CE4.7-A-CE3.2 | -1.52% | -1.41% | -1.58% | -1.57% |
|  | CE4.7-B-CE3.1 | -1.53% | -1.41% | -1.59% | -1.58% |
|  | CE4.7-B-CE3.2 | -1.53% | -1.41% | -1.59% | -1.58% |

# Crosschecker report

Table 7.1. Summary of cross-check reports

| Test | Crosschecker | Crosscheck report |
| --- | --- | --- |
| CE-1.1 | Alibaba | results: Cross-check results match with proponent’s results code verification: Consistent with CE description. draft text verification: further comments: |
| CE-1.2 | Sharp | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description. draft text verification: Same as the software further comments: |
| CE-1.4 | Sony | results: Cross-check results match code verification: Consistent with CE description draft text verification: Consistent with software further comments: |
| CE-1.5 | Qualcomm | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description draft text verification: Consistent with SW and CE description. further comments: |
| CE-1.6 | Qualcomm | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description draft text verification: Consistent with SW and CE description. further comments: |
| CE-1.7 | Qualcomm | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description draft text verification: Consistent with SW and CE description. |
| CE-2.1 | Qualcomm | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description draft text verification: Consistent with SW and CE description. further comments: |
| CE-2.2 | Kwai | results: Cross-check results match code verification: Consistent with CE description draft text verification: Consistent with software further comments: |
| CE-2.3 | Kwai | results: Cross-check results match code verification: Consistent with CE description draft text verification: Consistent with software further comments: |
| CE-3.1 | Sony | results: Cross-check results match code verification: Consistent with CE description draft text verification: Consistent with software further comments: |
| CE-3.2 | Sony, Qualcomm | results: Cross-check results match code verification: Consistent with CE description draft text verification: Consistent with software further comments: |
| CE-4.1 | Kwai | results: Cross-check results match code verification: Consistent with CE description draft text verification: Consistent with CE description further comments: |
| CE-4.2 | Qualcomm | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description draft text verification: Consistent with SW and CE description. further comments: |
| CE-4.3 | Qualcomm | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description draft text verification: Consistent with SW and CE description. further comments: |
| CE-4.4 | Kwai | results: Cross-check results match code verification: Consistent with CE description draft text verification: Consistent with software further comments: |
| CE-4.5 | Sony | results: Cross-check results match code verification: Consistent with CE description draft text verification: Consistent with software further comments: |
| CE-4.6 | Sharp | results: Cross-check results completely match proponent’s. code verification: Consistent with CE description. draft text verification: Same as the software further comments: |
| CE-4.7 | Sony | results: Cross-check results match code verification: Consistent with CE description draft text verification: Consistent with software further comments: |

# Complexity analysis and memory requirements

## CE-1

| Test | Complexity increase vs. VVC, per TU4x4. | Memory requirement | Dependencies |
| --- | --- | --- | --- |
| CE-1.1 | 32 | 1 LUT of 4 entries 1 LUT of 5 entries |  |
| CE-1.2 | 48 |  |  |
| CE1.4 | 68 (Total): - 48 in critical pass, - 20 in non-critical pass. | 3 variables (3x4 bits). | Cross-TU dependency. CE1.4-A: No cross-CTU. CE1.4-B: Cross-CTU. |
| CE1.5 | 95 (Total) - 32 in critical pass - 63 in non-critical pass | 8 variables (8 * 10 bits) 6 * 16 * 2 bits LUT | Cross-subTU Cross-TU Cross-CTU |
| CE1.6 | 95 (Total) - 32 in critical pass - 63 in non-critical pass | 8 variables (8 * 10 bits) 6 * 16 * 2 bits LUT | Cross-TU Cross-CTU |
| CE1.7 | 48 | No | No |

## CE-2

| Test | Complexity increase vs. VVC, per TU4x4. | Memory requirement | Dependencies |
| --- | --- | --- | --- |
| CE-2.1 | 1 condition and 1 add per slice | No memory increase | No Dependency |
| CE-2.2 | 61 (Total) - 16 in critical pass - 45 in non-critical pass | 2 variables (2 * 10 bits) | Cross-subTU Cross-TU Cross-CTU |
| CE-2.3 | 61 (Total) - 16 in critical pass - 45 in non-critical pass | 2 variables (2 * 10 bits) | Cross-TU Cross-CTU |

## CE-3

Proposed to review during the meeting.

## CE-4

Proposed to review during the meeting.

# CE-related

There are 4 contributions submitted to V-meeting are marked as CE-related. Three (3) of these contributions (JVET-V0084, JVET-V0085, JVET-V0106) propose either additional test results for RRC/TSRC methods currently being tested in CE or propose modifications and improvements on top of technologies currently tested in CE. Some of the related document address problem of the screen content coding.

JVET-V0084, CE-related: On Rice parameter selection for regular residual coding (RRC) at high bit depths,

A. Browne, S. Keating, K. Sharman (Sony).

This contribution describes modifications to the method for the selection of Rice parameters for regular residual coding, for consideration in VVC version 2. The modifications offer BD-rate gains when residual coefficient values become larger but do not affect version 1 operating points.  It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths. A further modification is described which alters the template function used to calculate a clipped sum of neighbouring coefficients used during Rice parameter evaluation. Results are provided both with and without this further modification.

JVET-V0085, CE-related: On Rice parameter selection for transform skip residual coding (TSRC) at high bit depths, A. Browne, S. Keating, K. Sharman (Sony).

This contribution describes modifications to the method for the selection of Rice parameters for transform skip residual coding, for consideration in VVC version 2. The modifications offer BD-rate gains, particularly for screen content, when residual coefficient values become larger but do not affect version 1 operating points. It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths.

JVET-V0106 CE-related: On history-enhanced method of Rice parameter derivation for regular residual coding (RRC) at high bit depths, D. Rusanovskyy, L. Pham Van, M. Coban, M. Karczewicz (Qualcomm)

This contribution describes a history-based extension of Rice parameters derivation for regular residual coding (RRC), for VVCv2 development. It is advocated that a local (template based) methods of Rice parameter derivation in RRC, can be further improved by enabling history (across TUs) utilization during the template computation. One example of such history-based extension is currently being studied in CE1.4, where local method of CE1.2 is being extended with a history usage. This proposal present additional example of history-based extension for local CE1.1 derivation and present improvement provided by the said extension compare to the local methods.

It is asserted by the proponent, that proposed history-enhancement for local RRC derivation provides a complexity and dependency scalable RRC parameter derivation method with each component (of the method) providing a gain in all tested class. Method is proposed for adoption to VVCv2.

JVET-V0123 CE-related: On prefix code length of remaining level coding for high bit depth and high bit rate coding	Y. Yu, Z. Xie, F. Wang, H. Yu, D. Wang (OPPO)

This contribution proposes a modification to the prefix code length of remaining level coding for high bit-depth and high bit-rate coding. A smaller fixed value equal to 1 or 4 is proposed to be used in the VTM12 or on the top of CE anchor. Simulations results show that there are XX% gain for XX. It is suggested that a smaller fixed number of prefix code length could be used for 16-bit video while retaining the current prefix code length unchanged for 12-bit input video.

# Input contributions for CE

JVET-V0046, “CE-1.7: Rice parameter derivation for high bit-depth coding”, T. Hashimoto, T. Ikai (Sharp)

JVET-V0047, “CE-3.1 and CE-3.2: Transform coefficients range extension for high bit-depth coding”, T. Zhou, T. Chujoh, T. Ikai (Sharp)

JVET-V0048, “CE-4.1: Combination of CE-3.1, CE-1.5 and CE-2.1”, T. Zhou, T. Chujoh, T. Ikai (Sharp)

JVET-V0049, “CE-4.2: Combination of CE-3.2, CE-1.5 and CE-2.1”, T. Zhou, T. Chujoh, T. Ikai (Sharp)

JVET-V0050, “CE-1.5, CE-1.6, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths”, A. Browne, S. Keating, K. Sharman (Sony)

JVET-V0051, “CE-4.3 and CE-4.4: Combinations of CE-1.6 and CE-2.3 with CE-3.2”, A. Browne, S. Keating, K. Sharman (Sony)

JVET-V0052, “CE-1.1, CE-1.2 and CE-1.4: On the Rice parameter derivation for high bit-depth coding”, D. Rusanovskyy, M. Karczewicz, L. P. Van, M. Coban (Qualcomm)

JVET-V0053, “CE-4.5, CE-4.6 and CE-4.7: Combinations of RRC tests CE-1.1, CE-1.2 and CE-1.4 with CE-3.1/CE-3.2”, D. Rusanovskyy, M. Karczewicz, L. P. Van, M. Coban (Qualcomm)

JVET-V0054, “CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang (Kwai Inc.)

JVET-V0134, “Crosscheck of CE-1.4 from JVET-V0052 (CE-1.1, CE-1.2 and CE-1.4: On the Rice parameter derivation for high bit-depth coding)”, A. Browne (Sony)

JVET-V0135, “Crosscheck of CE-4.5 and CE-4.7 from JVET-V0053 (CE-4.5, CE-4.6 and CE-4.7: Combinations of RRC tests CE-1.1, CE-1.2 and CE-1.4 with CE-3.1/CE-3.2)”, A. Browne (Sony)

JVET-V0136, “Crosscheck of JVET-V0047 (CE-3.1 and CE-3.2: Transform coefficients range extension for high bit-depth coding)”, A. Browne (Sony)

JVET-V0138, “Crosscheck of CE-1.2 from JVET-V0052 (CE-1.1, CE-1.2 and CE-1.4: On the Rice parameter derivation for high bit-depth coding)”, T. Hashimoto (Sharp)

JVET-V0139, “Cross-check report for CE1.5 and CE1.6 of JVET-V0050 and CE4.3 of JVET-V0051”, D.Rusanovskyy (Qualcomm)

JVET-V0140, “Cross-check report for CE-1.7 of JVET-V0046 and CE-4.2 of JVET-V0049”, D.Rusanovskyy (Qualcomm)

JVET-V0142, “Crosscheck of JVET-V0048 (CE-4.1: Combination of CE-3.1, CE-1.5 and CE-2.1)”, H.-J. Jhu (Kwai Inc.)

JVET-V0143, “Crosscheck of CE-2.2 and CE-2.3 from JVET-V0050 (CE-1.5, CE-1.6, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths)”, H.-J. Jhu (Kwai Inc.)

JVET-V0144, “Crosscheck of CE-4.4 from JVET-V0051 (CE-4.3 and CE-4.4: Combinations of CE-1.6 and CE-2.3 with CE-3.2)”, H.-J. Jhu (Kwai Inc.)

JVET-V0145, “Crosscheck of CE-4.6 from JVET-V0053 (CE-4.5, CE-4.6 and CE-4.7: Combinations of RRC tests CE-1.1, CE-1.2 and CE-1.4 with CE-3.1/CE-3.2)”, T. Zhou (Sharp)

JVET-V0155, “Crosscheck of CE-1.1 from JVET-V0052 (CE-1.1, CE-1.2 and CE-1.4: On the Rice parameter derivation for high bit-depth coding)”, M. G. Sarwer (Alibaba)

JVET-V0084, “CE-related: On Rice parameter selection for regular residual coding (RRC) at high bit depths”, A. Browne, S. Keating, K. Sharman (Sony).

JVET-V0085, “CE-related: On Rice parameter selection for transform skip residual coding (TSRC) at high bit depths”, A. Browne, S. Keating, K. Sharman (Sony).

JVET-V0106, “CE-related: On history-enhanced method of Rice parameter derivation for regular residual coding (RRC) at high bit depths”, D. Rusanovskyy, L. Pham Van, M. Coban, M. Karczewicz (Qualcomm)

JVET-V0123, “CE-related: On prefix code length of remaining level coding for high bit depth and high bit rate coding”, Y. Yu, Z. Xie, F. Wang, H. Yu, D. Wang (OPPO)