| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0023 |
| --- | --- |

| Title: | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. Ma and H. Wang | Email: | elena.alshina@huawei.com, shanl@tencent.com, chenwei06@kwai.com, yue.li@bytedance.com, ruling.lrl@alibaba-inc.com, mazhan@nju.edu.cn, hongtaow@qti.qualcomm.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report for the Exploration  Experiments on on Neural Network-based Video Coding. In total 6 tests have been completed within this EE between the JVET U and V meetings to study and evaluate NNVC technologies, analyze their performance and complexity aspects.

There are 2 cathegories of tests in this EE: in-loop filetrs and superresolution-like technologies.   Algorithms with different level of complexity were demonstrated in both categories.

First attempts to cross-check test presented in EE, by not just running code and checking performance, but also verifying complexity assessment were taken. Several aspects which are not absolutely clear in NN CTC (for example, MAC/pixel computation) have been clarified trhough cross-check activity.

As part of the preparation to JVET-V meeting viewing was prepared in coordination with AG05. It was noticed that the length of 4K sequences in NN CTC is 5 sec in RA and only 3 sec in LD-B/P tests, and it was asserted this is too short for visual tests.

# Exploration Experiment on NN-based in-loop filters

Proposals in this category were tested compared to VTM11.0, QP=22,..., 42. High level summary of results is shown in Table 1.

Table 1. Test results in NN-based in-loop filters cathegory.

| # | Performance | Dec.T. | Features | #params | kMAC /pxl |
| --- | --- | --- | --- | --- | --- |
| JVET-V0114 | RA:7.2%(Y); 13% (Ch) RA:7.5%(Y); 14% (Ch) | ×137 ×146 | Input 10 planes: 4*Y+U+V+QP+3*Bsinfo, 72×72, #channels K=96 Residual scalling (per color, per picture); Supplement: Refining training improves performance | 4×1M | 319 |
| JVET-V0115 | RA:8.3%(Y); 20% (Ch) RA:8.5%(Y); 22% (Ch) | ×337 ×344 | Same, as JVET-V0114, but with LongActivation = CONV1×1,K×M + LeakyReLu+CONV1×1,M×K ; M=216> K=72 Supplement: Refining training improves performance | 8×1M | 624 |
| JVET-V0137 | RA:1.2%(Y); 4% (Ch) | ×50 | Input YUV444, 4 planes + QP map, 64×64, DepthwiseSeparableConvolutions (DSC) in 4 Residual Blocks | 23K | 23 |

For the proposal which reports BD-rate computed based on those 2 quality metrics (both PSNR and MS-SSIM) performance difference was ±0.15% close. Interesting model refining hint, which provides ~0.3% compression performance improvement (likely to be useful for other proponents), was demonstrated by Qualcomm.

For the simplicity of comparison NN structures for all 3 tests in this cathegory are shown on Fig. 1-3.

Fig. 1. "NN-based de-block" from JVET-V0114.

Fig. 2. "NN-based de-block" with long activation function from JVET-V0115.

Fig. 3. Low-complexity NN-based in-loop filter with DSC from JVET-V0137.

For the training proponents used DIV2K and BVI-DVC.

# Exploration Experiment on superresolution-like techniques

Proposals in this category were tested compared to VTM11.0, QP=27,..., 47. High level summary of results is shown in Table 2.

Table 2. Test results of superresolution-like technologies in EE1.

| # | Performance | Dec.T. | Features | #params | kMAC /pxl |
| --- | --- | --- | --- | --- | --- |
| JVET-V0073 | RA(4K):2.5%(Y), ~ 30% drop (Ch) RA(4K):5.8%(Y), ~ 20% drop (Ch) RA(4K):6.5%(Y), ~ 10% drop (Ch) | ×0.3 ×1.5 ×1.5 | Test0: RPR2 (VVC) Test1: RPR2 (VVC) CNN const.param Test2: RPR2 (VVC) CNN signalled param NCTM5.0 used fo CNN param compression; Simplified ESRGAN | no info (483.6K) | no info (122) |
| JVET-V0096 | RA(4K):8.1%(Y), ~20% drop (Ch) AI(4K): 11% (Y), ~24% drop (Ch) | ×0.3 ×0.4 | RPR2 (VVC) CNN const.param Simplified EDSR | 2.7M (21.3M) | 344 |
| JVET-V0149 | RA(4K):9.6%(Y), ~3% drop (Ch) LD(4K):18%(Y), ~25% drop (Ch) | no info | DCS-based Video Coding: Intra = spatial texture frames are coded in orignal resolution, Inter = temporal motion frames frames are coded 2 , synthesis with Motion Compensaiton & Texture Transfer Networks | no info | no info |

Technologies in this category typically provide compression gain (measured in Y-PSNR) for 4K videos in NN CTC (classes A1, A2, H2). Authors of JVET-V0149 have managed to achieve gain for classes B and E as well. For most of the cases Luma gain is accomplished by loss in Chroma.

By just encoding 4K content from NN CTC “in quarter resolution” (donsampling 2, coding with VVC, upsamling with VVC filetrs designed for RPR2) one gets 2.5% gain. By changing re-sampling RPR2 filters to NN-based off-line trained ones (which also operate as enhancement filters), compression gain in a range 6% ... 8% (JVET-V0073, JVET-V0096) can be achieved. If additionally NN-filter parameters are tunned for concrete video sequences and included to the bit-stream (JVET-V0073) additionally ~1% compression gain can be achieved.

If Intra frames are coded in full resolution, Inter-frames are encoded in quarter resolution and Motion Compensaiton Network with Texture Transfer Network are used for full resolution reconstruction synthesis (JVET-V0149) gain of almost 10% (in average for 4K content in RA) can be achieved.

For the training mot fo the proponents in this category used BVI-DVC.

# Viewing for test materials in EE1

Candidates for viewing were suggested by EE proponents, discussed during AhG11 telco and made available via JVET ftp. All proponents selected 4K sequences to illustrate benefits of their proposals.

It was noticed that 4K sequences in NN CTC are coded as 5 sec video in RA configuration (Tango is even shorter). Low-delay configuration in NN CTC is optional for 4K sequences and only 3 sec of 4K video are encoded. Short length of video sequences definitely not sufficient for the comprehensive viewing. 5 sec can be used for viewing, 3 sec as asserted by AG05 is too short. It might be reasonable to equalize number of encoded frames in RA and LD configurations for 4K videos in NN CTC.

Bit-rate difference between anchor (VTM) and proposal is shown in Table 3. In NN-based in-loop filter category the difference is within 2%. In super-resolution cathegory difference is up to 10%. Note that for files name on ftp use JVET-U documents numbers were used.

Table. 3. Sequences selected for viewing.

| VTM11.0 | QP | kbps | Proposal # | QP | kbps | diff. % |
| --- | --- | --- | --- | --- | --- | --- |
| Campfire RA QP37 | 37 | 3492.338 | JVET-U0061 (Tencent) | 37 | 3509.302 | 0% |
| Campfire RA QP42 | 42 | 1643.751 | JVET-V0137 | 42 | 1652.949 | 1% |
| Campfire RA | 42 | 1643.751 | JVET-U0104 (Qualcomm) | 42 | 1645.79 | 0% |
| DayLightRoad_LB | 42 | 1092.04 | JVET-V0115 | 42 | 1086.296 | -1% |
| FoodMarket_LB | 42 | 1009.283 |  | 42 | 985.357 | -2% |
| Campfire RA | 42 | 1643.751 | JVET-U0115 (Qualcomm) | 42 | 1640.715 | 0% |
| DayLightRoad_LB | 42 | 1092.04 |  | 42 | 1081.968 | -1% |
| FoodMarket_LB | 42 | 1009.283 | JVET-V0114 | 42 | 984.477 | -2% |
| Tango RA | 44 | 898.2073 | JVET-U0053 (Sharp) | 37 | 924.7612 | 3% |
| CatRobot RA | 44 | 922.8992 | JVET-V0073 | 37 | 986.4184 | 7% |
| Campfire RA | 44 | 1210.127 | JVET-U0099 (Qualcomm) | 42 | 1331.5 | 10% |
| FoodMarket4 RA | 44 | 851.0048 | JVET-V0096 | 42 | 921.43 | 8% |
| Tango RA | 44 | 898.2073 |  | 42 | 925.31 | 3% |
| Campfire RA | 44 | 1210.127 | VTM11.0 RPR2 | 37 | 1323.618 | 9% |
| FoodMarket4 RA | 44 | 851.0048 |  | 37 | 915.4704 | 8% |
| Tango RA | 44 | 898.2073 |  | 37 | 922.3788 | 3% |
| CatRobot RA | 44 | 922.8992 |  | 37 | 983.9776 | 7% |

Viewing is supposed to be conducted during JVET-V meeting.

# Recommendation

- Conduct viewing of prepared video coded with NN-based technologies,

- Review EE contributions for more details,

- Discuss kMAC/pxl measurement procedure during BoG and clarify it in NN CTC document,

- Equalize number of coded frames in RA and LDB, LDP configuration in NN CTC.