| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0043 |
| --- | --- |

| Title: | AHG4: Status Report on HDR Video Verification Test Preparation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall Mathias Wien Vittorio Baroncini Kenneth Andersson | Tel: Email: | asegall@sharplabs.com wien@lfb.rwth-aachen.de baroncini@gmx.com kenneth.r.andersson@ericsson.com |
| Source: | AHG4 Coordinators |  |  |

_____________________________

# Abstract

This document summarizes the status of test sequence selection and rate point determination for the HDR VVC verification test category since the last JVET meeting.  Areas of progress include:

- Selection of additional HDR candidate sequences suitable for use in the verification test

- Selection of QP points for these sequences for use in a dry-run of the test

- Receipt of additional HDR candidates.

With the identification of the additional HDR candidate sequences, the number of HDR sequence candidates for the verification test has increased from 5 sequences to 10 sequences.

# Introduction

Preparation for the HDR verification test has made strong progress on identifying additional sequences for testing.  At the last meeting, five sequences had been identified as suitable from all of the content available to JVET at the time with all remaining sequences rejected.  Since then, an additional nine sequences have been identified and evaluated.  Four of those sequences have been identified as being suitable for further study in a dry-run; two of the sequences have been rejected.  The final three sequences have recently been received and are being converted for coding.

# Test Sequence Selection

Based on expert viewing, the following set of sequences have been identified and are under consideration for evaluation in the verification tests.

### HDR HLG test sequences

A set of HDR-HLG candidate test sequences was visually evaluated in multiple online meeting together with JVET experts.  The evaluated sequences were:

Table 1 – HDR HLG test sequences under consideration

| Test sequence | Resolution | fps | Frames | Transfer Function |
| --- | --- | --- | --- | --- |
| H3_AMS01_3840x2160_10bit_420_HLG (TramBridge) | 3840×2160 | 60 | 600 | HLG |
| H3_AMS06_3840x2160_10bit_420_HLG (CanalWithTrees) | 3840×2160 | 60 | 600 | HLG |
| H3_AMS11_3840x2160_10bit_420_HLG (CityView) | 3840×2160 | 60 | 600 | HLG |
| RiverByBoat_3840x2160_60fps_10bit_HLG_420_type2 | 3840×2160 | 60 | 500 | HLG |

A selection of proposed QP points was generated and reviewed in a session of experts. The proposed QP settings are listed in Table 2 below.

Table 2 – Proposed QPs for HDR HLG test sequences under consideration

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| H3_AMS01_3840x2160_10bit_420_HLG (TramOnBridge) | 39, 36, 32, 28, 24 | 42, 38, 34, 30, 26 |
| H3_AMS06_3840x2160_10bit_420_HLG (CanalWithTrees) | 40, 36, 32, 28, 24 | 42, 38, 34, 30, 26 |
| H3_AMS11_3840x2160_10bit_420_HLG (CityView) | 39, 35, 31, 27, 23 | 42, 38, 34, 30, 26 |
| RiverByBoat_3840x2160_60fps_10bit_HLG_420_type2 | 42, 38, 34, 30, 26 | 45, 41, 37, 33, 28 |

### HDR PQ test sequences

A set of HDR-PQ candidate test sequences was visually evaluated in multiple online meeting together with JVET experts.  The evaluated sequences were:

Table 9 – HDR PQ test sequences under consideration

| Test sequence | Resolution | fps | Frames |
| --- | --- | --- | --- |
| Chimera HDR7 (Dog) | 3840×2160 | 60 | 600 |
| Meridian HDR2 (Car) | 3840×2160 | 60 | 600 |
| Meridian HDR5 (Beach) | 3840×2160 | 60 | 600 |
| Sparks DirtLot | 4096x2160 | 60 | 600 |
| Sparts TruckPack | 4096x2160 | 60 | 554 |
| Sparks Welding | 4096x2160 | 60 | 600 |

A selection of proposed QP points was generated and reviewed in a session of experts. The proposed QP settings are listed in Table 8 below.

Table 10 – Proposed QPs for HDR PQ test sequences under consideration

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| Chimera HDR7 (Dog) | 44, 40, 36, 32, 28, 24 | 46, 42, 38, 34, 30, 26 |
| Meridian HDR2 (Car) | 43, 39, 35, 31, 27, 24 | 46, 42, 38, 34, 30, 26 |
| Meridian HDR5 (Beach) | 44, 40, 36, 32, 28, 24 | 46, 42, 38, 34, 30, 26 |
| Sparks DirtLot | 35, 31, 27, 23, 20 | 38, 34, 30, 26, 22 |
| Sparts TruckPack | 35, 31, 27, 22, 20 | 38, 34, 30, 26, 22 |
| Sparks Welding | 35, 32, 28, 24, 20 | 38, 34, 30, 26, 22 |

# Additional Content

In addition to the content discussed above, AHG4 has received three additional test sequences that are described in ITU-R BT.2245-9.  These sequences were selected by experts participating in the HDR verification test evaluations, and the sequences were delivered in a BT-2100 container using an HLG transfer function.  The sequences include Drama, Swim Race Backstrock and Paddock.  Examples of the sequences are shown below.

The sequences are currently being converted to YCbCr 4:2:0 and HM and VTM bit-streams are being generated.

Sequence 7 - Drama

Sequence 11 – Swim Race Backstroke

Sequence 16 - Paddock

# Additional Comments and Counter Measures

### Sequence Targets

An unofficial target of the AHG4 activity is to identify five sequences in both the HLG and PQ categories for inclusion in the verification test.  Currently, four HLG sequences and six PQ sequences have been identified, while an additional three HLG sequences have been procured and are now under evaluation.

In the event that the three HLG sequences do not result in another sequence, it is planned to convert one of the PQ sequences to HLG.  Since the PQ sequences are mastered to 1,000-nits, this conversion would not require a tone mapping operation and would rather be a near-lossless conversion.

### Software issues

During generation of the HM bit-streams, a bug has been identified in HM-16.22.  The bug results in a mis-match between parallel and sequential encodings when processing the PQ content.  (This is because the PQ content uses luma and chroma QP adaptation, which is not employed when coding HLG.)

A patch has been submitted and integrated into HM-16.23.

# Conclusions

It is suggested to proceed towards a dry-run evaluation for the PQ test category when possible due to the constraints of the COVID pandemic.  Furthermore, it is suggested to continue evaluation of available HLG test content to identify a fifth candidate for the verification test.  Once complete, it is also recommended to proceed towards a dry-run for the HLG test category.

# References

- M. Wien, V. Baroncini, A. Segall, and Y. Ye, “VVC verification test plan (Draft 5),” Doc. JVET-U2021, Joint Video Experts Team of VCEG and MPEG, 21st meeting, Jan. 2021.