| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0049 |
| --- | --- |

| Title: | CE-4.2: Combination of CE-3.2, CE-1.5 and CE-2.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-50-5213-6338 zhou.tianyang@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This document reports results for CE-4.2, a combination of CE-1.5 (RRC), CE2-1 (TSRC) and CE-3.2 (transform coefficients range extension, bitDepth + 5 without extension flag).

The experimental results are as follow:

Low QP

- 1.43 %, 0.67 % and 0.60 % in bdrateY in PQ, HLG and SVT in AI

- 0.85 %, 0.61 % and 0.51 % in bdrateY in PQ, HLG and SVT in LDB

- 2.40 %, 1.27 % and 1.12 % in bdrateY in PQ, HLG and SVT in RA

Test description

The aim of combining test is to produce a more comprehensive method for Rice parameter selection for high bit depth coding in VVC version 2.

CE-3.2

It is proposed to derive Log2TransformRange, bdShift1 (bdShift in 8.7.3	Scaling process for transform coefficients), bdShift2 (bdShift in 8.7.4	Transformation process for scaled transform coefficients), truncSuffixLen and maxPreExtLen as follows.

- Log2TransformRange = Max( 15, Min(20, BitDepthY + 5 ) )

- bdShift1 = BitDepthY + rectNonTsFlag +	( ( Log2 ( nTbW ) + Log2 ( nTbH ) ) / 2 ) + 10 – log2TransformRange+ sh_dep_quant_used_flag

- bdShift2 = ( nTbH > 1  &&  nTbW > 1 ) ? 5 + Log2TransformRange – BitDepth : 6 + Log2TransformRange – BitDepth

- truncSuffixLen = Log2TransformRange

- maxPreExtLen = 32 – 6 – truncSuffixLen = 26 – Log2TransformRange

The CE-3.2 derives without ExtendedPrecisionFlag.

Log2TransformRange = Max( 15, Min(20, BitDepthY + 5 ) )

CE-1.5 and CE-2.1

CE-1.5 proposes a modification to the Rice parameter selection method for RRC

CE-2.1 proposes a modification to the Rice parameter selection method for TSRC. Experiment

Experiment

●CE-3.2 + CE-1.5 + CE-2.1 vs cebase (CE-1.1 + CE-2.1)

Anchor: ExtendedPrecision = 0 in 12bit case, ExtendedPrecision = 1 in 16bit case, GolombRiceParameterAdaptation=1, Log2TransformRange = 15

Test: ExtendedPrecision = 1 in both 12bit and 16bit cases, GolombRiceParameterAdaptation=1, Log2TransformRange = Min (20, BitDepth + 6)

Low QP

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -1.41% | -1.89% | -2.02% | -1.43% | -1.91% | -2.04% | 104% | 96% |
| PQ422 | -1.23% | -1.43% | -1.54% | -1.25% | -1.43% | -1.54% | 104% | 97% |
| Overall | -1.32% | -1.66% | -1.78% | -1.34% | -1.67% | -1.79% | 104% | 96% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.69% | -0.73% | -0.81% | -0.70% | -0.74% | -0.82% | 103% | 99% |
| PQ422 | -0.64% | -0.56% | -0.63% | -0.64% | -0.56% | -0.62% | 103% | 99% |
| Overall | -0.66% | -0.65% | -0.72% | -0.67% | -0.65% | -0.72% | 103% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.62% | -0.69% | -0.78% | -0.62% | -0.68% | -0.77% | 105% | 96% |
| PQ422 | -0.58% | -0.55% | -0.59% | -0.58% | -0.55% | -0.59% | 104% | 96% |
| Overall | -0.60% | -0.62% | -0.68% | -0.60% | -0.61% | -0.68% | 104% | 96% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.88% | -1.01% | -1.03% | 114% | 96% |
| HLG422 | -0.81% | -0.92% | -0.95% | 114% | 98% |
| Overall | -0.85% | -0.96% | -0.99% | 114% | 97% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.66% | -0.88% | -0.94% | 106% | 96% |
| HLG422 | -0.57% | -0.66% | -0.69% | 105% | 97% |
| Overall | -0.61% | -0.77% | -0.81% | 105% | 97% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.55% | -0.79% | -0.85% | 104% | 91% |
| HLG422 | -0.46% | -0.61% | -0.67% | 105% | 93% |
| Overall | -0.51% | -0.70% | -0.76% | 105% | 92% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -2.96% | -2.21% | -2.18% | 121% | 128% |
| SVT12 | -1.85% | -1.73% | -1.75% | 105% | 103% |
| Overall | -2.40% | -1.97% | -1.97% | 113% | 116% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.39% | -1.41% | -1.40% | 122% | 119% |
| SVT12 | -1.16% | -0.98% | -0.99% | 105% | 100% |
| Overall | -1.27% | -1.19% | -1.19% | 113% | 110% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -1.22% | -1.36% | -1.35% | 121% | 107% |
| SVT12 | -1.02% | -0.92% | -0.94% | 104% | 90% |
| Overall | -1.12% | -1.14% | -1.14% | 113% | 99% |

Conclusions

This contribution reports a combined result of CE-1.5, CE-2.1 and CE-3.2.

References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10,” JVET-T2001, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 20th Meeting, by teleconference, 7 – 16 Oct. 2020.

[2] T. Zhou, T. Ikai, “AHG8: Transform coefficients range extension for high bit-depth coding”, JVET-U0052

[3] H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang, “CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, JVET-U0075

[4] A. Browne, S. Keating, K. Sharman, “CE-1.4, CE-1.5, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths”, JVET-U0057

[5] A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, “CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-U2022

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).