| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0054-r1 |
| --- | --- |

| Title: | CE-2.1: Slice based Rice parameter selection for transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Wei Chen, Che-Wei Kuo, Xianglin Wang | Email: | [{jhuhong-jheng, xiaoyuxiu, yiwenchen, chenwei06, cheweikuo, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment (CE) 2-1 [1]. In the CE test, slice based Rice parameter selection for transform skip residual coding (TSRC) is tested for high bit-depth coding. Instead of using fixed value, the Rice parameter used for coding the bypass bins of abs_remainder is explicitly signaled in the slice header of each slice. Under the high bit-depth common test condition (CTC), the simulation results compared to VTM-12.0 are summarized as below:

Low QP range:

SVT RGB-16: -22.27% AI, -13.97% LB, -12.67% RA

SVT RGB-12: -1.45% AI, -0.38% LB, -0.37% RA

HDR HLG: -0.21% AI, 0.05% LB, -0.04% RA

HDR PQ: -0.10% AI, 0.00% LB, 0.00% RA

SCC TGM: -3.02% AI, -1.74% LB, -1.47% RA

Normal QP range:

HDR H1: 0.01% RA, 0.00% AI

HDR H2: 0.01% RA, 0.00% AI

r1 update working draft text with a few minor edits.

# Introduction

In VVC version 1 [2], when transform skip residual coding (TSRC) is applied, the remainder of the absolute levels, i.e. abs_remainder, are binarized using the fixed Rice parameter 1. It is reported that such design causes a significant coding performance drop when the coding bit-depth is beyond 12-bit. To solve such issue, a slice based Rice parameter selection scheme was proposed in [3] for the TSRC to improve the efficiency of high bit-depth coding. The method was included in the core experiment (CE) test 2.1 for further evaluation. This document reports the simulation results of the CE test.

# Proposal

In the proposed method, one control flag is signaled in sequence parameter set (SPS) to indicate the Rice parameter used for the TSRC is allowed to be adjusted for each slice separately. When the flag is enabled, another syntax element is further signaled for each slice to indicate the Rice parameter used for the TSRC of that slice. Otherwise (the flag is disabled), no further syntax element is signaled at slice level and the default Rice parameter (e.g. 1) is used for all the slices.

Additionally, the following encoder scheme is applied to calculate the optimal Rice parameter for each slice based on the values of abs_remainder that are coded in its preceding slice according to the coding order. Specifically, after one slice is coded, the number of bins for the binarization of abs_remainder using different Rice parameters are computed which are then used to determine the Rice parameter of the following slice as follows: if the current slice and its preceding slice use one same QP, the Rice parameter which achieves the minimal bin number in the preceding slice will be selected for current slice; otherwise, the number of bins generated using the default Rice parameter (i.e., 1) in the preceding slice is scaled by 0.9 before being compared with other Rice parameters and the Rice parameter which leads to the minimal number of bins will be selected for current slice. Forthermore, when the current slice and its preceding slice use different QP, the default Rice parameter (i.e., 1) will be selected for current slice when the percentage of matched IBC hash block is smaller than a predefined value.

# Simulation Results

The proposed method was implemented based on the VTM-12.0 reference software [4] and tested using the test condition and evaluation criteria that are defined in common test conditions for high bit depth and high bit rate video coding [5].

Table 1: Results of Slice based Rice Parameters for SVT RGB

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -22.27% | -21.36% | -21.30% | 101% | 94% |
| SVT12 | -1.45% | -1.62% | -1.57% | 100% | 101% |
| Overall | -11.86% | -11.49% | -11.44% | 101% | 98% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -13.97% | -13.41% | -13.41% | 101% | 98% |
| SVT12 | -0.38% | -0.42% | -0.40% | 98% | 99% |
| Overall | -7.18% | -6.91% | -6.91% | 99% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -12.67% | -12.89% | -12.84% | 100% | 98% |
| SVT12 | -0.37% | -0.41% | -0.38% | 97% | 99% |
| Overall | -6.52% | -6.65% | -6.61% | 98% | 98% |
| Overall RGB | -8.52% | -8.35% | -8.32% | 100% | 98% |

Table 2: Results of Slice based Rice Parameters for HDR HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.18% | -0.12% | -0.13% | 99% | 99% |
| HLG422 | -0.23% | -0.17% | -0.18% | 99% | 99% |
| Overall | -0.21% | -0.15% | -0.15% | 99% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.05% | 0.00% | -0.05% | 100% | 101% |
| HLG422 | 0.05% | 0.00% | -0.03% | 100% | 100% |
| Overall | 0.05% | 0.00% | -0.04% | 100% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.04% | -0.01% | -0.01% | 100% | 100% |
| HLG422 | 0.02% | 0.00% | -0.03% | 101% | 100% |
| Overall | 0.03% | 0.00% | -0.02% | 100% | 100% |
| Overall HLG | -0.04% | -0.05% | -0.07% | 100% | 100% |

Table 3: Results of Slice based Rice Parameters for HDR PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.09% | -0.06% | -0.07% | -0.09% | -0.06% | -0.07% | 101% | 99% |
| PQ422 | -0.11% | -0.07% | -0.08% | -0.11% | -0.07% | -0.08% | 100% | 100% |
| Overall | -0.10% | -0.06% | -0.07% | -0.10% | -0.07% | -0.08% | 100% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.01% | 0.01% | 0.00% | 0.01% | 0.01% | 100% | 101% |
| PQ422 | 0.00% | 0.00% | -0.02% | -0.01% | 0.00% | -0.02% | 100% | 100% |
| Overall | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% | -0.01% | 100% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | 0.00% | -0.01% | 0.00% | 0.01% | -0.01% | 100% | 99% |
| PQ422 | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% | -0.01% | 101% | 100% |
| Overall | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% | -0.01% | 100% | 99% |
| Overall PQ | -0.03% | -0.02% | -0.03% | -0.03% | -0.02% | -0.03% | 100% | 100% |

Table 4: Results of Slice based Rice Parameters for HDR (Normal QP)

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-12.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class H2 |  |  |  |  |  | 0.01% | 0.01% | 0.01% | 100% | 101% |
| Overall | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 100% | 100% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM-12.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 5: Results of Slice based Rice Parameters for SCC TGM

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -5.22% | -5.06% | -5.08% | 100% | 97% |
| RGB 4:4:4 - TGM 720p | -0.83% | -0.54% | -0.54% | 101% | 96% |
| Overall | -3.02% | -2.80% | -2.81% | 101% | 97% |

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -2.91% | -2.82% | -2.84% | 102% | 99% |
| RGB 4:4:4 - TGM 720p | -0.02% | -0.33% | 0.03% | 101% | 98% |
| Overall | -1.47% | -1.58% | -1.41% | 101% | 98% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -2.70% | -2.62% | -2.65% | 101% | 98% |
| RGB 4:4:4 - TGM 720p | -0.78% | -0.03% | -0.02% | 101% | 100% |
| Overall | -1.74% | -1.32% | -1.33% | 101% | 99% |

# Working Draft Text

The changes to the VVC Draft [2] are highlighted below.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_sign_data_hiding_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) |  |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_virtual_boundaries_enabled_flag | u(1) |
| … |  |
| } |  |

sps_ts_residual_coding_rice_present_in_sh_flag equal to 1 specifies that sh_ts_residual_coding_rice_idx could be present in SH syntax structures referring to the SPS. sps_ts_residual_coding_rice_present_in_sh_flag equal to 0 specifies that sh_ts_residual_coding_rice_idx_minus1 is not present in SH syntax structures referring to the SPS. When sps_ts_residual_coding_rice_present_in_sh_flag is not present, the value of sps_ts_residual_coding_rice_present_in_sh_flag is inferred to be equal to 0.

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_transform_skip_enabled_flag && !sh_dep_quant_used_flag && !sh_sign_data_hiding_used_flag ) |  |
| sh_ts_residual_coding_disabled_flag | u(1) |
| if((!sh_ts_residual_coding_disabled_flag) && sps_ts_residual_coding_rice_present_in_sh_flag ) |  |
| sh_ts_residual_coding_rice_idx_minus1 | u(3) |
| … |  |
| } |  |

sh_ts_residual_coding_rice_idx_minus1 plus 1, specifies the rice parameter used for the residual_ts_coding( ) syntax structure. When sh_ts_residual_coding_rice_idx_minus1 is not present, the value of sh_ts_residual_coding_rice_idx_minus1 is inferred to be equal to 0.

### 9.3.3.11	Binarization process for abs_remainder[ ]

Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the binarization of the syntax element.

The variables lastAbsRemainder and lastRiceParam are derived as follows:

–	If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0.

–	Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this clause.

The rice parameter cRiceParam is derived as follows:

–	If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 and sh_ts_residual_coding_disabled_flag is equal to 0, the Rice parameter cRiceParam is set equal to sh_ts_residual_coding_rice_idx_minus1+1.

–	Otherwise, the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[ ] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

# Conclusions

- A slice based Rice parameter selection scheme for the TSRC is proposed in this CE test. Simulation results show that the proposed scheme provides significant BD-rate saving for high bit-depth coding with minimal changes to the existing design to VVC version 1. It is recommended to adopt the proposed method into VVC version 2.

# References

- A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, K. Kawamura and T. Zhou “ CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-U2022, 21th meeting, by teleconference, 6 – 15 Jan. 2021.

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “ Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, 20th meeting, by teleconference, 7 – 16 Oct. 2020.

- H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo and X. Wang, “ CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, JVET-U0075, 21th meeting, by teleconference, 6 – 15 Jan. 2021.

- VTM-12.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-12.0.

- A. Browne, T. Ikai, D. Rusanovskyy, M. Golam Sarwer and X. Xiu “ Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, 21th meeting, by teleconference, 6 – 15 Jan. 2021.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).