| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0059-v2 |
| --- | --- |

| Title: | AHG8: CABAC-bypass alignment for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Jie Chen Yan Ye Ru-Ling Liao 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1-408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

In the last JVET meeting, a CABAC bypass alignment flag (similar to HEVC range extension) was proposed in JVET-U0069 to improve the throughput of the CABAC engine in high bit-rate applications. It was generally agreed that CABAC bypass alignment method in JVET-U0069 could improve the throughput in very high bit-rate professional applications, however, further study was recommended to investigate at which level (i.e. profile level, sequence level etc) this functionality is desirable. This contribution proposed to enable CABAC bypass alignment method in VVC High Throughput 4:4:4 16 Intra Profile.

Same as JVET-U0069, this contribution proposes two alignment options. In the first option, the bypass alignment is applied only to coefficient coding within a coefficient group (CG), without affecting the coding of sb_coded_flag of a transform block (TB). In the second option, in addition to CABAC bypass alignment, it is also proposed to switch to bypass coding of sb_coded_flag after the limit of context coded bins has been reached for the current TB. It is asserted that with the second option, alignment is needed only once after the limit of the context coded bins has been reached for the TB.

Following results are reported. Note that only SVT 16-bit sequences will be affected by the proposed changes as all other sequences are 12-bit sequences.

As compared to CE-1 anchor (low QP):

- Option1: SVT-16 bit sequences:

  - All Intra: 0.47%(G), 0.49% (B), 0.49 % (R), 100% (EncT), 93%(DecT)

  - LDB: 0.55%(G), 0.55 % (B), 0.55 % (R), 100% (EncT), 94%(DecT)

  - Random access: 0.57%(G), 0.56 % (B), 0.56 % (R), 100% (EncT), 93%(DecT)

- Option2: SVT-16 bit sequences:

  - All Intra: 0.47%(G), 0.49 % (B), 0.49 % (R), 99% (EncT), 94%(DecT)

  - LDB: 0.55%(G), 0.54 % (B), 0.54 % (R), 100%(EncT), 94%%(DecT)

  - Random access: 0.57%(G), 0.56 % (B), 0.56 % (R), 100%(EncT), 94%(DecT)

The full simulation results are included in the v2 of this document.

# Problem Statement

The CABAC process in VVC is a sequential process where the evaluation of each iteration depends on the outcome of the previous iteration. At the higher bit depth and higher bit rate operating ranges (especially in 16-bit input), the serial nature of CABAC decoding process may impact the throughput of the codec. Since there is an upper-limit of the maximum number of context coded bins, at high bit-rate the bypass coding bins are the dominant portion in the bitstream, compared to the context coded bins.

The decoding process of a bypass coded bin is not so straight forward. In VVC, the status of the arithmetic decoding engine is represented by the variables ivlCurrRange and ivlOffset. In order to decode a bypass coded bin the parameter ivlCurrRange is updated first ( as shown in Figure 1) . Then, the updated ivlCurrRange is compared with the ivlOffset. If the ivlOffset is greater than or equal to ivlCurrRange, the bin value is equal to 1 and ivlOffset is updated; otherwise, the bin value is set to 0. This serial decoding of bypass coding may impact the throughput of the codec especially at high bit-rate.

Figure 1: Decoding process of a bypass coded bin in VVC

# Proposed methods

In order to improve the throughput of the CABAC engine, HEVC range extension (the High Throughput 4:4:4 16 Intra Profile) supports aligning the CABAC process prior to coding of bypass data, which allows easy, simultaneous decoding of multiple bypass bins[1]. In the last JVET meeting, JVET-U0069 was proposed to add the same CABAC bypass alignment concept as that in HEVC range extension into VVC [6].  JVET-U0069 proposed to use the same process as in HEVC range extension where the value of the ivlCurrRange of CABAC engine is set to 256 right before starting of the bypass coding of a coefficient group. After aligning the ivlCurrRange to 256, the decoding process of a bypass coded bin can be simplified (as shown in Figure 2).

Figure 2: Decoding of a bypass coded bin while ivlCurrRange == 256. The figure is extracted from [1]

As shown in Figure 2, the values of the bypass coded bins are directly visible from ivlOffset. Therefore, the bypass decoding process can be implemented using a shift register and multiple bypass coded bins can be decoded in parallel, as follows:

- ivlOffset ← (ivlOffset << n) | read_bits(n)

- binVals ← n most-significant-but-one bits of ivlOffset

- remove the n interpreted bits from ivlOffset

In last JVET meeting, it was generally agreed that CABAC-bypass alignment method can improve the throughput in professional applications, however, further study was recommended to investigate at which level (i.e. profile level, sequence level etc) this functionality is desirable. This contribution proposed to enable CABAC-bypass alignment method only in VVC High Throughput 4:4:4 16 Intra profile.

The CABAC bypass alignment method of this contribution is same as the methods proposed in JVET-U0069. Same as JVET-U0069, two alignment methods are proposed, one modifying the process for only coefficients within a CG, and the other also including switching sb_coded_flag to bypass coding when certain conditions are met.

## Option 1: CABAC bypass alignment

In option 1, the proposed CABAC bypass alignment is applied only to coefficients within a CG, and executed before starting of the bypass coding pass. The coding method of sb_coded_flag is exactly same as VVC. In the following pseudo code, the variable sps_cabac_bypass_alignment_enabled_flag is set to 1 to enable the proposed method. If the value of sps_cabac_bypass_alignment_enabled_flag is equal to 0, the proposed method is disabled and the behavior is same as current VVC. The variables remBinsPass1 and RemCcbs represent the remaining number of context coded bins available in the TB in regular and transform-skip residual coding, respectively. The red italic portions are the additional process as compared to the current VVC.

In case of regular residual coding:

- remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - context_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (remBinsPass1  >=  4 )

      - Pass 1:

        - sig_coeff_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 0 ] ; remBinsPass1--;

        - par_level_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 1 ] ; remBinsPass1--;

    - if (sps_cabac_bypass_alignment_enabled_flag)

      - CABAC bypass alignment (i.e.set ivlCurrRange to 256)

    - Pass 2: abs_remainder, dec_abs_level

    - Pass 3: coeff_sign_flag

In case of transform-skip residual coding:

- RemCcbs = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - context_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (RemCcbs >= 4)

      - Pass 1:

        - sig_coeff_flag; RemCcbs--;

        - coeff_sign_flag; RemCcbs--;

        - abs_level_gtx_flag[ ][ 0 ] ; RemCcbs--;

        - par_level_flag; RemCcbs--;

    - if (RemCcbs >= 4)

      - Pass 2:

        - abs_level_gtx_flag [ ][ 1 ]; RemCcbs--;

        - abs_level_gtx_flag [ ][ 2 ] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 3] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 4 ] ; RemCcbs--;

    - if (sps_cabac_bypass_alignment_enabled_flag)

      - CABAC-bypass alignment (i.e.set ivlCurrRange to 256)

    - pass 3: abs_remainder, coeff_sign_flag

Figure 3: Proposed option 1

Figure 3 shows the bit-stream format of option 1. The red arrows show the proposed alignment points within a transform block. The alignment is performed at each coefficient group before starting the decoding/encoding of the bypass coded coefficients. In this option, as depicted in Figure 3, it may be necessary to perform the alignment multiple times even after the maximum limit of the context coded bins has been reached for the current TB. This is because the sb_coded_flag is always context coded, so even after the limit of maximum number of context coded bins is reached, the switching between context coded bins and bypass coded bins means that bypass alignment could occur multiple times.

## Option 2: CABAC bypass alignment + context/bypass coding of sb_coded_flag

One drawback of option 1 is that the CABAC bypass alignment is required even after reaching to the maximum limit of the context coded bins. In order to guarantee that there is no context coded bins after reaching to the maximum limits, it was proposed in JVET-P0370 [4]that the sb_coded_flag (of both RRC and TSRC) be bypass coded after reaching to the number of context coded bins to the maximum limit. In order to reduce the number of bypass alignments, in option 2, the proposed alignment method (described in option 1) is combined with sb_coded_flag coding method proposed in JVET-P0370 when CABAC bypass alignment is enabled.

Following is the pseudo-code of option 2. The sps_cabac_bypass_alignment_enabled_flag is set to 1 to enable the proposed method. If the value of sps_cabac_bypass_alignment_enabled_flag is equal to 0, the proposed method is disabled and the behavior is same as current VVC. The variables remBinsPass1 and RemCcbs represent the remaining number of context coded bins available in the TB in regular and transform-skip residual coding, respectively. The red italic portions are the additional process as compared to the current VVC.

In case of regular residual coding:

- remBinsPass1 = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - if (remBinsPass1  >=  4 || sps_cabac_bypass_alignment_enabled_flag == 0)

    - CABACBypasIntervalAlign = 0

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (remBinsPass1  >=  4)

      - Pass 1:

        - sig_coeff_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 0 ] ; remBinsPass1--;

        - par_level_flag; remBinsPass1--;

        - abs_level_gtx_flag[ ][ 1 ] ; remBinsPass1--;

    - If (CABACBypasIntervalAlign == 0 && sps_cabac_bypass_alignment_enabled_flag)

      - CABAC bypass alignment (i.e.set ivlCurrRange to 256)

      - CABACBypasIntervalAlign = 1

    - Pass 2: abs_remainder, dec_abs_level

    - Pass 3: coeff_sign_flag

In case of transform-skip residual coding:

- RemCcbs = ( ( 1  <<  ( log2TbWidth + log2TbHeight ) ) * 7 )  >>  2

- For each coefficient group

  - if (RemCcbs >= 4 || sps_cabac_bypass_alignment_enabled_flag == 0)

    - CABACBypasIntervalAlign = 0

    - context_coding of sb_coded_flag

  - else

    - bypass_coding of sb_coded_flag

  - if (sb_coded_flag ==1)

    - if (RemCcbs >= 4)

      - Pass 1:

        - sig_coeff_flag; RemCcbs--;

        - coeff_sign_flag; RemCcbs--;

        - abs_level_gtx_flag[ ][ 0 ] ; RemCcbs--;

        - par_level_flag; RemCcbs--;

    - if (RemCcbs >= 4)

      - Pass 2:

        - abs_level_gtx_flag [ ][ 1 ]; RemCcbs--;

        - abs_level_gtx_flag [ ][ 2 ] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 3] ; RemCcbs--;

        - abs_level_gtx_flag [ ][ 4 ] ; RemCcbs--;

    - If (CABACBypasIntervalAlign == 0 && sps_cabac_bypass_alignment_enabled_flag)

      - CABAC-bypass alignment (i.e.set ivlCurrRange to 256)

      - CABACBypasIntervalAlign = 1

    - Pass 3: abs_remainder, coeff_sign_flag

Figure 4 shows an example of the bit-stream of option 2. It shows that after the number of context coded bins has reach the limit for the current TB, only bypass coding bins are processed and the bypass alignment is necessary at most once per TB. As shown in Figure 4, the proposed method reduces the number of alignments as compared to Figure 3.

Figure 4: Proposed option 2

## Proposed high level syntax changes

The proposed CABAC bypass alignment (i.e. set the value of ivlCurrRange to 256) has benefits of simplifying high-throughput design of the entropy decoder, however, it may not be suitable for all application scenarios. It is proposed to enable this method in High Throughput 4:4:4 16 Intra Profile only. Following signaling methods are proposed.

### SPS level signaling

The SPS flags are signaled to enable/disable the proposed alignment in the sequence level. This is the same as HEVC range extension. At first, sps_range_extension_flag is added to support the VVC range extension. If sps_range_extension_flag is equal to 1, sps_cabac_bypass_alignment_enabled_flag is signaled to control the proposed alignment process. If the sps_cabac_bypass_alignment_enabled_flag is equal to 1, the proposed alignment method may be enabled for that sequence. The sps_extension_7bits are reserved for other future extensions. Following are the semantic of the proposed flags (in the following semantic italic part are valid only for option 2):

sps_range_extension_flag equal to 1 specifies that the sps_range_extension( ) syntax structure is present in the SPS RBSP syntax structure. sps_range_extension_flag equal to 0 specifies that this syntax structure is not present. When not present, the value of sps_range_extension_flag is inferred to be equal to 0.

sps_cabac_bypass_alignment_enabled_flag equal to 1 specifies that the value of ivlCurrRange may be aligned prior to bypass decoding of the syntax elements sb_coded_flag[][],  abs_remainder[ ], dec_abs_level[ n ], and coeff_sign_flag[]. sps_cabac_bypass_alignment_enabled_flag equal to 0 specifies that the value of ivlCurrRange is not aligned prior to bypass decoding. When not present, the value of sps_cabac_bypass_alignment_enabled_flag is inferred to be equal to 0.

sps_extension_7bits equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. When present, sps_extension_7bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Values of sps_extension_7bits not equal to 0 are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of sps_extension_7bits to be not equal to 0 and shall ignore all sps_extension_data_flag syntax elements in an SPS NAL unit. When not present, the value of sps_extension_7bits is inferred to be equal to 0.

Table 1: SPS syntax of the proposed method

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| sps_extension_flag | u(1) |
| if (sps_extension_flag) |  |
| { |  |
| sps_range_extension_flag | u(1) |
| sps_extension_7bits | u(7) |
| } |  |
| if (sps_range_extension_flag) |  |
| sps_range_extension( ) |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2: SPS range extension syntax of the proposed method

| sps_range_extension( ) { | Descriptor |
| --- | --- |
| sps_cabac_bypass_alignment_enabled_flag | u(1) |
| } |  |

### Profile level

In HEVC, the CABAC bypass alignment is only supported in High Throughput 4:4:4 16 Intra profile. It is proposed to define a similar profile in VVC, and to enable the proposed CABAC bypass method in that profile.

# Simulation results

The proposed methods are implemented on top of CE-1 anchor [7]. The performance of the proposed methods are evaluated based on the common test conditions suggested in JVET-U2018 [2]. Both of the low QP and standard QP settings are tested.

## Low QP tests

Table 3, Table 4, Table 5 show the low QP results of option 1 of PQ, HLG, and SVT sequences, respectively as compared to CE-1 anchor. Note that only SVT 16-bit sequences will be affected by the proposed changes as all other sequences are 12-bit sequences. The results of option 2 are tabulated in Table 6, Table 7, and Table 8.

Table 3: results of low QP test of option 1 (HDR PQ sequences)

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1 anchor |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.20% | 1.25% | 1.23% | 1.19% | 1.24% | 1.21% | 100% | 98% |
| PQ422 | 1.10% | 1.13% | 1.11% | 1.09% | 1.12% | 1.09% | 101% | 99% |
| Overall | 1.15% | 1.19% | 1.17% | 1.14% | 1.18% | 1.15% | 101% | 98% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over CE-1 anchor |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.56% | 1.66% | 1.61% | 1.52% | 1.62% | 1.59% | 101% | 98% |
| PQ422 | 1.48% | 1.52% | 1.58% | 1.45% | 1.48% | 1.56% | 100% | 98% |
| Overall | 1.52% | 1.59% | 1.59% | 1.48% | 1.55% | 1.58% | 101% | 98% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over CE-1 anchor |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.55% | 1.65% | 1.60% | 1.51% | 1.62% | 1.58% | 101% | 98% |
| PQ422 | 1.46% | 1.51% | 1.52% | 1.43% | 1.48% | 1.51% | 101% | 99% |
| Overall | 1.50% | 1.58% | 1.56% | 1.47% | 1.55% | 1.55% | 101% | 98% |
| Overall PQ | 1.39% | 1.45% | 1.44% | 1.36% | 1.43% | 1.43% | 101% | 98% |

Table 4: results of low QP test of option 1 (HDR HLG  sequences)

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.90% | 0.97% | 0.98% | 101% | 99% |
| HLG422 | 0.84% | 0.90% | 0.91% | 101% | 99% |
| Overall | 0.87% | 0.94% | 0.95% | 101% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 1.45% | 1.44% | 1.50% | 100% | 98% |
| HLG422 | 1.37% | 1.43% | 1.49% | 100% | 99% |
| Overall | 1.41% | 1.44% | 1.49% | 100% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 1.46% | 1.45% | 1.51% | 100% | 98% |
| HLG422 | 1.39% | 1.46% | 1.46% | 100% | 98% |
| Overall | 1.42% | 1.46% | 1.49% | 100% | 98% |
| Overall HLG | 1.23% | 1.28% | 1.31% | 100% | 98% |

Table 5: results of low QP test of option 1 (SVT RGB  sequences)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.47% | 0.49% | 0.49% | 100% | 93% |
| SVT12 | 0.54% | 0.59% | 0.59% | 100% | 99% |
| Overall | 0.51% | 0.54% | 0.54% | 100% | 96% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.55% | 0.55% | 0.55% | 100% | 94% |
| SVT12 | 0.71% | 0.74% | 0.75% | 100% | 100% |
| Overall | 0.63% | 0.65% | 0.65% | 100% | 97% |
|  |  |  | RA |  |  |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.57% | 0.56% | 0.56% | 100% | 93% |
| SVT12 | 0.74% | 0.77% | 0.77% | 100% | 100% |
| Overall | 0.65% | 0.66% | 0.67% | 100% | 97% |
| Overall RGB | 0.60% | 0.62% | 0.62% | 100% | 97% |

Table 6: results of low QP test of option 2 (HDR PQ sequences)

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1 anchor |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.20% | 1.25% | 1.24% | 1.19% | 1.24% | 1.23% | 100% | 99% |
| PQ422 | 1.09% | 1.14% | 1.10% | 1.08% | 1.13% | 1.09% | 101% | 99% |
| Overall | 1.15% | 1.20% | 1.17% | 1.14% | 1.18% | 1.16% | 100% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over CE-1 anchor |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.58% | 1.68% | 1.61% | 1.53% | 1.64% | 1.58% | 101% | 99% |
| PQ422 | 1.46% | 1.51% | 1.56% | 1.43% | 1.47% | 1.55% | 100% | 99% |
| Overall | 1.52% | 1.59% | 1.59% | 1.48% | 1.56% | 1.57% | 101% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over CE-1 anchor |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.55% | 1.65% | 1.61% | 1.52% | 1.61% | 1.59% | 101% | 99% |
| PQ422 | 1.45% | 1.50% | 1.54% | 1.42% | 1.47% | 1.54% | 101% | 99% |
| Overall | 1.50% | 1.57% | 1.58% | 1.47% | 1.54% | 1.56% | 101% | 99% |
| Overall PQ | 1.39% | 1.45% | 1.44% | 1.36% | 1.43% | 1.43% | 101% | 99% |

Table 7: results of low QP test of option 2 (HDR HLG sequences)

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.90% | 0.97% | 0.98% | 101% | 100% |
| HLG422 | 0.83% | 0.91% | 0.91% | 101% | 100% |
| Overall | 0.87% | 0.94% | 0.94% | 101% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 1.44% | 1.45% | 1.50% | 100% | 99% |
| HLG422 | 1.38% | 1.44% | 1.49% | 100% | 99% |
| Overall | 1.41% | 1.44% | 1.50% | 100% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 1.47% | 1.45% | 1.51% | 100% | 99% |
| HLG422 | 1.39% | 1.46% | 1.47% | 100% | 99% |
| Overall | 1.43% | 1.45% | 1.49% | 100% | 99% |
| Overall HLG | 1.23% | 1.28% | 1.31% | 100% | 99% |

Table 8: results of low QP test of option 2 (SVT RGB sequences)

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.47% | 0.49% | 0.49% | 99% | 94% |
| SVT12 | 0.54% | 0.59% | 0.59% | 100% | 100% |
| Overall | 0.51% | 0.54% | 0.54% | 100% | 97% |
|  |  |  | LDB |  |  |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.55% | 0.54% | 0.54% | 100% | 94% |
| SVT12 | 0.71% | 0.74% | 0.75% | 100% | 101% |
| Overall | 0.63% | 0.64% | 0.64% | 100% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over CE-1 anchor |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.57% | 0.56% | 0.56% | 100% | 94% |
| SVT12 | 0.73% | 0.77% | 0.77% | 100% | 101% |
| Overall | 0.65% | 0.67% | 0.67% | 100% | 98% |
| Overall RGB | 0.60% | 0.61% | 0.62% | 100% | 98% |

## Standard QP tests

Table 9, and Table 10 shows the results of proposed methods of standard QP tests.

Table 9: results of option 1 ( standard QP test)

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE-1 anchor |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| PQ-420 | 2.95% | 1.51% | 1.50% | 2.76% | 2.87% | 1.49% | 2.69% | 2.84% | 99% | 100% |
| HLG-420 |  |  |  |  |  | 1.91% | 2.25% | 2.58% | 100% | 100% |
| Overall | 2.95% | 1.51% | 1.50% | 2.76% | 2.87% | 1.70% | 2.47% | 2.71% | 100% | 100% |
|  | All intra |  |  |  |  |  |  |  |  |  |
|  | Over CE-1 anchor |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| PQ-420 | 3.02% | 1.96% | 1.90% | 3.25% | 3.23% | 1.91% | 3.25% | 3.32% | 100% | 99% |
| HLG-420 |  |  |  |  |  | 2.15% | 2.67% | 3.14% | 100% | 99% |
| Overall | 3.02% | 1.96% | 1.90% | 3.25% | 3.23% | 2.03% | 2.96% | 3.23% | 100% | 99% |

Table 10:  results of option 2 ( standard QP test)

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE-1 anchor |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| PQ-420 | 2.85% | 1.51% | 1.49% | 2.57% | 2.80% | 1.49% | 2.55% | 2.77% | 99% | 100% |
| HLG-420 |  |  |  |  |  | 1.93% | 2.36% | 2.59% | 100% | 100% |
| Overall | 2.85% | 1.51% | 1.49% | 2.57% | 2.80% | 1.71% | 2.46% | 2.68% | 99% | 100% |
|  | All intra |  |  |  |  |  |  |  |  |  |
|  | Over CE-1 anchor |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| PQ-420 | 3.02% | 1.96% | 1.90% | 3.25% | 3.23% | 1.91% | 3.24% | 3.33% | 100% | 99% |
| HLG-420 |  |  |  |  |  | 2.15% | 2.66% | 3.16% | 100% | 99% |
| Overall | 3.02% | 1.96% | 1.90% | 3.25% | 3.23% | 2.03% | 2.95% | 3.24% | 100% | 99% |

# Conclusions

Same as JVET-U0069, this contribution proposes two options of CABAC bypass coding alignment to improve the throughput of the CABAC in the high bit-rate applications. In the first option, only the bypass alignment is applied to coefficient coding within a coefficient group (CG), without affecting the coding of sb_coded_flag of a transform block (TB). In the second option, in addition to CABAC bypass alignment, it is also proposed to switch to bypass coding of sb_coded_flag after the limit of context coded bins has been reached for the current TB. The number of alignments is reduced in the second option as compared to the first option. In both options since the bypass bins are directly visible in ivlOffset, N bypass coded bins can be decoded just by looking at the N most-significant-but-one bits of ivlOffset concatenated with the bit-stream which ultimately allows decoding of multiple bypass coded bins in parallel. It is proposed to enable the proposed method only in High Throughput 4:4:4 16 Intra Profile.

# References

K. Sharman, N. Saunders, J. Gamei, “AHG 5 and 18: Entropy Coding Throughput for High Bit Depths”, JCTVC-N0190, July 2013.

A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, Jan 2021

A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, “CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-T2022, Oct 2020.

M. G. Sarwer, R. L. Liao, J. Luo, Y. Ye, “ Non-CE7: Signaling of coded_sub_block_flag”, JVET-P0370, Oct 2019.

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, Oct 2020.

M. G. Sarwer, J. Chen, Y. Ye, R. -L. Liao, “AHG8: CABAC-bypass alignment for high bit-depth coding”, JVET-U0069, Jan 2021.

A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy, K. Kawamura, T. Zhou, “CE on Entropy Coding for High Bit Depth and High Bit Rate Coding”, JVET-U0022, Jan 2021

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).