| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0062 |
| --- | --- |

| Title: | AHG9: Picture quality metrics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Muhammed Coban, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the inclusion of picture quality metrics in the VSEI specification. The message provides a quality metric for each colour component of the current decoded picture.

# Introduction

Picture quality metrics are used to measure a vareity of picture distortions during the acquisition, processing, compression, storage, transmission and reproduction.

ISO/IEC 23001-10, carriage of timed metadata metrics of media in ISOBMFF, specifies the quality metrics of media such as PSNR, SSIM, MS-SSIM, VQM, PEVQ and MOS in file format.

The quality relevant quality ranking is specified in ISO/IEC 23090-2 OMAF to indicate the a relative quality order of quality ranking region. The value of the ranking may be derived from specific quality metrics.

ISO/IEC 23090-6, Immersive Media Metrics, also uses quality ranking to measure the viewport switching latency.

Some objective quality metrics such as PSNR and SSIM can only be obtained at the encoder side, a SEI message to carry such information is able to provide the relevant information to the application. In addition, such SEI message may also be used for the picture coding performance indication and verification, similar as picture HASH SEI message.

# Proposal

The proposed picture metrics SEI message is provided below:

Table 3 Picture quality metric SEI message

| Picture_quality_metrics( payloadSize ) { | Descriptor |
| --- | --- |
| pqm_metric_type | u(7) |
| pqm_single_component_flag | u(1) |
| for( cIdx = 0; cIdx < ( dph_sei_single_component_flag ? 1 : 3 ); cIdx++ ){ |  |
| if( pqm_sei_type = = 0 ) |  |
| pqm_psnr[ cIdx ] | u(16) |
| else if ( pqm_sei_type = = 1 ) |  |
| pqm_ssim[ cIdx ] | u(8) |
| else if ( pqm_sei_type == 2 ) |  |
| pqm_ms_ssim[ cIdx ] | u(8) |
| } |  |
| } |  |

This message provides quality metric for each colour component of the current decoded picture.

pqm_metric_type indicates the quality metric type associated with the component as specified in Table 5. The value of pqm_metric_type from 3 to 127 are reserved for future use by ITU-T | ISO/IEC and shall not be present in payload data conforming to this version of this Specification.

Table 5 Interpretation of pqm_metric_type

| pqm_metric_type | metric |
| --- | --- |
| 0 | PNSR |
| 1 | SSIM |
| 2 | MS-SSIM |

pqm_single_component_flag equal to 1 specifies that the picture associated with the picture quality metrics SEI message contains a single colour component. pqm_ single_component_flag equal to 0 specifies that the picture associated with the picture quality metrics SEI message contains three colour components. The value of pqm_single_component_flag shall be equal to (ChromaFormatIdc = = 0), where ChromaFormatIdc is described in subclause 7.3.

pqm_psnr[ cIdx ]  specifies the value of PSNR metric. The corresponding PSNR value (in dB) of the colour component cIdx of the decoded picture is derived as follows (expressed in floating point):

PSNR = pqm_psnr[ cIdx ] / 100; with the exception of PSNR = infinity for pqm_psnr[ cIdx ] is equal to 0

pqm_ssim[ cIdx ]  specifies the value of SSIM. The corresponding SSIM of the colour component cIdx of the decoded picture is derived as follows (expressed in floating point):

SSIM = ( pqm_ssim[ cIdx ] – 127 ) / 128

pqm_ms_ssim[ cIdx ]  specifies the value of MS-SSIM. The corresponding MS-SSIM of the colour component cIdx of the decoded picture is derived as follows (expressed in floating point):

SSIM = ( pqm_msssim[ cIdx ] – 127 ) / 128

# Patent rights declaration(s)

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).