| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0069 |
| --- | --- |

| Title: | AHG9: Decoded GDR clean area hash SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Limin Wang Seungwook Hong Krit Panusopone Miska M. Hannuksela | Email: | limin.2.wang@nokia.com seungwook.hong@nokia.com krit.panusopone@nokia.com miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution asserts that only the clean (or refreshed) areas of a GDR picture and the associated recovering pictures are important to meet the exact match requirement when starting the decoding from the GDR picture. Hence, it is proposed that a decoded GDR clean area hash SEI message is specified so that an encoder can generate and indicate hashes for the clean areas for GDR pictures and recovering pictures. Consequently, when starting the decoding from a GDR picture, a decoder would only need to check hashes for the clean areas for GDR pictures and recovering pictures.

Two options for the decoded GDR clean (or refreshed) area hash are proposed, summarized as follows:

- Decoded GDR clean area hash SEI message that contains a decoded_picture_hash() syntax structure derived from the clean area only.

The clean area is defined through the virtual boundary indicated in the picture header: it is either the left side of the picture (when a single vertical virtual boundary is in use in the picture) or the top of the picture (when a single horizontal virtual boundary is in use in the picture).

- Decoded GDR clean area hash SEI message that contains

- a vertical or horizontal boundary position defining the clean area, and

- a decoded_picture_hash() syntax structure derived from the clean area only.

The clean area is either the left side of the picture (when a vertical boundary is indicated in the SEI message) or the top of the picture (when a horizontal boundary is indicated in the SEI message).

# Introduction

In the current VVC [1], the decoded picture hash is used to verify if the reconstructed pictures at encoder and decoder are matched or not. The decoded picture hash is calculated based upon the entire picture.

Specifically, for each picture,

- encoder generates a hash for the reconstructed picture, and signals the hash via an SEI message (which is called the decoded picture hash SEI message in VSEI),

- decoder also generates a hash for the reconstructed picture, and compares the generated hash with the received hash from SEI message,

- if the hashes at encoder and decoder are matched, the reconstructed pictures at encoder and decoder are matched, otherwise not matched.

In VVC, GDR progressively refreshes pictures by spreading intra coded areas over several pictures. Fig. 1 illustrates the basic concept of (vertical) GDR, where a GDR period starts with picture of POC (n) and ends with picture of POC(n+N-1), including N pictures in total. The first picture of POC(n) within the GDR period is called GDR picture. Forced intra coded areas (green) gradually spread over the N pictures of the GDR period from the left to the right. The picture of POC(n+N-1) at recovery point is called recovery point picture. The pictures between GDR picture of POC(n) and recovery point picture of POC(n+N-1) are called recovering pictures of GDR picture of POC(n).

Fig. 1. A GDR period starts with a GDR picture of POC(n) and ends with picture of POC(n+N-1). A GDR/recovering picture consists of a clean area and a dirty area separated by a virtual boundary.

Typically, a GDR/recovering picture consists of a clean (or refreshed) area and a dirty (or non-refreshed) area, where the clean area may contain a forced intra area next to the dirty area for progressive intra refresh (PIR), as shown in picture of POC(n+1) of Fig. 1. The boundary between clean area and dirty area is signaled by virtual boundary syntax in Picture Header.

One of requirements for VVC GDR is so-called “exact match” at the recovery point. Or specifically, the reconstructed recovery point pictures at encoder and decoder are matched at the recovery point. To meet the exact match requirement, the clean (or refreshed) areas of GDR pictures and recovering pictures need to be the same at encoder and decoder, but which is not necessary for the dirty areas.

The picture-based hash may not be suitable for GDR pictures and recovering pictures because when starting the decoding from a GDR picture, only the clean (or refreshed) areas at decoder need to be checked. For example, for a GDR picture or a recovering picture, picture-based hash values at encoder and decoder may indicate mismatch of reconstructed pictures at encoder and decoder, but the reconstructed recovering pictures can still be good as long as the clean (or refreshed) areas at encoder and decoder are matched.

It is asserted that the proposed clean (or refreshed) area hash signalling would be useful in discovering mismatches between encoder and decoder implementations at the coded picture within the GDR period where the mismatch occurs, whereas the current decoded picture hash SEI message may allow detecting a mismatch in a GDR picture or any recovering picture only at a recovery point picture.

# GDR Clean Area Hash

It is proposed that for GDR pictures and recovering pictures, hashes are calculated for the clean (or refreshed) areas only. Below are two proposed options, which can be summarized as follows:

- Decoded GDR clean area hash SEI message that contains a decoded_picture_hash() syntax structure derived from the clean area only.

The clean area is defined through the virtual boundary indicated in the picture header: it is either the left side of the picture (when a single vertical virtual boundary is in use in the picture) or the top of the picture (when a single horizontal virtual boundary is in use in the picture).

- Decoded GDR clean area hash SEI message that contains

- a vertical or horizontal boundary position defining the clean area, and

- a decoded_picture_hash() syntax structure derived from the clean area only.

The clean area is either the left side of the picture (when a vertical boundary is indicated in the SEI message) or the top of the picture (when a horizontal boundary is indicated in the SEI message).

Option 1

This option creates a new SEI message for GDR clean area hash. This option still uses some syntax in picture header.

Add the following text in red to the VVC specification:

D.2 General SEI payload

D.2.1 General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| …… |  |
| else if( payloadType = = 204 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sample_aspect_ratio_info( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 132 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 233) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| decoded_gdr_clean_area_hash( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| …… |  |
| } |  |
| } |  |

Add the following to the VSEI specification:

8.19 Decoded GDR clean area hash SEI message

| decoded_gdr_clean_area_hash( payloadSize ) { | Descriptor |
| --- | --- |
| decoded_picture_hash( payloadSize ) |  |
| } |  |

This message provides a hash for each colour component of the clean area of a GDR picture or a recovering picture. The clean area is an area that has an exact match of decoded sample values when starting the decoding process from the GDR picture compared to starting the decoding process from the previous IRAP picture in decoding order, when present. This message shall not be present in any picture unit that does not contain a GDR picture or a recovering picture.

It is a requirement of bitstream conformance that when this SEI message is present in a picture unit, the following constraints apply for the picture header present in the picture unit:

- ph_virtual_boundaries_present_flag shall be equal to 1.

- ( ph_num_ver_virtual_boundaries + ph_num_hor_virtual_boundaries ) shall be equal to 1.

When this SEI message is present and ph_num_ver_virtual_boundaries is equal to 1, the clean area consists of the left side of the picture up to the virtual boundary VirtualBoundaryPosX[ 0 ], exclusive, in luma sample units. When this SEI message is present and ph_num_hor_virtual_boundaries is equal to 1, the clean area consists of the top of the picture up to the virtual boundary VirtualBoundaryPosY[ 0 ], exclusive, in luma sample units.

Add the following to the VVC specification (red text indicating the difference compared to the respective subclause on decoded picture hash SEI message):

D.9.7 Use of decoded GDR clean area hash SEI message

For purposes of interpretation of the decoded picture hash SEI message, the following variables are specified:

- if( ph_virtual_boundaries_present_flag ) {

if ( ph_num_ver_virtual_boundaries )

PicWidthInLumaSamples = (ph_virtual_boundary_pos_x_minus1[ 0 ] + 1) * 8

else

PicWidthInLumaSamples = pps_pic_width_in_luma_samples

if ( ph_num_hor_virtual_boundaries )

PicHeightInLumaSamples = (ph_virtual_boundary_pos_y_minus1[ 0 ] + 1) * 8

else

PicHeigtInLumaSamples = pps_pic_height_in_luma_samples

}

else {

PicWidthInLumaSamples = pps_pic_width_in_luma_samples

PicHeigtInLumaSamples = pps_pic_height_in_luma_samples

}

- ChromaFormatIdc is set equal to sps_chroma_format_idc0

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	ComponentSample[ cIdx ] is set to be the 2-dimension array of decoded sample values of the cIdx-th component of a decoded picture.

Option 2

This option creates a new SEI message for GDR clean area hash. This option avoids to use syntax in picture header, with introducing a few additional syntax elements.

Add the following text in red to the VVC specification:

D.2 General SEI payload

D.2.1 General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| ….. |  |
| else if( payloadType = = 204 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sample_aspect_ratio_info( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 132 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 233 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| decoded_gdr_clean_area_hash( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| …… |  |
| } |  |
| } |  |

Add the following to the VSEI specification:

8.19 Decoded GDR clean area hash SEI message

| decoded_gdr_clean_area_hash( payloadSize ) { | Descriptor |
| --- | --- |
| cah_ver_boundary_flag | u(1) |
| if( cah_ver_boundary_flag ) |  |
| cah_ver_boundary_pos_x_minus1 | u(15) |
| else |  |
| cah_hor_boundary_pos_y_minus1 | u(15) |
| decoded_picture_hash( payloadSize − 2 ) |  |
| } |  |

This message provides a hash for each colour component of the clean area of a GDR picture or a recovering picture. The clean area is an area that has an exact match of decoded sample values when starting the decoding process from the GDR picture compared to starting the decoding process from the previous IRAP picture in decoding order, when present. This message shall not be present in any picture unit that does not contain a GDR picture or a recovering picture.

cah_ver_boundary_flag equal to 0 indicates that the boundary between clean and dirty areas is horizontal. cah_ver_boundary_flag equal to 1 indicates that the boundary between clean and dirty areas is vertical.

cah_ver_boundary_pos_x_minus1 plus 1 specifies the location of the vertical boundary in units of luma samples divided by 8. The clean area is horizontally from 0 to ( cah_ver_boundary_pos_x_minus1 + 1 ) * 8, exclusive, in luma sample coordinates and vertically the picture height.

cah_hor_boundary_pos_y_minus1 plus 1 specifies the location of the horizontal boundary in units of luma samples divided by 8. The clean area is vertically from 0 to ( cah_hor_boundary_pos_y_minus1 + 1 ) * 8, exclusive, in luma sample coordinates and horizontally the picture width.

Add the following to the VVC specification (red text indicating the difference compared to the respective subclause on decoded picture hash SEI message):

D.9.7 Use of decoded GDR clean area hash SEI message

For purposes of interpretation of the decoded picture hash SEI message, the following variables are specified:

–	if( cah_ver_boundary_flag) {

picWidthInLumaSamples = ( cah_ver_boundary_pos_x_minus1 + 1 ) * 8

picHeightInLumaSamples = pps_pic_height_in_luma_samples

} else {

picWidthInLumaSamples = pps_pic_width_in_luma_samples

picHeightInLumaSamples = ( cah_hor_boundary_pos_y_minus1 + 1 ) * 8

}

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	ComponentSample[ cIdx ] is set to be the 2-dimension array of decoded sample values of the cIdx-th component of a decoded picture.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).