| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0073-v1 |
| --- | --- |

| Title: | EE1.2: Additional experimental results of NN-based super resolution (JVET-U0053) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-50-5213-6334 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In this contribution, several additional experimental results of EE1.2 NN-based super-resolution (JVET-U0053) are reported. JVET-U0053 was based on a framework for introducing a super-resolution post-filter using CNN (Neural Network). In VVC, RPR (Reference Picture Re-sampling) has been introduced, and in several 4K sequences with low bit-rate, it was reported that there are some coding gains by changing the resolution of the whole sequence. In this experiment, sending partial network parameters as network updated by each sequence and QP are tested. As experimental results, by sending partial network parameters, the coding efficiency has been improved obviously compared to the fixed network.

EE1.2 NN-based super resolution (JVET-U0053) [1]

We proposed a framework of super-resolution post-filter using a neural network [2]. In that framework, an SEI which includes network parameters was proposed.  In this contribution, a super-resolution network using the above experiments is compressed by using NCTM-5.0 [3] which is the reference software of MPEG NNR. Both the configuration parameters, approx_method and enc_method are set to baseline.

Figure 1 A framework of super-resolution neural network

- Test 1: Constant NN parameter version (same NN parameters for all sequences, no signalling)

- Test 2: NN parameters are partially updated by each sequence and each QP (signalled to the decoder in bit-stream)

The neural network structure of Test2 is the same as that of Test 1. Except for the last convolutional layer's network parameter, the same network parameters as Test 1 are used. The last convolutional layer parameters are re-trained by each sequence and each QP, and those parameters are encoded using NCTM. The bit rate of the encoded parameters is included in the resulting bit rate.

Table 1 shows the experimental results of RPR with half-resolution compared to VTM-11.0 (QP=27, … ,47). in several 4K sequences with low bit-rate, it is shown that there are some coding gains by changing the resolution of the whole sequence. Table 2 shows the results of Test 1 compared to VTM-11.0. Table 3 shows the results of Test 1 compared to RPR, which means the gains of super-resolution compared to that of up-sampling filtering of RPR. Table 4 shows the results of Text 2 compared to VTM-11.0. Table 5 shows the results of Test 2 compared to Test 1, which means the gain of sending partial network parameters as network updated by each sequence and QP.

Table 1  Experimental results of VTM-11.0 RPR with half resolution (QP=22, … ,42)

|  |  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0 QP=27, ... ,47 |  |  |  |  |  |  |  |
|  |  | BD-rate5 |  |  | BD-rate4 q2345 (piecewise cubic) |  |  | EncT | DecT |
|  |  | Y | U | V | Y | U | V |  |  |
| Cass A1 4K | Tango2 | -8.16% | -3.38% | 3.70% | -9.44% | -3.07% | 2.49% | 50% | 30% |
|  | FoodMarket4 | -7.27% | 3.99% | 4.12% | -9.93% | 2.52% | 2.89% |  |  |
|  | Campfire | -10.57% | 35.99% | 14.48% | -13.09% | 15.79% | 7.71% |  |  |
| Class A2 4K | CatRobot1 | 5.53% | 28.31% | 38.25% | -0.34% | 22.92% | 29.23% | 44% | 30% |
|  | DaylightRoad2 | 11.77% | 8.14% | 14.03% | 4.40% | 3.05% | 3.55% |  |  |
|  | ParkRunning3 | -6.25% | 157.25% | 66.27% | -8.94% | 121.76% | 45.88% |  |  |

Table 2  Experimental results of Test1 (QP=22, … ,42)

|  |  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0 QP=27, ... ,47 |  |  |  |  |  |  |  |
|  |  | BD-rate5 |  |  | BD-rate4 q2345 (piecewise cubic) |  |  | EncT | DecT |
|  |  | Y | U | V | Y | U | V |  |  |
| Cass A1 4K | Tango2 | -8.44% | -7.21% | 1.01% | -9.78% | -7.64% | -0.21% | 50% | 162% |
|  | FoodMarket4 | -8.25% | 5.06% | 2.64% | -10.77% | 2.04% | 0.55% |  |  |
|  | Campfire | -13.40% | 34.29% | 10.12% | -15.34% | 13.61% | 3.29% |  |  |
| Class A2 4K | CatRobot1 | -0.21% | 24.26% | 32.38% | -4.63% | 18.22% | 23.62% | 44% | 151% |
|  | DaylightRoad2 | 5.17% | 5.16% | 11.27% | -0.57% | -0.03% | 1.09% |  |  |
|  | ParkRunning3 | -9.42% | 149.57% | 72.45% | -11.17% | 114.24% | 51.05% |  |  |

Table 3  Experimental results of Test1 compared to VTM-11.0 RPR with half resolution

|  |  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0 RPR with half resolution QP=22, ... ,42 |  |  |  |  |  |  |  |
|  |  | BD-rate5 |  |  | BD-rate4 q2345 (piecewise cubic) |  |  | EncT | DecT |
|  |  | Y | U | V | Y | U | V |  |  |
| Cass A1 4K | Tango2 | -0.29% | -3.97% | -2.62% | -0.37% | -4.72% | -2.67% | 100% | 540% |
|  | FoodMarket4 | -1.13% | 1.12% | -1.44% | -0.98% | -0.62% | -2.36% |  |  |
|  | Campfire | -3.77% | -1.08% | -3.52% | -2.65% | -1.77% | -4.27% |  |  |
| Class A2 4K | CatRobot1 | -6.86% | -3.08% | -4.50% | -4.96% | -4.09% | -4.63% | 100% | 506% |
|  | DaylightRoad2 | -8.45% | -2.89% | -3.03% | -5.99% | -3.13% | -2.48% |  |  |
|  | ParkRunning3 | -3.51% | -1.69% | 6.93% | -2.44% | -4.07% | 4.11% |  |  |

Table 4 Experimental results of Test2

|  |  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0 QP=27, … ,47 |  |  |  |  |  |  |  |
|  |  | BD-rate5 |  |  | BD-rate4 q2345 (piecewise cubic) |  |  | EncT | DecT |
|  |  | Y | U | V | Y | U | V |  |  |
| Cass A1 | Tango2 | -9.89% | -11.28% | -2.07% | -10.67% | -11.86% | -3.01% | 50% | 168% |
| 4K | FoodMarket4 | -9.07% | -0.70% | -1.58% | -11.13% | -3.17% | -3.30% |  |  |
|  | Campfire | -14.61% | 31.24% | -4.00% | -16.32% | 11.65% | -8.41% |  |  |
| Class A2 | CatRobot1 | -0.66% | 18.71% | 28.44% | -4.86% | 13.00% | 20.28% | 44% | 151% |
| 4K | DaylightRoad2 | 4.98% | -5.76% | 9.07% | -0.81% | -10.59% | -1.27% |  |  |
|  | ParkRunning3 | -9.89% | 32.50% | 39.02% | -11.25% | 20.05% | 22.35% |  |  |

Table 5 Experimental results of Test2 compared to Test 1

|  |  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over Test 1 QP=22, ... ,42 |  |  |  |  |  |  |  |
|  |  | BD-rate5 |  |  | BD-rate4 q2345 (piecewise cubic) |  |  | EncT | DecT |
|  |  | Y | U | V | Y | U | V |  |  |
| Cass A1 | Tango2 | -1.59% | -4.82% | -3.13% | -0.99% | -5.02% | -2.84% | 100% | 104% |
| 4K | FoodMarket4 | -1.06% | -5.84% | -4.31% | -0.47% | -5.16% | -3.87% |  |  |
|  | Campfire | -1.69% | -5.77% | -14.99% | -1.17% | -3.63% | -12.12% |  |  |
| Class A2 | CatRobot1 | -0.67% | -5.16% | -4.00% | -0.31% | -4.63% | -3.02% | 100% | 100% |
| 4K | DaylightRoad2 | -0.44% | -11.29% | -3.15% | -0.27% | -11.00% | -2.45% |  |  |
|  | ParkRunning3 | -0.53% | -53.31% | -26.96% | -0.07% | -47.94% | -20.51% |  |  |

Reference

[1]	E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. Ma and H. Wang, “Exploration Experiments on Neural Network-based Video Coding”, Joint Video Experts Team Document, JVET-U2023, teleconference, Jan. 2021.

[2]	T. Chujoh, E. Sasaki, T. Suzuki and T. Ikai, “AHG9/AHG11: Level information for super-resolution neural network”, Joint Video Experts Team Document, JVET-U0053, teleconference, Jan. 2021.

[3]	“Test Model 5 of Compression of Neural Networks for Multimedia Content Description and Analysis”, ISO/IEC JTC 1/SC29/WG11 Documents, N 19500, Online, July 2020.

[4]	“Text of ISO/IEC DIS 15938-17 Compression of Neural Networks for Multimedia Content Description and Analysis”, ISO/IEC JTC 1/SC29 Documents, MDS19764_WG04_N00016, Online, Oct. 2020.

[5]	T. Suzuki, E. Sasaki, T. Chujoh, T. Ikai and H. Watanabe, “Coded Video Transmission using Super-Resolution and MPEG-NNR Deep Learning Metadata”, the 35th Picture Coding Symposium of Japan/the 25th Image Media Processing Symposium (PCSJ/IMPS 2020), Nov. 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).